3GPP TSG RAN WG2 Meeting #124                                                 R2-2313590

# Chicago, USA, November 13-17, 2023

Agenda item:	7.4.1

Source:	Intel Corporation

Title:	Discussion and TP on L2/3 UE capabilities for NR further mobility enhancements

Document for:	Discussion and decision

# Introduction

This document discusses the possible UE capability signalling required for feMob WI based on the agreements made so far.  Some of the discussion points are still FFS and will need to take into consideration additional agreements when they are made.

# Discussion

The sections below cover the three different objectives of the feMob WI and the possible capability signalling that could be defined for each with aim to reach agreement on what capability needs to be signalled and the indication from this signalling.  Apart from the UE implementation aspects, the discussion also considers the possible network deployment aspects in terms of opportunity for interoperability testing and whether an IOT/capability bit could be useful for indication of successful IOT.

The tentative list of RAN1 capabilities is shown below for information:

| Index | Feature group |
| --- | --- |
| 45-1 | Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure[—processing capability] |
| 45-1a | Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure[—processing capability] |
| 45-2 | Inclusion of current SpCell in the L1 measurement report |
| 45-3 | LTM beam indication with joint DL/UL TCI states |
| 45-3a | MAC-CE activated joint LTM TCI states |
| 45-4 | LTM beam indication with separate DL/UL TCI states |
| 45-4a | MAC-CE activated DL/UL TCI states |
| 45-5 | RACH-based early TA acquisition |
| 45-6 | UE-based TA measurement |
| 45-7 | TA indication in cell switch command with and without RACH |

## LTM:

The discussion below on possible new capabilities.  Note that Reference configuration being optional is already agreed and not discussed in this document.

| Feature/ scenario | Capability signalling | Other aspects | Related RAN1 features | Discussion |
| --- | --- | --- | --- | --- |
| MCG LTM | Optional feature Supported components: MAC CE based cell switch command; Preconfiguration of LTM candidate cell | Per UE, no FRx/xDD differentiation | Supports RAN1 intra-frequency L1 measurement and report (45-1) or inter-frequency L1 measurement and report (45-1a), both of which are per band/BC. | RAN2 UE specific LTM capability separate from RAN1 capabilities |
| SCG LTM | MAC CE based cell switch command; Preconfiguration of LTM candidate cell | Per UE, no FRx/xDD differentiation | Supports RAN1 intra-frequency L1 measurement and report (45-1) or inter-frequency L1 measurement and report (45-1a), both of which are per band/BC. | Separate capability bit for SCG LTM |
| RACHless LTM with DG for MCG | Optional feature Dependencies: UE shall indicate support of MCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | None of the RAN1 features cover this directly. Hence need a capability just for this. |
| RACHless LTM with CG for MCG | Optional feature Dependencies: UE shall indicate support of MCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | None of the RAN1 features cover this directly. Hence need a capability just for this. |
| RACHless LTM with DG for SCG | Optional feature Dependencies: UE shall indicate support of SCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | Separate capability for SCG |
| RACHless LTM with CG for SCG | Optional feature Dependencies: UE shall indicate support of SCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | Separate capability for SCG |

Proposal #1: It is proposed to support the above defined capability bits for LTM with the associated functionalities/dependencies.

## Subsequent CPAC in NR-DC

There are no related RAN1 features for this objective.

| Feature/ scenario | Capability signalling | Other aspects | Discussion |
| --- | --- | --- | --- |
| SCPAC MN configured with MN event | Optional feature Intra and inter SN (also supports change/list of SK-counter) | Per UE, no FRx/xDD differentiation | Capability for the main feature |
| SCPAC MN configured with SN event | Optional feature Intra and inter SN (also supports change/list of SK-counter) | Per UE, no FRx/xDD differentiation | Capability for the main feature |
| SCPAC SN configured | Optional feature Intra and inter SN (also supports change/list of SK-counter) | Per UE, no FRx/xDD differentiation | Capability for the main feature |
| Reference configuration for MN configured SCPAC | Optional | Per UE, no FRx/xDD differentiation | As this reference configuration handling is different to LTM (separate section in 331) and also because this reference configuration may include MCG and SCG configurations, it seems reasonable to have a separate capability for the reference configuration for SCPAC |
| Reference configuration for SN configured SCPAC | Optional | Per UE, no FRx/xDD differentiation | As this reference configuration handling is different to LTM (separate section in 331) and also because this reference configuration may include MCG and SCG configurations, it seems reasonable to have a separate capability for the reference configuration for SCPAC |

Proposal #3: It is proposed to support the above defined capability bits for SCPAC with the associated functionalities/dependencies.

Interaction of SCPAC with legacy CPAC brings up some additional scenarios, some of which are listed below.  It could be useful to check if they need separate capability bits, perhaps from IOT perspective.  They can be considered for further discussion.

| Feature/ scenario | Capability signalling | Other aspects | Discussion |
| --- | --- | --- | --- |
| Support of simultaneous configuration of SCPAC and legacy CPAC |  |  | As networks and UEs that support SCPAC will also support CPAC, it should be possible to also test for simultaneous configurations of both. A separate capability bit does not seem essential. |
| Interaction with legacy CPAC scenarios |  |  | There are several legacy CPAC scenario that have dedicated capability bits: - condPSCellChangeTwoTriggerEvents-r16 - mn-InitiatedCondPSCellChangeNRDC-r17 - sn-InitiatedCondPSCellChangeNRDC-r17 - condPSCellChangeFDD-TDD-r16 - condPSCellChangeFR1-FR2-r16 - inter-SN-condPSCellChangeFDD-TDD-NRDC-r17 - inter-SN-condPSCellChangeFR1-FR2-NRDC-r17 - condPSCellAdditionNRDC-r17 If there is only a single capability bit for SCPAC, it implies that UE that indicates support of SCPAC will also need to support SCPAC in combination with each of the supported legacy CPAC features. This seems to be a reasonable assumption. Hence it does not seem essential to have separate capability or IOT bits for combinations of SCPAC with the legacy CPAC features. |

## CHO including target MCG and candidate SCGs for CPC CPA in NR-DC

| Feature/ scenario | Capability signalling | Other aspects | Discussion |
| --- | --- | --- | --- |
| CHO with candidate SCG for CPC/CPA | Optional feature UE shall support CHO | Per UE, no FRx/xDD differentiation | Capability for the main feature |

Proposal #5: It is proposed to support the above defined capability bit for combination of CHO with CPAC with the associated dependencies.

Interaction of legacy CHO and CPAC causes many additional combinations and it could be useful to double check if they need separate capability bits, perhaps from IOT perspective.

The following are the legacy CHO and CPAC features that have dedicated capabilities

CHO: condHandoverTwoTriggerEvents-r16, eventA4BasedCondHandover-r17, locationBasedCondHandover-r17, timeBasedCondHandover-r17, condHandoverFDD-TDD-r16, condHandoverFR1-FR2-r16

CPAC: condPSCellChangeTwoTriggerEvents-r16, mn-InitiatedCondPSCellChangeNRDC-r17, sn-InitiatedCondPSCellChangeNRDC-r17, condPSCellChangeFDD-TDD-r16, condPSCellChangeFR1-FR2-r16, inter-SN-condPSCellChangeFDD-TDD-NRDC-r17, inter-SN-condPSCellChangeFR1-FR2-NRDC-r17

When we have a combination of CHO+CPAC, each of the above CHO could be considered in combination with any of the CPAC features, resulting in many combinations.  But it seems reasonable to assume that if a UE indicates supports for these legacy features (i.e., the network and UE supports them) and when a combination of CHO+CPAC  is supported, it should be possible to support and test with all of the supported CHO and CPAC capabilities.  Hence it does not seem essential to support separate capability bits for each of these combinations.

Proposal #6: Confirm that separate capability bits are not needed for the combination of CHO+CPAC with each of the legacy CHO and CPAC features.

Another open issue is the Max # of condCells that is captured as an FFS as below.  It is already agreed that UE shall support (at least) 8 candidate cells.

FFS whether any optional additional UE cap for higher number is needed.

Proposal #7: Optional additional UE cap for more than 8 candidate cells is not needed.

## Inclusion of the features in TS 38.306

As LTM features are related to measurement and mobility, it is proposed to include the LTM related features in the MeasAndMobParameters.

Proposal #8: it is proposed to include the LTM related features in the MeasAndMobParameters.

As the SCPAC and CHO with CPAC are related to MRDC, it is proposed to include the SCPAC and CHO+CPAC in the MeasAndMobParametersMRDC.

Proposal #9: It is proposed to include the SCPAC and CHO+CPAC in the MeasAndMobParametersMRDC.

## Topics that need further discussion

The following capability bits may need further discussion (this is not a complete list and additional capability bits can be considered).   These were mentioned by companies but there is no immediate consensus or where companies need more time before taking a decision.

- MCG LTM with SCG release at LTM execution

- MCG LTM with SCG remaining at LTM execution

- Support of the new "LTM cell switch execution" procedure.

- Deactivated Scell as LTM candidate cell

- Additional granularity for SCPAC (including whether per band should be considered for some)

- Dependency/interaction on legacy CPA and CPC capabilities.  This may require functional discussion on interaction with legacy features before capability dependency can be discussed

- …

# Summary and proposals

The following proposals were made.

Proposal #1: It is proposed to support the following capability bits for LTM with the associated functionalities/dependencies.

| Feature/ scenario | Capability signalling | Other aspects | Related RAN1 features | Discussion |
| --- | --- | --- | --- | --- |
| MCG LTM | Optional feature Supported components: MCG LTM MAC CE based cell switch command; Preconfiguration of LTM candidate cell | Per UE, no FRx/xDD differentiation | Supports RAN1 intra-frequency L1 measurement and report (45-1) or inter-frequency L1 measurement and report (45-1a), both of which are per band/BC. | RAN2 UE specific LTM capability separate from RAN1 capabilities |
| SCG LTM |  | Per UE, no FRx/xDD differentiation | Supports RAN1 intra-frequency L1 measurement and report (45-1) or inter-frequency L1 measurement and report (45-1a), both of which are per band/BC. | Separate capability bit for SCG LTM in addition to MCG LTM seems useful, at least for IOT purpose |
| RACHless LTM with DG for MCG | Optional feature Dependencies: UE shall indicate support of MCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | None of the RAN1 features cover this directly. Hence need a capability just for this. |
| RACHless LTM with CG for MCG | Optional feature Dependencies: UE shall indicate support of MCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | None of the RAN1 features cover this directly. Hence need a capability just for this. |
| RACHless LTM with DG for SCG | Optional feature Dependencies: UE shall indicate support of SCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | Separate capability for SCG |
| RACHless LTM with CG for SCG | Optional feature Dependencies: UE shall indicate support of SCG LTM | Per UE, no FRx/xDD differentiation | Supports RAN1 capability of TCI state in MAC CE (45-3 or 45-4 or both) | Separate capability for SCG |

Proposal #3: It is proposed to support the following capability bits for SCPAC with the associated functionalities/dependencies.

| Feature/ scenario | Capability signalling | Other aspects | Discussion |
| --- | --- | --- | --- |
| SCPAC MN configured with MN event | Optional feature Intra and inter SN (also supports change/list of SK-counter) | Per UE, no FRx/xDD differentiation | Capability for the main feature |
| SCPAC MN configured with SN event | Optional feature Intra and inter SN (also supports change/list of SK-counter) | Per UE, no FRx/xDD differentiation | Capability for the main feature |
| SCPAC SN configured | Optional feature Intra and inter SN (also supports change/list of SK-counter) | Per UE, no FRx/xDD differentiation | Capability for the main feature |
| Reference configuration for MN configured SCPAC | Optional | Per UE, no FRx/xDD differentiation | As this reference configuration handling is different to LTM (separate section in 331) and also because this reference configuration may include MCG and SCG configurations, it seems reasonable to have a separate capability for the reference configuration for SCPAC |
| Reference configuration for SN configured SCPAC | Optional | Per UE, no FRx/xDD differentiation | As this reference configuration handling is different to LTM (separate section in 331) and also because this reference configuration may include MCG and SCG configurations, it seems reasonable to have a separate capability for the reference configuration for SCPAC |

Proposal #5: It is proposed to support the following capability bit for combination of CHO with CPAC with the associated dependencies.

| Feature/ scenario | Capability signalling | Other aspects | Discussion |
| --- | --- | --- | --- |
| CHO with candidate SCG for CPC/CPA | Optional feature UE shall support CHO | Per UE, no FRx/xDD differentiation | Capability for the main feature |

Proposal #6: Confirm that separate capability bits are not needed for the combination of CHO+CPAC with each of the legacy CHO and CPAC features.

Proposal #7: Optional additional UE cap for more than 8 candidate cells is not needed.

Proposal #8: it is proposed to include the LTM related features in the MeasAndMobParameters.

Proposal #9: It is proposed to include the SCPAC and CHO+CPAC in the MeasAndMobParametersMRDC.

# Proposed TP for each feature (not in alphabetical order)

## TS38.306

### LTM:

| ltm-MCG-r18 Indicates whether the UE supports LTM for MCG as defined in TS 38.331 [9]. UE indicating support for this feature shall support MAC CE based cell swich command. UE shall support intra-frequency L1 measurement and report (45-1) or inter-frequency L1 measurement and report (45-1a) | UE | No | No | No |
| --- | --- | --- | --- | --- |
| ltm-SCG-r18 Indicates whether the UE supports LTM for SCG as defined in TS 38.331 [9]. UE indicating support for this feature shall support MAC CE based cell swich command. UE shall support intra-frequency L1 measurement and report (45-1) or inter-frequency L1 measurement and report (45-1a). | UE | No | No | No |
| rachlessLTM-DG-MCG-r18 Indicates whether the UE supports RACHless LTM with dynamic grant for MCG LTM. UE indicating support for this feature shall also indicate support for ltm-MCG-r18. UE indicating support for this feature shall also supports RACH based early TA (45-5) or UE based TA acquisition (45-6) | UE | No | No | No |
| rachlessLTM-CG-MCG-r18 Indicates whether the UE supports RACHless LTM with configured Grant for MCG LTM. UE indicating support for this feature shall also indicate support for ltm-MCG-r18. UE indicating support for this feature shall also indicate support for TCI state in MAC CE (45-3 or 45-4 or both) | UE | No | No | No |
| rachlessLTM-DG-SCG-r18 Indicates whether the UE supports RACHless LTM with dynamic grant for SCG LTM. UE indicating support for this feature shall also indicate support for ltm-SCG-r18. UE indicating support for this feature shall also indicate support for TCI state in MAC CE (45-3 or 45-4 or both) | UE | No | No | No |
| rachlessLTM-CG-SCG-r18 Indicates whether the UE supports RACHless LTM with configured Grant for SCG LTM. UE indicating support for this feature shall also indicate support for ltm-SCG-r18. UE indicating support for this feature shall also indicate support for TCI state in MAC CE (45-3 or 45-4 or both) | UE | No | No | No |

### SCPAC

| mn-Configured-MN-trigger-SCPAC-r18 Indicates whether the UE supports subsequent CPAC as defined in TS 38.331 [9] for MN configured conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using MN configured measurement as triggering condition. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| mn-Configured-SN-trigger-SCPAC-r18 Indicates whether the UE supports subsequent CPAC as defined in TS 38.331 [9] for MN configured conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using SN configured measurement as triggering condition. | UE | No | No | No |
| sn-Configured-SCPAC-r18 Indicates whether the UE supports subsequent CPAC as defined in TS 38.331 [9] for SN configured conditional PSCell change in NR-DC. | UE | No | No | No |
| mn-Configured-referenceConfigSCPAC-r18 Indicates whether the UE supports reference configuration for mn-Configured-MN-trigger-SCPAC-r18 and mn-Configured-SN-trigger-SCPAC-r18 as defined in TS 38.331 [9]. | UE | No | No | No |
| sn-Configured-referenceConfig-SCPAC-r18 Indicates whether the UE supports reference configuration for sn-Configured-SCPAC-r18 as defined in TS 38.331 [9]. | UE | No | No | No |

### CHO+CPAC

| condHandoverWithCandSCG-NRDC-r18 Indicates whether the UE supports conditional handover with NR SCG configuration and with PSCell execution condition (i.e. condExecutionCondPSCell) for NR-DC | UE | No | No | No |
| --- | --- | --- | --- | --- |

## TS38.331

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE {

concurrentPerUE-OnlyMeasGap-r17         ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17    ENUMERATED {supported}

}                                                                               OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

nr-NeedForGapNCSG-Reporting-r17         ENUMERATED {supported}                  OPTIONAL,

eutra-NeedForGapNCSG-Reporting-r17      ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-1 per FR Network controlled small gap (NCSG)

ncsg-MeasGapPerFR-r17                   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-2 Network controlled small gap (NCSG) supported patterns

ncsg-MeasGapPatterns-r17                BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-1-3 Network controlled small gap (NCSG) supported NR-only patterns

ncsg-MeasGapNR-Patterns-r17             BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL,

rrm-RelaxationRRC-ConnectedRedCap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 25-3: Parallel measurements with multiple measurement gaps

parallelMeasurementGap-r17              ENUMERATED {n2}                         OPTIONAL,

condHandoverWithSCG-NRDC-r17            ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-r17              ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-ENDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NEDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NRDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NPN-r17          ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R4 25-1: Parallel measurements on multiple SMTC-s for a single frequency carrier

parallelSMTC-r17                        ENUMERATED {n4}                         OPTIONAL,

-- R4 19-2-1 Concurrent measurement gaps for EUTRA

concurrentMeasGapEUTRA-r17              ENUMERATED {supported}                  OPTIONAL,

serviceLinkPropDelayDiffReporting-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-4 Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter

ncsg-SymbolLevelScheduleRestrictionInter-r17  ENUMERATED {supported}            OPTIONAL

]],

[[

eventD1-MeasReportTrigger-r17           ENUMERATED {supported}                  OPTIONAL,

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               OPTIONAL

]],

[[

interSatMeas-r17                            ENUMERATED {supported}              OPTIONAL,

deriveSSB-IndexFromCellInterNon-NCSG-r17    ENUMERATED {supported}              OPTIONAL

]] ,

[[

ltm-MCG-r18                                  ENUMERATED {supported}              OPTIONAL,

ltm-SCG-r18                                  ENUMERATED {supported}              OPTIONAL,

rachlessLTM-DG-MCG-r18                            ENUMERATED {supported}              OPTIONAL,

rachlessLTM-CG-MCG-r18                            ENUMERATED {supported}              OPTIONAL,

rachlessLTM-DG-SCG-r18                            ENUMERATED {supported}              OPTIONAL,

rachlessLTM-CG-SCG-r18                            ENUMERATED {supported}              OPTIONAL,

ltm-ReferenceConfig-r18                     ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=        SEQUENCE {

intraAndInterF-MeasAndReport            ENUMERATED {supported}                  OPTIONAL,

eventA-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

...,

[[

handoverInterF                          ENUMERATED {supported}                  OPTIONAL,

handoverLTE-EPC                         ENUMERATED {supported}                  OPTIONAL,

handoverLTE-5GC                         ENUMERATED {supported}                  OPTIONAL

]],

[[

sftd-MeasNR-Neigh                       ENUMERATED {supported}                  OPTIONAL,

sftd-MeasNR-Neigh-DRX                   ENUMERATED {supported}                  OPTIONAL

]],

[[

dummy                                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                  ENUMERATED {supported}              OPTIONAL,

dummy                                       ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                       ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}          OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL,

-- R4 6-2: Support of beam level Early Measurement Reporting

idleInactiveNR-MeasBeamReport-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

increasedNumberofCSIRSPerMO-r16             ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersFR2-2-r17 ::=           SEQUENCE {

handoverInterF-r17                          ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC-r17                         ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC-r17                         ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r17               ENUMERATED {supported}              OPTIONAL,

...

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560      OPTIONAL

}

MeasAndMobParametersMRDC-v1610 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1610      MeasAndMobParametersMRDC-Common-v1610        OPTIONAL,

interNR-MeasEUTRA-IAB-r16                  ENUMERATED {supported}                       OPTIONAL

}

MeasAndMobParametersMRDC-v1700 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1700      MeasAndMobParametersMRDC-Common-v1700        OPTIONAL

}

MeasAndMobParametersMRDC-v1730 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1730   MeasAndMobParametersMRDC-Common-v1730           OPTIONAL

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1610 ::=   SEQUENCE {

condPSCellChangeParametersCommon-r16        SEQUENCE {

condPSCellChangeFDD-TDD-r16                 ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeFR1-FR2-r16                 ENUMERATED {supported}                  OPTIONAL

}                                                                                       OPTIONAL,

pscellT312-r16                              ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1700 ::=   SEQUENCE {

condPSCellChangeParameters-r17              SEQUENCE {

inter-SN-condPSCellChangeFDD-TDD-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFDD-TDD-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL

}                                                                                       OPTIONAL,

condHandoverWithSCG-ENDC-r17                        ENUMERATED {supported}              OPTIONAL,

condHandoverWithSCG-NEDC-r17                        ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1730 ::= SEQUENCE {

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                             OPTIONAL

}

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                          OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}                   OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}                   OPTIONAL

}

MeasAndMobParametersMRDC-v1800 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1800      MeasAndMobParametersMRDC-Common-v1800        OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1800 ::=   SEQUENCE {

mn-Configured-MN-trigger-SCPAC-r18            ENUMERATED {supported}                OPTIONAL,

mn-Configured-SN-trigger-SCPAC -r18           ENUMERATED {supported}                OPTIONAL,

sn-Configured-SCPAC -r18                      ENUMERATED {supported}                OPTIONAL,

mn-Configured-referenceConfig-SCPAC-r18        ENUMERATED {supported}                OPTIONAL,

sn-Configured-referenceConfig-SCPAC-r18        ENUMERATED {supported}                OPTIONAL,

condHandoverWithCandSCG-NRDC-r18              ENUMERATED {supported}                OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [26].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=              SEQUENCE {

measAndMobParametersMRDC            MeasAndMobParametersMRDC                                                        OPTIONAL,

phy-ParametersMRDC-v1530            Phy-ParametersMRDC                                                              OPTIONAL,

rf-ParametersMRDC                   RF-ParametersMRDC,

generalParametersMRDC               GeneralParametersMRDC-XDD-Diff                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

tdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

fr1-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

fr2-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

featureSetCombinations              SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

pdcp-ParametersMRDC-v1530           PDCP-ParametersMRDC                                                             OPTIONAL,

lateNonCriticalExtension            OCTET STRING (CONTAINING UE-MRDC-Capability-v15g0)                              OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1560                                                        OPTIONAL

}

-- Regular non-critical extensions:

UE-MRDC-Capability-v1560 ::=        SEQUENCE {

receivedFilters                     OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                         OPTIONAL,

measAndMobParametersMRDC-v1560      MeasAndMobParametersMRDC-v1560                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

tdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1610                                                        OPTIONAL

}

UE-MRDC-Capability-v1610 ::=        SEQUENCE {

measAndMobParametersMRDC-v1610      MeasAndMobParametersMRDC-v1610                                                  OPTIONAL,

generalParametersMRDC-v1610         GeneralParametersMRDC-v1610                                                     OPTIONAL,

pdcp-ParametersMRDC-v1610           PDCP-ParametersMRDC-v1610                                                       OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1700                                                        OPTIONAL

}

UE-MRDC-Capability-v1700 ::=        SEQUENCE {

measAndMobParametersMRDC-v1700      MeasAndMobParametersMRDC-v1700,

nonCriticalExtension                UE-MRDC-Capability-v1730                                                        OPTIONAL

}

UE-MRDC-Capability-v1730 ::=        SEQUENCE {

measAndMobParametersMRDC-v1730      MeasAndMobParametersMRDC-v1730                                                  OPTIONAL,

nonCriticalExtension   UE-MRDC-Capability-v1800                                                 OPTIONAL

}

UE-MRDC-Capability-v1800 ::=        SEQUENCE {

measAndMobParametersMRDC-v1800      MeasAndMobParametersMRDC-v1800,

nonCriticalExtension                SEQUENCE {}                                                                    OPTIONAL

}

-- Late non-critical extensions:

UE-MRDC-Capability-v15g0 ::=        SEQUENCE {

rf-ParametersMRDC-v15g0             RF-ParametersMRDC-v15g0                                                         OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v15n0                                                        OPTIONAL

}

UE-MRDC-Capability-v15n0 ::=        SEQUENCE {

rf-ParametersMRDC-v15n0             RF-ParametersMRDC-v15n0                                                         OPTIONAL,

-- Following field is only for REL-15 late non-critical extensions

lateNonCriticalExtension            OCTET STRING                                                                    OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v16e0                                                        OPTIONAL

}

UE-MRDC-Capability-v16e0 ::=        SEQUENCE {

rf-ParametersMRDC-v16e0             RF-ParametersMRDC-v16e0                                                         OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff                                           OPTIONAL,

generalParametersMRDC-XDD-Diff          GeneralParametersMRDC-XDD-Diff                                              OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode-v1560 ::=    SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560                                  OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path             ENUMERATED {supported}                                                          OPTIONAL,

splitDRB-withUL-Both-MCG-SCG        ENUMERATED {supported}                                                          OPTIONAL,

srb3                                ENUMERATED {supported}                                                          OPTIONAL,

dummy                               ENUMERATED {supported}                                                          OPTIONAL,

...

}

GeneralParametersMRDC-v1610 ::= SEQUENCE {

f1c-OverEUTRA-r16                   ENUMERATED {supported}                                                          OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

# Annex: (for information) Extract from 38.306 of related capabilities

| condHandover-r16 Indicates whether the UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| condHandoverFailure-r16 Indicates whether the UE supports conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | No | N/A | N/A |
| condHandoverTwoTriggerEvents-r16 Indicates whether the UE supports 2 trigger events for same execution condition. This feature is mandatory supported if the UE supports condHandover-r16. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | CY | N/A | N/A |
| condPSCellChange-r16 Indicates whether the UE supports conditional PSCell change including execution condition, candidate cell configuration and maximum 8 candidate cells. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| condPSCellChangeTwoTriggerEvents-r16 Indicates whether the UE supports 2 trigger events for same execution condition. This feature is mandatory supported if the UE supports condPSCellChange-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | CY | N/A | N/A |
| eventA4BasedCondHandover-r17 Indicates whether the UE supports Event A4 based conditional handover in NTN bands, i.e., CondEvent A4 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | No | N/A | N/A |
| locationBasedCondHandover-r17 Indicates whether the UE supports location based conditional handover, i.e., CondEvent D1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | No | N/A | N/A |
| mn-InitiatedCondPSCellChangeNRDC-r17 Indicates whether the UE supports MN initiated conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using MN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. | Band | No | N/A | N/A |
| sn-InitiatedCondPSCellChangeNRDC-r17 Indicates whether the UE supports SN initiated inter-SN conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using SN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in SN initiated inter-SN conditional PSCell change in NR-DC. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. | Band | No | N/A | N/A |
| timeBasedCondHandover-r17 Indicates whether the UE supports time based conditional handover, i.e., CondEvent T1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | No | N/A | N/A |

#### 4.2.7.9	MRDC-Parameters

| condPSCellAdditionENDC-r17 Indicates whether the UE supports conditional PSCell addition in EN-DC. The UE supporting this feature shall also support 2 trigger events for same execution condition in conditional PSCell addition in EN-DC. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| condPSCellAdditionNRDC-r17 Indicates whether the UE supports conditional PSCell addition in NR-DC. The UE supporting this feature shall also support 2 trigger events for same execution condition in conditional PSCell addition in NR-DC. | BC | No | No | No |
| --- | --- | --- | --- | --- |

### 4.2.9	MeasAndMobParameters

| condHandoverFDD-TDD-r16 Indicates whether the UE supports conditional handover between FDD and TDD cells. The parameter can only be set if condHandover-r16 is set for both FDD and TDD. The UE that indicates support of this feature shall also indicate support of handoverFDD-TDD. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| condHandoverFR1-FR2-r16 Indicates whether the UE supports conditional handover HO between FR1 and FR2. The parameter can only be set if condHandover-r16 is set for both FR1 and FR2. The UE that indicates support of this feature shall also indicate support of handoverFR1-FR2. | UE | No | No | No |
| condHandoverWithSCG-NRDC-r17 Indicates whether the UE supports conditional handover with NR SCG configuration for NR-DC. The UE indicating support of this feature shall also indicate the support of condHandover-r16 and support of at least one NR-DC band combination. | UE | No | No | No |

### 4.2.9a	MeasAndMobParametersMRDC

| condHandoverWithSCG-ENDC-r17 Indicates whether the UE supports conditional handover with NR SCG configuration for EN-DC. The UE indicating support of this feature shall also indicate the support of cho-r16 as specified in TS 36.306 [15] and at least one EN-DC band combination. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| condHandoverWithSCG-NEDC-r17 Indicates whether the UE supports conditional handover with E-UTRA SCG configuration for NE-DC. The UE indicating support of this feature shall also indicate the support of condHandover-r16 and at least one NE-DC band combination. | UE | No | No | No |
| condPSCellChangeFDD-TDD-r16 Indicates whether the UE supports conditional PSCell change between FDD and TDD cells. The parameter can only be set if condPSCellChange-r16 is set for both FDD and TDD. | UE | No | No | No |
| condPSCellChangeFR1-FR2-r16 Indicates whether the UE supports conditional PSCell change between FR1 and FR2. The parameter can only be set if condPSCellChange-r16 is set for both FR1 and FR2. | UE | No | No | No |
| inter-SN-condPSCellChangeFDD-TDD-ENDC-r17 Indicates whether the UE supports inter SN conditional PSCell change between FDD and TDD cells in EN-DC. The parameter can only be set - if mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17 is supported and at least one of mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17 and mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17 is supported; or - if sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17 is supported and at least one of sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17 and sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17 is supported. | UE | No | No | No |
| inter-SN-condPSCellChangeFDD-TDD-NRDC-r17 Indicates whether the UE supports inter SN conditional PSCell change between FDD and TDD cells in NR-DC. The parameter can only be set if mn-InitiatedCondPSCellChangeNRDC-r17 is set for FDD band(s) and TDD band(s), or sn-InitiatedCondPSCellChangeNRDC-r17 is set for FDD band(s) and TDD band(s). | UE | No | No | No |
| inter-SN-condPSCellChangeFR1-FR2-ENDC-r17 Indicates whether the UE supports inter SN conditional PSCell change between FR1 and FR2 cells in EN-DC. The parameter can only be set: - if mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17 is supported and at least one of mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17 and mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17 is supported; or - if sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17 is supported and at least one of sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17 and sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17 is supported. | UE | No | No | No |
| inter-SN-condPSCellChangeFR1-FR2-NRDC-r17 Indicates whether the UE supports inter SN conditional PSCell change between FR1 and FR2 cells. The parameter can only be set if mn-InitiatedCondPSCellChangeNRDC-r17 is set for FR1 band(s) and FR2 band(s), or sn-InitiatedCondPSCellChangeNRDC-r17 is set for FR1 band(s) and FR2 band(s). | UE | No | No | No |
| mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17 Indicates whether the UE supports MN initiated conditional PSCell change within all supported FR1-FDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using MN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. | UE | No | No | No |
| mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17 Indicates whether the UE supports MN initiated conditional PSCell change within all supported FR1-TDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using MN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. | UE | No | No | No |
| mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17 Indicates whether the UE supports MN initiated conditional PSCell change within all supported FR2-TDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using MN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in MN initiated conditional PSCell change in EN-DC. | UE | No | No | No |
| pscellT312-r16 Indicates whether the UE supports T312 based fast failure recovery for PSCell. | UE | No | No | No |
| sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17 Indicates whether the UE supports SN initiated inter-SN conditional PSCell change within all supported FR1-FDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using SN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in SN initiated inter-SN conditional PSCell change in EN-DC. | UE | No | No | No |
| sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17 Indicates whether the UE supports SN initiated inter-SN conditional PSCell change within all supported FR1-TDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using SN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in SN initiated inter-SN conditional PSCell change in EN-DC. | UE | No | No | No |
| sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17 Indicates whether the UE supports SN initiated inter-SN conditional PSCell change within all supported FR2-TDD bands in EN-DC, which is configured by E-UTRA conditionalReconfiguration field using SN configured measurement as triggering condition. The UE supporting this feature shall also support 2 trigger events for same execution condition in SN initiated inter-SN conditional PSCell change in EN-DC. | UE | No | No | No |