3GPP TSG-RAN WG2 Meeting #125                                                        R2-2400105

Athens, Greece, 26th Feb - 1st Mar, 2024

Source:              CATT

Title:	             Leftover Issues on BSR

Agenda Item:	7.5.3.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss two leftover issues on BSR.

Discussion

Whether new BSR cancellation condition is needed?

Since the trigger conditions of BSR and DSR are different, it is possible that both DSR and BSR are triggered and pending, e.g, periodical BSR and DSR. In this case, if UL grant is received and the UL grant can only accommodate the DSR based on the LCP procedure, only DSR MAC CE can be transmitted. If the data in the buffer are all belonging to delay-critical data, the content of BSR equals to DSR. Hence, it is unnecessary to report the BSR again. In this case, it had better cancel the triggered BSR and corresponding SR (if any).

Proposal 1: BSR and corresponding SR should be cancelled when the total data volume it reports is already reported in a DSR MAC CE.

When to report Long BSR and when to report Refined Long BSR in padding BSR?

According to TS38.321, the long BSR MAC CE and Refined long BSR MAC CE format are as below:

Figure -1 Long BSR MAC CE

Figure -2 Refined Long BSR MAC CE

It is obvious that Long BSR MAC CE and Refined Long BSR MAC CE have different lengths (The refined long BSR MAC CE is one byte more than the long BSR MAC CE).

In TS38.321, for regular BSR and periodical BSR, it has been updated to describe when to report the Refined Long BSR MAC CE and when to report the Long BSR MAC CE, listed below:

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall: 1> if for at least one LCG configured with additionalBSR-TableAllowed, the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3: 2> report Refined Long BSR for all LCGs which have data available for transmission; 1> else: 2> if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built: 3> report Long BSR for all LCGs which have data available for transmission. 2> else: 3> report Short BSR.

But for padding BSR, it is still unclear when to report the Refined Long BSR MAC CE and when to report the Long BSR MAC CE, listed below:

For Padding BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall: 1> if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader: 2> if more than one LCG has data available for transmission when the BSR is to be built: 3> if the number of padding bits is equal to the size of the Short BSR plus its subheader: 4> report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission. 3> else: 4> report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID. 2> else: 3> report Short BSR. 1> else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader: 2> report Long BSR for all LCGs which have data available for transmission.

For padding BSR, considering the lengths of Long BSR MAC CE and Refined Long BSR MAC CE are different, it had better clarify when to report Refined Long BSR MAC CE and when to report Long BSR MAC CE.

Furthermore, regarding to padding BSR for IAB-MT, since whether IAB-MT can support XR or not has not been discussed, the corresponding description was not changed.

Proposal 2: For Padding BSR, clarify in MAC spec when to report Refined Long BSR MAC CE and when to report Long BSR MAC CE.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: BSR and corresponding SR should be cancelled when the total data volume it reports is already reported in a DSR MAC CE.

Proposal 2: For Padding BSR, clarify in MAC spec when to report Refined Long BSR MAC CE and when to report Long BSR MAC CE.

The corresponding TP(TS38.321) for Proposal 1 and Proposal 2 are attached in section 4.

Text Proposal

--------------------------------------- Start of the text proposal------------------------------------------------------

### 5.4.5	Buffer Status Reporting

The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity.

RRC configures the following parameters to control the BSR:

-	periodicBSR-Timer;

-	retxBSR-Timer;

-	logicalChannelSR-DelayTimerApplied;

-	logicalChannelSR-DelayTimer;

-	logicalChannelSR-Mask;

-	logicalChannelGroup, logicalChannelGroupIAB-Ext;

-	sdt-LogicalChannelSR-DelayTimer;

-	additionalBSR-TableAllowed.

Each logical channel may be allocated to an LCG using the logicalChannelGroup. The maximum number of LCGs is eight except for IAB-MTs configured with logicalChannelGroupIAB-Ext, for which the maximum number of LCGs is 256.

The MAC entity determines the amount of UL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4].

A BSR shall be triggered if any of the following events occur for activated cell group:

-	UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either

-	this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or

-	none of the logical channels which belong to an LCG contains any available UL data.

in which case the BSR is referred below to as 'Regular BSR';

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR';

-	retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR';

-	periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'.

NOTE 1:	When Regular BSR triggering events occur for multiple logical channels simultaneously, each logical channel triggers one separate Regular BSR.

For Regular BSR, the MAC entity shall:

1>	if the BSR is triggered for a logical channel for which logicalChannelSR-DelayTimerApplied with value true is configured by upper layers and SDT procedure is not on-going according to clause 5.27:

2>	start or restart the logicalChannelSR-DelayTimer.

1>	else if BSR is triggered for a logical channel for which logicalChannelSR-DelayTimerApplied with value true is configured by upper layers and SDT procedure is on-going according to clause 5.27:

2>	start or restart logicalChannelSR-DelayTimer with the value as configured by the sdt-LogicalChannelSR-DelayTimer.

1>	else:

2>	if running, stop the logicalChannelSR-DelayTimer.

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall:

1>	if for at least one LCG configured with additionalBSR-TableAllowed, the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3:

2>	report Refined Long BSR for all LCGs which have data available for transmission;

1>	else:

2>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

3>	report Long BSR for all LCGs which have data available for transmission.

2>	else:

3>	report Short BSR.

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is configured by upper layers shall:

1>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

2>	if the maximum LCG ID among the configured LCGs is 7 or lower:

3>	report Long BSR for all LCGs which have data available for transmission.

2>	else:

3>	report Extended Long BSR for all LCGs which have data available for transmission.

1>	else:

2>	report Extended Short BSR.

For Padding BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Short BSR.

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2> if for at least one LCG configured with additionalBSR-TableAllowed, the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3 and the number of padding bits is equal to or larger than the size of the Refined Long BSR plus its subheader:

3>  report Refined Long BSR for all LCGs which have data available for transmission;

2> else:

3>  report Long BSR for all LCGs which have data available for transmission.

For Padding BSR, the MAC entity for which logicalChannelGroupIAB-Ext is configured by upper layers shall:

1>	if the number of padding bits is equal to or larger than the size of the Extended Short BSR plus its subheader but smaller than the size of the Extended Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is smaller than the size of the Extended Long Truncated BSR with zero Buffer Size field plus its subheader:

4>	report Extended Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Extended Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Extended Short BSR.

1>	else if the number of padding bits is equal to or larger than the size of the Extended Long BSR plus its subheader:

2> report Extended Long BSR for all LCGs which have data available for transmission.

For BSR triggered by retxBSR-Timer expiry, the MAC entity considers that the logical channel that triggered the BSR is the highest priority logical channel that has data available for transmission at the time the BSR is triggered.

The MAC entity shall:

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the BSR MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s) as defined in clause 6.1.3.1;

3>	start or restart periodicBSR-Timer except when all the generated BSRs are long or short Truncated or Extended long or short Truncated BSRs;

3>	start or restart retxBSR-Timer.

2>	if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running:

3>	if there is no UL-SCH resource available for a new transmission; or

3>	if the MAC entity is configured with configured uplink grant(s) and the Regular BSR was triggered for a logical channel for which logicalChannelSR-Mask is set to false; or

3>	if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions (see clause 5.4.3.1) configured for the logical channel that triggered the BSR:

4>	trigger a Scheduling Request.

NOTE 2:	UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one BSR MAC CE, even when multiple events have triggered a BSR. The Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon reception of a grant for transmission of new data on any UL-SCH.

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader or all the pending data for transmission has already been reported by DSR. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

NOTE 3:	MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. BSR and SR can be triggered after the assembly of a MAC PDU which contains a BSR MAC CE, but before the transmission of this MAC PDU. In addition, BSR and SR can be triggered during MAC PDU assembly.

NOTE 4:	Void

NOTE 5:	If a HARQ process is configured with cg-RetransmissionTimer and if the BSR is already included in a MAC PDU for transmission on configured grant by this HARQ process, but not yet transmitted by lower layers, it is up to UE implementation how to handle the BSR content.

--------------------------------------------- End of the text proposal ------------------------------------------------