3GPP TSG-RAN WG2 Meeting #125                                                        R2-2400106

Athens, Greece, 26th Feb - 1st Mar, 2024

Source:              CATT

Title:	             Further Discussion on DSR

Agenda Item:	7.5.3.2

Document for:	Discussion and Decision

Introduction

In this contribution, how to determine the logical channel that triggered the DSR will be discussed.

Discussion

In section 5.4.9 of TS38.321, the following DSR triggering condition is specified. Based on the following description, it is obvious that DSR is triggered based on LCG.

If an LCG is configured for delay status reporting, the MAC entity shall: - if the smallest remaining value of the PDCP discardTimers among all the data buffered for the LCG that has not been transmitted in any MAC PDU or reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the LCG since the last transmission of a DSR MAC CE: - trigger a DSR for the LCG.

Observation 1: Based on the DSR trigger condition, it is obvious that DSR is triggered based on LCG.

However, in section 5.4.4 of TS38.321, when select the SR configuration for DSR, it specified that the SR configuration of the logical channel that triggered a DSR is considered as corresponding SR configuration.

The SR configuration of the logical channel that triggered a BSR (clause 5.4.5) or a DSR (clause 5.4.9) or the SCell beam failure recovery or the beam failure recovery of a BFD-RS set or the consistent LBT failure recovery (clause 5.21) (if such a configuration exists) or positioning measurement gap activation/deactivation request (clause 5.25) is considered as corresponding SR configuration for the triggered SR.

Furthermore, in section 5.4.9, it also mentioned that DSR is corresponding to logical channel.

1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request.

Observation 2: When selecting the SR resource for DSR, it specified the SR configuration of the logical channel triggered the DSR is used.

Based on the above Observation 1 and Observation 2, one issue raised that is how to decide the logical channel triggered the DSR since DSR is triggered based on LCG. This issue should be addressed;

Proposal 1: It should clarify how to decide the logical channel that triggered the DSR.

Regarding to the issue in Proposal 1, there are two options:

- Option 1: Specify in spec that the logical channel which the data triggered the DSR belongs to is considered as the logical channel that triggered the DSR.

- Option 2: Use “the logical channel which the data triggered the DSR belongs to” instead of “the logical channel that triggered a DSR”.

The similar issue was also discussed in the previous meeting that is how to define the logical channel that triggered a BSR in case of the BSR is triggered by retxBSR-Timer expiry. And the following was captured in TS38.321 to solve this problem:

For BSR triggered by retxBSR-Timer expiry, the MAC entity considers that the logical channel that triggered the BSR is the highest priority logical channel that has data available for transmission at the time the BSR is triggered.

Based on the same principle, Option 1 is slightly preferred.

Proposal 2: Specify in spec that the logical channel which the data triggered the DSR belongs to is considered as the logical channel triggered the DSR.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: It should clarify how to decide the logical channel that triggered the DSR.

Proposal 2: Specify in spec that the logical channel which the data triggered the DSR belongs to is considered as the logical channel triggered the DSR.

The corresponding TP (TS 38.321) for Proposal 2 is attached in section 4.

Text Proposal

--------------------------------------- Start of the text proposal----------------------------------------------------

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the PDCP discardTimers among SDUs buffered for the LCG as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold: the threshold on remaining time for triggering a DSR for an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall:

- if the smallest remaining value of the PDCP discardTimers among all the data buffered for the LCG that has not been transmitted in any MAC PDU or reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>   if there is no DSR pending for the LCG since the last transmission of a DSR MAC CE:

2> trigger a DSR for the LCG.

The MAC entity considers the logical channel which the data triggered the DSR belongs to as the logical channel triggered the DSR.

If there is at least one DSR pending, the MAC entity shall:

- if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE;

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

An SDU is considered to be associated with a DSR if it is associated with the LCG which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold.

A MAC PDU shall contain at most one DSR MAC CE. The MAC entity shall not include a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate the SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, either when all the SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes either all the SDUs associated with the DSR or a DSR MAC CE that contains the delay information of all the SDUs associated with the DSR (as described in the clause 6.1.3.72).

--------------------------------------------- End of the text proposal ------------------------------------------------