3GPP TSG-RAN WG2 Meeting #125	R2-2400155

Athens, Greece, Feb 26th – Mar 1st, 2024

Source:	vivo

Title:	Discussion on LMF involvement in SL-PRS resource allocation

Agenda Item:	7.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#123 meeting, the mechanism of SL-PRS resource allocation when LMF is involved is FFS:

When aperiodic/one-shot SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, design a new MAC CE for the UE to send to the gNB for SL-PRS resource request. (12/14) FFS when LMF is involved.

At least when periodic SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, the UE sends an RRC message to the gNB for providing the assistance information for CG configuration. (13/14) FFS when the LMF is involved.

At the RAN3#122 meeting, the signalling design on SL-PRS resource allocation for scheme 1 was discussed and RAN3 sent the LS [1] to ask RAN2 about the mechanism.

- 1 Overall description RAN3 discussed the signalling design in RAN3 on SL-PRS resource allocation for scheme 1. RAN3 would like to ask RAN2 whether LMF is involved in the SL-PRS resource allocation, and if yes, whether RAN2 expects NRPPa impacts. - 2 Actions To RAN2 ACTION: RAN3 kindly asks RAN2 to provide feedback on the above questions to RAN3.

This contribution provides our understanding of LMF-involved SL-PRS resource allocation for scheme 1.

- Discussion

For UL-based positioning, the LMF will send the requested SRS characteristic to serving gNB for SRS configuration to a target UE. Similarly, having the full knowledge of the QoS requirement of location service request and therefore the corresponding SL-PRS characteristics, LMF can request gNB to allocate an appropriate SL grant for the TX UE through the provision of SL-PRS characteristics to gNB.

The detailed procedures are illustrated in Figure 2-1.

Figure 2-1 LMF-involved SL-PRS resource allocation for scheme 1

Step 1: LMF may send SL-PRS characteristics to the serving gNB to request CG for TX UE via NRPPa message, e.g., POSITONING INFORMATION REQUEST.

Step 2: The serving gNB configures TX UE with resource pool for SL-PRS, and Type 1 Sidelink Configured Grant via RRCReconfiguration.

Step 3: The serving gNB provides feedback to LMF via NRPPa message, e.g., POSITONING INFORMATION RESPONSE, and CG information can be optionally provided.

Step 4: LMF provides TX UE with SL-PRS characteristics to trigger SL-PRS transmission, and SL-PRS sequence ID and/or CG index are optionally provided.

Step 5: LMF provides RX UE with sequence ID for SL-PRS reception.

Step 6: TX UE transmits SL-PRS to RX UE.

In step 1, the content of SL-PRS characteristics that LMF provides to gNB may take RAN1 agreement for reference. Based on current progress in RAN1 and RAN2, we think the SL-PRS characteristics should at least include priority and bandwidth of SL-PRS, other parameters should align with the UAI from TX UE to gNB, e.g., periodicity, delay budget, symbol number, comb size, and repetition number.

Agreement In resource allocation in scheme 1, for a dedicated resource pool - in the DCI, introduce at least the following fields: - Resource pool index – number of bits same to SL communications - Time gap - 3 bits - SCI format 1-B fields: - Time resource assignment for SL-PRS future reservation(s) - SL-PRS resource ID (s) for the future 1 or 2 reservations - SL-PRS resource ID for the first SL-PRS transmission - Configuration index – number of bits same to SL communications - Padding bits, if required - For configured grant type 1 resource allocation, - RRC is used for indicating at least the following: - Info-1: the periodicity, - Info-2: the slot offset relative to a logical slot defined by Info-3, - Info-3: SFN used for determination of the slot offset, - Info-4: Resource pool index - Info-5: Time resource assignment for SL-PRS future reservation(s) - Info-6: SL-PRS resource ID (s) for the future 1 or 2 reservations - Info-7: SL-PRS resource ID for the first SL-PRS transmission - For configured grant type 2 resource allocation, - RRC is used for indicating at least the following: - Info 1: the periodicity - DCI is used for the activation/release of the configured grant resources

In step 2, the serving gNB should take the SL-PRS characteristics into consideration to configure the SL grant for scheme-1 transmitting UE or to adjust the configuration of the SL-PRS resource pool accordingly.

Based on the above procedure, we propose:

Proposal 1: Reply to RAN3 that LMF can be involved in the SL-PRS resource allocation.

Proposal 2: Reply to RAN3 with the following NRPPa impacts:

- LMF may provide SL-PRS characteristics to serving gNB for SL-PRS resource allocation, including the configuration of the resource pool and/or CG for SL-PRS transmission. The SL-PRS characteristics at least include priority and bandwidth, other parameters should align with UAI from UE to gNB.

- After receiving SL-PRS characteristics from LMF and allocating resources for TX UE, gNB may feedback to LMF with corresponding SL CG information (e.g., SL CG index) via NRPPa message.

Proposal 3: When triggering SL-PRS transmission, LMF may also provide SL CG index to TX UE.

- Conclusion

Proposal 1: Reply to RAN3 that LMF can be involved in the SL-PRS resource allocation.

Proposal 2: Reply to RAN3 with the following NRPPa impacts:

- LMF may provide SL-PRS characteristics to serving gNB for SL-PRS resource allocation, including the configuration of the resource pool and/or CG for SL-PRS transmission. The SL-PRS characteristics at least include priority and bandwidth, other parameters should align with UAI from UE to gNB.

- After receiving SL-PRS characteristics from LMF and allocating resources for TX UE, gNB may feedback to LMF with corresponding SL CG information (e.g., SL CG index) via NRPPa message.

Proposal 3: When triggering SL-PRS transmission, LMF may also provide SL CG index to TX UE.

- Reference

- R2-2400038	LS on LMF involvement in SL-PRS resource allocation