3GPP TSG-RAN WG2 Meeting #125	                                                                   R2-2400268

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Source:	CATT

Title:	Discussion on SA2 LS on RedCap UE MBS Broadcast Reception

Agenda Item:	7.24.1

Document for:	Discussion and Decision

- 1. Introduction

This paper discusses the issues mentioned in the SA2 LS on RedCap UE MBS Broadcast Reception from SA2 [1].

- 2. Discussion

In the SA2 LS [1], Questions are raised to RAN2 as follows,

SA2 thanks RAN2 for the feedback. RAN2 has replied to SA2´s question 1: An indication that an MBS broadcast session is intended to be received by both non-RedCap UE and RedCap UE may assist the gNB to decide when to transmit the session on both default and RedCap CFR and avoid waste of resources when this is not needed. SA2 would like to inform that it agreed the attached CR against TS 23.247 to reflect that reply. SA2 understands that for an MBS session intended for both RedCap and non-RedCap UEs separate radio resources may be allocated by RAN nodes for RedCap and non-RedCap UEs. It is, however, unclear whether a single MBS Frequency Selection Area (FSA) ID can apply to RedCap UEs and non-RedCap UEs in the same MBS session. If it is not the case, whether it is feasible to adopt different MBS FSA IDs for RedCap and non-RedCap UEs. SA2 would thus like to ask RAN2 to answer the following related questions: Q1: SA2 would like to ask RAN2 to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session . Q2: If the answer to Q1 is no, could RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s)? SA2 would thus like to ask RAN3 to answer the following related question: Q3: If the answer to Q1 is no, and RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s), is there a need for CN to indicate to NG-RAN which FSA ID is aimed for RedCap UEs and which for non-RedCap UEs?

SA2 confirmed that the CN can provide the indication to the gNB whether a certain broadcast session is intended for non-redcap UE and/or redcap UE. This can assist the gNB to give an efficient scheduling for the broadcast service.

Regarding the SA2 question on the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session, from RAN2 perspective we do not see the needs to use different MBS FSA ID for them.

If to use different MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session, there will big ASN.1 signaling impact (e.g. enhancement to SIB21 to include FSAI mapping for redcap UEs) to RAN2 which is not expected to RAN2.

The draft of reply LS to SA2 is provided in the Annex 1.

Observation 1: From RAN2 perspective, there is no need to use different MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

Proposal 1: Same MBS FSA ID is used for the RedCap UEs and non-RedCap UEs in the same MBS session.

- 3. Conclusion

In this paper, we give the discussion on the question mentioned in the SA2 LS [1]. The observation and proposal are as follows,

Observation 1: From RAN2 perspective, there is no need to use different MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

Proposal 1: Same MBS FSA ID is used for the RedCap UEs and non-RedCap UEs in the same MBS session.

- 4. Reference

[1] S2-2401506 Reply LS on RedCap UE MBS Broadcast reception

- Annex 1: Draft reply LS to SA2

1. Overall Description:

RAN2 thanks the LS from SA2 with the following question,

SA2 would thus like to ask RAN2 to answer the following related questions: Q1: SA2 would like to ask RAN2 to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session . Q2: If the answer to Q1 is no, could RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s)?

RAN2 Answer to Q1:

Same MBS FSA ID is used for the RedCap UEs and non-RedCap UEs in the same MBS session..

2. Actions

To SA2 group

ACTION: 	RAN2 kindly asks SA2 to take the above information into consideration.

To RAN3 group

ACTION: 	RAN2 kindly asks RAN3 to take the above information into consideration.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #125bis    	15 April – 19 April 2024		                                 China

TSG RAN WG2 Meeting #126		20 May – 24 May 2024		                                 Japan