3GPP TSG-RAN WG2 Meeting #125 	   R2-2400382

Athens, Greece, Feb 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE capability 38.331 for Rel-18 R1 feature lists and corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core, NR_cov_enh2, NR_Mob_enh2-Core, NR_SL_enh2-Core, NR_MC_enh, NR_BWP_wor, NR_DualTxRx_MUSIM-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture further Release-18 UE capabilities based on the RAN1 UE feature list (R1-2312707) and further editorial corrections based on ASN.1 review. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New Release-18 capabilities from RAN1 are added based on the latest RAN1 feature lists. - Editorial corrections based on ASN.1 review. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New RAN1 related UE capabilities and editorial corrections from ASN.1 review will not be captured in specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.306 CRx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF 1st CHANGE

### 6.3.3	UE capability information elements

#### –	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, rel16, rel17, rel18, spare4, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

#### –	AerialParameters

The IE AerialParameters is used to convey the capabilities supported by the UE for aerial operation.

AerialParameters information element

-- ASN1START

-- TAG-AERIALPARAMETERS-START

AerialParameters-r18 ::=                      SEQUENCE {

-- Support of Aerial UE features

aerialUE-Capability-r18                       ENUMERATED {supported}                               OPTIONAL,

-- Support of altitude measurement and event H1/H2-triggered reporting

altitudeMeas-r18                              ENUMERATED {supported}                               OPTIONAL,

-- Support of altitude based measurement configuration of SSB-ToMeasure

altitudeBasedSSB-ToMeasure-r18                ENUMERATED {supported}                               OPTIONAL,

-- Support of events A3H1, A3H2, A4H1, A4H2, A5H1, A5H2

eventAxHy-r18                                 ENUMERATED {supported}                               OPTIONAL,

-- Support of flight path reporting

flightPathReporting-r18                       ENUMERATED {supported}                               OPTIONAL,

-- Support of flight path availability indication via UAI

flightPathAvailabilityIndicationUAI-r18       ENUMERATED {supported}                               OPTIONAL,

-- Support of numberOfTriggeringCells for eventA3, eventA4, and eventA5, and additionally, if the UE supports eventAxHy-r18,

-- support of numberOfTriggeringCells for eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, and eventA5H2

multipleCellsMeasExtension-r18                ENUMERATED {supported}                               OPTIONAL,

-- Support aerial-specific Ns and Pmax list broadcasted by the cell

nr-NS-PmaxListAerial-r18                      ENUMERATED {supported}                               OPTIONAL, -- Editor's Note:

-- Understanding is that a UE that doesn't support any frequency band that requires an aerial specific NS value doesn't need to

-- implement the procedure for aerial specific NS value. Whether indication is needed is still FFS. This is only shown as

-- placeholder. Support of reporting only the measurement report corresponding to the event with the smallest value between the

-- altitude of the UAV and the altitude threshold for which the altitude-related entering condition e.g. A3H1-2 is satisfied, when

-- multiple events of the same type (Hx or AxHy) for the same MO (for AxHy) are triggered simultaneously.

simulMultiTriggerSingleMeasReport-r18         ENUMERATED {supported}                               OPTIONAL,

-- Support of A2X service(s) using PC5 Sidelink and dedicated resource pool for A2X service(s)

sl-A2X-Service-r18                            ENUMERATED {brid, daa, bridAndDAA}                   OPTIONAL,

-- Editor's Note: Granularity of this capability, e.g. per UE/band/FS is still FFS. Depending on the conclusion, this may need to

-- be moved.

...

}

-- TAG-AERIALPARAMETERS-STOP

-- ASN1STOP

#### –	AppLayerMeasParameters

The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements.

AppLayerMeasParameters information element

-- ASN1START

-- TAG-APPLAYERMEASPARAMETERS-START

AppLayerMeasParameters-r17 ::=            SEQUENCE {

qoe-Streaming-MeasReport-r17              ENUMERATED {supported}                                             OPTIONAL,

qoe-MTSI-MeasReport-r17                   ENUMERATED {supported}                                             OPTIONAL,

qoe-VR-MeasReport-r17                     ENUMERATED {supported}                                             OPTIONAL,

ran-VisibleQoE-Streaming-MeasReport-r17   ENUMERATED {supported}                                             OPTIONAL,

ran-VisibleQoE-VR-MeasReport-r17          ENUMERATED {supported}                                             OPTIONAL,

ul-MeasurementReportAppLayer-Seg-r17      ENUMERATED {supported}                                             OPTIONAL,

...,

[[

qoe-IdleInactiveMeasReport-r18            ENUMERATED {supported}                                             OPTIONAL,

qoe-NRDC-MeasReport-r18                   ENUMERATED {supported}                                             OPTIONAL,

qoe-AdditionalMemoryMeasReport-r18        ENUMERATED {kB128, kB256, kB512, kB1024}                           OPTIONAL,

qoe-PriorityBasedDiscarding-r18           ENUMERATED {supported}                                             OPTIONAL,

srb5-r18                                  ENUMERATED {supported}                                             OPTIONAL

]]

}

-- TAG-APPLAYERMEASPARAMETERS-STOP

-- ASN1STOP

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA, NR non-CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-v1540 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540

BandCombinationList-v1550 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1550

BandCombinationList-v1560 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1560

BandCombinationList-v1570 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1570

BandCombinationList-v1580 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1580

BandCombinationList-v1590 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1590

BandCombinationList-v15g0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v15g0

BandCombinationList-v15n0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v15n0

BandCombinationList-v1610 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610

BandCombinationList-v1630 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1630

BandCombinationList-v1640 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1640

BandCombinationList-v1650 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1650

BandCombinationList-v1680 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1680

BandCombinationList-v1690 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1690

BandCombinationList-v16a0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v16a0

BandCombinationList-v1700 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1700

BandCombinationList-v1720 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1720

BandCombinationList-v1730 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1730

BandCombinationList-v1740 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1740

BandCombinationList-v1760 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1760

BandCombinationList-v1770 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1770

BandCombinationList-v1800 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1800

BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16

BandCombinationList-UplinkTxSwitch-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1630

BandCombinationList-UplinkTxSwitch-v1640 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1640

BandCombinationList-UplinkTxSwitch-v1650 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1650

BandCombinationList-UplinkTxSwitch-v1670 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1670

BandCombinationList-UplinkTxSwitch-v1690 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1690

BandCombinationList-UplinkTxSwitch-v16a0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v16a0

BandCombinationList-UplinkTxSwitch-v16e0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v16e0

BandCombinationList-UplinkTxSwitch-v1700 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1700

BandCombinationList-UplinkTxSwitch-v1720 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1720

BandCombinationList-UplinkTxSwitch-v1730 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1730

BandCombinationList-UplinkTxSwitch-v1740 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1740

BandCombinationList-UplinkTxSwitch-v1760 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1760

BandCombinationList-UplinkTxSwitch-v1770 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1770

BandCombinationList-UplinkTxSwitch-v1800 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1800

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-v1540::=            SEQUENCE {

bandList-v1540                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540,

ca-ParametersNR-v1540               CA-ParametersNR-v1540                       OPTIONAL

}

BandCombination-v1550 ::=           SEQUENCE {

ca-ParametersNR-v1550               CA-ParametersNR-v1550

}

BandCombination-v1560::=            SEQUENCE {

ne-DC-BC                                ENUMERATED {supported}                 OPTIONAL,

ca-ParametersNRDC                       CA-ParametersNRDC                      OPTIONAL,

ca-ParametersEUTRA-v1560                CA-ParametersEUTRA-v1560               OPTIONAL,

ca-ParametersNR-v1560                   CA-ParametersNR-v1560                  OPTIONAL

}

BandCombination-v1570 ::=           SEQUENCE {

ca-ParametersEUTRA-v1570            CA-ParametersEUTRA-v1570

}

BandCombination-v1580 ::=           SEQUENCE {

mrdc-Parameters-v1580               MRDC-Parameters-v1580

}

BandCombination-v1590::=            SEQUENCE {

supportedBandwidthCombinationSetIntraENDC  BIT STRING (SIZE (1..32))           OPTIONAL,

mrdc-Parameters-v1590                      MRDC-Parameters-v1590

}

BandCombination-v15g0::=            SEQUENCE {

ca-ParametersNR-v15g0               CA-ParametersNR-v15g0                      OPTIONAL,

ca-ParametersNRDC-v15g0             CA-ParametersNRDC-v15g0                    OPTIONAL,

mrdc-Parameters-v15g0               MRDC-Parameters-v15g0                      OPTIONAL

}

BandCombination-v15n0::=            SEQUENCE {

mrdc-Parameters-v15n0               MRDC-Parameters-v15n0

}

BandCombination-v1610 ::=           SEQUENCE {

bandList-v1610                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610  OPTIONAL,

ca-ParametersNR-v1610               CA-ParametersNR-v1610                  OPTIONAL,

ca-ParametersNRDC-v1610             CA-ParametersNRDC-v1610                OPTIONAL,

powerClass-v1610                    ENUMERATED {pc1dot5}                   OPTIONAL,

powerClassNRPart-r16                ENUMERATED {pc1, pc2, pc3, pc5}        OPTIONAL,

featureSetCombinationDAPS-r16       FeatureSetCombinationId                OPTIONAL,

mrdc-Parameters-v1620               MRDC-Parameters-v1620                  OPTIONAL

}

BandCombination-v1630 ::=                   SEQUENCE {

ca-ParametersNR-v1630                       CA-ParametersNR-v1630                                             OPTIONAL,

ca-ParametersNRDC-v1630                     CA-ParametersNRDC-v1630                                           OPTIONAL,

mrdc-Parameters-v1630                       MRDC-Parameters-v1630                                             OPTIONAL,

supportedTxBandCombListPerBC-Sidelink-r16   BIT STRING (SIZE (1..maxBandComb))                                OPTIONAL,

supportedRxBandCombListPerBC-Sidelink-r16   BIT STRING (SIZE (1..maxBandComb))                                OPTIONAL,

scalingFactorTxSidelink-r16                 SEQUENCE (SIZE (1..maxBandComb)) OF ScalingFactorSidelink-r16     OPTIONAL,

scalingFactorRxSidelink-r16                 SEQUENCE (SIZE (1..maxBandComb)) OF ScalingFactorSidelink-r16     OPTIONAL

}

BandCombination-v1640 ::=                   SEQUENCE {

ca-ParametersNR-v1640                       CA-ParametersNR-v1640                                             OPTIONAL,

ca-ParametersNRDC-v1640                     CA-ParametersNRDC-v1640                                           OPTIONAL

}

BandCombination-v1650 ::=          SEQUENCE {

ca-ParametersNRDC-v1650             CA-ParametersNRDC-v1650                 OPTIONAL

}

BandCombination-v1680 ::=          SEQUENCE {

intrabandConcurrentOperationPowerClass-r16 SEQUENCE (SIZE (1..maxBandComb)) OF IntraBandPowerClass-r16     OPTIONAL

}

BandCombination-v1690 ::=          SEQUENCE {

ca-ParametersNR-v1690              CA-ParametersNR-v1690                 OPTIONAL

}

BandCombination-v16a0 ::=          SEQUENCE {

ca-ParametersNR-v16a0              CA-ParametersNR-v16a0                    OPTIONAL,

ca-ParametersNRDC-v16a0            CA-ParametersNRDC-v16a0                  OPTIONAL

}

BandCombination-v1700 ::=          SEQUENCE {

ca-ParametersNR-v1700              CA-ParametersNR-v1700                    OPTIONAL,

ca-ParametersNRDC-v1700            CA-ParametersNRDC-v1700                  OPTIONAL,

mrdc-Parameters-v1700              MRDC-Parameters-v1700                    OPTIONAL,

bandList-v1710                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1710  OPTIONAL,

supportedBandCombListPerBC-SL-RelayDiscovery-r17      BIT STRING (SIZE (1..maxBandComb))               OPTIONAL,

supportedBandCombListPerBC-SL-NonRelayDiscovery-r17   BIT STRING (SIZE (1..maxBandComb))               OPTIONAL

}

BandCombination-v1720 ::=          SEQUENCE {

ca-ParametersNR-v1720              CA-ParametersNR-v1720                    OPTIONAL,

ca-ParametersNRDC-v1720            CA-ParametersNRDC-v1720                  OPTIONAL

}

BandCombination-v1730 ::=          SEQUENCE {

ca-ParametersNR-v1730              CA-ParametersNR-v1730                    OPTIONAL,

ca-ParametersNRDC-v1730            CA-ParametersNRDC-v1730                  OPTIONAL,

bandList-v1730                     SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1730  OPTIONAL

}

BandCombination-v1740 ::=          SEQUENCE {

ca-ParametersNR-v1740              CA-ParametersNR-v1740                    OPTIONAL

}

BandCombination-v1760 ::=          SEQUENCE {

ca-ParametersNR-v1760              CA-ParametersNR-v1760,

ca-ParametersNRDC-v1760            CA-ParametersNRDC-v1760

}

BandCombination-v1770::=            SEQUENCE {

bandList-v1770                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1770,

mrdc-Parameters-v1770               MRDC-Parameters-v1770                      OPTIONAL,

ca-ParametersNR-v1770               CA-ParametersNR-v1770                      OPTIONAL

}

BandCombination-v1800 ::=          SEQUENCE {

ca-ParametersNR-v1800               CA-ParametersNR-v1800                      OPTIONAL,

ca-ParametersNRDC-v1800             CA-ParametersNRDC-v1800                    OPTIONAL,

supportedBandCombListPerBC-SL-U2U-RelayDiscovery-r18   BIT STRING (SIZE (1..maxBandComb))         OPTIONAL,

bandList-v1810                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1810

}

BandCombination-UplinkTxSwitch-r16 ::= SEQUENCE {

bandCombination-r16                 BandCombination,

bandCombination-v1540               BandCombination-v1540                      OPTIONAL,

bandCombination-v1560               BandCombination-v1560                      OPTIONAL,

bandCombination-v1570               BandCombination-v1570                      OPTIONAL,

bandCombination-v1580               BandCombination-v1580                      OPTIONAL,

bandCombination-v1590               BandCombination-v1590                      OPTIONAL,

bandCombination-v1610               BandCombination-v1610                      OPTIONAL,

supportedBandPairListNR-r16         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r16,

uplinkTxSwitching-OptionSupport-r16 ENUMERATED {switchedUL, dualUL, both}      OPTIONAL,

uplinkTxSwitching-PowerBoosting-r16 ENUMERATED {supported}                     OPTIONAL,

...,

[[

-- R4 16-5 UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching

uplinkTxSwitching-PUSCH-TransCoherence-r16     ENUMERATED {nonCoherent, fullCoherent}   OPTIONAL

]]

}

BandCombination-UplinkTxSwitch-v1630 ::=    SEQUENCE {

bandCombination-v1630                       BandCombination-v1630              OPTIONAL

}

BandCombination-UplinkTxSwitch-v1640 ::=    SEQUENCE {

bandCombination-v1640                       BandCombination-v1640              OPTIONAL

}

BandCombination-UplinkTxSwitch-v1650 ::= SEQUENCE {

bandCombination-v1650               BandCombination-v1650                      OPTIONAL

}

BandCombination-UplinkTxSwitch-v1670 ::= SEQUENCE {

bandCombination-v15g0                    BandCombination-v15g0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1690 ::=  SEQUENCE {

bandCombination-v1690                     BandCombination-v1690                OPTIONAL

}

BandCombination-UplinkTxSwitch-v16a0 ::= SEQUENCE {

bandCombination-v16a0                    BandCombination-v16a0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v16e0 ::= SEQUENCE {

bandCombination-v15n0                    BandCombination-v15n0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1700 ::= SEQUENCE {

bandCombination-v1700                    BandCombination-v1700                      OPTIONAL,

-- R4 16-1/16-2/16-3 Dynamic Tx switching between 2CC/3CC 2Tx-2Tx/1Tx-2Tx switching

supportedBandPairListNR-v1700            SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v1700  OPTIONAL,

-- R4 16-6: UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching

uplinkTxSwitchingBandParametersList-v1700 SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF UplinkTxSwitchingBandParameters-v1700  OPTIONAL

}

BandCombination-UplinkTxSwitch-v1720 ::= SEQUENCE {

bandCombination-v1720                    BandCombination-v1720                 OPTIONAL,

uplinkTxSwitching-OptionSupport2T2T-r17  ENUMERATED {switchedUL, dualUL, both} OPTIONAL

}

BandCombination-UplinkTxSwitch-v1730 ::= SEQUENCE {

bandCombination-v1730                    BandCombination-v1730                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1740 ::= SEQUENCE {

bandCombination-v1740                    BandCombination-v1740                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1760 ::= SEQUENCE {

bandCombination-v1760                    BandCombination-v1760                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1770 ::= SEQUENCE {

bandCombination-v1770                    BandCombination-v1770                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1800 ::=     SEQUENCE {

bandCombination-v1800                                      BandCombination-v1800                                           OPTIONAL,

supportedBandPairListNR-r18                  SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r18   OPTIONAL,

-- R1 49-Y: Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots

uplinkTxSwitchingMinimumSeparationTime-r18   ENUMERATED {n0us, n500us}                                                     OPTIONAL,

uplinkTxSwitchingAdditionalPeriodDualUL-List-r18 SEQUENCE (SIZE (1..maxULTxSwitchingBetweenBandPairs-r18)) OF

UplinkTxSwitchingAdditionalPeriodDualUL-r18                     OPTIONAL

}

ULTxSwitchingBandPair-r16 ::=       SEQUENCE {

bandIndexUL1-r16                    INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r16                    INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingPeriod-r16         ENUMERATED {n35us, n140us, n210us},

uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL

}

ULTxSwitchingBandPair-v1700 ::=     SEQUENCE {

uplinkTxSwitchingPeriod2T2T-r17     ENUMERATED {n35us, n140us, n210us}     OPTIONAL

}

ULTxSwitchingBandPair-r18 ::=                             SEQUENCE {

bandCombination-v1800                                      BandCombination-v1800                                        OPTIONAL,

bandIndexUL1-r18                                           INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r18                                           INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingOptionForBandPair-r18                     ENUMERATED {switchedUL, dualUL, both},

-- R1 49-X: Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands

uplinkTxSwitchingPeriodForBandPair-r18                     SEQUENCE {

switchingPeriodFor2T-r18                                 ENUMERATED {n35us, n140us, n210us}                       OPTIONAL,

switchingPeriodFor1T-r18                                 ENUMERATED {n35us, n140us, n210us}

},

uplinkTxSwitching-DL-Interruption-r18                      BIT STRING (SIZE(1..maxSimultaneousBands))                   OPTIONAL,

uplinkTxSwitchingPeriodUnaffectedBandDualUL-List-r18       SEQUENCE (SIZE (1..maxSimultaneousBands-2-r18)) OF

SwitchingPeriodUnaffectedBandDualUL-r18            OPTIONAL

}

UplinkTxSwitchingBandParameters-v1700 ::=                 SEQUENCE {

bandIndex-r17                                              INTEGER(1..maxSimultaneousBands),

uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17             ENUMERATED {nonCoherent, fullCoherent}                       OPTIONAL

}

UplinkTxSwitchingAdditionalPeriodDualUL-r18::=            SEQUENCE {

uplinkTxSwitchingBetweenBandPairs-r18                     SEQUENCE {

bandPairIndex1-r18                                        INTEGER(1.. maxULTxSwitchingBandPairs),

anotherBandPairOrBand-r18                                 CHOICE {

bandPairIndex2-r18                                        INTEGER(1.. maxULTxSwitchingBandPairs),

bandIndex-r18                                             INTEGER(1..maxSimultaneousBands)

}

},

switchingAdditionalPeriodDualUL-r18                   ENUMERATED {n35us, n140us, n210us}

}

SwitchingPeriodUnaffectedBandDualUL-r18::=                SEQUENCE {

bandIndexUnaffected-r18                                   INTEGER(1..maxSimultaneousBands),

periodUnaffectedBandDualUL-r18                            CHOICE {

maintainedUL-Trans-r18                                    NULL,

periodOnULBands-r18                                       ENUMERATED {n35us, n140us, n210us}

}

}

CombinationCarrierType-r18 ::=                  SEQUENCE {

schedulingCellCarrierType-r18                   ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1, fr2-2},

scheduledCellCarrierType-r18                    ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1, fr2-2}

}

BandParameters ::=                      CHOICE {

eutra                               SEQUENCE {

bandEUTRA                           FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL,

ca-BandwidthClassUL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL

},

nr                                  SEQUENCE {

bandNR                              FreqBandIndicatorNR,

ca-BandwidthClassDL-NR              CA-BandwidthClassNR                    OPTIONAL,

ca-BandwidthClassUL-NR              CA-BandwidthClassNR                    OPTIONAL

}

}

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1610 ::=         SEQUENCE {

srs-TxSwitch-v1610               SEQUENCE {

supportedSRS-TxPortSwitch-v1610  ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL

}

BandParameters-v1710 ::=         SEQUENCE {

-- R1 23-8-3	SRS Antenna switching for >4Rx

srs-AntennaSwitchingBeyond4RX-r17                     SEQUENCE {

-- 1. Support of SRS antenna switching xTyR with y>4

supportedSRS-TxPortSwitchBeyond4Rx-r17                BIT STRING (SIZE (11)),

-- 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL

entryNumberAffectBeyond4Rx-r17                        INTEGER (1..32)      OPTIONAL,

-- 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL

entryNumberSwitchBeyond4Rx-r17                        INTEGER (1..32)      OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1730 ::= SEQUENCE {

-- R1 39-3-2	Affected bands for inter-band CA during SRS carrier switching

srs-SwitchingAffectedBandsListNR-r17    SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingAffectedBandsNR-r17

}

BandParameters-v1770 ::=         SEQUENCE {

ca-BandwidthClassDL-NR-r17       CA-BandwidthClassNR-r17                    OPTIONAL,

ca-BandwidthClassUL-NR-r17       CA-BandwidthClassNR-r17                    OPTIONAL

}

BandParameters-v1810 ::=         SEQUENCE {

-- R1 40-5-4: SRS 8 Tx ports—antenna switching

srs-AntennaSwitching8T8R-r18                    SEQUENCE {

8T8R-AntennaSwitch-r18                         ENUMERATED {noTdm, tdmAndNoTdm},

downGradeConfig-r18                            BIT STRING (SIZE (11)),

entryNumberAffect-r18                          INTEGER (1..32),

entryNumberSwtich-r18                          INTEGER (1..32)

}                                                                                         OPTIONAL

}

ScalingFactorSidelink-r16 ::=       ENUMERATED {f0p4, f0p75, f0p8, f1}

IntraBandPowerClass-r16 ::=         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

SRS-SwitchingAffectedBandsNR-r17 ::= BIT STRING (SIZE (1..maxSimultaneousBands))

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

| BandCombination field descriptions |
| --- |
| BandCombinationList-v1540, BandCombinationList-v1550, BandCombinationList-v1560, BandCombinationList-v1570, BandCombinationList-v1580, BandCombinationList-v1590, BandCombinationList-v15g0, BandCombinationList-v15n0, BandCombinationList-v1610, BandCombinationList-v1630, BandCombinationList-v1640, BandCombinationList-v1650, BandCombinationList-v1680, BandCombinationList-v1690, BandCombinationList-v16a0, BandCombinationList-v1700, BandCombinationList-v1720, BandCombinationList-v1730, BandCombinationList-v1760, BandCombinationList-v1770, BandCombinationList-v1800 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix). If the field is included in supportedBandCombinationListNEDC-Only-v1610, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList of supportedBandCombinationListNEDC-Only (without suffix) field. If the field is included in supportedBandCombinationListNEDC-Only-v15a0, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix) of supportedBandCombinationListNEDC-Only (without suffix) field. |
| BandCombinationList-UplinkTxSwitch-r16, BandCombinationList-UplinkTxSwitch-v1630, BandCombinationList-UplinkTxSwitch-v1640, BandCombinationList-UplinkTxSwitch-v1650, BandCombinationList-UplinkTxSwitch-v1690, BandCombinationList-UplinkTxSwitch-v16a0, BandCombinationList-UplinkTxSwitch-v16e0, BandCombinationList-UplinkTxSwitch-v1700, BandCombinationList-UplinkTxSwitch-v1720, BandCombinationList-UplinkTxSwitch-v1730, BandCombinationList-UplinkTxSwitch-v1760, BandCombinationList-UplinkTxSwitch-v1770, BandCombinationList-UplinkTxSwitch-v1800 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList-UplinkTxSwitch-r16. For the field of supportedBandCombinationList-UplinkTxSwitch-v1700, if the UE does not support 2Tx-2Tx switching for a given band combination, the field of supportedBandPairListNR-v1700 in the corresponding entry is absent. |
| ca-ParametersNRDC If the field is included for a band combination in the NR capability container, the field indicates support of NR-DC. Otherwise, the field is absent. |
| featureSetCombinationDAPS If this field is present for a band combination, it reports the feature set combination supported for the band combination when any DAPS bearer is configured. |
| ne-DC-BC If the field is included for a band combination in the MR-DC capability container, the field indicates support of NE-DC. Otherwise, the field is absent. |
| supportedBandPairListNR-r16, supportedBandPairListNR-v1700 Indicates a list of band pair supporting UL Tx switching as defined in TS 38.101-1 [15] for a given band combination. A UE supporting 2Tx-2Tx switching should include both of supportedBandPairListNR-r16 and supportedBandPairListNR-v1700. And the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r16. If the UE does not support 2Tx-2Tx switching for a given band pair, the field of uplinkTxSwitchingPeriod2T2T in the corresponding entry is absent. |
| supportedBandPairListNR-r18 Indicates a list of band pair supporting UL Tx switching up to 4 bands as defined in TS 38.101-1 [15] for a given band combination. The UE shall include all the possible band pairs. For a band pair only supporting 1Tx-1Tx switching, the UE should include switchingPeriodFor1T in ULTxSwitchingBandPair-r18. For a band pair supporting 1Tx-2Tx switching, the UE always supports 1Tx-1Tx switching, and the UE should include switchingPeriodFor1T in ULTxSwitchingBandPair-r18. For a band pair supporting 2Tx-2Tx switching, the UE always supports 1Tx-2Tx switching and 1Tx-1Tx switching, the UE should include switchingPeriodFor2T as well as switchingPeriodFor1T in ULTxSwitchingBandPair-r18. |
| srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on |
| srs-SwitchingTimesListEUTRA Indicates, for a particular pair of E-UTRA bands, the RF retuning time when switching between an E-UTRA carrier corresponding to this band entry and another (PUSCH-less) E-UTRA carrier corresponding to the band entry in the order indicated below: - For the first E-UTRA band, the UE shall include the same number of entries for E-UTRA bands as in bandList, i.e. first entry corresponds to first E-UTRA band in bandList and so on, - For the second E-UTRA band, the UE shall include one entry less, i.e. first entry corresponds to the second E-UTRA band in bandList and so on - And so on |
| srs-TxSwitch Indicates supported SRS antenna switch capability for the associated band. If the UE indicates support of SRS-SwitchingTimeNR, the UE is allowed to set this field for a band with associated FeatureSetUplinkId set to 0 for SRS carrier switching. |  |
| uplinkTxSwitchingBandParametersList-v1700 Indicates a list of per band per band combination capabilities for UL Tx switching. |  |

#### –	BandCombinationListSidelinkEUTRA-NR

The IE BandCombinationListSidelinkEUTRA-NR contains a list of V2X sidelink and NR sidelink band combinations.

BandCombinationListSidelinkEUTRA-NR information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-START

BandCombinationListSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-r16

BandCombinationListSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1630

BandCombinationListSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1710

BandCombinationParametersSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-r16

BandCombinationParametersSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1630

BandCombinationParametersSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1710

BandParametersSidelinkEUTRA-NR-r16 ::= CHOICE {

eutra                                  SEQUENCE {

bandParametersSidelinkEUTRA1-r16       OCTET STRING                         OPTIONAL,

bandParametersSidelinkEUTRA2-r16       OCTET STRING                         OPTIONAL

},

nr                                     SEQUENCE {

bandParametersSidelinkNR-r16           BandParametersSidelink-r16

}

}

BandParametersSidelinkEUTRA-NR-v1630 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

tx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

rx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

sl-CrossCarrierScheduling-r16            ENUMERATED {supported}                          OPTIONAL

}

}

BandParametersSidelinkEUTRA-NR-v1710 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

--32-4

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--32-2a:  Receiving NR sidelink of PSFCH

rx-sidelinkPSFCH-r17                     ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL,

--32-5b-1

tx-IUC-Scheme2-Mode2Sidelink-r17         ENUMERATED {n4, n8, n16}                          OPTIONAL

}

}

BandParametersSidelink-r16 ::= SEQUENCE {

freqBandSidelink-r16           FreqBandIndicatorNR

}

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-STOP

-- ASN1STOP

BandParametersSidelinkEUTRA-NR field descriptions

bandParametersSidelinkEUTRA1, bandParametersSidelinkEUTRA2 This field includes the V2X-BandParameters-r14 and V2X-BandParameters-v1530 IE as specified in 36.331 [10]. It is used for reporting the per-band capability for V2X sidelink communication.

#### –	BandCombinationListSL-Discovery

The IE BandCombinationListSL-Discovery contains a list of NR Sidelink discovery band combinations.

BandCombinationListSidelinkSL-Discovery information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSLDISCOVERY-START

BandCombinationListSL-Discovery-r17 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkDiscovery-r17

BandParametersSidelinkDiscovery-r17 ::= SEQUENCE {

sl-CrossCarrierScheduling-r17            ENUMERATED {supported}                            OPTIONAL,

--R1 32-4: Transmitting NR sidelink mode 2 with partial sensing

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--R1 32-5a-1: Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL

}

-- TAG-BANDCOMBINATIONLISTSLDISCOVERY-STOP

-- ASN1STOP

#### –	CA-BandwidthClassEUTRA

The IE CA-BandwidthClassEUTRA indicates the E-UTRA CA bandwidth class as defined in TS 36.101 [22], table 5.6A-1.

CA-BandwidthClassEUTRA information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=          ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

#### –	CA-BandwidthClassNR

The IE CA-BandwidthClassNR indicates the NR CA bandwidth class as defined in TS 38.101-1 [15], table 5.3A.5-1 and TS 38.101-2 [39], table 5.3A.4-1.

CA-BandwidthClassNR information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

-- R4 17-6: new CA BW Classes R2-R12

-- R4 17-7: new CA BW Classes V, W

CA-BandwidthClassNR ::=             ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...,r2-v1730, r3-v1730, r4-v1730, r5-v1730, r6-v1730, r7-v1730, r8-v1730, r9-v1730, r10-v1730, r11-v1730, r12-v1730,v-v1770, w-v1770 }

CA-BandwidthClassNR-r17 ::=         ENUMERATED {r, s, t, u, ...}

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

#### –	CA-ParametersEUTRA

The IE CA-ParametersEUTRA contains the E-UTRA part of band combination parameters for a given MR-DC band combination.

NOTE:	If additional E-UTRA band combination parameters are defined in TS 36.331 [10], which are supported for MR-DC, they will be defined here as well.

CA-ParametersEUTRA information element

-- ASN1START

-- TAG-CA-PARAMETERSEUTRA-START

CA-ParametersEUTRA ::=                          SEQUENCE {

multipleTimingAdvance                           ENUMERATED {supported}                          OPTIONAL,

simultaneousRx-Tx                               ENUMERATED {supported}                          OPTIONAL,

supportedNAICS-2CRS-AP                          BIT STRING (SIZE (1..8))                        OPTIONAL,

additionalRx-Tx-PerformanceReq                  ENUMERATED {supported}                          OPTIONAL,

ue-CA-PowerClass-N                              ENUMERATED {class2}                             OPTIONAL,

supportedBandwidthCombinationSetEUTRA-v1530     BIT STRING (SIZE (1..32))                       OPTIONAL,

...

}

CA-ParametersEUTRA-v1560 ::=                    SEQUENCE {

fd-MIMO-TotalWeightedLayers                     INTEGER (2..128)                                OPTIONAL

}

CA-ParametersEUTRA-v1570 ::=                    SEQUENCE {

dl-1024QAM-TotalWeightedLayers                  INTEGER (0..10)                                 OPTIONAL

}

-- TAG-CA-PARAMETERSEUTRA-STOP

-- ASN1STOP

#### –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)     OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)    OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

CA-ParametersNR-v1550 ::=           SEQUENCE {

dummy                               ENUMERATED {supported}                      OPTIONAL

}

CA-ParametersNR-v1560 ::=           SEQUENCE {

diffNumerologyWithinPUCCH-GroupLargerSCS      ENUMERATED {supported}            OPTIONAL

}

CA-ParametersNR-v15g0 ::=           SEQUENCE {

simultaneousRxTxInterBandCAPerBandPair        SimultaneousRxTxPerBandPair       OPTIONAL,

simultaneousRxTxSULPerBandPair                SimultaneousRxTxPerBandPair       OPTIONAL

}

CA-ParametersNR-v1610 ::=           SEQUENCE {

-- R1 9-3: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA

parallelTxMsgA-SRS-PUCCH-PUSCH-r16                ENUMERATED {supported}        OPTIONAL,

-- R1 9-4: MsgA operation in a band combination including SUL

msgA-SUL-r16                                      ENUMERATED {supported}        OPTIONAL,

-- R1 10-9c: Joint search space group switching across multiple cells

jointSearchSpaceSwitchAcrossCells-r16             ENUMERATED {supported}        OPTIONAL,

-- R1 14-5: Half-duplex UE behaviour in TDD CA for same SCS

half-DuplexTDD-CA-SameSCS-r16                     ENUMERATED {supported}        OPTIONAL,

-- R1 18-4: SCell dormancy within active time

scellDormancyWithinActiveTime-r16                 ENUMERATED {supported}        OPTIONAL,

-- R1 18-4a: SCell dormancy outside active time

scellDormancyOutsideActiveTime-r16                ENUMERATED {supported}        OPTIONAL,

-- R1 18-6: Cross-carrier A-CSI RS triggering with different SCS

crossCarrierA-CSI-trigDiffSCS-r16                 ENUMERATED {higherA-CSI-SCS,lowerA-CSI-SCS,both}   OPTIONAL,

-- R1 18-6a: Default QCL assumption for cross-carrier A-CSI-RS triggering

defaultQCL-CrossCarrierA-CSI-Trig-r16             ENUMERATED {diffOnly, both}   OPTIONAL,

-- R1 18-7: CA with non-aligned frame boundaries for inter-band CA

interCA-NonAlignedFrame-r16                       ENUMERATED {supported}        OPTIONAL,

simul-SRS-Trans-BC-r16                            ENUMERATED {n2}               OPTIONAL,

interFreqDAPS-r16                                 SEQUENCE {

interFreqAsyncDAPS-r16                            ENUMERATED {supported}    OPTIONAL,

interFreqDiffSCS-DAPS-r16                         ENUMERATED {supported}    OPTIONAL,

interFreqMultiUL-TransmissionDAPS-r16             ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode1-r16     ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode2-r16     ENUMERATED {supported}    OPTIONAL,

interFreqDynamicPowerSharingDAPS-r16              ENUMERATED {short, long}  OPTIONAL,

interFreqUL-TransCancellationDAPS-r16             ENUMERATED {supported}    OPTIONAL

}                                                                               OPTIONAL,

codebookParametersPerBC-r16                       CodebookParameters-v1610      OPTIONAL,

-- R1 16-2a-10 Value of R for BD/CCE

blindDetectFactor-r16                             INTEGER (1..2)                OPTIONAL,

-- R1 11-2a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r16                            SEQUENCE {

maxNumberOfMonitoringCC-r16                       INTEGER (2..16),

supportedSpanArrangement-r16                      ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                               OPTIONAL,

-- R1 11-2c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers

pdcch-BlindDetectionCA-Mixed-r16                  SEQUENCE {

pdcch-BlindDetectionCA1-r16                       INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                       INTEGER (1..15),

supportedSpanArrangement-r16                      ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                               OPTIONAL,

-- R1 11-2d: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for

-- SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-BlindDetectionMCG-UE-r16                    INTEGER (1..14)               OPTIONAL,

pdcch-BlindDetectionSCG-UE-r16                    INTEGER (1..14)               OPTIONAL,

-- R1 11-2e: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and

-- Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMCG-UE-Mixed-r16              SEQUENCE {

pdcch-BlindDetectionMCG-UE1-r16                   INTEGER (0..15),

pdcch-BlindDetectionMCG-UE2-r16                   INTEGER (0..15)

}                                                                               OPTIONAL,

pdcch-BlindDetectionSCG-UE-Mixed-r16              SEQUENCE {

pdcch-BlindDetectionSCG-UE1-r16                   INTEGER (0..15),

pdcch-BlindDetectionSCG-UE2-r16                   INTEGER (0..15)

}                                                                               OPTIONAL,

-- R1 18-5 cross-carrier scheduling with different SCS in DL CA

crossCarrierSchedulingDL-DiffSCS-r16              ENUMERATED {low-to-high, high-to-low, both} OPTIONAL,

-- R1 18-5a Default QCL assumption for cross-carrier scheduling

crossCarrierSchedulingDefaultQCL-r16              ENUMERATED {diff-only, both}  OPTIONAL,

-- R1 18-5b cross-carrier scheduling with different SCS in UL CA

crossCarrierSchedulingUL-DiffSCS-r16              ENUMERATED {low-to-high, high-to-low, both} OPTIONAL,

-- R1 13.19a Simultaneous positioning SRS and MIMO SRS transmission for a given BC

simul-SRS-MIMO-Trans-BC-r16                       ENUMERATED {n2}               OPTIONAL,

-- R1 16-3a, 16-3a-1, 16-3b, 16-3b-1: New Individual Codebook

codebookParametersAdditionPerBC-r16               CodebookParametersAdditionPerBC-r16         OPTIONAL,

-- R1 16-8: Mixed codebook

codebookComboParametersAdditionPerBC-r16          CodebookComboParametersAdditionPerBC-r16    OPTIONAL

}

CA-ParametersNR-v1630 ::= SEQUENCE {

-- R1 22-5b: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for inter-band UL CA

-- R1 22-5d: Simultaneous transmission of SRS for antenna switching for inter-band UL CA

simulTX-SRS-AntSwitchingInterBandUL-CA-r16        SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R4 8-5: supported beam management type for inter-band CA

beamManagementType-r16                            ENUMERATED {ibm, dummy}                       OPTIONAL,

-- R4 7-3a: UL frequency separation class with aggregate BW and Gap BW

intraBandFreqSeparationUL-AggBW-GapBW-r16         ENUMERATED {classI, classII, classIII}      OPTIONAL,

-- RAN 89: Case B in case of Inter-band CA with non-aligned frame boundaries

interCA-NonAlignedFrame-B-r16                     ENUMERATED {supported}                      OPTIONAL

}

CA-ParametersNR-v1640 ::= SEQUENCE {

-- R4 7-5: Support of reporting UL Tx DC locations for uplink intra-band CA.

uplinkTxDC-TwoCarrierReport-r16                               ENUMERATED {supported}          OPTIONAL,

-- RAN 22-6: Support of up to 3 different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA

-- where UE is not configured with two NR PUCCH groups

maxUpTo3Diff-NumerologiesConfigSinglePUCCH-grp-r16            PUCCH-Grp-CarrierTypes-r16      OPTIONAL,

-- RAN 22-6a: Support of up to 4 different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA

-- where UE is not configured with two NR PUCCH groups

maxUpTo4Diff-NumerologiesConfigSinglePUCCH-grp-r16            PUCCH-Grp-CarrierTypes-r16      OPTIONAL,

-- RAN 22-7: Support two PUCCH groups for NR-CA with 3 or more bands with at least two carrier types

twoPUCCH-Grp-ConfigurationsList-r16 SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r16)) OF TwoPUCCH-Grp-Configurations-r16 OPTIONAL,

-- R1 22-7a: Different numerology across NR PUCCH groups

diffNumerologyAcrossPUCCH-Group-CarrierTypes-r16              ENUMERATED {supported}          OPTIONAL,

-- R1 22-7b: Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of smaller SCS

diffNumerologyWithinPUCCH-GroupSmallerSCS-CarrierTypes-r16    ENUMERATED {supported}          OPTIONAL,

-- R1 22-7c: Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of larger SCS

diffNumerologyWithinPUCCH-GroupLargerSCS-CarrierTypes-r16     ENUMERATED {supported}          OPTIONAL,

-- R1 11-2f: add the replicated FGs of 11-2a/c with restriction for non-aligned span case

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-NonAlignedSpan-r16                         INTEGER (2..16)                 OPTIONAL,

-- R1 11-2g: add the replicated FGs of 11-2a/c with restriction for non-aligned span case

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16               SEQUENCE {

pdcch-BlindDetectionCA1-r16                                   INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                                   INTEGER (1..15)

}                                                                                             OPTIONAL

}

CA-ParametersNR-v1690 ::= SEQUENCE {

csi-ReportingCrossPUCCH-Grp-r16          SEQUENCE {

computationTimeForA-CSI-r16              ENUMERATED {sameAsNoCross, relaxed},

additionalSymbols-r16                    SEQUENCE {

scs-15kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-30kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-60kHz-additionalSymbols-r16          ENUMERATED {s14, s28, s56}       OPTIONAL,

scs-120kHz-additionalSymbols-r16         ENUMERATED {s14, s28, s56}       OPTIONAL

}                                                                             OPTIONAL,

sp-CSI-ReportingOnPUCCH-r16              ENUMERATED {supported}               OPTIONAL,

sp-CSI-ReportingOnPUSCH-r16              ENUMERATED {supported}               OPTIONAL,

carrierTypePairList-r16                  SEQUENCE (SIZE (1..maxCarrierTypePairList-r16)) OF CarrierTypePair-r16

}                                                                                 OPTIONAL

}

CA-ParametersNR-v16a0 ::= SEQUENCE {

pdcch-BlindDetectionMixedList-r16    SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF PDCCH-BlindDetectionMixedList-r16

}

CA-ParametersNR-v1700 ::= SEQUENCE {

-- R1 23-9-1: Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) per band combination information

codebookParametersfetype2PerBC-r17               CodebookParametersfetype2PerBC-r17           OPTIONAL,

-- R4 18-4: Support of enhanced Demodulation requirements for CA in HST SFN FR1

demodulationEnhancementCA-r17                    ENUMERATED {supported}                       OPTIONAL,

-- R4 20-1: Maximum uplink duty cycle for NR inter-band CA power class 2

maxUplinkDutyCycle-interBandCA-PC2-r17           ENUMERATED {n50, n60, n70, n80, n90, n100}   OPTIONAL,

-- R4 20-2: Maximum uplink duty cycle for NR SUL combination power class 2

maxUplinkDutyCycle-SULcombination-PC2-r17        ENUMERATED {n50, n60, n70, n80, n90, n100}   OPTIONAL,

beamManagementType-CBM-r17                       ENUMERATED {supported}                       OPTIONAL,

-- R1 25-18: Parallel PUCCH and PUSCH transmission across CCs in inter-band CA

parallelTxPUCCH-PUSCH-r17                        ENUMERATED {supported}      OPTIONAL,

-- R1 23-9-5	Active CSI-RS resources and ports for mixed codebook types in any slot per band combination

codebookComboParameterMixedTypePerBC-r17         CodebookComboParameterMixedTypePerBC-r17     OPTIONAL,

-- R1 23-7-1	Basic Features of CSI Enhancement for Multi-TRP

mTRP-CSI-EnhancementPerBC-r17                    SEQUENCE {

maxNumNZP-CSI-RS-r17                             INTEGER (2..8),

cSI-Report-mode-r17                              ENUMERATED {mode1, mode2, both},

supportedComboAcrossCCs-r17                      SEQUENCE (SIZE (1..16)) OF CSI-MultiTRP-SupportedCombinations-r17,

codebookMode-NCJT-r17	ENUMERATED{mode1,mode1And2}

}                                                                                             OPTIONAL,

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

codebookComboParameterMultiTRP-PerBC-r17         CodebookComboParameterMultiTRP-PerBC-r17     OPTIONAL,

-- R1 24-8b: 32 DL HARQ processes for FR 2-2 - maximum number of component carriers

maxCC-32-DL-HARQ-ProcessFR2-2-r17                ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32} OPTIONAL,

-- R1 24-9b: 32 UL HARQ processes for FR 2-2 - maximum number of component carriers

maxCC-32-UL-HARQ-ProcessFR2-2-r17                ENUMERATED {n1, n2, n3, n4, n5, n8, n16, n32}  OPTIONAL,

-- R1 34-2: Cross-carrier scheduling from SCell to PCell/PSCell (Type B)

crossCarrierSchedulingSCell-SpCellTypeB-r17      CrossCarrierSchedulingSCell-SpCell-r17       OPTIONAL,

-- R1 34-1: Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A)

crossCarrierSchedulingSCell-SpCellTypeA-r17      CrossCarrierSchedulingSCell-SpCell-r17       OPTIONAL,

-- R1 34-1a: DCI formats on PCell/PSCell USS set(s) support

dci-FormatsPCellPSCellUSS-Sets-r17               ENUMERATED {supported}                       OPTIONAL,

-- R1 34-3: Disabling scaling factor alpha when sSCell is deactivated

disablingScalingFactorDeactSCell-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 34-4: Disabling scaling factor alpha when sSCell is deactivated

disablingScalingFactorDormantSCell-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 34-5: Non-aligned frame boundaries between PCell/PSCell and sSCell

non-AlignedFrameBoundaries-r17 SEQUENCE {

scs15kHz-15kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs15kHz-30kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs15kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs30kHz-30kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs30kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs60kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL

}                                                                                             OPTIONAL

}

CA-ParametersNR-v1720 ::= SEQUENCE {

-- R1 39-1: Parallel SRS and PUCCH/PUSCH transmission across CCs in intra-band non-contiguous CA

parallelTxSRS-PUCCH-PUSCH-intraBand-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 39-2: Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA

parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 25-9: Semi-static PUCCH cell switching for a single PUCCH group only

semiStaticPUCCH-CellSwitchSingleGroup-r17        SEQUENCE {

pucch-Group-r17                                ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                           PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-9a: Semi-static PUCCH cell switching for two PUCCH groups

semiStaticPUCCH-CellSwitchTwoGroups-r17    SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17 OPTIONAL,

-- R1 25-10: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots for a single

-- PUCCH group only

dynamicPUCCH-CellSwitchSameLengthSingleGroup-r17 SEQUENCE {

pucch-Group-r17                                  ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                       PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-10a: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots

-- for a single PUCCH group only

dynamicPUCCH-CellSwitchDiffLengthSingleGroup-r17 SEQUENCE {

pucch-Group-r17                                  ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                           PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-10b: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots for two PUCCH

-- groups

dynamicPUCCH-CellSwitchSameLengthTwoGroups-r17   SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17

OPTIONAL,

-- R1 25-10c: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots for two

-- PUCCH groups

dynamicPUCCH-CellSwitchDiffLengthTwoGroups-r17   SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17

OPTIONAL,

-- R1 33-2a: ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based

-- feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticast-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 33-2d: PTP retransmission for multicast dynamic scheduling

ptp-Retx-Multicast-r17                           ENUMERATED {supported}                       OPTIONAL,

-- R1 33-4: NACK-only based HARQ-ACK feedback for RRC-based enabling/disabling multicast with ACK/NACK transforming

nack-OnlyFeedbackForMulticast-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 33-4a: NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission

nack-OnlyFeedbackSpecificResourceForMulticast-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 33-5-1a: ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback

-- for SPS group-common PDSCH for multicast

ack-NACK-FeedbackForSPS-Multicast-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 33-5-1d: PTP retransmission for SPS group-common PDSCH for multicast

ptp-Retx-SPS-Multicast-r17                       ENUMERATED {supported}                       OPTIONAL,

-- R4 26-1: Higher Power Limit CA DC

higherPowerLimit-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 39-4: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA

parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17     ENUMERATED {supported}                       OPTIONAL,

-- R1 24-11a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when

-- configured with DL CA with Rel-17 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r17                           INTEGER (4..16)                              OPTIONAL,

-- R1 24-11f: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs for MCG and for SCG

-- when configured for NR-DC operation with Rel-17 PDCCH monitoring capability on all the serving cells

pdcch-BlindDetectionMCG-SCG-List-r17             SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMCG-SCG-r17

OPTIONAL,

-- R1 24-11c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 15 PDCCH monitoring capabilities on

-- different Carriers

-- R1 24-11g: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and

-- Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList1-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed-r17

OPTIONAL,

-- R1 24-11d: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 16 PDCCH monitoring capabilities on

-- different Carriers

-- R1 24-11h: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and

-- Rel. 16 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList2-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed-r17

OPTIONAL,

-- R1 24-11e: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17, Rel. 16 and Rel. 15 PDCCH monitoring

-- capabilities on different carriers

-- R1 24-11i: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17,

-- Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList3-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed1-r17

OPTIONAL

}

CA-ParametersNR-v1730 ::= SEQUENCE {

-- R1 30-4a: DM-RS bundling for PUSCH repetition type A (per BC)

dmrs-BundlingPUSCH-RepTypeAPerBC-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4b: DM-RS bundling for PUSCH repetition type B(per BC)

dmrs-BundlingPUSCH-RepTypeBPerBC-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4c: DM-RS bundling for TB processing over multi-slot PUSCH(per BC)

dmrs-BundlingPUSCH-multiSlotPerBC-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4d: DMRS bundling for PUCCH repetitions(per BC)

dmrs-BundlingPUCCH-RepPerBC-r17                        ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4g: Restart DM-RS bundling (per BC)

dmrs-BundlingRestartPerBC-r17                          ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4h: DM-RS bundling for non-back-to-back transmission (per BC)

dmrs-BundlingNonBackToBackTX-PerBC-r17                 ENUMERATED {supported}                         OPTIONAL,

-- R1 39-3-1: Stay on the target CC for SRS carrier switching

stayOnTargetCC-SRS-CarrierSwitch-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-3a: FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

fdm-CodebookForMux-UnicastMulticastHARQ-ACK-r17        ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-3b: Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

mode2-TDM-CodebookForMux-UnicastMulticastHARQ-ACK-r17  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-4: Mode 1 for type1 codebook generation

mode1-ForType1-CodebookGeneration-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-5-1j: NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission

-- for SPS group-commmon PDSCH for multicast

nack-OnlyFeedbackSpecificResourceForSPS-Multicast-r17  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-2: Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast

multiPUCCH-ConfigForMulticast-r17                      ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-3: PUCCH resource configuration for multicast feedback for SPS GC-PDSCH

pucch-ConfigForSPS-Multicast-r17                       ENUMERATED {supported}                         OPTIONAL,

-- The following parameter is associated with R1 33-2a, R1 33-3-3a, and R1 33-3-3b, and is not a RAN1 FG.

maxNumberG-RNTI-HARQ-ACK-Codebook-r17                  INTEGER (1..4)                                 OPTIONAL,

-- R1 33-3-5: Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook

-- type

mux-HARQ-ACK-UnicastMulticast-r17                      ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1740 ::= SEQUENCE {

-- R1 33-5-1f: NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback

-- for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-Multicast-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-1: PUCCH resource configuration for multicast feedback for dynamically scheduled multicast

singlePUCCH-ConfigForMulticast-r17                     ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1760 ::= SEQUENCE {

prioSCellPRACH-OverSP-PeriodicSRS-Support-r17          ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1770 ::= SEQUENCE {

parallelTxPUCCH-PUSCH-SamePriority-r17                 ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1800 ::= SEQUENCE {

codebookParametersetype2DopplerCSI-PerBC-r18  CodebookParametersetype2DopplerCSI-r18                  OPTIONAL,

codebookParametersfetype2DopplerCSI-PerBC-r18 CodebookParametersfetype2DopplerCSI-r18                 OPTIONAL,

codebookParametersetype2CJT-PerBC-r18         CodebookParametersetype2CJT-r18                         OPTIONAL,

codebookParametersfetype2CJT-PerBC-r18        CodebookParametersfetype2CJT-r18                        OPTIONAL,

codebookComboParametersCJT-PerBC-r18          CodebookComboParametersCJT-r18                          OPTIONAL,

-- R1 40-2-8: Maximum number of TAGs across all CCs

maxNumberTAG-AcrossCC-r18                     INTEGER (2..4)                                          OPTIONAL,

-- R1 40-3-1-24: Timeline for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI

timelineRelax-CJT-CSI-CA-r18                  ENUMERATED {n0,n2}                                      OPTIONAL,

-- R1 49-1: Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling

-- cell and cells in the set

multiCell-PDSCH-DCI-1-3-SameSCS-r18           SEQUENCE {

coScheduledCelSCS-r18                         ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1,

fr2-2, licensed-fdd-tdd-fr1},

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

harqFeedbackType-r18                          ENUMERATED {type1, type2, type1And2},

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both}

}                                                                                                     OPTIONAL,

-- R1 49-1b: Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different

-- SCS/carrier type between scheduling cell and cells in the set

multiCell-PDSCH-DCI-1-3-DiffSCS-r18           SEQUENCE {

coScheduledCellSCS-r18                        ENUMERATED {lowScheduling-highScheduled, highScheduling-lowScheduled, both},

combinationCarrierType-r18                    SEQUENCE (SIZE(1..maxSchedulingBandCombination-r18)) OF

CombinationCarrierType-r18,

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

harqFeedbackType-r18                          ENUMERATED {type1, type2, type1And2},

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both}

}                                                                                                     OPTIONAL,

-- R1 49-2: Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell

-- and cells in the set

multiCell-PUSCH-DCI-0-3-SameSCS-r18           SEQUENCE {

coScheduledCelSCS-r18                         ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1,

fr2-2, licensed-fdd-tdd},

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both}

}                                                                                                     OPTIONAL,

-- R1 49-2b: Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with

-- different SCS/carrier type between scheduling cell and cells in the set

multiCell-PUSCH-DCI-0-3-DiffSCS-r18           SEQUENCE {

coScheduledCellSCS-r18                        ENUMERATED {lowScheduling-highScheduled, highScheduling-lowScheduled, both},

combinationCarrierType-r18                    SEQUENCE (SIZE(1..maxSchedulingBandCombination-r18)) OF

CombinationCarrierType-r18,

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both}

}                                                                                                      OPTIONAL,

-- R1 55-6a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r18                        SEQUENCE {

maxNumberOfMonitoringCC-r18                   INTEGER (2..16),

supportedSpanArrangement-r18                  ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                                                     OPTIONAL,

-- R1 55-6c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers

pdcch-BlindDetectionCA-Mixed-r18              SEQUENCE {

blindDetectionCA-Mixed-r18                    SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetection-r17)) OF

PDCCH-BlindDetectionCA-Mixed-r18,

supportedSpanArrangement-r18                  ENUMERATED{ alignedOnly, alignedAndNonAligned }

}                                                                                                     OPTIONAL,

-- R1 55-6e: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16

-- and Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMCG-SCG-List-r18         SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMCG-SCG-r18

OPTIONAL,

-- R1 55-6g: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers with restriction for non-aligned span case

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r18  SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetection-r17)) OF

PDCCH-BlindDetectionCA-Mixed-r18                OPTIONAL,

-- R1 55-6f: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells with restriction for non-aligned span case

pdcch-MonitoringCA-NonAlignedSpan-r18         INTEGER (2..16)                                         OPTIONAL,

-- R4 33-1: Support of intra-band non-collocated NR CA operation

intraBandNR-CA-non-collocated-r18             ENUMERATED {supported}                                  OPTIONAL

}

CrossCarrierSchedulingSCell-SpCell-r17 ::= SEQUENCE {

supportedSCS-Combinations-r17              SEQUENCE {

scs15kHz-15kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs15kHz-30kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs15kHz-60kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs30kHz-30kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL,

scs30kHz-60kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL,

scs60kHz-60kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL

},

pdcch-MonitoringOccasion-r17               ENUMERATED {val1, val2}

}

PDCCH-BlindDetectionMixedList-r16::=       SEQUENCE {

pdcch-BlindDetectionCA-MixedExt-r16        CHOICE {

pdcch-BlindDetectionCA-Mixed-v16a0                PDCCH-BlindDetectionCA-MixedExt-r16,

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-v16a0 PDCCH-BlindDetectionCA-MixedExt-r16

}                                                                                             OPTIONAL,

pdcch-BlindDetectionCG-UE-MixedExt-r16     SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-v16a0                PDCCH-BlindDetectionCG-UE-MixedExt-r16,

pdcch-BlindDetectionSCG-UE-Mixed-v16a0            PDCCH-BlindDetectionCG-UE-MixedExt-r16

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCA-MixedExt-r16 ::=    SEQUENCE {

pdcch-BlindDetectionCA1-r16                INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                INTEGER (1..15)

}

PDCCH-BlindDetectionCG-UE-MixedExt-r16 ::= SEQUENCE {

pdcch-BlindDetectionCG-UE1-r16             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r16             INTEGER (0..15)

}

PDCCH-BlindDetectionMCG-SCG-r17 ::=        SEQUENCE {

pdcch-BlindDetectionMCG-UE-r17             INTEGER (1..15),

pdcch-BlindDetectionSCG-UE-r17             INTEGER (1..15)

}

PDCCH-BlindDetectionMixed-r17::=           SEQUENCE {

pdcch-BlindDetectionCA-Mixed-r17           PDCCH-BlindDetectionCA-Mixed-r17                   OPTIONAL,

pdcch-BlindDetectionCG-UE-Mixed-r17        SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-v17       PDCCH-BlindDetectionCG-UE-Mixed-r17,

pdcch-BlindDetectionSCG-UE-Mixed-v17       PDCCH-BlindDetectionCG-UE-Mixed-r17

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCG-UE-Mixed-r17 ::=    SEQUENCE {

pdcch-BlindDetectionCG-UE1-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r17             INTEGER (0..15)

}

PDCCH-BlindDetectionCA-Mixed-r17 ::=       SEQUENCE {

pdcch-BlindDetectionCA1-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA2-r17                INTEGER (1..15)                                    OPTIONAL

}

PDCCH-BlindDetectionMixed1-r17::=          SEQUENCE {

pdcch-BlindDetectionCA-Mixed1-r17          PDCCH-BlindDetectionCA-Mixed1-r17                  OPTIONAL,

pdcch-BlindDetectionCG-UE-Mixed1-r17       SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed1-v17      PDCCH-BlindDetectionCG-UE-Mixed1-r17,

pdcch-BlindDetectionSCG-UE-Mixed1-v17      PDCCH-BlindDetectionCG-UE-Mixed1-r17

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCG-UE-Mixed1-r17 ::=   SEQUENCE {

pdcch-BlindDetectionCG-UE1-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE3-r17             INTEGER (0..15)

}

PDCCH-BlindDetectionCA-Mixed1-r17 ::=      SEQUENCE {

pdcch-BlindDetectionCA1-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA2-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA3-r17                INTEGER (1..15)                                    OPTIONAL

}

PDCCH-BlindDetectionMCG-SCG-r18 ::=        SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-r18       PDCCH-BlindDetectionCG-UE-Mixed-r18,

pdcch-BlindDetectionSCG-UE-Mixed-r18       PDCCH-BlindDetectionCG-UE-Mixed-r18

}

PDCCH-BlindDetectionCA-Mixed-r18 ::=       SEQUENCE {

pdcch-BlindDetectionCA1-r18                INTEGER (1..15),

pdcch-BlindDetectionCA2-r18                INTEGER (1..15)

}

PDCCH-BlindDetectionCG-UE-Mixed-r18 ::=    SEQUENCE {

pdcch-BlindDetectionCG-UE1-r18             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r18             INTEGER (0..15)

}

SimulSRS-ForAntennaSwitching-r16 ::= SEQUENCE {

supportSRS-xTyR-xLessThanY-r16       ENUMERATED {supported}                     OPTIONAL,

supportSRS-xTyR-xEqualToY-r16        ENUMERATED {supported}                     OPTIONAL,

supportSRS-AntennaSwitching-r16      ENUMERATED {supported}                     OPTIONAL

}

TwoPUCCH-Grp-Configurations-r16 ::=  SEQUENCE {

pucch-PrimaryGroupMapping-r16        TwoPUCCH-Grp-ConfigParams-r16,

pucch-SecondaryGroupMapping-r16      TwoPUCCH-Grp-ConfigParams-r16

}

TwoPUCCH-Grp-Configurations-r17 ::=  SEQUENCE {

primaryPUCCH-GroupConfig-r17         PUCCH-Group-Config-r17,

secondaryPUCCH-GroupConfig-r17       PUCCH-Group-Config-r17

}

TwoPUCCH-Grp-ConfigParams-r16 ::=    SEQUENCE {

pucch-GroupMapping-r16               PUCCH-Grp-CarrierTypes-r16,

pucch-TX-r16                         PUCCH-Grp-CarrierTypes-r16

}

CarrierTypePair-r16 ::=             SEQUENCE {

carrierForCSI-Measurement-r16       PUCCH-Grp-CarrierTypes-r16,

carrierForCSI-Reporting-r16         PUCCH-Grp-CarrierTypes-r16

}

PUCCH-Grp-CarrierTypes-r16 ::=       SEQUENCE {

fr1-NonSharedTDD-r16                 ENUMERATED {supported}                     OPTIONAL,

fr1-SharedTDD-r16                    ENUMERATED {supported}                     OPTIONAL,

fr1-NonSharedFDD-r16                 ENUMERATED {supported}                     OPTIONAL,

fr2-r16                              ENUMERATED {supported}                     OPTIONAL

}

PUCCH-Group-Config-r17 ::=           SEQUENCE {

fr1-FR1-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL,

fr2-FR2-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL,

fr1-FR2-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL

}

CombinationCarrierType-r18 ::=                  SEQUENCE {

schedulingCellCarrierType-r18                   ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1, fr2-2},

scheduledCellCarrierType-r18                    ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1, fr2-2}

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

CA-ParametersNR field description

codebookParametersPerBC For a given supported band combination, this field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type, amongst the supported CSI-RS resources included in codebookParametersPerBand in MIMO-ParametersPerBand.

#### –	CA-ParametersNRDC

The IE CA-ParametersNRDC contains dual connectivity related capabilities that are defined per band combination.

CA-ParametersNRDC information element

-- ASN1START

-- TAG-CA-PARAMETERS-NRDC-START

CA-ParametersNRDC ::=                         SEQUENCE {

ca-ParametersNR-ForDC                       CA-ParametersNR                              OPTIONAL,

ca-ParametersNR-ForDC-v1540                 CA-ParametersNR-v1540                        OPTIONAL,

ca-ParametersNR-ForDC-v1550                 CA-ParametersNR-v1550                        OPTIONAL,

ca-ParametersNR-ForDC-v1560                 CA-ParametersNR-v1560                        OPTIONAL,

featureSetCombinationDC                     FeatureSetCombinationId                      OPTIONAL

}

CA-ParametersNRDC-v15g0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v15g0                   CA-ParametersNR-v15g0                        OPTIONAL

}

CA-ParametersNRDC-v1610 ::= SEQUENCE {

-- R1 18-1: Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode1-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1a: Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode2-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1b: Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-DynamicPwrSharing-r16      ENUMERATED {short, long}       OPTIONAL,

asyncNRDC-r16                            ENUMERATED {supported}         OPTIONAL

}

CA-ParametersNRDC-v1630 ::=                         SEQUENCE {

ca-ParametersNR-ForDC-v1610                 CA-ParametersNR-v1610                        OPTIONAL,

ca-ParametersNR-ForDC-v1630                 CA-ParametersNR-v1630                        OPTIONAL

}

CA-ParametersNRDC-v1640 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1640                  CA-ParametersNR-v1640                        OPTIONAL

}

CA-ParametersNRDC-v1650 ::=                  SEQUENCE {

supportedCellGrouping-r16                    BIT STRING (SIZE (1..maxCellGroupings-r16))  OPTIONAL

}

CA-ParametersNRDC-v16a0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v16a0                  CA-ParametersNR-v16a0                        OPTIONAL

}

CA-ParametersNRDC-v1700 ::=                   SEQUENCE {

-- R1 31-9: Indicates the support of simultaneous transmission and reception of an IAB-node from multiple parent nodes

simultaneousRxTx-IAB-MultipleParents-r17     ENUMERATED {supported}                       OPTIONAL,

condPSCellAdditionNRDC-r17                   ENUMERATED {supported}                       OPTIONAL,

scg-ActivationDeactivationNRDC-r17           ENUMERATED {supported}                       OPTIONAL,

scg-ActivationDeactivationResumeNRDC-r17     ENUMERATED {supported}                       OPTIONAL,

beamManagementType-CBM-r17                   ENUMERATED {supported}                       OPTIONAL

}

CA-ParametersNRDC-v1720 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1700                  CA-ParametersNR-v1700                        OPTIONAL,

ca-ParametersNR-ForDC-v1720                  CA-ParametersNR-v1720                        OPTIONAL

}

CA-ParametersNRDC-v1730 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1730                   CA-ParametersNR-v1730                       OPTIONAL

}

CA-ParametersNRDC-v1760 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1760                  CA-ParametersNR-v1760

}

CA-ParametersNRDC-v1800 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1800                  CA-ParametersNR-v1800                        OPTIONAL,

-- R1 55-6d: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for

-- SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring on all the serving cells

pdcch-BlindDetectionNRDC-r18                 SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed1-r18

OPTIONAL

}

PDCCH-BlindDetectionMixed1-r18::=            SEQUENCE {

pdcch-BlindDetectionCG-UE-Mixed-r18          SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-r18         PDCCH-BlindDetectionCG-UE-Mixed-r18,

pdcch-BlindDetectionSCG-UE-Mixed-r18         PDCCH-BlindDetectionCG-UE-Mixed-r18

}

}

-- TAG-CA-PARAMETERS-NRDC-STOP

-- ASN1STOP

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If a version of this field (i.e., with or without suffix) is absent for a band combination, the corresponding ca-ParametersNR field version in BandCombination is applicable to the UE configured with NR-DC for the band combination. If a version of this field (i.e., with or without suffix) is present for a band combination but does not contain any parameters, the UE does not support the corresponding field version when configured with NR-DC for the band combination.

featureSetCombinationDC If this field is present for a band combination, it reports the feature set combination supported for the band combination when NR-DC is configured. If this field is absent for a band combination, the featureSetCombination in BandCombination (without suffix) is applicable to the UE configured with NR-DC for the band combination.

#### –	CarrierAggregationVariant

The IE CarrierAggregationVariant informs the network about supported "placement" of the SpCell in an NR cell group.

CarrierAggregationVariant information element

-- ASN1START

-- TAG-CARRIERAGGREGATIONVARIANT-START

CarrierAggregationVariant ::=          SEQUENCE {

fr1fdd-FR1TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1FDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1TDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR2TDD  ENUMERATED {supported}                      OPTIONAL

}

-- TAG-CARRIERAGGREGATIONVARIANT-STOP

-- ASN1STOP

#### –	CodebookParameters

The IE CodebookParameters is used to convey codebook related parameters.

CodebookParameters information element

-- ASN1START

-- TAG-CODEBOOKPARAMETERS-START

CodebookParameters ::=             SEQUENCE {

type1                                  SEQUENCE {

singlePanel                           SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode1andMode2},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

},

multiPanel                            SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode2, both},

nrofPanels                            ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

}                                                                                                               OPTIONAL

},

type2                                  SEQUENCE {

supportedCSI-RS-ResourceList        SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                 ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}              OPTIONAL

}                                                                                                                   OPTIONAL,

type2-PortSelection                  SEQUENCE {

supportedCSI-RS-ResourceList        SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                 ENUMERATED {wideband, widebandAndSubband}

}                                                                                                                   OPTIONAL

}

CodebookParameters-v1610 ::=        SEQUENCE {

supportedCSI-RS-ResourceListAlt-r16  SEQUENCE {

type1-SinglePanel-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type1-MultiPanel-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-r16                            SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-PortSelection-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL

}                                                                                                                                       OPTIONAL

}

CodebookParametersAddition-r16 ::=      SEQUENCE {

etype2-r16                             SEQUENCE {

-- R1 16-3a Regular eType 2 R=1

etype2R1-r16                           SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 16-3a-1 Regular eType 2 R=2

etype2R2-r16                           SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                  OPTIONAL,

-- R1 16-3a-2: Support of parameter combinations 7-8

paramComb7-8-r16                       ENUMERATED {supported}      OPTIONAL,

-- R1 16-3a-3: Support of rank 3,4

rank3-4-r16                            ENUMERATED {supported}      OPTIONAL,

-- R1 16-3a-4: CBSR with soft amplitude restriction

amplitudeSubsetRestriction-r16         ENUMERATED {supported}      OPTIONAL

}                                                                      OPTIONAL,

etype2-PS-r16                          SEQUENCE {

-- R1 16-3b Regular eType 2 R=1 PortSelection

etype2R1-PortSelection-r16             SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 16-3b-1 Regular eType 2 R=2 PortSelection

etype2R2-PortSelection-r16             SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                  OPTIONAL,

-- R1 16-3b-2: Support of rank 3,4

rank3-4-r16                            ENUMERATED {supported}      OPTIONAL

}                                                                      OPTIONAL

}

CodebookComboParametersAddition-r16 ::= SEQUENCE {

-- R1 16-8 Mixed codebook types

type1SP-Type2-null-r16                 SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-Type2PS-null-r16               SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R1-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R2-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R1PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R2PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-Type2-Type2PS-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2-null-r16                 SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2PS-null-r16               SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R1-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R2-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R1PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R2PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2-Type2PS-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL

}

CodebookParametersfetype2-r17 ::= SEQUENCE {

-- R1 23-9-1  Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

fetype2basic-r17        SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16),

-- R1 23-9-2  Support of M=2 and R=1 for FeType-II

fetype2R1-r17           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-4  Support of R = 2 for FeType-II

fetype2R2-r17           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-3  Support of rank 3, 4 for FeType-II

fetype2Rank3Rank4-r17   ENUMERATED {supported}     OPTIONAL

}

CodebookComboParameterMixedType-r17 ::= SEQUENCE {

-- R1 23-9-5 Active CSI-RS resources and ports for mixed codebook types in any slot

type1SP-feType2PS-null-r17              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R1-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R2-null-r1          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M1-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M1-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M2R1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-null-r17              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R1-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R2-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M1-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M1-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M2R1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMultiTRP-r17::= SEQUENCE {

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

--  {Codebook 2, Codebook 3} =(NULL, NULL}

nCJT-null-null                        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-null-null                     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--    {Codebook 2, Codebook 3} = {( {"Rel 16 combinations in FG 16-8"}

nCJT-Type2-null-r16                   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2PS-null-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-Type2PS-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-null-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2-null-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1PS-null-r16           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2PS-null-r16           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-Type2PS-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- {Codebook 2, Codebook 3} = {"New Rel17 combinations in FG 23-9-5"}

nCJT-feType2PS-null-r17               SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R1-null-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R2-null-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M1-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M2R1-r17        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M2R1-r17     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-null-r17            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R1-null-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R2-null-r1        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M2R1-r17     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M2R1-r17  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersAdditionPerBC-r16::=  SEQUENCE {

-- R1 16-3a Regular eType 2 R=1

etype2R1-r16                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3a-1 Regular eType 2 R=2

etype2R2-r16                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3b Regular eType 2 R=1 PortSelection

etype2R1-PortSelection-r16      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3b-1 Regular eType 2 R=2 PortSelection

etype2R2-PortSelection-r16      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParametersAdditionPerBC-r16::= SEQUENCE {

-- R1 16-8 Mixed codebook types

type1SP-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersfetype2PerBC-r17 ::= SEQUENCE {

-- R1 23-9-1	Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

fetype2basic-r17    SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16),

-- R1 23-9-2	Support of M=2 and R=1 for FeType-II

fetype2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-4	Support of R = 2 for FeType-II

fetype2R2-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMixedTypePerBC-r17 ::= SEQUENCE {

-- R1 23-9-5 Active CSI-RS resources and ports for mixed codebook types in any slot

type1SP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R2-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R2-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMultiTRP-PerBC-r17::= SEQUENCE {

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

--  {Codebook 2, Codebook 3} =(NULL, NULL}

nCJT-null-null               SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-null-null            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--    {Codebook 2, Codebook 3} = {( {"Rel 16 combinations in FG 16-8"}

nCJT-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-null-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2-null-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1PS-null-r16  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2PS-null-r16  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-Type2PS-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- {Codebook 2, Codebook 3} = {"New Rel17 combinations in FG 23-9-5"}

nCJT-feType2PS-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R1-null-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R2-null-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M2R1-r17  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R2-null-r1  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersetype2DopplerCSI-r18 ::= SEQUENCE {

-- R1 40-3-2-1: Support of Rel-16-based doppler CSI

eType2Doppler-r18                          SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

valueY-P-SP-CSI-RS-r18                 INTEGER (1..3),

valueY-A-CSI-RS-r18                    INTEGER (1..3),

scalingfactor-r18                      ENUMERATED {n1, n2, n4}

},

-- R1 40-3-2-1a: Support of Rel-16-based doppler measurement with N4>1

eType2DopplerN4-r18  SEQUENCE {

supportedCSI-RS-ReportSettingList1-r18 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

SupportedCSI-RS-ReportSetting-r18,

supportedCSI-RS-ReportSettingList2-r18 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

SupportedCSI-RS-ReportSetting-r18

}                                                                                                            OPTIONAL,

-- R1 40-3-2-1a-1: DD unit size when A-CSI-RS is configured for CMR N4>1

ddUnitSize-A-CSI-RS-CMR-r18                ENUMERATED {supported}                                            OPTIONAL,

-- R1 40-3-2-2: Support R=2 for Rel-16-based doppler codebook

eType2DopplerR2-r18  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 40-3-2-3: Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook

eType2DopplerX1-r18                        ENUMERATED {supported}                                            OPTIONAL,

-- R1 40-3-2-3a: Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook

eType2DopplerX2-r18                        ENUMERATED {supported}                                            OPTIONAL,

--R1 40-3-2-7: support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook

eType2DopplerL-N4D1-r18                    ENUMERATED {supported}                                            OPTIONAL

}

CodebookParametersfetype2DopplerCSI-r18 ::= SEQUENCE {

-- R1 40-3-2-4: Support of Rel-17-based doppler CSI

feType2Doppler-r18  SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

valueY-A-CSI-RS-r18                    INTEGER (1..3),

scalingfactor-r18                      ENUMERATED {n1, n2, n4}

},

-- R1 40-3-2-5: Support of M=2 and R=1 for Rel-17-based doppler codebook

feType2DopplerM2R1-r18                     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 40-3-2-6: Support R=2 for Rel-17-based doppler codebook

feType2DopplerR2-r18  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--R1 40-3-2-7a: Support of l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook

feType2DopplerL-N4D1-r18  ENUMERATED {supported}                                                             OPTIONAL

}

CodebookParametersetype2CJT-r18 ::= SEQUENCE {

-- R1 40-3-1-1: Basic feature for Rel-16-based CJT type-II codebook

eType2CJT-r18                          SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

scalingfactor-r18                      ENUMERATED {n1, f1p5, n2},

maxNumberNZP-CSI-RS-MultiTRP-CJT-r18   INTEGER (2..4)

}

-- R1 40-3-1-1a: Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset

eType2CJT-FD-IO-r18                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-2: Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1

eType2CJT-FD-FO-r18                    ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-3: Support R=2 for Rel-16-based CJT codebook

eType2CJT-R2-r18                            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-4: Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook

eType2CJT-PV-Beta-r18                      ENUMERATED {supported}                                                         OPTIONAL,

-- R1 40-3-1-9: Support for 2NN1N2 >32 for Rel-16 based CJT codebook

eType2CJT-2NN1N2-r18                       ENUMERATED {n64,n96,n128}                                                      OPTIONAL,

-- R1 40-3-1-12: Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook

eType2CJT-Rank3Rank4-r18                   ENUMERATED {supported}                                                         OPTIONAL,

-- R1 40-3-1-14: Support of Support of L=6 for Rel-16-based CJT type-II codebook

eType2CJT-L6-r18                            ENUMERATED {supported}                                                         OPTIONAL,

-- R1 40-3-1-15: dynamic selection of N<=N_TRP for Rel-16-based CJT type-II codebook

eType2CJT-NN-r18                            ENUMERATED {supported}                                                          OPTIONAL,

-- R1 40-3-1-17: Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-16-based CJT

-- type-II codebook

eType2CJT-NL-SD-r18                         ENUMERATED {n2,n4}                                                              OPTIONAL,

-- R1 40-3-1-23: Unequal number of spatial basis selection configuration for multi-TRP CJT

eType2CJT-Unequal-r18                   ENUMERATED {supported}                                                OPTIONAL

}

CodebookParametersfetype2CJT-r18 ::= SEQUENCE {

-- R1 40-3-1-5: Basic feature for Rel-17-based CJT type-II codebook

feType2CJT-r18                               SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

scalingfactor-r18                       ENUMERATED {n1, f1p5, n2},

maxNumberNZP-CSI-RS-MultiTRP-CJT-r18    INTEGER (2..4)

}

-- R1 40-3-1-5a: Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset

feType2CJT-FD-IO-r18                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-6: Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1

feType2CJT-FD-FO-r18                    ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-7: Support of M=2 and R=1 for Rel-17-based CJT codebook

feType2CJT-M2R1-r18                         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-8: Support of R=2 for Rel-17-based CJT codebook

feType2CJT-R2-r18                           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-9a: Support for 2NN1N2 >32 for Rel-17 based CJT codebook

feType2CJT-2NN1N2-r18                       ENUMERATED {n64,n96,n128}                                                     OPTIONAL,

-- R1 40-3-1-13: Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook

feType2CJT-Rank3Rank4-r18                  ENUMERATED {supported}                                                         OPTIONAL,

-- R1 40-3-1-16: dynamic selection of N<=N_TRP for Rel-17-based CJT type-II codebook

feType2CJT-NN-r18                            ENUMERATED {supported}                                                         OPTIONAL,

-- R1 40-3-1-18: Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-17-based CJT

-- type-II codebook

feType2CJT-NL-r18                           ENUMERATED {n2,n4}                                                              OPTIONAL,

-- R1 40-3-1-23a: Unequal number of port selection configuration for multi-TRP CJT

feType2CJT-Unequal-r18                   ENUMERATED {supported}                                               OPTIONAL

}

CodebookComboParametersCJT-r18::= SEQUENCE {

-- R1 40-3-1-11: Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot

--  {Codebook 1} = Type I SP

cjt-Type1SP-eType2R1-null             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-eType2R2-null             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-feType2R1M1-null          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-feType2R1M2-null          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-feType2R2M2-null          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--  {Codebook 1} = Type I MP

cjt-Type1MP-eType2R1-null             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-eType2R2-null             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-feType2R1M1-null          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-feType2R1M2-null          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-feType2R2M2-null          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookVariantsList-r16 ::= SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-Resource

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

SupportedCSI-RS-ReportSetting-r18 ::= SEQUENCE {

maxN4-r18                            ENUMERATED {n1, n2, n4, n8},

maxNumberTxPortsPerResource-r18      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand-r18        INTEGER (1..64),

totalNumberTxPortsPerBand-r18        INTEGER (2..256)

}

-- TAG-CODEBOOKPARAMETERS-STOP

-- ASN1STOP

CodebookParameters field descriptions

supportedCSI-RS-ResourceListAlt This field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS-Resource defined in CodebookVariantsList. The value 0 corresponds to the first entry of CodebookVariantsList. The value 1 corresponds to the second entry of CodebookVariantsList, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC (but optional for single CC) and codebookParametersPerBand.

#### –	ERedCapParameters

The IE ERedCapParameters is used to indicate the UE capabilities supported by eRedCap UEs.

ERedCapParameters information element

-- ASN1START

-- TAG-EREDCAPPARAMETERS-START

ERedCapParameters-r18::=                   SEQUENCE {

-- R1 48-1: eRedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1

supportOfERedCap-r18                       ENUMERATED {supported},

-- R1 48-2: eRedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1

eRedCapNotReducedBB-BW-r18                 ENUMERATED {supported}                            OPTIONAL,

eRedCapIgnoreCapabilityFiltering-r18       ENUMERATED {supported}                            OPTIONAL

}

-- TAG-EREDCAPPARAMETERS-STOP

-- ASN1STOP

#### –	FeatureSetCombination

The IE FeatureSetCombination is a two-dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

Each FeatureSet contains either a pair of NR or E-UTRA feature set IDs for UL and DL.

In case of NR, the actual feature sets for UL and DL are defined in the FeatureSets IE and referred to from here by their ID, i.e., their position in the featureSetsUplink / featureSetsDownlink list in the FeatureSet IE.

In case of E-UTRA, the feature sets referred to from this list are defined in TS 36.331 [10] and conveyed as part of the UE-EUTRA-Capability container.

The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Ids and FeatureSetDownlinkPerCC-Ids. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of carriers supported by the UE is also restricted by the bandwidth class indicated in the associated BandCombination, if present.

In feature set combinations the UE shall exclude entries with same or lower capabilities, since the network may anyway assume that the UE supports those.

NOTE 1:	The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:	The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

NOTE 3:	The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

FeatureSetCombination information element

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

#### –	FeatureSetCombinationId

The IE FeatureSetCombinationId identifies a FeatureSetCombination. The FeatureSetCombinationId of a FeatureSetCombination is the position of the FeatureSetCombination in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability). The FeatureSetCombinationId = 0 refers to the first entry in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability).

NOTE:	The FeatureSetCombinationId = 1024 is not used due to the maximum entry number of featureSetCombinations.

FeatureSetCombinationId information element

-- ASN1START

-- TAG-FEATURESETCOMBINATIONID-START

FeatureSetCombinationId ::=         INTEGER (0.. maxFeatureSetCombinations)

-- TAG-FEATURESETCOMBINATIONID-STOP

-- ASN1STOP

#### –	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                           ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy8                                  ENUMERATED {supported}                                                  OPTIONAL,

scellWithoutSSB                         ENUMERATED {supported}                                                  OPTIONAL,

csi-RS-MeasSCellWithoutSSB              ENUMERATED {supported}                                                  OPTIONAL,

dummy1                                  ENUMERATED {supported}                                                  OPTIONAL,

type1-3-CSS                             ENUMERATED {supported}                                                  OPTIONAL,

pdcch-MonitoringAnyOccasions            ENUMERATED {withoutDCI-Gap, withDCI-Gap}                                OPTIONAL,

dummy2                                  ENUMERATED {supported}                                                  OPTIONAL,

ue-SpecificUL-DL-Assignment             ENUMERATED {supported}                                                  OPTIONAL,

searchSpaceSharingCA-DL                 ENUMERATED {supported}                                                  OPTIONAL,

timeDurationForQCL                      SEQUENCE {

scs-60kHz                           ENUMERATED {s7, s14, s28}                                               OPTIONAL,

scs-120kHz                          ENUMERATED {s14, s28}                                                   OPTIONAL

}                                                                                                           OPTIONAL,

pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-30kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-60kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-120kHz                              ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL

}                                                                                                           OPTIONAL,

dummy3                                  DummyA                                                                  OPTIONAL,

dummy4                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyB                        OPTIONAL,

dummy5                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyC                        OPTIONAL,

dummy6                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyD                        OPTIONAL,

dummy7                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyE                        OPTIONAL

}

FeatureSetDownlink-v1540 ::= SEQUENCE {

oneFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

additionalDMRS-DL-Alt                   ENUMERATED {supported}                       OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-DL       ENUMERATED {supported}                       OPTIONAL,

pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE {

scs-15kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-30kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-60kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-120kHz                              ENUMERATED {set1, set2, set3}                OPTIONAL

}                                                                                    OPTIONAL,

pdsch-SeparationWithGap                 ENUMERATED {supported}                       OPTIONAL,

pdsch-ProcessingType2                   SEQUENCE {

scs-15kHz                               ProcessingParameters                         OPTIONAL,

scs-30kHz                               ProcessingParameters                         OPTIONAL,

scs-60kHz                               ProcessingParameters                         OPTIONAL

} OPTIONAL,

pdsch-ProcessingType2-Limited           SEQUENCE {

differentTB-PerSlot-SCS-30kHz           ENUMERATED {upto1, upto2, upto4, upto7}

} OPTIONAL,

dl-MCS-TableAlt-DynamicIndication       ENUMERATED {supported}                       OPTIONAL

}

FeatureSetDownlink-v15a0 ::= SEQUENCE {

supportedSRS-Resources              SRS-Resources                                    OPTIONAL

}

FeatureSetDownlink-v1610 ::=   SEQUENCE {

-- R1 22-4e/4f/4g/4h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 1

cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16   SEQUENCE {

scs-15kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-30kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-60kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-120kHz-r16       ENUMERATED {one, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

-- R1 22-3e/3f/3g/3h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 2

cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16   SEQUENCE {

scs-15kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-30kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-60kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-120kHz-r16       ENUMERATED {one, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

intraFreqDAPS-r16                  SEQUENCE {

intraFreqDiffSCS-DAPS-r16          ENUMERATED {supported}            OPTIONAL,

intraFreqAsyncDAPS-r16             ENUMERATED {supported}            OPTIONAL

}                                                                        OPTIONAL,

intraBandFreqSeparationDL-v1620    FreqSeparationClassDL-v1620           OPTIONAL,

intraBandFreqSeparationDL-Only-r16 FreqSeparationClassDL-Only-r16        OPTIONAL,

-- R1 11-2: Rel-16 PDCCH monitoring capability

pdcch-Monitoring-r16               SEQUENCE {

pdsch-ProcessingType1-r16          SEQUENCE {

scs-15kHz-r16                      PDCCH-MonitoringOccasions-r16 OPTIONAL,

scs-30kHz-r16                      PDCCH-MonitoringOccasions-r16 OPTIONAL

}                                                                    OPTIONAL,

pdsch-ProcessingType2-r16      SEQUENCE {

scs-15kHz-r16                  PDCCH-MonitoringOccasions-r16     OPTIONAL,

scs-30kHz-r16                  PDCCH-MonitoringOccasions-r16     OPTIONAL

}                                                                    OPTIONAL

}                                                                        OPTIONAL,

-- R1 11-2b: Mix of Rel. 16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers

pdcch-MonitoringMixed-r16          ENUMERATED {supported}                OPTIONAL,

-- R1 18-5c: Processing up to X unicast DCI scheduling for DL per scheduled CC

crossCarrierSchedulingProcessing-DiffSCS-r16  SEQUENCE {

scs-15kHz-120kHz-r16               ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-15kHz-60kHz-r16                ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-30kHz-120kHz-r16               ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-15kHz-30kHz-r16                ENUMERATED {n2}                   OPTIONAL,

scs-30kHz-60kHz-r16                ENUMERATED {n2}                   OPTIONAL,

scs-60kHz-120kHz-r16               ENUMERATED {n2}                   OPTIONAL

}                                                                        OPTIONAL,

-- R1 16-2b-1: Support of single-DCI based SDM scheme

singleDCI-SDM-scheme-r16           ENUMERATED {supported}                OPTIONAL

}

FeatureSetDownlink-v1700 ::= SEQUENCE {

-- R1 36-2: Scaling factor to be applied to 1024QAM for FR1

scalingFactor-1024QAM-FR1-r17 ENUMERATED {f0p4, f0p75, f0p8}             OPTIONAL,

-- R1 24 feature for existing UE cap to include new SCS

timeDurationForQCL-v1710     SEQUENCE {

scs-480kHz                   ENUMERATED {s56, s112}                  OPTIONAL,

scs-960kHz                   ENUMERATED {s112, s224}                 OPTIONAL

}                                                                        OPTIONAL,

-- R1 23-6-1	SFN scheme A (scheme 1) for PDSCH and PDCCH

sfn-SchemeA-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1-1	SFN scheme A (scheme 1) for PDCCH only

sfn-SchemeA-PDCCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1a	Dynamic switching - scheme A

sfn-SchemeA-DynamicSwitching-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1b	SFN scheme A (scheme 1) for PDSCH only

sfn-SchemeA-PDSCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2	SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH

sfn-SchemeB-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2a	Dynamic switching - scheme B

sfn-SchemeB-DynamicSwitching-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2b	SFN scheme B (TRP based pre-compensation) for PDSCH only

sfn-SchemeB-PDSCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-2-1d	PDCCH repetition for Case 2 PDCCH monitoring with a span gap

mTRP-PDCCH-Case2-1SpanGap-r17    SEQUENCE {

scs-15kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-30kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-60kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-120kHz-r17                   PDCCH-RepetitionParameters-r17      OPTIONAL

}                                                                        OPTIONAL,

-- R1 23-2-1e	PDCCH repetition for Rel-16 PDCCH monitoring

mTRP-PDCCH-legacyMonitoring-r17  SEQUENCE {

scs-15kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-30kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL

}                                                                        OPTIONAL,

-- R1  23-2-4	Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP

mTRP-PDCCH-multiDCI-multiTRP-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 33-2:	Dynamic scheduling for multicast for PCell

dynamicMulticastPCell-r17        ENUMERATED {supported}                  OPTIONAL,

-- R1 23-2-1	PDCCH repetition

mTRP-PDCCH-Repetition-r17        SEQUENCE {

numBD-twoPDCCH-r17               INTEGER (2..3),

maxNumOverlaps-r17               ENUMERATED {n1,n2,n3,n5,n10,n20,n40}

}                                                                        OPTIONAL

}

FeatureSetDownlink-v1720 ::=                SEQUENCE {

-- R1 25-19: RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS

rtt-BasedPDC-CSI-RS-ForTracking-r17         ENUMERATED {supported}                                                   OPTIONAL,

-- R1 25-19a: RTT-based Propagation delay compensation based on DL PRS for RTT-based PDC and SRS

rtt-BasedPDC-PRS-r17                        SEQUENCE {

maxNumberPRS-Resource-r17                   ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPRS-ResourceProcessedPerSlot-r17   SEQUENCE {

scs-15kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-30kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-60kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-120kHz-r17                              ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL

}

}                                                                                                                    OPTIONAL,

-- R1 33-5-1: SPS group-common PDSCH for multicast on PCell

sps-Multicast-r17                           ENUMERATED {supported}                                                   OPTIONAL

}

FeatureSetDownlink-v1730 ::=                SEQUENCE {

-- R1 25-19b: Support of PRS as spatial relation RS for SRS

prs-AsSpatialRelationRS-For-SRS-r17         ENUMERATED {supported}                                                   OPTIONAL

}

FeatureSetDownlink-v1800 ::=                    SEQUENCE {

-- R1 40-3-3-6: Maximum number of TRS resource sets in a report configuration

maxNumberTRS-ResourceSet-r18                    INTEGER (2..3)                                                           OPTIONAL,

-- R1 40-4-1: Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A

pdsch-TypeA-DMRS-r18                            ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1a: Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B

pdsch-TypeB-DMRS-r18                            ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1b: 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH

pdsch-1SymbolFL-DMRS-Addition2Symbol-r18        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1c: Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH

pdsch-AlternativeDMRS-Coexistence-r18           ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1d: 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH

pdsch-2SymbolFL-DMRS-r18                        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1e: 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH

pdsch-2SymbolFL-DMRS-Addition2Symbol-r18        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1f: 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH

pdsch-1SymbolFL-DMRS-Addition3Symbol-r18        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1g: DMRS type for Rel.18 enhanced DMRS ports for PDSCH

pdsch-DMRS-Type-r18                             ENUMERATED {etype1, etype1And2}                                          OPTIONAL,

-- R1 40-4-1h: 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8

pdsch-1PortDL-PTRS-r18                          ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1i: 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8

pdsch-2PortDL-PTRS-r18                          ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1j: Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A

mappingTypeA-1SymbolFL-DMRS-Addition2Symbol-r18 ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-4: Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports

pdsch-ReceptionWithoutSchedulingRestriction-r18 ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-4a: Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA

pdsch-ReceptionSchemeA-r18                      ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-4b: Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB

pdsch-ReceptionSchemeB-r18                      ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-5: Rel-18 DL DMRS with single DCI based M-TRP

dmrs-MultiTRP-SingleDCI-r18                     ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-5a: Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP

dmrs-MultiTRP-AddtionRows-r18                   ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-7: Rel-18 DL DMRS with M-DCI based M-TRP

dmrs-MultiTRP-MultiDCI-r18                      ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-12: Support of Rel-18 DMRS and PDSCH processing capability 2 simultaneously

simulDMRS-PDSCH-r18                             SEQUENCE {

scs-15kHz-r18                                   INTEGER (0..4)                                                       OPTIONAL,

scs-30kHz-r18                               INTEGER (0..5)                                                       OPTIONAL,

scs-60kHz-r18                                   INTEGER (0..7)                                                       OPTIONAL

}                                                                                                                        OPTIONAL,

-- R1 53-1: Support RLM/BM/BFD and gapless L3 intra-frequency measurements based on CD-SSB outside active BWP without interruptions

bwpOperationMeasWithoutInterrupt-r18            ENUMERATED {supported}                                                   OPTIONAL,

-- R1 55-6: (2, 2) span-based PDCCH monitoring with additional restriction(s)

pdcch-MonitoringSpan2-2-r18                     SEQUENCE{

pdsch-ProcessingType1-r18                       SEQUENCE{

scs-15kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL,

scs-30kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL

},

pdsch-ProcessingType2-r18                       SEQUENCE{

scs-15kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL,

scs-30kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL

}

}                                                                                                                        OPTIONAL,

-- R1 55-6b: Mix of Rel-16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers

pdcch-MonitoringMixed-r18                       ENUMERATED {supported}                                                   OPTIONAL,

multicastInactive-r18                           ENUMERATED {supported}                                                   OPTIONAL

}

PDCCH-MonitoringOccasions-r16 ::= SEQUENCE {

period7span3-r16                  ENUMERATED {supported}                 OPTIONAL,

period4span3-r16                  ENUMERATED {supported}                 OPTIONAL,

period2span2-r16                  ENUMERATED {supported}                 OPTIONAL

}

PDCCH-RepetitionParameters-r17 ::= SEQUENCE {

supportedMode-r17                  ENUMERATED {intra-span, inter-span, both},

limitX-PerCC-r17                   ENUMERATED {n4, n8, n16, n32, n44, n64, nolimit}                      OPTIONAL,

limitX-AcrossCC-r17                ENUMERATED {n4, n8, n16, n32, n44, n64, n128, n256, n512, nolimit}    OPTIONAL

}

DummyA ::=      SEQUENCE {

maxNumberNZP-CSI-RS-PerCC                   INTEGER (1..32),

maxNumberPortsAcrossNZP-CSI-RS-PerCC        ENUMERATED {p2, p4, p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256},

maxNumberCS-IM-PerCC                        ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousCSI-RS-ActBWP-AllCC    ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26,

n28, n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52,

n54, n56, n58, n60, n62, n64},

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC ENUMERATED {p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256}

}

DummyB ::=       SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode1AndMode2},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyC ::=        SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p8, p16, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode2, both},

supportedNumberPanels               ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyD ::=                 SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}                          OPTIONAL,

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyE ::=    SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

FeatureSetDownlink field descriptions

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

supportedSRS-Resources Indicates supported SRS resources for SRS carrier switching to the band associated with this FeatureSetDownlink. The UE is only allowed to set this field for a band with associated FeatureSetUplinkId set to 0.

#### –	FeatureSetDownlinkId

The IE FeatureSetDownlinkId identifies a downlink feature set. The FeatureSetDownlinkId of a FeatureSetDownlink is the index position of the FeatureSetDownlink in the featureSetsDownlink list in the FeatureSets IE. The first element in that list is referred to by FeatureSetDownlinkId = 1. The FeatureSetDownlinkId=0 is not used by an actual FeatureSetDownlink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetDownlinkId information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKID-START

FeatureSetDownlinkId ::=            INTEGER (0..maxDownlinkFeatureSets)

-- TAG-FEATURESETDOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetDownlinkPerCC information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-START

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                                                  OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                                           OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                                         OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                                                   OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                                                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1700 ::=   SEQUENCE {

supportedMinBandwidthDL-r17             SupportedBandwidth-v1700                                                OPTIONAL,

broadcastSCell-r17                     ENUMERATED {supported}                                                  OPTIONAL,

-- R1 33-2g: MIMO layers for multicast PDSCH

maxNumberMIMO-LayersMulticastPDSCH-r17  ENUMERATED {n2, n4, n8}                                                 OPTIONAL,

-- R1 33-2h: Dynamic scheduling for multicast for SCell

dynamicMulticastSCell-r17               ENUMERATED {supported}                                                  OPTIONAL,

supportedBandwidthDL-v1710              SupportedBandwidth-v1700                                                OPTIONAL,

-- R4 24-1/24-2/24-3/24-4/24-5

supportedCRS-InterfMitigation-r17       CRS-InterfMitigation-r17                                                OPTIONAL

}

FeatureSetDownlinkPerCC-v1720 ::=   SEQUENCE {

-- R1 33-2j: Supported maximum modulation order used for maximum data rate calculation for multicast PDSCH

maxModulationOrderForMulticastDataRateCalculation-r17  ENUMERATED {qam64, qam256, qam1024}                  OPTIONAL,

-- R1 33-1-2: FDM-ed unicast PDSCH and group-common PDSCH for broadcast

fdm-BroadcastUnicast-r17            ENUMERATED {supported}                                                  OPTIONAL,

-- R1 33-3-2: FDM-ed unicast PDSCH and one group-common PDSCH for multicast

fdm-MulticastUnicast-r17            ENUMERATED {supported}                                                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1730 ::=           SEQUENCE {

-- R1 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

intraSlotTDM-UnicastGroupCommonPDSCH-r17    ENUMERATED {yes, no}                    OPTIONAL,

-- R1 33-5-3: One SPS group-common PDSCH configuration for multicast for SCell

sps-MulticastSCell-r17                      ENUMERATED {supported}                  OPTIONAL,

-- R1 33-5-4: Up to 8 SPS group-common PDSCH configurations per CFR for multicast for SCell

sps-MulticastSCellMultiConfig-r17           INTEGER (1..8)                          OPTIONAL,

-- R1 33-1-1: Dynamic slot-level repetition for broadcast MTCH

dci-BroadcastWith16Repetitions-r17          ENUMERATED {supported}                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1800 ::=           SEQUENCE {

-- R1 40-2-1: Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement

multiDCI-IntraCellMultiTRP-TwoTA-r18        ENUMERATED {supported}                                          OPTIONAL,

-- R1 40-2-2: Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement

multiDCI-InterCellMultiTRP-TwoTA-r18        INTEGER (1..2)                                                  OPTIONAL,

-- R1 40-2-6: Rx timing difference larger than CP length

rxTimingDiff-r18                            ENUMERATED {supported}                                          OPTIONAL,

-- R1 55-7: Two QCL TypeD for CORESET monitoring in multi-DCI based multi-TRP

multiDCI-MultiTRP-CORESET-Monitoring-r18    ENUMERATED {supported}                                          OPTIONAL,

broadcastNonServingCell-r18                 ENUMERATED {supported}                                          OPTIONAL

}

MultiDCI-MultiTRP-r16 ::=           SEQUENCE {

maxNumberCORESET-r16                ENUMERATED {n2, n3, n4, n5},

maxNumberCORESETPerPoolIndex-r16    INTEGER (1..3),

maxNumberUnicastPDSCH-PerPool-r16   ENUMERATED {n1, n2, n3, n4, n7}

}

CRS-InterfMitigation-r17 ::=        SEQUENCE {

-- R4 24-1 CRS-IM (Interference Mitigation) in DSS scenario

crs-IM-DSS-15kHzSCS-r17             ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-2 CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-15kHzSCS-r17          ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-3 CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-NWA-15kHzSCS-r17      ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-4 CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-30kHzSCS-r17          ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-5 CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-NWA-30kHzSCS-r17      ENUMERATED {supported}                                                  OPTIONAL

}

-- TAG-FEATURESETDOWNLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC-Id

The IE FeatureSetDownlinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetDownlinkPerCC-Id of a FeatureSetDownlinkPerCC is the index position of the FeatureSetDownlinkPerCC in the featureSetsDownlinkPerCC. The first element in the list is referred to by FeatureSetDownlinkPerCC-Id = 1, and so on.

FeatureSetDownlinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-ID-START

FeatureSetDownlinkPerCC-Id ::=      INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETDOWNLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-DownlinkId

The IE FeatureSetEUTRA-DownlinkId identifies a downlink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-DownlinkId = 1. The FeatureSetEUTRA-DownlinkId=0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-DownlinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRADOWNLINKID-START

FeatureSetEUTRA-DownlinkId ::=      INTEGER (0..maxEUTRA-DL-FeatureSets)

-- TAG-FEATURESETEUTRADOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-UplinkId

The IE FeatureSetEUTRA-UplinkId identifies an uplink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-UplinkId = 1. The FeatureSetEUTRA-UplinkId =0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-UplinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRAUPLINKID-START

FeatureSetEUTRA-UplinkId ::=                    INTEGER (0..maxEUTRA-UL-FeatureSets)

-- TAG-FEATURESETEUTRAUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports for that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present). The number of entries in the new list(s) shall be the same as in the original list(s).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=    SEQUENCE {

featureSetsDownlink                 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink               OPTIONAL,

featureSetsDownlinkPerCC            SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC            OPTIONAL,

featureSetsUplink                   SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink                   OPTIONAL,

featureSetsUplinkPerCC              SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC              OPTIONAL,

...,

[[

featureSetsDownlink-v1540           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540         OPTIONAL,

featureSetsUplink-v1540             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540             OPTIONAL,

featureSetsUplinkPerCC-v1540        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540        OPTIONAL

]],

[[

featureSetsDownlink-v15a0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0         OPTIONAL

]],

[[

featureSetsDownlink-v1610           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610         OPTIONAL,

featureSetsUplink-v1610             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1610             OPTIONAL,

featureSetDownlinkPerCC-v1620       SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620      OPTIONAL

]],

[[

featureSetsUplink-v1630             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1630             OPTIONAL

]],

[[

featureSetsUplink-v1640             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1640             OPTIONAL

]],

[[

featureSetsDownlink-v1700           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1700         OPTIONAL,

featureSetsDownlinkPerCC-v1700      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1700      OPTIONAL,

featureSetsUplink-v1710             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1710             OPTIONAL,

featureSetsUplinkPerCC-v1700        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1700        OPTIONAL

]],

[[

featureSetsDownlink-v1720           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1720         OPTIONAL,

featureSetsDownlinkPerCC-v1720      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1720      OPTIONAL,

featureSetsUplink-v1720             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1720             OPTIONAL

]],

[[

featureSetsDownlink-v1730           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1730         OPTIONAL,

featureSetsDownlinkPerCC-v1730      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1730      OPTIONAL

]],

[[

featureSetsDownlink-v1800           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1800         OPTIONAL,

featureSetsDownlinkPerCC-v1800      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1800      OPTIONAL,

featureSetsUplink-v1800             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1800             OPTIONAL,

featureSetsUplinkPerCC-v1800        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1800        OPTIONAL

]]

}

FeatureSets-v16d0 ::=    SEQUENCE {

featureSetsUplink-v16d0             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v16d0             OPTIONAL

}

-- TAG-FEATURESETS-STOP

-- ASN1STOP

#### –	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=                SEQUENCE {

featureSetListPerUplinkCC           SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor                       ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy3                              ENUMERATED {supported}                                                  OPTIONAL,

intraBandFreqSeparationUL           FreqSeparationClass                                                     OPTIONAL,

searchSpaceSharingCA-UL             ENUMERATED {supported}                                                  OPTIONAL,

dummy1                              DummyI                                                                  OPTIONAL,

supportedSRS-Resources              SRS-Resources                                                           OPTIONAL,

twoPUCCH-Group                      ENUMERATED {supported}                                                  OPTIONAL,

dynamicSwitchSUL                    ENUMERATED {supported}                                                  OPTIONAL,

simultaneousTxSUL-NonSUL            ENUMERATED {supported}                                                  OPTIONAL,

pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-30kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-60kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-120kHz                                ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL

}                                                                                                           OPTIONAL,

dummy2                               DummyF                                                                 OPTIONAL

}

FeatureSetUplink-v1540 ::=           SEQUENCE {

zeroSlotOffsetAperiodicSRS           ENUMERATED {supported}                     OPTIONAL,

pa-PhaseDiscontinuityImpacts         ENUMERATED {supported}                     OPTIONAL,

pusch-SeparationWithGap              ENUMERATED {supported}                     OPTIONAL,

pusch-ProcessingType2                SEQUENCE {

scs-15kHz                            ProcessingParameters                       OPTIONAL,

scs-30kHz                            ProcessingParameters                       OPTIONAL,

scs-60kHz                            ProcessingParameters                       OPTIONAL

}                                                                               OPTIONAL,

ul-MCS-TableAlt-DynamicIndication    ENUMERATED {supported}                     OPTIONAL

}

FeatureSetUplink-v1610 ::=       SEQUENCE {

-- R1 11-5: PUsCH repetition Type B

pusch-RepetitionTypeB-r16        SEQUENCE {

maxNumberPUSCH-Tx-r16            ENUMERATED {n2, n3, n4, n7, n8, n12},

hoppingScheme-r16                ENUMERATED {interSlotHopping, interRepetitionHopping, both}

}                                                                              OPTIONAL,

-- R1 11-7: UL cancelation scheme for self-carrier

ul-CancellationSelfCarrier-r16       ENUMERATED {supported}                    OPTIONAL,

-- R1 11-7a: UL cancelation scheme for cross-carrier

ul-CancellationCrossCarrier-r16      ENUMERATED {supported}                    OPTIONAL,

-- R1 16-5c: The maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for Mode 2

ul-FullPwrMode2-MaxSRS-ResInSet-r16  ENUMERATED {n1, n2, n4}                   OPTIONAL,

-- R1 22-4a/4b/4c/4d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 1

cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16    SEQUENCE {

scs-15kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-30kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-60kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-120kHz-r16       ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL

} OPTIONAL,

-- R1 22-3a/3b/3c/3d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 2

cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16    SEQUENCE {

scs-15kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-30kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-60kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-120kHz-r16       ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL

} OPTIONAL,

supportedSRS-PosResources-r16              SRS-AllPosResources-r16             OPTIONAL,

intraFreqDAPS-UL-r16                             SEQUENCE {

dummy                                            ENUMERATED {supported}    OPTIONAL,

intraFreqTwoTAGs-DAPS-r16                        ENUMERATED {supported}    OPTIONAL,

dummy1                                           ENUMERATED {supported}    OPTIONAL,

dummy2                                           ENUMERATED {supported}    OPTIONAL,

dummy3                                           ENUMERATED {short, long}  OPTIONAL

}                                                                              OPTIONAL,

intraBandFreqSeparationUL-v1620                  FreqSeparationClassUL-v1620   OPTIONAL,

-- R1 11-3: More than one PUCCH for HARQ-ACK transmission within a slot

multiPUCCH-r16                        SEQUENCE {

sub-SlotConfig-NCP-r16                ENUMERATED {set1, set2}              OPTIONAL,

sub-SlotConfig-ECP-r16                ENUMERATED {set1, set2}              OPTIONAL

}                                                                              OPTIONAL,

-- R1 11-3c: 2 PUCCH of format 0 or 2 for a single 7*2-symbol subslot based HARQ-ACK codebook

twoPUCCH-Type1-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3d: 2 PUCCH of format 0 or 2 for a single 2*7-symbol subslot based HARQ-ACK codebook

twoPUCCH-Type2-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3e: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for a single 2*7-symbol HARQ-ACK codebooks

twoPUCCH-Type3-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3f: 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and

-- 11-3e

twoPUCCH-Type4-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3g: SR/HARQ-ACK multiplexing once per subslot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK

-- are supposed to be sent with different starting symbols in a subslot

mux-SR-HARQ-ACK-r16                   ENUMERATED {supported}                   OPTIONAL,

dummy1                                ENUMERATED {supported}                   OPTIONAL,

dummy2                                ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4c: 2 PUCCH of format 0 or 2 for two HARQ-ACK codebooks with one 7*2-symbol sub-slot based HARQ-ACK codebook

twoPUCCH-Type5-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4d: 2 PUCCH of format 0 or 2 in consecutive symbols for two HARQ-ACK codebooks with one 2*7-symbol sub-slot based HARQ-ACK

-- codebook

twoPUCCH-Type6-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4e: 2 PUCCH of format 0 or 2 for two subslot based HARQ-ACK codebooks

twoPUCCH-Type7-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4f: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for HARQ-ACK codebooks with one 2*7-symbol

-- subslot based HARQ-ACK codebook

twoPUCCH-Type8-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4g: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for two subslot based HARQ-ACK codebooks

twoPUCCH-Type9-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4h: 2 PUCCH transmissions in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot which are not covered

-- by 11-4c and 11-4e

twoPUCCH-Type10-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4i: 2 PUCCH transmissions in the same subslot for two subslot based HARQ-ACK codebooks which are not covered by 11-4d and

-- 11-4f

twoPUCCH-Type11-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 12-1: UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer

ul-IntraUE-Mux-r16                    SEQUENCE {

pusch-PreparationLowPriority-r16      ENUMERATED {sym0, sym1, sym2},

pusch-PreparationHighPriority-r16     ENUMERATED {sym0, sym1, sym2}

}                                                                              OPTIONAL,

-- R1 16-5a: Supported UL full power transmission mode of fullpower

ul-FullPwrMode-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 18-5d: Processing up to X unicast DCI scheduling for UL per scheduled CC

crossCarrierSchedulingProcessing-DiffSCS-r16    SEQUENCE {

scs-15kHz-120kHz-r16                  ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-15kHz-60kHz-r16                   ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-30kHz-120kHz-r16                  ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-15kHz-30kHz-r16                   ENUMERATED {n2}                      OPTIONAL,

scs-30kHz-60kHz-r16                   ENUMERATED {n2}                      OPTIONAL,

scs-60kHz-120kHz-r16                  ENUMERATED {n2}                      OPTIONAL

}                                                                              OPTIONAL,

-- R1 16-5b: Supported UL full power transmission mode of fullpowerMode1

ul-FullPwrMode1-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 16-5c-2: Ports configuration for Mode 2

ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16  ENUMERATED {p1-2, p1-4, p1-2-4} OPTIONAL,

-- R1 16-5c-3: TPMI group for Mode 2

ul-FullPwrMode2-TPMIGroup-r16         SEQUENCE {

twoPorts-r16                          BIT STRING(SIZE(2))                      OPTIONAL,

fourPortsNonCoherent-r16              ENUMERATED{g0, g1, g2, g3}               OPTIONAL,

fourPortsPartialCoherent-r16          ENUMERATED{g0, g1, g2, g3, g4, g5, g6}   OPTIONAL

}                                                                                  OPTIONAL

}

FeatureSetUplink-v1630 ::=       SEQUENCE {

-- R1 22-8: For SRS for CB PUSCH and antenna switching on FR1 with symbol level offset for aperiodic SRS transmission

offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8a: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot and constrained timeline for SRS for CB

-- PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16           ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8b: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16    ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8c: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 with a DCI gap and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16       ENUMERATED {supported}                   OPTIONAL,

dummy                                                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-9: Cancellation of PUCCH, PUSCH or PRACH with a DCI scheduling a PDSCH or CSI-RS or a DCI format 2_0 for SFI

partialCancellationPUCCH-PUSCH-PRACH-TX-r16                 ENUMERATED {supported}                   OPTIONAL

}

FeatureSetUplink-v1640 ::=              SEQUENCE {

-- R1 11-4: Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based +

-- sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE

twoHARQ-ACK-Codebook-type1-r16          SubSlot-Config-r16      OPTIONAL,

-- R1 11-4a: Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different

-- priorities at a UE

twoHARQ-ACK-Codebook-type2-r16          SubSlot-Config-r16      OPTIONAL,

-- R1 22-8d: All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap and constrained timeline

-- for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16 SEQUENCE {

scs-15kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL,

scs-30kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL,

scs-60kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL

}                                                                                                           OPTIONAL

}

FeatureSetUplink-v16d0 ::=       SEQUENCE {

pusch-RepetitionTypeB-v16d0      SEQUENCE {

maxNumberPUSCH-Tx-Cap1-r16       ENUMERATED {n2, n3, n4, n7, n8, n12},

maxNumberPUSCH-Tx-Cap2-r16       ENUMERATED {n2, n3, n4, n7, n8, n12}

}                                                                                         OPTIONAL

}

FeatureSetUplink-v1710 ::= SEQUENCE {

-- R1 23-3-1	Multi-TRP PUSCH repetition (type A) -codebook based

mTRP-PUSCH-TypeA-CB-r17                ENUMERATED {n1,n2,n4}                              OPTIONAL,

-- R1 23-3-1-2	Multi-TRP PUSCH repetition (type A) - non-codebook based

mTRP-PUSCH-RepetitionTypeA-r17         ENUMERATED {n1,n2,n3,n4}                           OPTIONAL,

-- R1 23-3-3	Multi-TRP PUCCH repetition-intra-slot

mTRP-PUCCH-IntraSlot-r17               ENUMERATED {pf0-2, pf1-3-4, pf0-4}                 OPTIONAL,

-- R1 23-8-4	Maximum 2 SP and 1 periodic SRS sets for antenna switching

srs-AntennaSwitching2SP-1Periodic-r17  ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-9	Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R

srs-ExtensionAperiodicSRS-r17          ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-10	1 aperiodic SRS resource set for 1T4R

srs-OneAP-SRS-r17                      ENUMERATED {supported}                             OPTIONAL,

-- R4 16-8 UE power class per band per band combination

ue-PowerClassPerBandPerBC-r17          ENUMERATED {pc1dot5, pc2, pc3}                     OPTIONAL,

-- R4 17-8 UL transmission in FR2 bands within an UL gap when the UL gap is activated

tx-Support-UL-GapFR2-r17               ENUMERATED {supported}                             OPTIONAL

}

FeatureSetUplink-v1720 ::= SEQUENCE {

-- R1 25-3: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8

pucch-Repetition-F0-1-2-3-4-RRC-Config-r17         ENUMERATED {supported}                 OPTIONAL,

-- R1 25-3a: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication

pucch-Repetition-F0-1-2-3-4-DynamicIndication-r17  ENUMERATED {supported}                 OPTIONAL,

-- R1 25-3b: Inter-subslot frequency hopping for PUCCH repetitions

interSubslotFreqHopping-PUCCH-r17                  ENUMERATED {supported}                 OPTIONAL,

-- R1 25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH

semiStaticHARQ-ACK-CodebookSub-SlotPUCCH-r17       ENUMERATED {supported}                 OPTIONAL,

-- R1 25-14: PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH

phy-PrioritizationLowPriorityDG-HighPriorityCG-r17 INTEGER(1..16)                         OPTIONAL,

-- R1 25-15: PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH

phy-PrioritizationHighPriorityDG-LowPriorityCG-r17 SEQUENCE {

pusch-PreparationLowPriority-r17                   ENUMERATED{sym0, sym1, sym2},

additionalCancellationTime-r17                     SEQUENCE {

scs-15kHz-r17                                      ENUMERATED{sym0, sym1, sym2}   OPTIONAL,

scs-30kHz-r17                                      ENUMERATED{sym0, sym1, sym2, sym3, sym4}    OPTIONAL,

scs-60kHz-r17                                      ENUMERATED{sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7, sym8} OPTIONAL,

scs-120kHz-r17                                     ENUMERATED{sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7, sym8, sym9,

sym10, sym11, sym12, sym13, sym14, sym15, sym16}    OPTIONAL

},

maxNumberCarriers-r17                              INTEGER(1..16)

}                                                                                         OPTIONAL,

-- R4 17-5 Support of UL DC location(s) report

extendedDC-LocationReport-r17                      ENUMERATED {supported}                 OPTIONAL

}

FeatureSetUplink-v1800 ::= SEQUENCE {

-- R1 40-3-3-1a: Supported maximum delay value larger than D_basic

maxDelayValueBeyondD-Basic-r18                     ENUMERATED {sl2,sl3,sl4,sl5,sl6,sl10}  OPTIONAL,

-- R1 40-3-3-2: Number of delay values

tdcpNumberDelayValue-r18                           INTEGER (2..4)                         OPTIONAL,

-- R1 40-3-3-4: Phase report

phaseReportMoreThanOne-r18                         ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6: Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports

pusch-TypeA-DMRS-r18                                           SEQUENCE {

-- R1 40-4-6d: 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH

pusch-2SymbolFL-DMRS-r18                           ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6e: 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH

pusch-2SymbolFL-DMRS-Addition2Symbol-r18           ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6f: 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH

pusch-1SymbolFL-DMRS-Addition3Symbol-r18           ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6g: 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4

pusch-rank-1-4-1Port-r18                           ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6h: 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8

pusch-rank-5-8-1Port-r18                           ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6i: 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4

pusch-rank-1-4-2Port-r18                           ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-6j: 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8

pusch-rank-5-8-2Port-r18                           ENUMERATED {supported}                 OPTIONAL

}                                                                                                                             OPTIONAL,

-- R1 40-4-6a: Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports

pusch-TypeB-DMRS-r18                                     ENUMERATED {supported}                     OPTIONAL,

-- R1 40-4-12: Support Rel-18 UL DMRS with single-DCI based M-TRP

ul-DMRS-SingleDCI-M-TRP-r18                        ENUMERATED {supported}                 OPTIONAL,

-- R1 40-4-13: Support Rel-18 UL DMRS with M-DCI based M-TRP

ul-DMRS-M-DCI-M-TRP-r18                            ENUMERATED {supported}                 OPTIONAL,

-- R1 40-5-5: Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching

srs-AntennaSwitching8T8R2SP-1Periodic-r18                   ENUMERATED {supported}                 OPTIONAL,

-- R1 40-6-4: Single-DCI based STx2P SFN scheme for PUCCH

pucch-SingleDCI-STx2P-SFN-r18                      ENUMERATED {pf0-2, pf1-3-4, pf0-4}     OPTIONAL,

-- R4 27-1 TxDiversity for 4Tx

txDiversity4Tx-r18                                 ENUMERATED {supported}                 OPTIONAL,

-- R4 44-1 TxDiversity for 2Tx

txDiversity2Tx-r18                                 ENUMERATED {supported}                 OPTIONAL

}

SubSlot-Config-r16 ::=                  SEQUENCE {

sub-SlotConfig-NCP-r16                  ENUMERATED {n4,n5,n6,n7}              OPTIONAL,

sub-SlotConfig-ECP-r16                  ENUMERATED {n4,n5,n6}                 OPTIONAL

}

SRS-AllPosResources-r16 ::=               SEQUENCE {

srs-PosResources-r16                      SRS-PosResources-r16,

srs-PosResourceAP-r16                     SRS-PosResourceAP-r16                OPTIONAL,

srs-PosResourceSP-r16                     SRS-PosResourceSP-r16                OPTIONAL

}

SRS-PosResources-r16 ::=                       SEQUENCE {

maxNumberSRS-PosResourceSetPerBWP-r16                ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSRS-ResourcesPerBWP-PerSlot-r16             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16          ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r16  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceAP-r16 ::=                SEQUENCE {

maxNumberAP-SRS-PosResourcesPerBWP-r16         ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-PerSlot-r16 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceSP-r16 ::=                       SEQUENCE {

maxNumberSP-SRS-PosResourcesPerBWP-r16               ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSP-SRS-PosResourcesPerBWP-PerSlot-r16       ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-Resources ::=                           SEQUENCE {

maxNumberAperiodicSRS-PerBWP                ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot        INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP                 ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot         INTEGER (1..6),

maxNumberSemiPersistentSRS-PerBWP           ENUMERATED {n1, n2, n4, n8, n16},

maxNumberSemiPersistentSRS-PerBWP-PerSlot   INTEGER (1..6),

maxNumberSRS-Ports-PerResource              ENUMERATED {n1, n2, n4}

}

DummyF ::=                                  SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP           INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP          INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP     INTEGER (0..4),

simultaneousCSI-ReportsAllCC                INTEGER (5..32)

}

-- TAG-FEATURESETUPLINK-STOP

-- ASN1STOP

FeatureSetUplink field descriptions

featureSetListPerUplinkCC Indicates which features the UE supports on the individual UL carriers of the feature set (and hence of a band entry that refers to the feature set). The UE shall hence include at least as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetUplinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

#### –	FeatureSetUplinkId

The IE FeatureSetUplinkId identifies an uplink feature set. The FeatureSetUplinkId of a FeatureSetUplink is the index position of the FeatureSetUplink in the featureSetsUplink list in the FeatureSets IE. The first element in the list is referred to by FeatureSetUplinkId = 1, and so on. The FeatureSetUplinkId =0 is not used by an actual FeatureSetUplink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetUplinkId information element

-- ASN1START

-- TAG-FEATURESETUPLINKID-START

FeatureSetUplinkId ::=                  INTEGER (0..maxUplinkFeatureSets)

-- TAG-FEATURESETUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC

The IE FeatureSetUplinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetUplinkPerCC information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-START

FeatureSetUplinkPerCC ::=               SEQUENCE {

supportedSubcarrierSpacingUL            SubcarrierSpacing,

supportedBandwidthUL                    SupportedBandwidth,

channelBW-90mhz                         ENUMERATED {supported}                      OPTIONAL,

mimo-CB-PUSCH                           SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH            MIMO-LayersUL                               OPTIONAL,

maxNumberSRS-ResourcePerSet             INTEGER (1..2)

}                                                                                   OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH         MIMO-LayersUL                               OPTIONAL,

supportedModulationOrderUL              ModulationOrder                             OPTIONAL

}

FeatureSetUplinkPerCC-v1540 ::=       SEQUENCE {

mimo-NonCB-PUSCH                      SEQUENCE {

maxNumberSRS-ResourcePerSet           INTEGER (1..4),

maxNumberSimultaneousSRS-ResourceTx   INTEGER (1..4)

} OPTIONAL

}

FeatureSetUplinkPerCC-v1700 ::=   SEQUENCE {

supportedMinBandwidthUL-r17       SupportedBandwidth-v1700                          OPTIONAL,

-- R1 23-3-1-3	FeMIMO: Multi-TRP PUSCH repetition (type B) - non-codebook based

mTRP-PUSCH-RepetitionTypeB-r17    ENUMERATED {n1,n2,n3,n4}                          OPTIONAL,

-- R1 23-3-1-1 -codebook based Multi-TRP PUSCH repetition (type B)

mTRP-PUSCH-TypeB-CB-r17           ENUMERATED {n1,n2,n4}                             OPTIONAL,

supportedBandwidthUL-v1710        SupportedBandwidth-v1700                          OPTIONAL

}

FeatureSetUplinkPerCC-v1800 ::=   SEQUENCE {

-- R1 40-2-7: Two TAs for multi-DCI STxMP PUSCH+PUSCH

twoPUSCH-MultiDCI-STxMP-TwoTA-r18      ENUMERATED {supported}                       OPTIONAL,

-- R1 40-6-1: Single-DCI based STx2P SDM scheme for PUSCH—codebook

pusch-CB-SingleDCI-STx2P-SDM-r18       SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             ENUMERATED {n1,n2,n4},

maxNumberLayerPerPanel-r18                  INTEGER (1..2),

maxNumberNZP-PUSCH-PortsPerSet-r18          ENUMERATED {n1,n2,n4},

maxNumberSRS-AntennaPortsPerSet-r18         ENUMERATED {n1,n2,n4}

}                                                                                   OPTIONAL,

-- R1 40-6-1a: Single-DCI based STx2P SDM scheme for PUSCH—noncodebook

pusch-NonCB-SingleDCI-STx2P-SDM-r18    SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             INTEGER (1..4),

maxNumberLayerPerPanel-r18                  INTEGER (1..2),

maxNumberSimulSRS-ResourcePerSet-r18        INTEGER (1..4)

}                                                                                   OPTIONAL,

-- R1 40-6-2: Single-DCI based STx2P SFN scheme for PUSCH—codebook

pusch-CB-SingleDCI-STx2P-SFN-r18       SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             ENUMERATED {n1,n2,n4},

maxNumberLayerPerSet-r18                    INTEGER (1..2),

maxNumberSRS-AntennaPortsPerSet-r18         ENUMERATED {n1,n2,n4},

maxNumberNZP-PUSCH-PortsPerSet-r18          ENUMERATED {n1,n2,n4}

}                                                                                   OPTIONAL,

-- R1 40-6-2a: Single-DCI based STx2P SFN scheme for PUSCH—noncodebook

pusch-NonCB-SingleDCI-STx2P-SFN-r18    SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             INTEGER (1..4),

maxNumberLayerPerSet-r18                    INTEGER (1..2),

maxNumberSimulSRS-ResourcePerSet-r18        INTEGER (1..4)

}                                                                                   OPTIONAL,

-- R1 40-6-3a: codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18   SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             ENUMERATED {n1, n2, n4},

maxNumberLayerOverlapping-r18               INTEGER (1..2),

maxNumberNZP-PUSCH-Overlapping-r18          ENUMERATED {n1, n2, n4},

maxNumberPUSCH-PerCORESET-PerSlot-r18       SEQUENCE {

scs-60kHz-r18                             ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL,

scs-120kHz-r18                            ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL

}                                                                              OPTIONAL,

maxNumberTotalLayerOverlapping-r18          INTEGER (2..4),

maxNumberSRS-AntennaPortsPerSet-r18         ENUMERATED {n1,n2,n4}

}                                                                                   OPTIONAL,

-- R1 40-6-3b: Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18    SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             INTEGER (1..4),

maxNumberLayerOverlapping-r18               INTEGER (1..2),

maxNumberSimulSRS-ResourcePerSet-r18        INTEGER (1..4),

maxNumberPUSCH-PerCORESET-PerSlot-r18       SEQUENCE {

scs-60kHz-r18                             ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL,

scs-120kHz-r18                            ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL

}                                                                              OPTIONAL,

maxNumberTotalLayerOverlapping-r18          INTEGER (2..4)

}                                                                                   OPTIONAL,

-- R1 40-6-6: Out-of-order operation for multi-DCI based STx2P PUSCH+PUSCH

twoPUSCH-MultiDCI-STx2P-OutOfOrder-r18           ENUMERATED {supported}             OPTIONAL,

-- R1 40-7-1a: Codebook-based 8Tx PUSCH—codebook1

codebook1-8TxPUSCH-r18               ENUMERATED {n4-1,n2-2,both}                 OPTIONAL,

-- R1 40-7-1b: Codebook-based 8Tx PUSCH—codebook2

codebook2-8TxPUSCH-r18               ENUMERATED {supported}                      OPTIONAL,

-- R1 40-7-1c: Codebook-based 8Tx PUSCH—codebook3

codebook3-8TxPUSCH-r18               ENUMERATED {supported}                      OPTIONAL,

-- R1 40-7-1d: Codebook-based 8Tx PUSCH—codebook4

codebook4-8TxPUSCH-r18               ENUMERATED {supported}                      OPTIONAL

-- R1 40-7-1e: UL full power transmission mode 0

ul-FullPwrTransMode0-r18             ENUMERATED {supported}                      OPTIONAL,

-- R1 40-7-1f: UL full power transmission mode 1

ul-FullPwrTransMode1-r18             ENUMERATED {supported}                      OPTIONAL,

-- R1 40-7-1g: UL full power transmission mode 2 with 1/2/4 resources

ul-FullPwrTransMode2-r18             ENUMERATED {n1,n2,n4}                       OPTIONAL,

-- R1 40-7-2: Basic features for Non-Codebook-based 8Tx PUSCH

noneCodebook-8TxPUSCH-r18            ENUMERATED {supported}                      OPTIONAL,

-- R1 40-7-3: CBG based 2 CWs PUSCH with rank >4

cgb-2CW-PUSCH-r18                    ENUMERATED {supported}                      OPTIONAL

}

-- TAG-FEATURESETUPLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC-Id

The IE FeatureSetUplinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetUplinkPerCC-Id of a FeatureSetUplinkPerCC is the index position of the FeatureSetUplinkPerCC in the featureSetsUplinkPerCC. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.

FeatureSetUplinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-ID-START

FeatureSetUplinkPerCC-Id ::=            INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETUPLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQBANDINDICATOREUTRA-START

FreqBandIndicatorEUTRA ::=  INTEGER (1..maxBandsEUTRA)

-- TAG-FREQBANDINDICATOREUTRA-STOP

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used by the network to request NR CA, NR non-CA and/or MR-DC band combinations for specific NR and/or E-UTRA frequency bands and/or up to a specific number of carriers and/or up to specific aggregated bandwidth. This is also used to request feature sets (for NR) and feature set combinations (for NR and MR-DC). For NR sidelink communication, this is used by the initiating UE to request sidelink UE radio access capabilities from the peer UE. This is also used to request lower MSD capability for specific NR frequency bands for the UE supporting lower MSD.

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=                SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::=         CHOICE {

bandInformationEUTRA            FreqBandInformationEUTRA,

bandInformationNR               FreqBandInformationNR

}

FreqBandInformationEUTRA ::=    SEQUENCE {

bandEUTRA                       FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL,   -- Need N

ca-BandwidthClassUL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL    -- Need N

}

FreqBandInformationNR ::=       SEQUENCE {

bandNR                          FreqBandIndicatorNR,

maxBandwidthRequestedDL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxBandwidthRequestedUL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxCarriersRequestedDL          INTEGER (1..maxNrofServingCells)        OPTIONAL,   -- Need N

maxCarriersRequestedUL          INTEGER (1..maxNrofServingCells)        OPTIONAL    -- Need N

}

AggregatedBandwidth ::=         ENUMERATED {mhz50, mhz100, mhz150, mhz200, mhz250, mhz300, mhz350,

mhz400, mhz450, mhz500, mhz550, mhz600, mhz650, mhz700, mhz750, mhz800}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

#### –	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::= ENUMERATED { mhz800, mhz1200, mhz1400, ..., mhz400-v1650, mhz600-v1650}

FreqSeparationClassDL-v1620 ::= ENUMERATED {mhz1000, mhz1600, mhz1800, mhz2000, mhz2200, mhz2400}

FreqSeparationClassUL-v1620 ::= ENUMERATED {mhz1000}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	FreqSeparationClassDL-Only

The IE FreqSeparationClassDL-Only is used to indicate the frequency separation between lower edge of lowest CC and upper edge of highest CC of DL only frequency spectrum in a frequency band.

FreqSeparationClassDL-Only information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASSDL-Only-START

FreqSeparationClassDL-Only-r16 ::= ENUMERATED {mhz200, mhz400, mhz600, mhz800, mhz1000, mhz1200}

-- TAG-FREQSEPARATIONCLASSDL-Only-STOP

-- ASN1STOP

#### –	FR2-2-AccessParamsPerBand

The IE FR2-2-AccessParamsPerBand is used to convey FR2-2 related parameters specific for a certain frequency band (not per feature set or band combination).

FR2-2-AccessParamsPerBand information element

-- ASN1START

-- TAG-FR2-2-ACCESSPARAMSPERBAND-START

FR2-2-AccessParamsPerBand-r17 ::=       SEQUENCE {

-- R1 24-1: Basic FR2-2 DL support

dl-FR2-2-SCS-120kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-1a: Basic FR2-2 UL support

ul-FR2-2-SCS-120kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-2: 120KHz SSB support for initial access in FR2-2

initialAccessSSB-120kHz-r17             ENUMERATED {supported}            OPTIONAL,

-- R1 24-1b: Wideband PRACH for 120 kHz in FR2-2

widebandPRACH-SCS-120kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-1c: Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2

multiRB-PUCCH-SCS-120kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-1d: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2

multiPDSCH-SingleDCI-FR2-2-SCS-120kHz-r17 ENUMERATED {supported}          OPTIONAL,

-- R1 24-1e: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2

multiPUSCH-SingleDCI-FR2-2-SCS-120kHz-r17 ENUMERATED {supported}          OPTIONAL,

-- R1 24-4: 480KHz SCS support for DL

dl-FR2-2-SCS-480kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-4a: 480KHz SCS support for UL

ul-FR2-2-SCS-480kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-3: 480KHz SSB support for initial access in FR2-2

initialAccessSSB-480kHz-r17             ENUMERATED {supported}            OPTIONAL,

-- R1 24-4b: Wideband PRACH for 480 kHz in FR2-2

widebandPRACH-SCS-480kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-4c: Multi-RB support PUCCH format 0/1/4 for 480 kHz in FR2-2

multiRB-PUCCH-SCS-480kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-4f: Enhanced PDCCH monitoring for 480KHz in FR2-2

enhancedPDCCH-monitoringSCS-480kHz-r17  ENUMERATED {supported}            OPTIONAL,

-- R1 24-5: 960KHz SCS support for DL

dl-FR2-2-SCS-960kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-5a: 960KHz SCS support for UL

ul-FR2-2-SCS-960kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-5c: Multi-RB support PUCCH format 0/1/4 for 960 kHz in FR2-2

multiRB-PUCCH-SCS-960kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-5f: Enhanced PDCCH monitoring for 960KHz in FR2-2

enhancedPDCCH-monitoringSCS-960kHz-r17  SEQUENCE {

pdcch-monitoring4-1-r17                 ENUMERATED {supported}        OPTIONAL,

pdcch-monitoring4-2-r17                 ENUMERATED {supported}        OPTIONAL,

pdcch-monitoring8-4-r17                 ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

-- R1 24-6: Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access

type1-ChannelAccess-FR2-2-r17           ENUMERATED {supported}            OPTIONAL,

-- R1 24-7: Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access

type2-ChannelAccess-FR2-2-r17           ENUMERATED {supported}            OPTIONAL,

-- R1 24-10: Reduced beam switching time delay

reduced-BeamSwitchTiming-FR2-2-r17      ENUMERATED {supported}            OPTIONAL,

-- R1 24-8: 32 DL HARQ processes for FR 2-2

support32-DL-HARQ-ProcessPerSCS-r17     SEQUENCE {

scs-120kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-480kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-960kHz-r17                          ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

-- R1 24-9: 32 UL HARQ processes for FR 2-2

support32-UL-HARQ-ProcessPerSCS-r17		SEQUENCE {

scs-120kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-480kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-960kHz-r17                          ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

...,

[[

-- R4 15-1: 64QAM for PUSCH for FR2-2

modulation64-QAM-PUSCH-FR2-2-r17            ENUMERATED {supported}        OPTIONAL

]]

}

-- TAG-FR2-2-ACCESSPARAMSPERBAND-STOP

-- ASN1STOP

#### –	HighSpeedParameters

The IE HighSpeedParameters is used to convey capabilities related to high speed scenarios.

HighSpeedParameters information element

-- ASN1START

-- TAG-HIGHSPEEDPARAMETERS-START

HighSpeedParameters-r16 ::= SEQUENCE {

measurementEnhancement-r16       ENUMERATED {supported}   OPTIONAL,

demodulationEnhancement-r16      ENUMERATED {supported}   OPTIONAL

}

HighSpeedParameters-v1650 ::= CHOICE {

intraNR-MeasurementEnhancement-r16       ENUMERATED {supported},

interRAT-MeasurementEnhancement-r16      ENUMERATED {supported}

}

HighSpeedParameters-v1700 ::= SEQUENCE {

-- R4 18-1: Enhanced RRM requirements specified for CA for FR1 HST

measurementEnhancementCA-r17            ENUMERATED {supported}   OPTIONAL,

-- R4 18-2: Enhanced RRM requirements specified for inter-frequency measurement in connected mode for FR1 HST

measurementEnhancementInterFreq-r17     ENUMERATED {supported}   OPTIONAL

}

-- TAG-HIGHSPEEDPARAMETERS-STOP

-- ASN1STOP

#### –	IMS-Parameters

The IE IMS-Parameters is used to convey capabilities related to IMS.

IMS-Parameters information element

-- ASN1START

-- TAG-IMS-PARAMETERS-START

IMS-Parameters ::=         SEQUENCE {

ims-ParametersCommon       IMS-ParametersCommon                  OPTIONAL,

ims-ParametersFRX-Diff     IMS-ParametersFRX-Diff                OPTIONAL,

...

}

IMS-Parameters-v1700 ::=   SEQUENCE {

ims-ParametersFR2-2-r17    IMS-ParametersFR2-2-r17               OPTIONAL

}

IMS-ParametersCommon ::=   SEQUENCE {

voiceOverEUTRA-5GC                  ENUMERATED {supported}                OPTIONAL,

...,

[[

voiceOverSCG-BearerEUTRA-5GC        ENUMERATED {supported}                OPTIONAL

]],

[[

voiceFallbackIndicationEPS-r16       ENUMERATED {supported}                   OPTIONAL

]]

}

IMS-ParametersFRX-Diff ::= SEQUENCE {

voiceOverNR                ENUMERATED {supported}                OPTIONAL,

...

}

IMS-ParametersFR2-2-r17 ::= SEQUENCE {

voiceOverNR-r17             ENUMERATED {supported}               OPTIONAL,

...

}

-- TAG-IMS-PARAMETERS-STOP

-- ASN1STOP

#### –	InterRAT-Parameters

The IE InterRAT-Parameters is used convey UE capabilities related to the other RATs.

InterRAT-Parameters information element

-- ASN1START

-- TAG-INTERRAT-PARAMETERS-START

InterRAT-Parameters ::=             SEQUENCE {

eutra                               EUTRA-Parameters                OPTIONAL,

...,

[[

utra-FDD-r16                        UTRA-FDD-Parameters-r16         OPTIONAL

]]

}

EUTRA-Parameters ::=                SEQUENCE {

supportedBandListEUTRA          SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA,

eutra-ParametersCommon              EUTRA-ParametersCommon                                      OPTIONAL,

eutra-ParametersXDD-Diff            EUTRA-ParametersXDD-Diff                                    OPTIONAL,

...

}

EUTRA-ParametersCommon ::=      SEQUENCE {

mfbi-EUTRA                          ENUMERATED {supported}          OPTIONAL,

modifiedMPR-BehaviorEUTRA           BIT STRING (SIZE (32))          OPTIONAL,

multiNS-Pmax-EUTRA                  ENUMERATED {supported}          OPTIONAL,

rs-SINR-MeasEUTRA                   ENUMERATED {supported}          OPTIONAL,

...,

[[

ne-DC                               ENUMERATED {supported}          OPTIONAL

]],

[[

nr-HO-ToEN-DC-r16                   ENUMERATED {supported}          OPTIONAL

]]

}

EUTRA-ParametersXDD-Diff ::=        SEQUENCE {

rsrqMeasWidebandEUTRA               ENUMERATED {supported}          OPTIONAL,

...

}

UTRA-FDD-Parameters-r16 ::=                SEQUENCE {

supportedBandListUTRA-FDD-r16              SEQUENCE (SIZE (1..maxBandsUTRA-FDD-r16)) OF SupportedBandUTRA-FDD-r16,

...

}

SupportedBandUTRA-FDD-r16 ::=           ENUMERATED {

bandI, bandII, bandIII, bandIV, bandV, bandVI,

bandVII, bandVIII, bandIX, bandX, bandXI,

bandXII, bandXIII, bandXIV, bandXV, bandXVI,

bandXVII, bandXVIII, bandXIX, bandXX,

bandXXI, bandXXII, bandXXIII, bandXXIV,

bandXXV, bandXXVI, bandXXVII, bandXXVIII,

bandXXIX, bandXXX, bandXXXI, bandXXXII}

-- TAG-INTERRAT-PARAMETERS-STOP

-- ASN1STOP

#### –	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-Parameters-v1610 ::= SEQUENCE {

mac-ParametersFRX-Diff-r16      MAC-ParametersFRX-Diff-r16  OPTIONAL

}

MAC-Parameters-v1700 ::= SEQUENCE {

mac-ParametersFR2-2-r17         MAC-ParametersFR2-2-r17     OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                         ENUMERATED {supported}      OPTIONAL,

dummy                                   ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction                  ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate                      ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery                 ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16         ENUMERATED {supported}     OPTIONAL,

preEmptiveBSR-r16                        ENUMERATED {supported}     OPTIONAL,

autonomousTransmission-r16               ENUMERATED {supported}     OPTIONAL,

lch-PriorityBasedPrioritization-r16      ENUMERATED {supported}     OPTIONAL,

lch-ToConfiguredGrantMapping-r16         ENUMERATED {supported}     OPTIONAL,

lch-ToGrantPriorityRestriction-r16       ENUMERATED {supported}     OPTIONAL,

singlePHR-P-r16                          ENUMERATED {supported}     OPTIONAL,

ul-LBT-FailureDetectionRecovery-r16      ENUMERATED {supported}     OPTIONAL,

-- R4 8-1: MPE

tdd-MPE-P-MPR-Reporting-r16              ENUMERATED {supported}     OPTIONAL,

lcid-ExtensionIAB-r16                    ENUMERATED {supported}     OPTIONAL

]],

[[

spCell-BFR-CBRA-r16                      ENUMERATED {supported}     OPTIONAL

]],

[[

srs-ResourceId-Ext-r16                   ENUMERATED {supported}     OPTIONAL

]],

[[

enhancedUuDRX-forSidelink-r17            ENUMERATED {supported}     OPTIONAL,

--27-10: Support of UL MAC CE based MG activation request for PRS measurements

mg-ActivationRequestPRS-Meas-r17         ENUMERATED {supported}     OPTIONAL,

--27-11: Support of DL MAC CE based MG activation request for PRS measurements

mg-ActivationCommPRS-Meas-r17            ENUMERATED {supported}     OPTIONAL,

intraCG-Prioritization-r17               ENUMERATED {supported}     OPTIONAL,

jointPrioritizationCG-Retx-Timer-r17     ENUMERATED {supported}     OPTIONAL,

survivalTime-r17                         ENUMERATED {supported}     OPTIONAL,

lcg-ExtensionIAB-r17                     ENUMERATED {supported}     OPTIONAL,

harq-FeedbackDisabled-r17                ENUMERATED {supported}     OPTIONAL,

uplink-Harq-ModeB-r17                    ENUMERATED {supported}     OPTIONAL,

sr-TriggeredBy-TA-Report-r17             ENUMERATED {supported}     OPTIONAL,

extendedDRX-CycleInactive-r17            ENUMERATED {supported}     OPTIONAL,

simultaneousSR-PUSCH-DiffPUCCH-groups-r17 ENUMERATED {supported}    OPTIONAL,

lastTransmissionUL-r17                   ENUMERATED {supported}     OPTIONAL

]],

[[

sr-TriggeredByTA-ReportATG-r18           ENUMERATED {supported}     OPTIONAL,

-- similar to R1 26-4: UE reporting of information related to TA pre-compensation defined for ATG

uplinkTA-ReportingATG-r18                ENUMERATED {supported}     OPTIONAL,

extendedDRX-CycleInactive-r18            ENUMERATED {supported}     OPTIONAL

]]

}

MAC-ParametersFRX-Diff-r16 ::=  SEQUENCE {

directMCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

-- R1 19-1: DRX Adaptation

drx-Adaptation-r16          SEQUENCE {

non-SharedSpectrumChAccess-r16      MinTimeGap-r16              OPTIONAL,

sharedSpectrumChAccess-r16          MinTimeGap-r16              OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersFR2-2-r17 ::=  SEQUENCE {

directMCG-SCellActivation-r17           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r17     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r17           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r17     ENUMERATED {supported}      OPTIONAL,

drx-Adaptation-r17       SEQUENCE {

non-SharedSpectrumChAccess-r17      MinTimeGapFR2-2-r17         OPTIONAL,

sharedSpectrumChAccess-r17          MinTimeGapFR2-2-r17         OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic                     ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer             ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                           ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                          ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations               ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants                ENUMERATED {supported}     OPTIONAL,

...,

[[

secondaryDRX-Group-r16                  ENUMERATED {supported}     OPTIONAL

]],

[[

enhancedSkipUplinkTxDynamic-r16         ENUMERATED {supported}     OPTIONAL,

enhancedSkipUplinkTxConfigured-r16      ENUMERATED {supported}     OPTIONAL

]],

[[

ptm-Retransmission-r18                  ENUMERATED {supported}     OPTIONAL,

ptm-RetransmissionInactive-r18          ENUMERATED {supported}     OPTIONAL

]]

}

MinTimeGap-r16 ::=    SEQUENCE {

scs-15kHz-r16                         ENUMERATED {sl1, sl3}        OPTIONAL,

scs-30kHz-r16                         ENUMERATED {sl1, sl6}        OPTIONAL,

scs-60kHz-r16                         ENUMERATED {sl1, sl12}       OPTIONAL,

scs-120kHz-r16                        ENUMERATED {sl2, sl24}       OPTIONAL

}

MinTimeGapFR2-2-r17 ::= SEQUENCE {

scs-120kHz-r17                        ENUMERATED {sl2, sl24}       OPTIONAL,

scs-480kHz-r17                        ENUMERATED {sl8, sl96}       OPTIONAL,

scs-960kHz-r17                        ENUMERATED {sl16, sl192}     OPTIONAL

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE {

concurrentPerUE-OnlyMeasGap-r17         ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17    ENUMERATED {supported}

}                                                                               OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

nr-NeedForGapNCSG-Reporting-r17         ENUMERATED {supported}                  OPTIONAL,

eutra-NeedForGapNCSG-Reporting-r17      ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-1 per FR Network controlled small gap (NCSG)

ncsg-MeasGapPerFR-r17                   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-2 Network controlled small gap (NCSG) supported patterns

ncsg-MeasGapPatterns-r17                BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-1-3 Network controlled small gap (NCSG) supported NR-only patterns

ncsg-MeasGapNR-Patterns-r17             BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL,

rrm-RelaxationRRC-ConnectedRedCap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 25-3: Parallel measurements with multiple measurement gaps

parallelMeasurementGap-r17              ENUMERATED {n2}                         OPTIONAL,

condHandoverWithSCG-NRDC-r17            ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-r17              ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-ENDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NEDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NRDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NPN-r17          ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R4 25-1: Parallel measurements on multiple SMTC-s for a single frequency carrier

parallelSMTC-r17                        ENUMERATED {n4}                         OPTIONAL,

-- R4 19-2-1 Concurrent measurement gaps for EUTRA

concurrentMeasGapEUTRA-r17              ENUMERATED {supported}                  OPTIONAL,

serviceLinkPropDelayDiffReporting-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-4 Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter

ncsg-SymbolLevelScheduleRestrictionInter-r17  ENUMERATED {supported}            OPTIONAL

]],

[[

eventD1-MeasReportTrigger-r17           ENUMERATED {supported}                  OPTIONAL,

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               OPTIONAL

]],

[[

interSatMeas-r17                            ENUMERATED {supported}              OPTIONAL,

deriveSSB-IndexFromCellInterNon-NCSG-r17    ENUMERATED {supported}              OPTIONAL

]],

[[

-- R4 31-1 Enhanced L3 measurement reporting for unknown SCell activation if the valid L3 measurement results are available

l3-MeasUnknownSCellActivation-r18           ENUMERATED {supported}              OPTIONAL,

-- R4 31-3 Shorter measurement interval for unknown SCell activation

shortMeasInterval-r18                       ENUMERATED {supported}              OPTIONAL,

nr-NeedForInterruptionReport-r18            ENUMERATED {supported}              OPTIONAL,

measSequenceConfig-r18                      ENUMERATED {supported}              OPTIONAL,

cellIndividualOffsetPerMeasEvent-r18        ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=        SEQUENCE {

intraAndInterF-MeasAndReport            ENUMERATED {supported}                  OPTIONAL,

eventA-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

...,

[[

handoverInterF                          ENUMERATED {supported}                  OPTIONAL,

handoverLTE-EPC                         ENUMERATED {supported}                  OPTIONAL,

handoverLTE-5GC                         ENUMERATED {supported}                  OPTIONAL

]],

[[

sftd-MeasNR-Neigh                       ENUMERATED {supported}                  OPTIONAL,

sftd-MeasNR-Neigh-DRX                   ENUMERATED {supported}                  OPTIONAL

]],

[[

dummy                                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                  ENUMERATED {supported}              OPTIONAL,

dummy                                       ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                       ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}          OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL,

-- R4 6-2: Support of beam level Early Measurement Reporting

idleInactiveNR-MeasBeamReport-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

increasedNumberofCSIRSPerMO-r16             ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersFR2-2-r17 ::=           SEQUENCE {

handoverInterF-r17                          ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC-r17                         ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC-r17                         ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r17               ENUMERATED {supported}              OPTIONAL,

...

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560      OPTIONAL

}

MeasAndMobParametersMRDC-v1610 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1610      MeasAndMobParametersMRDC-Common-v1610        OPTIONAL,

interNR-MeasEUTRA-IAB-r16                  ENUMERATED {supported}                       OPTIONAL

}

MeasAndMobParametersMRDC-v1700 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1700      MeasAndMobParametersMRDC-Common-v1700        OPTIONAL

}

MeasAndMobParametersMRDC-v1730 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1730   MeasAndMobParametersMRDC-Common-v1730           OPTIONAL

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1610 ::=   SEQUENCE {

condPSCellChangeParametersCommon-r16        SEQUENCE {

condPSCellChangeFDD-TDD-r16                 ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeFR1-FR2-r16                 ENUMERATED {supported}                  OPTIONAL

}                                                                                       OPTIONAL,

pscellT312-r16                              ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1700 ::=   SEQUENCE {

condPSCellChangeParameters-r17              SEQUENCE {

inter-SN-condPSCellChangeFDD-TDD-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFDD-TDD-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL

}                                                                                       OPTIONAL,

condHandoverWithSCG-ENDC-r17                        ENUMERATED {supported}              OPTIONAL,

condHandoverWithSCG-NEDC-r17                        ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1730 ::= SEQUENCE {

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                             OPTIONAL

}

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                          OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}                   OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}                   OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

#### –	MIMO-Layers

The IE MIMO-Layers is used to convey the number of supported MIMO layers.

MIMO-Layers information element

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=   ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=   ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

#### –	MIMO-ParametersPerBand

The IE MIMO-ParametersPerBand is used to convey MIMO related parameters specific for a certain band (not per feature set or band combination).

MIMO-ParametersPerBand information element

-- ASN1START

-- TAG-MIMO-PARAMETERSPERBAND-START

MIMO-ParametersPerBand ::=          SEQUENCE {

tci-StatePDSCH                      SEQUENCE {

maxNumberConfiguredTCI-StatesPerCC  ENUMERATED {n4, n8, n16, n32, n64, n128}                                   OPTIONAL,

maxNumberActiveTCI-PerBWP           ENUMERATED {n1, n2, n4, n8}                                                OPTIONAL

}                                                                                                              OPTIONAL,

additionalActiveTCI-StatePDCCH              ENUMERATED {supported}                                             OPTIONAL,

pusch-TransCoherence                        ENUMERATED {nonCoherent, partialCoherent, fullCoherent}            OPTIONAL,

beamCorrespondenceWithoutUL-BeamSweeping    ENUMERATED {supported}                                             OPTIONAL,

periodicBeamReport                          ENUMERATED {supported}                                             OPTIONAL,

aperiodicBeamReport                         ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUCCH                          ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUSCH                          ENUMERATED {supported}                                             OPTIONAL,

dummy1                                      DummyG                                                             OPTIONAL,

maxNumberRxBeam                             INTEGER (2..8)                                                     OPTIONAL,

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

scs-15kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-30kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-60kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-120kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL

}                                                                                                              OPTIONAL,

maxNumberNonGroupBeamReporting              ENUMERATED {n1, n2, n4}                                            OPTIONAL,

groupBeamReporting                          ENUMERATED {supported}                                             OPTIONAL,

uplinkBeamManagement                        SEQUENCE {

maxNumberSRS-ResourcePerSet-BM              ENUMERATED {n2, n4, n8, n16},

maxNumberSRS-ResourceSet                    INTEGER (1..8)

}                                                                                                              OPTIONAL,

maxNumberCSI-RS-BFD                 INTEGER (1..64)                                                            OPTIONAL,

maxNumberSSB-BFD                    INTEGER (1..64)                                                            OPTIONAL,

maxNumberCSI-RS-SSB-CBD             INTEGER (1..256)                                                           OPTIONAL,

dummy2                              ENUMERATED {supported}                                                     OPTIONAL,

twoPortsPTRS-UL                     ENUMERATED {supported}                                                     OPTIONAL,

dummy5                              SRS-Resources                                                              OPTIONAL,

dummy3                              INTEGER (1..4)                                                             OPTIONAL,

beamReportTiming                    SEQUENCE {

scs-15kHz                           ENUMERATED {sym2, sym4, sym8}                                              OPTIONAL,

scs-30kHz                           ENUMERATED {sym4, sym8, sym14, sym28}                                      OPTIONAL,

scs-60kHz                           ENUMERATED {sym8, sym14, sym28}                                            OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym56}                                           OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetDL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationDL                                               OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetUL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationUL                                               OPTIONAL

}                                                                                                              OPTIONAL,

dummy4                              DummyH                                                                     OPTIONAL,

aperiodicTRS                        ENUMERATED {supported}                                                     OPTIONAL,

...,

[[

dummy6                              ENUMERATED {true}                                                          OPTIONAL,

beamManagementSSB-CSI-RS            BeamManagementSSB-CSI-RS                                                   OPTIONAL,

beamSwitchTiming                    SEQUENCE {

scs-60kHz                           ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL

}                                                                                                              OPTIONAL,

codebookParameters                  CodebookParameters                                                         OPTIONAL,

csi-RS-IM-ReceptionForFeedback      CSI-RS-IM-ReceptionForFeedback                                             OPTIONAL,

csi-RS-ProcFrameworkForSRS          CSI-RS-ProcFrameworkForSRS                                                 OPTIONAL,

csi-ReportFramework                 CSI-ReportFramework                                                        OPTIONAL,

csi-RS-ForTracking                  CSI-RS-ForTracking                                                         OPTIONAL,

srs-AssocCSI-RS                     SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource  OPTIONAL,

spatialRelations                    SpatialRelations                                                           OPTIONAL

]],

[[

-- R1 16-2b-0: Support of default QCL assumption with two TCI states

defaultQCL-TwoTCI-r16               ENUMERATED {supported}                                                     OPTIONAL,

codebookParametersPerBand-r16       CodebookParameters-v1610                                                   OPTIONAL,

-- R1 16-1b-3: Support of PUCCH resource groups per BWP for simultaneous spatial relation update

simul-SpatialRelationUpdatePUCCHResGroup-r16    ENUMERATED {supported}                                         OPTIONAL,

-- R1 16-1f: Maximum number of SCells configured for SCell beam failure recovery simultaneously

maxNumberSCellBFR-r16                           ENUMERATED {n1,n2,n4,n8}                                       OPTIONAL,

-- R1 16-2c: Supports simultaneous reception with different Type-D for FR2 only

simultaneousReceptionDiffTypeD-r16              ENUMERATED {supported}                                         OPTIONAL,

-- R1 16-1a-1: SSB/CSI-RS for L1-SINR measurement

ssb-csirs-SINR-measurement-r16      SEQUENCE {

maxNumberSSB-CSIRS-OneTx-CMR-r16    ENUMERATED {n8, n16, n32, n64},

maxNumberCSI-IM-NZP-IMR-res-r16     ENUMERATED {n8, n16, n32, n64},

maxNumberCSIRS-2Tx-res-r16          ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberSSB-CSIRS-res-r16          ENUMERATED {n8, n16, n32, n64, n128},

maxNumberCSI-IM-NZP-IMR-res-mem-r16 ENUMERATED {n8, n16, n32, n64, n128},

supportedCSI-RS-Density-CMR-r16     ENUMERATED {one, three, oneAndThree},

maxNumberAperiodicCSI-RS-Res-r16    ENUMERATED {n2, n4, n8, n16, n32, n64},

supportedSINR-meas-r16              ENUMERATED {ssbWithCSI-IM, ssbWithNZP-IMR, csirsWithNZP-IMR, csi-RSWithoutIMR}  OPTIONAL

}                                                                                                              OPTIONAL,

-- R1 16-1a-2: Non-group based L1-SINR reporting

nonGroupSINR-reporting-r16              ENUMERATED {n1, n2, n4}                                                OPTIONAL,

-- R1 16-1a-3: Non-group based L1-SINR reporting

groupSINR-reporting-r16                 ENUMERATED {supported}                                                 OPTIONAL,

multiDCI-multiTRP-Parameters-r16        SEQUENCE {

-- R1 16-2a-0: Overlapping PDSCHs in time and fully overlapping in frequency and time

overlapPDSCHsFullyFreqTime-r16          INTEGER (1..2)                                                     OPTIONAL,

-- R1 16-2a-1: Overlapping PDSCHs in time and partially overlapping in frequency and time

overlapPDSCHsInTimePartiallyFreq-r16    ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-2: Out of order operation for DL

outOfOrderOperationDL-r16               SEQUENCE {

supportPDCCH-ToPDSCH-r16                ENUMERATED {supported}                                         OPTIONAL,

supportPDSCH-ToHARQ-ACK-r16             ENUMERATED {supported}                                         OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 16-2a-3: Out of order operation for UL

outOfOrderOperationUL-r16               ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-5: Separate CRS rate matching

separateCRS-RateMatching-r16            ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-6: Default QCL enhancement for multi-DCI based multi-TRP

defaultQCL-PerCORESETPoolIndex-r16      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-7: Maximum number of activated TCI states

maxNumberActivatedTCI-States-r16        SEQUENCE {

maxNumberPerCORESET-Pool-r16            ENUMERATED {n1, n2, n4, n8},

maxTotalNumberAcrossCORESET-Pool-r16    ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL

}                                                                                                              OPTIONAL,

singleDCI-SDM-scheme-Parameters-r16         SEQUENCE {

-- R1 16-2b-1b: Single-DCI based SDM scheme - Support of new DMRS port entry

supportNewDMRS-Port-r16                     ENUMERATED {supported1, supported2, supported3}                                        OPTIONAL,

-- R1 16-2b-1a: Support of s-port DL PTRS

supportTwoPortDL-PTRS-r16                   ENUMERATED {supported}                                         OPTIONAL

}                                                                                                              OPTIONAL,

-- R1 16-2b-2: Support of single-DCI based FDMSchemeA

supportFDM-SchemeA-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2b-3a: Single-DCI based FDMSchemeB CW soft combining

supportCodeWordSoftCombining-r16            ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2b-4: Single-DCI based TDMSchemeA

supportTDM-SchemeA-r16                      ENUMERATED {kb3, kb5, kb10, kb20, noRestriction}                   OPTIONAL,

-- R1 16-2b-5: Single-DCI based inter-slot TDM

supportInter-slotTDM-r16                    SEQUENCE {

supportRepNumPDSCH-TDRA-r16                 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16},

maxTBS-Size-r16                             ENUMERATED {kb3, kb5, kb10, kb20, noRestriction},

maxNumberTCI-states-r16                     INTEGER (1..2)

}                                                                                                              OPTIONAL,

-- R1 16-4: Low PAPR DMRS for PDSCH

lowPAPR-DMRS-PDSCH-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6a: Low PAPR DMRS for PUSCH without transform precoding

lowPAPR-DMRS-PUSCHwithoutPrecoding-r16      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6b: Low PAPR DMRS for PUCCH

lowPAPR-DMRS-PUCCH-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6c: Low PAPR DMRS for PUSCH with transform precoding & pi/2 BPSK

lowPAPR-DMRS-PUSCHwithPrecoding-r16         ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings

csi-ReportFrameworkExt-r16                  CSI-ReportFrameworkExt-r16                                         OPTIONAL,

-- R1 16-3a, 16-3a-1, 16-3b, 16-3b-1, 16-8: Individual new codebook types

codebookParametersAddition-r16              CodebookParametersAddition-r16                                     OPTIONAL,

-- R1 16-8: Mixed codebook types

codebookComboParametersAddition-r16         CodebookComboParametersAddition-r16                                OPTIONAL,

-- R4 8-2: SSB based beam correspondence

beamCorrespondenceSSB-based-r16             ENUMERATED {supported}                                             OPTIONAL,

-- R4 8-3: CSI-RS based beam correspondence

beamCorrespondenceCSI-RS-based-r16          ENUMERATED {supported}                                             OPTIONAL,

beamSwitchTiming-r16                        SEQUENCE {

scs-60kHz-r16                               ENUMERATED {sym224, sym336}                                    OPTIONAL,

scs-120kHz-r16                              ENUMERATED {sym224, sym336}                                    OPTIONAL

}                                                                                                              OPTIONAL

]],

[[

-- R1 16-1a-4: Semi-persistent L1-SINR report on PUCCH

semi-PersistentL1-SINR-Report-PUCCH-r16     SEQUENCE {

supportReportFormat1-2OFDM-syms-r16         ENUMERATED {supported}                                     OPTIONAL,

supportReportFormat4-14OFDM-syms-r16        ENUMERATED {supported}                                     OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 16-1a-5: Semi-persistent L1-SINR report on PUSCH

semi-PersistentL1-SINR-Report-PUSCH-r16     ENUMERATED {supported}                                         OPTIONAL

]],

[[

-- R1 16-1h: Support of 64 configured PUCCH spatial relations

spatialRelations-v1640                      SEQUENCE {

maxNumberConfiguredSpatialRelations-v1640   ENUMERATED {n96, n128, n160, n192, n224, n256, n288, n320}

}                                                                                                          OPTIONAL,

-- R1 16-1i: Support of 64 configured candidate beam RSs for BFR

support64CandidateBeamRS-BFR-r16            ENUMERATED {supported}                                         OPTIONAL

]],

[[

-- R1 16-2a-9: Interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP

maxMIMO-LayersForMulti-DCI-mTRP-r16         ENUMERATED {supported}                                         OPTIONAL

]],

[[

supportedSINR-meas-v1670                    BIT STRING (SIZE (4))                                          OPTIONAL

]],

[[

-- R1 23-8-5	Increased repetition for SRS

srs-increasedRepetition-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-6	Partial frequency sounding of SRS

srs-partialFrequencySounding-r17            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-7	Start RB location hopping for partial frequency SRS

srs-startRB-locationHoppingPartial-r17      ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-8	Comb-8 SRS

srs-combEight-r17                           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-9-1	Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) per band information

codebookParametersfetype2-r17               CodebookParametersfetype2-r17                                  OPTIONAL,

-- R1 23-3-1-2a    Two associated CSI-RS resources

mTRP-PUSCH-twoCSI-RS-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2    Multi-TRP PUCCH repetition scheme 1 (inter-slot)

mTRP-PUCCH-InterSlot-r17                    ENUMERATED {pf0-2, pf1-3-4, pf0-4}                             OPTIONAL,

-- R1 23-3-2b    Cyclic mapping for multi-TRP PUCCH repetition

mTRP-PUCCH-CyclicMapping-r17                ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2c    Second TPC field for multi-TRP PUCCH repetition

mTRP-PUCCH-SecondTPC-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-5-2    MTRP BFR based on two BFD-RS set

mTRP-BFR-twoBFD-RS-Set-r17                  SEQUENCE {

maxBFD-RS-resourcesPerSetPerBWP-r17         ENUMERATED {n1, n2},

maxBFR-r17                                  INTEGER (1..9),

maxBFD-RS-resourcesAcrossSetsPerBWP-r17     ENUMERATED {n2, n3, n4}

}                                                                                                          OPTIONAL,

-- R1 23-5-2a    PUCCH-SR resources for MTRP BFRQ - Max number of PUCCH-SR resources for MTRP BFRQ per cell group

mTRP-BFR-PUCCH-SR-perCG-r17                 ENUMERATED{n1, n2}                                             OPTIONAL,

-- R1 23-5-2b    Association between a BFD-RS resource set on SpCell and a PUCCH SR resource

mTRP-BFR-association-PUCCH-SR-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-3    Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC)

sfn-SimulTwoTCI-AcrossMultiCC-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-4    Default DL beam setup for SFN

sfn-DefaultDL-BeamSetup-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-4a    Default UL beam setup for SFN PDCCH(FR2 only)

sfn-DefaultUL-BeamSetup-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-1    SRS triggering offset enhancement

srs-TriggeringOffset-r17                    ENUMERATED {n1, n2, n4}                                        OPTIONAL,

-- R1 23-8-2    Triggering SRS only in DCI 0_1/0_2

srs-TriggeringDCI-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-9-5    Active CSI-RS resources and ports for mixed codebook types in any slot per band information

codebookComboParameterMixedType-r17         CodebookComboParameterMixedType-r17                            OPTIONAL,

-- R1 23-1-1    Unified TCI [with joint DL/UL TCI update] for intra-cell beam management

unifiedJointTCI-r17                         SEQUENCE{

maxConfiguredJointTCI-r17                   ENUMERATED {n8, n12, n16, n24, n32, n48, n64, n128},

maxActivatedTCIAcrossCC-r17                 ENUMERATED {n1, n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1  23-1-1b    Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE

unifiedJointTCI-multiMAC-CE-r17             SEQUENCE{

minBeamApplicationTime-r17                  ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336}

OPTIONAL,

maxNumMAC-CE-PerCC                          ENUMERATED {n2, n3, n4, n5, n6, n7, n8}

} OPTIONAL,

-- R1 23-1-1d    Per BWP TCI state pool configuration for CA mode

unifiedJointTCI-perBWP-CA-r17               ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1e    TCI state pool configuration with TCI pool sharing for CA mode

unifiedJointTCI-ListSharingCA-r17           ENUMERATED {n1,n2,n4,n8}                                       OPTIONAL,

-- R1 23-1-1f    Common multi-CC TCI state ID update and activation

unifiedJointTCI-commonMultiCC-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1g    Beam misalignment between the DL source RS in the TCI state

unifiedJointTCI-BeamAlignDLRS-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1h    Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS

unifiedJointTCI-PC-association-r17          ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1i    Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH

unifiedJointTCI-Legacy-r17                  ENUMERATED {supported}                                         OPTIONAL,

-- 23-1-1m    Indication/configuration of R17 TCI states for SRS

unifiedJointTCI-Legacy-SRS-r17              ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1j    Indication/configuration of R17 TCI states for CORESET #0

unifiedJointTCI-Legacy-CORESET0-r17         ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1c    SCell BFR with unified TCI framework  (NOTE; pre-requisite is empty)

unifiedJointTCI-SCellBFR-r17                ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1a    Unified TCI with joint DL/UL TCI update for inter-cell beam management

unifiedJointTCI-InterCell-r17               SEQUENCE{

additionalMAC-CE-PerCC-r17                  ENUMERATED {n0, n1, n2, n4},

additionalMAC-CE-AcrossCC-r17               ENUMERATED {n0, n1, n2, n4}

}                                                                                                          OPTIONAL,

-- R1  23-10-1    Unified TCI with separate DL/UL TCI update for intra-cell beam management

unifiedSeparateTCI-r17                      SEQUENCE{

maxConfiguredDL-TCI-r17                     ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n128},

maxConfiguredUL-TCI-r17                     ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64},

maxActivatedDL-TCIAcrossCC-r17              ENUMERATED {n1, n2, n4, n8, n16},

maxActivatedUL-TCIAcrossCC-r17              ENUMERATED {n1, n2, n4, n8, n16}

} OPTIONAL,

-- R1  23-10-1b    Unified TCI with separate DL/UL TCI update for intra-cell beam management with more than one MAC-CE

unifiedSeparateTCI-multiMAC-CE-r17          SEQUENCE{

minBeamApplicationTime-r17                  ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336},

maxActivatedDL-TCIPerCC-r17                 INTEGER (2..8),

maxActivatedUL-TCIPerCC-r17                 INTEGER (2..8)

}                                                                                                          OPTIONAL,

-- R1 23-10-1d    Per BWP DL/UL-TCI state pool configuration for CA mode

unifiedSeparateTCI-perBWP-CA-r17            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-10-1e    TCI state pool configuration with DL/UL-TCI pool sharing for CA mode

unifiedSeparateTCI-ListSharingCA-r17        SEQUENCE {

maxNumListDL-TCI-r17                        ENUMERATED {n1,n2,n4,n8}                                   OPTIONAL,

maxNumListUL-TCI-r17                        ENUMERATED {n1,n2,n4,n8}                                   OPTIONAL

} OPTIONAL,

-- R1 23-10-1f    Common multi-CC DL/UL-TCI state ID update and activation with separate DL/UL TCI update

unifiedSeparateTCI-commonMultiCC-r17    ENUMERATED {supported}                                             OPTIONAL,

-- 23-10-1m    Unified TCI with separate DL/UL TCI update for inter-cell beam management with more than one MAC-CE

unifiedSeparateTCI-InterCell-r17            SEQUENCE {

k-DL-PerCC-r17                              ENUMERATED {n0, n1, n2, n4},

k-UL-PerCC-r17                              ENUMERATED {n0, n1, n2, n4},

k-DL-AcrossCC-r17                           ENUMERATED {n0, n1, n2, n4},

k-UL-AcrossCC-r17                           ENUMERATED {n0, n1, n2, n4}

}                                                                                                          OPTIONAL,

-- R1  23-1-2    Inter-cell beam measurement and reporting (for inter-cell BM and mTRP)

unifiedJointTCI-mTRP-InterCell-BM-r17       SEQUENCE {

maxNumAdditionalPCI-L1-RSRP-r17             INTEGER (1..7),

maxNumSSB-ResourceL1-RSRP-AcrossCC-r17      ENUMERATED {n1,n2,n4,n8}

}                                                                                                          OPTIONAL,

-- R1  23-1-3    MPE mitigation

mpe-Mitigation-r17                          SEQUENCE {

maxNumP-MPR-RI-pairs-r17                    INTEGER (1..4),

maxNumConfRS-r17                            ENUMERATED {n1, n2, n4, n8, n12, n16, n28, n32, n48, n64}

}                                                                                                          OPTIONAL,

-- R1  23-1-4    UE capability value reporting

srs-PortReport-r17                          SEQUENCE {

capVal1-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal2-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal3-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal4-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 23-2-1a    Monitoring of individual candidates

mTRP-PDCCH-individual-r17                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-2-1b    PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot

mTRP-PDCCH-anySpan-3Symbols-r17             ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-2-2    Two QCL TypeD for CORESET monitoring in PDCCH repetition

mTRP-PDCCH-TwoQCL-TypeD-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1-2b    CSI-RS processing framework for SRS with two associated CSI-RS resources

mTRP-PUSCH-CSI-RS-r17                       SEQUENCE {

maxNumPeriodicSRS-r17                       INTEGER (1..8),

maxNumAperiodicSRS-r17                      INTEGER (1..8),

maxNumSP-SRS-r17                            INTEGER (0..8),

numSRS-ResourcePerCC-r17                    INTEGER (1..16),

numSRS-ResourceNonCodebook-r17              INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 23-3-1a    Cyclic mapping for Multi-TRP PUSCH repetition

mTRP-PUSCH-cyclicMapping-r17                ENUMERATED {typeA,typeB,both}                                  OPTIONAL,

-- R1 23-3-1b    Second TPC field for Multi-TRP PUSCH repetition

mTRP-PUSCH-secondTPC-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1c     Two PHR reporting

mTRP-PUSCH-twoPHR-Reporting-r17             ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1e    A-CSI report

mTRP-PUSCH-A-CSI-r17                        ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1f    SP-CSI report

mTRP-PUSCH-SP-CSI-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1g    CG PUSCH transmission

mTRP-PUSCH-CG-r17                           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2d    Updating two Spatial relation or two sets of power control parameters for PUCCH group

mTRP-PUCCH-MAC-CE-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2e    Maximum number of power control parameter sets configured for multi-TRP PUCCH repetition in FR1

mTRP-PUCCH-maxNum-PC-FR1-r17                INTEGER (3..8)                                                 OPTIONAL,

-- R1 23-4    IntCell-mTRP

mTRP-inter-Cell-r17                         SEQUENCE {

maxNumAdditionalPCI-Case1-r17               INTEGER (1..7),

maxNumAdditionalPCI-Case2-r17               INTEGER (0..7)

}                                                                                                          OPTIONAL,

-- R1 23-5-1    Group based L1-RSRP reporting enhancements

mTRP-GroupBasedL1-RSRP-r17                  SEQUENCE {

maxNumBeamGroups-r17                        INTEGER (1..4),

maxNumRS-WithinSlot-r17                     ENUMERATED {n2,n3,n4,n8,n16,n32,n64},

maxNumRS-AcrossSlot-r17                     ENUMERATED {n8, n16, n32, n64, n128}

}                                                                                                          OPTIONAL,

-- R1 23-5-2c    MAC-CE based update of explicit BFD-RS    mTRP-PUCCH-IntraSlot-r17  => per band

mTRP-BFD-RS-MAC-CE-r17                      ENUMERATED {n4, n8, n12, n16, n32, n48, n64 }                  OPTIONAL,

-- R1 23-7-1    Basic Features of CSI Enhancement for Multi-TRP

mTRP-CSI-EnhancementPerBand-r17             SEQUENCE {

maxNumNZP-CSI-RS-r17                        INTEGER (2..8),

cSI-Report-mode-r17                         ENUMERATED {mode1, mode2, both},

supportedComboAcrossCCs-r17                 SEQUENCE (SIZE (1..16)) OF CSI-MultiTRP-SupportedCombinations-r17,

codebookModeNCJT-r17                        ENUMERATED{mode1,mode1And2}

}                                                                                                          OPTIONAL,

-- R1 23-7-1b    Active CSI-RS resources and ports in the presence of multi-TRP CSI

codebookComboParameterMultiTRP-r17          CodebookComboParameterMultiTRP-r17                             OPTIONAL,

-- R1 23-7-1a    Additional CSI report mode 1

mTRP-CSI-additionalCSI-r17                  ENUMERATED{x1,x2}                                              OPTIONAL,

-- R1 23-7-4    Support of Nmax=2 for Multi-TRP CSI

mTRP-CSI-N-Max2-r17                         ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-7-5    CMR sharing

mTRP-CSI-CMR-r17                            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-11    Partial frequency sounding of SRS for non-frequency hopping case

srs-partialFreqSounding-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1-24 feature: Extend beamSwitchTiming for FR2-2

beamSwitchTiming-v1710                      SEQUENCE {

scs-480kHz                                  ENUMERATED {sym56, sym112, sym192, sym896, sym1344}        OPTIONAL,

scs-960kHz                                  ENUMERATED {sym112, sym224, sym384, sym1792, sym2688}      OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature: Extend beamSwitchTiming-r16 for FR2-2

beamSwitchTiming-r17                        SEQUENCE {

scs-480kHz-r17                              ENUMERATED {sym896, sym1344}                               OPTIONAL,

scs-960kHz-r17                              ENUMERATED {sym1792, sym2688}                              OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature: Extend beamReportTiming for FR2-2

beamReportTiming-v1710                      SEQUENCE {

scs-480kHz-r17                              ENUMERATED {sym56, sym112, sym224}                         OPTIONAL,

scs-960kHz-r17                              ENUMERATED {sym112, sym224, sym448}                        OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature:    Extend maximum number of RX/TX beam switch DL for FR2-2

maxNumberRxTxBeamSwitchDL-v1710             SEQUENCE {

scs-480kHz-r17                              ENUMERATED {n2, n4, n7}                                    OPTIONAL,

scs-960kHz-r17                              ENUMERATED {n1, n2, n4, n7}                                OPTIONAL

}                                                                                                          OPTIONAL

]],

[[

-- R1-23-1-4a:	Semi-persistent/aperiodic capability value report

srs-PortReportSP-AP-r17                     ENUMERATED {supported}                                         OPTIONAL,

maxNumberRxBeam-v1720                       INTEGER (9..12)                                                OPTIONAL,

-- R1-23-6-5	Support implicit configuration of RS(s) with two TCI states for beam failure detection

sfn-ImplicitRS-twoTCI-r17                   ENUMERATED {supported}                                         OPTIONAL,

-- R1-23-6-6	QCL-TypeD collision handling with CORESET with 2 TCI states

sfn-QCL-TypeD-Collision-twoTCI-r17          ENUMERATED {supported}                                         OPTIONAL,

-- R1-23-7-1c	Basic Features of CSI Enhancement for Multi-TRP - number of CPUs

mTRP-CSI-numCPU-r17                         ENUMERATED {n2, n3, n4}                                        OPTIONAL

]],

[[

supportRepNumPDSCH-TDRA-DCI-1-2-r17         ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                   OPTIONAL

]],

[[

codebookParametersetype2DopplerCSI-r18      CodebookParametersetype2DopplerCSI-r18                         OPTIONAL,

codebookParametersfetype2DopplerCSI-r18     CodebookParametersfetype2DopplerCSI-r18                        OPTIONAL,

codebookParametersetype2CJT-r18             CodebookParametersetype2CJT-r18                                OPTIONAL,

codebookParametersfetype2CJT-r18            CodebookParametersfetype2CJT-r18                               OPTIONAL,

codebookComboParametersCJT-r18              CodebookComboParametersCJT-r18                                 OPTIONAL,

-- R1 40-1-1: Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI

-- codepoint per CC

tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 SEQUENCE {

maxNumberConfigJointTCIPerCC-PerBWP-r18     ENUMERATED {n8,n12,n16,n24,n32,n48,n64,n128},

maxNumberActiveJointTCI-AcrossCC-r18        ENUMERATED {n2,n4,n6,n8,n16,n32}

}                                                                                                          OPTIONAL,

-- R1 40-1-1a: Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI

-- codepoints per CC

tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18 SEQUENCE {

tci-StateInd-r18                            ENUMERATED {withAssignment, withoutAssignment},

maxNumberActiveJointTCI-PerCC-r18           INTEGER (2..8)

}                                                                                                           OPTIONAL,

-- R1 40-1-1b: TRP-specific BFR with unified TCI framework with Unified TCI

tci-TRP-BFR-r18                             ENUMERATED {supported}                                          OPTIONAL,

-- R1 40-1-1c: DCI format 1_1 and if supported 1_2 configured with TCI selection field

tci-SelectionDCI-r18                            ENUMERATED {supported}                                      OPTIONAL,

-- R1 40-1-2: Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI

-- codepoint per CC

tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18 SEQUENCE {

maxNumConfigDL-TCI-PerCC-PerBWP-r18         ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n128},

maxNumConfigUL-TCI-PerCC-PerBWP-r18         ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64},

maxNumActiveDL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16},

maxNumActiveUL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-2a: Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple

-- activated TCI codepoints per CC

tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18  SEQUENCE {

maxNumActiveDL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16},

maxNumActiveUL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-3: Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP

tci-SelectionAperiodicCSI-RS-r18            ENUMERATED {perResource, perResourceSet, both}                 OPTIONAL,

-- R1 40-1-4: Two TCI states for CJT Tx scheme for PDSCH

twoTCI-StatePDSCH-CJT-TxScheme-r18          ENUMERATED {cjtSchemeA, cjtSchemeB, both}                      OPTIONAL,

-- R1 40-1-7: Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI

-- codepoint per CORESETPoolIndex per CC

tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18   SEQUENCE {

mTRP-Operation-r18                                        ENUMERATED {intraCell, intraCellAndInterCell},

maxNumberConfigJointTCIPerCC-PerBWP-r18                   ENUMERATED {n8,n12,n16,n24,n32,n48,n64,n128},

maxNumberActiveJointTCIAcrossCC-PerCORESET                ENUMERATED {n1,n2,n4,n8,n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-7a: Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI

-- codepoints per CORESETPoolIndex per CC

tci-JointTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18        INTEGER (2..8)                               OPTIONAL,

-- R1 40-1-9: Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI

-- codepoint per CORESETPoolIndex per CC

tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18   SEQUENCE {

maxNumConfigDL-TCI-PerCC-PerBWP-r18         ENUMERATED {n8, n12, n16, n24, n32, n48, n64, n128},

maxNumConfigUL-TCI-PerCC-PerBWP-r18         ENUMERATED {n8, n12, n16, n24, n32, n48, n64},

maxNumActiveDL-TCI-AcrossCC-r18             ENUMERATED {n1, n2, n4, n8, n16},

maxNumActiveUL-TCI-AcrossCC-r18             ENUMERATED {n1, n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-9a: Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI

-- codepoints per CORESETPoolIndex per CC

tci-SeperateTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18   SEQUENCE {

maxNumConfigDL-TCI-PerCC-PerBWP-r18         INTEGER (1..8),

maxNumConfigUL-TCI-PerCC-PerBWP-r18         INTEGER (1..8)

}                                                                                                          OPTIONAL,

-- R1 40-1-12: Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP

commonTCI-SingleDCI-r18                     INTEGER (1..4)                                                 OPTIONAL,

-- R1 40-1-13: Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP

commonTCI-MultiDCI-r18                      INTEGER (1..4)                                                 OPTIONAL,

-- R1 40-2-3: TAG ID indication via absolute TA command MAC CE

spCell-TAG-Ind-r18                          ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-4: PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for

-- inter-cell

interCellCrossTRP-PDCCH-OrderCFRA-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-4a: PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for

-- intra-cell

intraCellCrossTRP-PDCCH-OrderCFRA-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-9: Overlapping UL transmission reduction

overlapUL-TransReduction-r18                ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-3-1-24: Timeline for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI

timelineRelax-CJT-CSI-r18                   ENUMERATED {n0,n2}                                             OPTIONAL,

-- R1 40-4-11: Joint configuration of Rel.18 DMRS ports and Rel.18 dynamic switching between DFT-S-OFDM and CP-OFDM for PUSCH

jointConfigDMRSPortDynamicSwitching-r18     ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-1: SRS comb offset hopping

srs-combOffsetHopping-r18                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-1a: Comb offset hopping time-domain behavior when repetition factor R>1

srs-combOffsetInTime-r18                    ENUMERATED {srs, rsrs, both}                                   OPTIONAL,

-- R1 40-5-1b: SRS comb offset hopping combined with legacy group/sequence hopping

srs-combOffsetCombinedGroupSequence-r18     ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-1c: Comb offset hopping within a subset

srs-combOffsetHoppingWithinSubset-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2: SRS cyclic shift hopping

srs-cyclicShiftHopping-r18                  ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2a: Smaller cyclic shift granularity for cyclic shift hopping

srs-cyclicShiftHoppingSmallGranularity-r18  ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2b: SRS cyclic shift hopping combined with legacy group/sequence hopping

srs-cyclicShiftCombinedGroupSequence-r18    ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2c: Cyclic shift hopping within a subset

cyclicShiftHoppingWithinSubset-r18          ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-3: SRS cyclic shift hopping combined with SRS comb offset hopping

srs-cyclicShiftCombinedCombOffset-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-1-1: 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook

pusch-CB-2PTRS-SingleDCI-STx2P-SDM-r18      ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-1a-1: 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook

pusch-NonCB-2PTRS-SingleDCI-STx2P-SDM-r18   ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-1b: Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH

pusch-NonCB-SingleDCI-STx2P-SDM-CSI-RS-SRS-r18 SEQUENCE {

maxNumberPeriodicSRS-Resource-PerBWP-r18      INTEGER (1..8),

maxNumberAperiodicSRS-Resource-PerBWP-r18     INTEGER (1..8),

maxNumberSemiPersistentSRS-ResourcePerBWP-r18 INTEGER (0..8),

valueY-SRS-ResourceAssociate-r18              INTEGER (1..16),

valueX-CSI-RS-ResourceAssociate-r18           INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 40-6-1-2: New DMRS port entry for single-DCI based SDM scheme

dmrs-PortEntrySingleDCI-SDM-r18             ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-2-1: 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook

pusch-CB-2PTRS-SingleDCI-STx2P-SFN-r18      ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-2a-1: 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook

pusch-NonCB-2PTRS-SingleDCI-STx2P-SFN-r18   ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-2b: Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH

pusch-NonCB-SingleDCI-STx2P-SFN-CSI-RS-SRS-r18 SEQUENCE {

maxNumberPeriodicSRS-Resource-PerBWP-r18      INTEGER (1..8),

maxNumberAperiodicSRS-Resource-PerBWP-r18     INTEGER (1..8),

maxNumberSemiPersistentSRS-ResourcePerBWP-r18 INTEGER (0..8),

valueY-SRS-ResourceAssociate-r18              INTEGER (1..16),

valueX-CSI-RS-ResourceAssociate-r18           INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 40-6-3b-1: Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH

twoPUSCH-NoneCB-Multi-DCI-STx2P-CSI-RS-Resource-r18       SEQUENCE {

maxNumberPeriodicSRS-r18                                            INTEGER (1..8),

maxNumberAperiodicSRS-r18                                           INTEGER (1..8),

maxNumberSemiPersistentSRS-r18                                     INTEGER (0..8),

simultaneousSRS-PerCC-r18                                           INTEGER (1..16),

simultaneousCSI-RS-NoneCodebook-r18                               INTEGER (1..2)

}                                                                                                                                              OPTIONAL,

-- R1 40-6-3c: Codebook multi-DCI based STx2P PUSCH+PUSCH –Fully overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18       ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3d: Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and partially overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18    ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3e: Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18    ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3f: Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, partially overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3g: Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, partially or non-overlapping

-- in frequency

twoPUSCH-CB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18     ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3h: Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG

twoPUSCH-CB-MultiDCI-STx2P-CG-CG-r18                         ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3i: Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG

twoPUSCH-CB-MultiDCI-STx2P-CG-DG-r18                         ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3j: Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-NoneCB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18   ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3k: Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping PUSCHs in time and partially overlapping in frequency

twoPUSCH-NoneCB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18 ENUMERATED {supported}                       OPTIONAL,

-- R1 40-6-3l: Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-NoneCB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18 ENUMERATED {supported}                       OPTIONAL,

-- R1 40-6-3m: Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, partially overlapping in frequency

twoPUSCH-NoneCB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 ENUMERATED {supported}                    OPTIONAL,

-- R1 40-6-3n: Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially overlapping PUSCHs in time, non-overlapping in frequency

twoPUSCH-NoneCB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18     ENUMERATED {supported}                    OPTIONAL,

-- R1 40-6-3o: Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG

twoPUSCH-NoneCB-MultiDCI-STx2P-CG-CG-r18                     ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3p: Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG

twoPUSCH-NoneCB-MultiDCI-STx2P-CG-DG-r18                     ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-4a: Dynamic indication of repetition number for SFN scheme for PUCCH

pucch-RepetitionDynamicIndicationSFN-r18                     ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-5: Support grouped-based beam reporting for STx2P

groupBeamReporting-STx2P-r18         SEQUENCE {

groupL1-RSRP-Reporting-r18           ENUMERATED {jointULandDL, ulOnly, both},

maxNumberBeamGroups-r18              INTEGER (1..4),

maxNumberResWithinSlotAcrossCC-r18   ENUMERATED {n2,n3,n4,n8,n16,n32,n64},

maxNumberResAcrossCC-r18             ENUMERATED {n8,n16,n32,n64,n128}

}                                                                                                                                                OPTIONAL

]]

}

DummyG ::=                          SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx   ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}

}

BeamManagementSSB-CSI-RS ::=        SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n0, n8, n16, n32, n64},

maxNumberCSI-RS-Resource            ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberCSI-RS-ResourceTwoTx       ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}                                       OPTIONAL,

maxNumberAperiodicCSI-RS-Resource   ENUMERATED {n0, n1, n4, n8, n16, n32, n64}

}

DummyH ::=                          SEQUENCE {

burstLength                         INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..128)

}

CSI-RS-ForTracking ::=              SEQUENCE {

maxBurstLength                      INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..256)

}

CSI-RS-IM-ReceptionForFeedback ::=              SEQUENCE {

maxConfigNumberNZP-CSI-RS-PerCC                 INTEGER (1..64),

maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC      INTEGER (2..256),

maxConfigNumberCSI-IM-PerCC                     ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousNZP-CSI-RS-PerCC           INTEGER (1..64),

totalNumberPortsSimultaneousNZP-CSI-RS-PerCC    INTEGER (2..256)

}

CSI-RS-ProcFrameworkForSRS ::=                  SEQUENCE {

maxNumberPeriodicSRS-AssocCSI-RS-PerBWP         INTEGER (1..4),

maxNumberAperiodicSRS-AssocCSI-RS-PerBWP        INTEGER (1..4),

maxNumberSP-SRS-AssocCSI-RS-PerBWP              INTEGER (0..4),

simultaneousSRS-AssocCSI-RS-PerCC               INTEGER (1..8)

}

CSI-ReportFramework ::=                         SEQUENCE {

maxNumberPeriodicCSI-PerBWP-ForCSI-Report       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForCSI-Report      INTEGER (1..4),

maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4),

maxNumberPeriodicCSI-PerBWP-ForBeamReport       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForBeamReport      INTEGER (1..4),

maxNumberAperiodicCSI-triggeringStatePerCC      ENUMERATED {n3, n7, n15, n31, n63, n128},

maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4),

simultaneousCSI-ReportsPerCC                    INTEGER (1..8)

}

CSI-ReportFrameworkExt-r16 ::=                      SEQUENCE {

maxNumberAperiodicCSI-PerBWP-ForCSI-ReportExt-r16   INTEGER (5..8)

}

PTRS-DensityRecommendationDL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29),

sampleDensity1                      INTEGER (1..276),

sampleDensity2                      INTEGER (1..276),

sampleDensity3                      INTEGER (1..276),

sampleDensity4                      INTEGER (1..276),

sampleDensity5                      INTEGER (1..276)

}

SpatialRelations ::=                    SEQUENCE {

maxNumberConfiguredSpatialRelations     ENUMERATED {n4, n8, n16, n32, n64, n96},

maxNumberActiveSpatialRelations         ENUMERATED {n1, n2, n4, n8, n14},

additionalActiveSpatialRelationPUCCH    ENUMERATED {supported}                              OPTIONAL,

maxNumberDL-RS-QCL-TypeD                ENUMERATED {n1, n2, n4, n8, n14}

}

DummyI ::=               SEQUENCE {

supportedSRS-TxPortSwitch           ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, tr-equal},

txSwitchImpactToRx                  ENUMERATED {true}                                       OPTIONAL

}

CSI-MultiTRP-SupportedCombinations-r17 ::= SEQUENCE {

maxNumTx-Ports-r17                         ENUMERATED {n2, n4, n8, n12, n16, n24, n32},

maxTotalNumCMR-r17                         INTEGER (2..64),

maxTotalNumTx-PortsNZP-CSI-RS-r17          INTEGER (2..256)

}

-- TAG-MIMO-PARAMETERSPERBAND-STOP

-- ASN1STOP

MIMO-ParametersPerBand field descriptions

codebookParametersPerBand For a given frequency band, this field this field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resources indicated by this field are referred by codebookParametersperBC in CA-ParametersNR to indicate the supported CSI-RS resource per band combination.

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework CSI related capabilities which the UE supports on each of the carriers operated on this band. If the network configures the UE with serving cells on both FR1 and FR2 bands these values may be further limited by the corresponding fields in fr1-fr2-Add-UE-NR-Capabilities.

supportNewDMRS-Port Presence of this field set to supported1, supported2 or supported3 indicates that the UE supports the new DMRS port entry {0,2,3}.

#### –	ModulationOrder

The IE ModulationOrder is used to convey the maximum supported modulation order.

ModulationOrder information element

-- ASN1START

-- TAG-MODULATIONORDER-START

ModulationOrder ::= ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATIONORDER-STOP

-- ASN1STOP

#### –	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]]

}

MRDC-Parameters-v1580 ::= SEQUENCE {

dynamicPowerSharingNEDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v1590 ::=	SEQUENCE {

interBandContiguousMRDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v15g0 ::=   SEQUENCE {

simultaneousRxTxInterBandENDCPerBandPair   SimultaneousRxTxPerBandPair  OPTIONAL

}

MRDC-Parameters-v15n0 ::= SEQUENCE {

intraBandENDC-Support-UL            ENUMERATED {non-contiguous, both}   OPTIONAL

}

MRDC-Parameters-v1620 ::=    SEQUENCE {

maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16    SEQUENCE{

eutra-TDD-Config0-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config1-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config2-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config3-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config4-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config5-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config6-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL

}                                                                                    OPTIONAL,

-- R1 18-2 Single UL TX operation for TDD PCell in EN-DC

tdm-restrictionTDD-endc-r16          ENUMERATED {supported}                          OPTIONAL,

-- R1 18-2a Single UL TX operation for FDD PCell in EN-DC

tdm-restrictionFDD-endc-r16          ENUMERATED {supported}                          OPTIONAL,

--  R1 18-2b Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE

singleUL-HARQ-offsetTDD-PCell-r16    ENUMERATED {supported}                          OPTIONAL,

--  R1 18-3 Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE)

tdm-restrictionDualTX-FDD-endc-r16   ENUMERATED {supported}                          OPTIONAL

}

MRDC-Parameters-v1630 ::= SEQUENCE {

-- R4 2-20 Maximum uplink duty cycle for FDD+TDD EN-DC power class 2

maxUplinkDutyCycle-interBandENDC-FDD-TDD-PC2-r16  SEQUENCE {

maxUplinkDutyCycle-FDD-TDD-EN-DC1-r16             ENUMERATED {n30, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

maxUplinkDutyCycle-FDD-TDD-EN-DC2-r16             ENUMERATED {n30, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL

}                                                                                                             OPTIONAL,

-- R4 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum

interBandMRDC-WithOverlapDL-Bands-r16       ENUMERATED {supported}                   OPTIONAL

}

MRDC-Parameters-v1700 ::=	SEQUENCE {

condPSCellAdditionENDC-r17                  ENUMERATED {supported}                   OPTIONAL,

scg-ActivationDeactivationENDC-r17          ENUMERATED {supported}                   OPTIONAL,

scg-ActivationDeactivationResumeENDC-r17    ENUMERATED {supported}                   OPTIONAL

}

MRDC-Parameters-v1770 ::=	SEQUENCE {

-- R4 26-1: Higher Power Limit CA DC

higherPowerLimitMRDC-r17                    ENUMERATED {supported}                   OPTIONAL

}

-- TAG-MRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	NCR-Parameters

The IE NCR-Parameters is used to indicate the UE capabilities supported by NCR-MT.

NCR-Parameters information element

-- ASN1START

-- TAG-NCR-PARAMETERS-START

NCR-Parameters-r18::=                   SEQUENCE {

inactiveStateNCR-r18                    ENUMERATED {supported}                                  OPTIONAL,

supportedNumberOfDRBs-NCR-r18           ENUMERATED {n1,n16}                                     OPTIONAL,

nonDRB-NCR-r18                          ENUMERATED {supported}                                  OPTIONAL

}

-- TAG-NCR-PARAMETERS-STOP

-- ASN1STOP

#### –	NRDC-Parameters

The IE NRDC-Parameters contains parameters specific to NR-DC, i.e., which are not applicable to NR SA.

NRDC-Parameters information element

-- ASN1START

-- TAG-NRDC-PARAMETERS-START

NRDC-Parameters ::=                 SEQUENCE {

measAndMobParametersNRDC            MeasAndMobParametersMRDC                    OPTIONAL,

generalParametersNRDC               GeneralParametersMRDC-XDD-Diff              OPTIONAL,

fdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

tdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

fr1-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

fr2-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

dummy2                              OCTET STRING                                OPTIONAL,

dummy                               SEQUENCE {}                                 OPTIONAL

}

NRDC-Parameters-v1570 ::=           SEQUENCE {

sfn-SyncNRDC                        ENUMERATED {supported}                      OPTIONAL

}

NRDC-Parameters-v15c0 ::=           SEQUENCE {

pdcp-DuplicationSplitSRB            ENUMERATED {supported}                      OPTIONAL,

pdcp-DuplicationSplitDRB            ENUMERATED {supported}                      OPTIONAL

}

NRDC-Parameters-v1610 ::=           SEQUENCE {

measAndMobParametersNRDC-v1610      MeasAndMobParametersMRDC-v1610              OPTIONAL

}

NRDC-Parameters-v1700   ::=         SEQUENCE {

f1c-OverNR-RRC-r17                  ENUMERATED {supported}                      OPTIONAL,

measAndMobParametersNRDC-v1700      MeasAndMobParametersMRDC-v1700

}

-- TAG-NRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	NTN-Parameters

The IE NTN-Parameters is used to convey the subset of UE Radio Access Capability Parameters that apply to NTN access when there is a difference compared to TN access.

NTN-Parameters information element

-- ASN1START

-- TAG-NTN-PARAMETERS-START

NTN-Parameters-r17 ::= SEQUENCE {

inactiveStateNTN-r17                ENUMERATED {supported}                                OPTIONAL,

ra-SDT-NTN-r17                      ENUMERATED {supported}                                OPTIONAL,

srb-SDT-NTN-r17                     ENUMERATED {supported}                                OPTIONAL,

measAndMobParametersNTN-r17         MeasAndMobParameters                                  OPTIONAL,

mac-ParametersNTN-r17               MAC-Parameters                                        OPTIONAL,

phy-ParametersNTN-r17               Phy-Parameters                                        OPTIONAL,

fdd-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddXDD-Mode                           OPTIONAL,

fr1-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddFRX-Mode                           OPTIONAL,

ue-BasedPerfMeas-ParametersNTN-r17  UE-BasedPerfMeas-Parameters-r16                       OPTIONAL,

son-ParametersNTN-r17               SON-Parameters-r16                                    OPTIONAL

}

-- TAG-NTN-PARAMETERS-STOP

-- ASN1STOP

NTN-Parameters field descriptions

fdd-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fdd-Add-UE-NR-Capabilities applies to NTN.

fr1-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fr1-Add-UE-NR-Capabilities applies to NTN.

mac-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, mac-Parameters applies to NTN.

measAndMobParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, measAndMobParameters applies to NTN.

phy-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, phy-Parameters applies to NTN.

son-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, son-Parameters-r16 applies to NTN.

ue-BasedPerfMeas-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, ue-BasedPerfMeas-Parameters-r16 applies to NTN.

#### –	OLPC-SRS-Pos

The IE OLPC-SRS-Pos is used to convey OLPC SRS positioning related parameters specific for a certain band.

OLPC-SRS-Pos information element

-- ASN1START

-- TAG-OLPC-SRS-POS-START

OLPC-SRS-Pos-r16 ::=        SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16         ENUMERATED {supported}               OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16           ENUMERATED {supported}               OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16           ENUMERATED {supported}               OPTIONAL,

maxNumberPathLossEstimatePerServing-r16    ENUMERATED {n1, n4, n8, n16}         OPTIONAL

}

--TAG-OLPC-SRS-POS-STOP

-- ASN1STOP

#### –	PDCP-Parameters

The IE PDCP-Parameters is used to convey capabilities related to PDCP.

PDCP-Parameters information element

-- ASN1START

-- TAG-PDCP-PARAMETERS-START

PDCP-Parameters ::=         SEQUENCE {

supportedROHC-Profiles      SEQUENCE {

profile0x0000               BOOLEAN,

profile0x0001               BOOLEAN,

profile0x0002               BOOLEAN,

profile0x0003               BOOLEAN,

profile0x0004               BOOLEAN,

profile0x0006               BOOLEAN,

profile0x0101               BOOLEAN,

profile0x0102               BOOLEAN,

profile0x0103               BOOLEAN,

profile0x0104               BOOLEAN

},

maxNumberROHC-ContextSessions       ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64,

cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles             ENUMERATED {supported}      OPTIONAL,

continueROHC-Context                ENUMERATED {supported}      OPTIONAL,

outOfOrderDelivery                  ENUMERATED {supported}      OPTIONAL,

shortSN                             ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSRB                 ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMCG-OrSCG-DRB       ENUMERATED {supported}      OPTIONAL,

...,

[[

drb-IAB-r16                         ENUMERATED {supported}      OPTIONAL,

non-DRB-IAB-r16                     ENUMERATED {supported}      OPTIONAL,

extendedDiscardTimer-r16            ENUMERATED {supported}      OPTIONAL,

continueEHC-Context-r16             ENUMERATED {supported}      OPTIONAL,

ehc-r16                             ENUMERATED {supported}      OPTIONAL,

maxNumberEHC-Contexts-r16           ENUMERATED {cs2, cs4, cs8, cs16, cs32, cs64, cs128, cs256, cs512,

cs1024, cs2048, cs4096, cs8192, cs16384, cs32768, cs65536}    OPTIONAL,

jointEHC-ROHC-Config-r16            ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMoreThanTwoRLC-r16  ENUMERATED {supported}      OPTIONAL

]],

[[

longSN-RedCap-r17                   ENUMERATED {supported}      OPTIONAL,

udc-r17                             SEQUENCE {

standardDictionary-r17              ENUMERATED {supported}  OPTIONAL,

operatorDictionary-r17              SEQUENCE {

versionOfDictionary-r17             INTEGER (0..15),

associatedPLMN-ID-r17               PLMN-Identity

}                                                           OPTIONAL,

continueUDC-r17                     ENUMERATED {supported}  OPTIONAL,

supportOfBufferSize-r17             ENUMERATED {kbyte4, kbyte8}  OPTIONAL

}                                                               OPTIONAL

]],

[[

longSN-NCR-r18                      ENUMERATED {supported}      OPTIONAL

]]

}

-- TAG-PDCP-PARAMETERS-STOP

-- ASN1STOP

#### –	PDCP-ParametersMRDC

The IE PDCP-ParametersMRDC is used to convey PDCP related capabilities for MR-DC.

PDCP-ParametersMRDC information element

-- ASN1START

-- TAG-PDCP-PARAMETERSMRDC-START

PDCP-ParametersMRDC ::=                 SEQUENCE {

pdcp-DuplicationSplitSRB                ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSplitDRB                ENUMERATED {supported}      OPTIONAL

}

PDCP-ParametersMRDC-v1610 ::= SEQUENCE {

scg-DRB-NR-IAB-r16                  ENUMERATED {supported}          OPTIONAL

}

-- TAG-PDCP-PARAMETERSMRDC-STOP

-- ASN1STOP

#### –	Phy-Parameters

The IE Phy-Parameters is used to convey the physical layer capabilities.

Phy-Parameters information element

-- ASN1START

-- TAG-PHY-PARAMETERS-START

Phy-Parameters ::=                  SEQUENCE {

phy-ParametersCommon                Phy-ParametersCommon                        OPTIONAL,

phy-ParametersXDD-Diff              Phy-ParametersXDD-Diff                      OPTIONAL,

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                      OPTIONAL,

phy-ParametersFR1                   Phy-ParametersFR1                           OPTIONAL,

phy-ParametersFR2                   Phy-ParametersFR2                           OPTIONAL

}

Phy-Parameters-v16a0 ::=            SEQUENCE {

phy-ParametersCommon-v16a0          Phy-ParametersCommon-v16a0                  OPTIONAL

}

Phy-ParametersCommon ::=            SEQUENCE {

csi-RS-CFRA-ForHO                   ENUMERATED {supported}                      OPTIONAL,

dynamicPRB-BundlingDL               ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUCCH                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUSCH                  ENUMERATED {supported}                      OPTIONAL,

nzp-CSI-RS-IntefMgmt                ENUMERATED {supported}                      OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH     ENUMERATED {supported}                      OPTIONAL,

precoderGranularityCORESET          ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-Codebook            ENUMERATED {supported}                      OPTIONAL,

semiStaticHARQ-ACK-Codebook         ENUMERATED {supported}                      OPTIONAL,

spatialBundlingHARQ-ACK             ENUMERATED {supported}                      OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI   ENUMERATED {supported}                      OPTIONAL,

pucch-Repetition-F1-3-4             ENUMERATED {supported}                      OPTIONAL,

ra-Type0-PUSCH                      ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH       ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH       ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeA                  ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeB                  ENUMERATED {supported}                      OPTIONAL,

interleavingVRB-ToPRB-PDSCH         ENUMERATED {supported}                      OPTIONAL,

interSlotFreqHopping-PUSCH          ENUMERATED {supported}                      OPTIONAL,

type1-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

type2-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

pusch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

pdsch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

downlinkSPS                         ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType1             ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType2             ENUMERATED {supported}                      OPTIONAL,

pre-EmptIndication-DL               ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-DL              ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-UL              ENUMERATED {supported}                      OPTIONAL,

cbg-FlushIndication-DL              ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL   ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetSemi-Static     ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetDynamic         ENUMERATED {supported}                      OPTIONAL,

bwp-SwitchingDelay                  ENUMERATED {type1, type2}                   OPTIONAL,

...,

[[

dummy                               ENUMERATED {supported}                      OPTIONAL

]],

[[

maxNumberSearchSpaces               ENUMERATED {n10}                            OPTIONAL,

rateMatchingCtrlResrcSetDynamic     ENUMERATED {supported}                      OPTIONAL,

maxLayersMIMO-Indication            ENUMERATED {supported}                      OPTIONAL

]],

[[

spCellPlacement                             CarrierAggregationVariant           OPTIONAL

]],

[[

-- R1 9-1: Basic channel structure and procedure of 2-step RACH

twoStepRACH-r16                             ENUMERATED {supported}              OPTIONAL,

-- R1 11-1: Monitoring DCI format 1_2 and DCI format 0_2

dci-Format1-2And0-2-r16                     ENUMERATED {supported}              OPTIONAL,

-- R1 11-1a: Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space

monitoringDCI-SameSearchSpace-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 11-10: Type 2 configured grant release by DCI format 0_1

type2-CG-ReleaseDCI-0-1-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 11-11: Type 2 configured grant release by DCI format 0_2

type2-CG-ReleaseDCI-0-2-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 12-3: SPS release by DCI format 1_1

sps-ReleaseDCI-1-1-r16                      ENUMERATED {supported}              OPTIONAL,

-- R1 12-3a: SPS release by DCI format 1_2

sps-ReleaseDCI-1-2-r16                      ENUMERATED {supported}              OPTIONAL,

-- R1 14-8: CSI trigger states containing non-active BWP

csi-TriggerStateNon-ActiveBWP-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 20-2: Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities

separateSMTC-InterIAB-Support-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 20-3: Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors

separateRACH-IAB-Support-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 20-5a: Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatSemiStatic-IAB-r16  ENUMERATED {supported}              OPTIONAL,

-- R1 20-5b: Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatDynamics-IAB-r16    ENUMERATED {supported}              OPTIONAL,

dft-S-OFDM-WaveformUL-IAB-r16               ENUMERATED {supported}              OPTIONAL,

-- R1 20-6: Support DCI Format 2_5 based indication of soft resource availability to an IAB node

dci-25-AI-RNTI-Support-IAB-r16              ENUMERATED {supported}              OPTIONAL,

-- R1 20-7: Support T_delta reception.

t-DeltaReceptionSupport-IAB-r16             ENUMERATED {supported}              OPTIONAL,

-- R1 20-8: Support of Desired guard symbol reporting and provided guard symbok reception.

guardSymbolReportReception-IAB-r16          ENUMERATED {supported}              OPTIONAL,

-- R1 18-8 HARQ-ACK codebook type and spatial bundling per PUCCH group

harqACK-CB-SpatialBundlingPUCCH-Group-r16   ENUMERATED {supported}              OPTIONAL,

-- R1 19-2: Cross Slot Scheduling

crossSlotScheduling-r16                     SEQUENCE {

non-SharedSpectrumChAccess-r16              ENUMERATED {supported}          OPTIONAL,

sharedSpectrumChAccess-r16                  ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16  ENUMERATED {n1, n4, n8, n16}         OPTIONAL,

extendedCG-Periodicities-r16                ENUMERATED {supported}              OPTIONAL,

extendedSPS-Periodicities-r16               ENUMERATED {supported}              OPTIONAL,

codebookVariantsList-r16                    CodebookVariantsList-r16            OPTIONAL,

-- R1 11-6: PUSCH repetition Type A

pusch-RepetitionTypeA-r16                   SEQUENCE {

sharedSpectrumChAccess-r16                  ENUMERATED {supported}          OPTIONAL,

non-SharedSpectrumChAccess-r16              ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

-- R1 11-4b: DL priority indication in DCI with mixed DCI formats

dci-DL-PriorityIndicator-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 12-1a: UL priority indication in DCI with mixed DCI formats

dci-UL-PriorityIndicator-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 16-1e: Maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC for MAC-CE based pathloss reference RS update

maxNumberPathlossRS-Update-r16              ENUMERATED {n4, n8, n16, n32, n64}  OPTIONAL,

-- R1 18-9: Usage of the PDSCH starting time for HARQ-ACK type 2 codebook

type2-HARQ-ACK-Codebook-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 16-1g-1: Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges

maxTotalResourcesForAcrossFreqRanges-r16    SEQUENCE {

maxNumberResWithinSlotAcrossCC-AcrossFR-r16 ENUMERATED {n2, n4, n8, n12, n16, n32, n64, n128}        OPTIONAL,

maxNumberResAcrossCC-AcrossFR-r16           ENUMERATED {n2, n4, n8, n12, n16, n32, n40, n48, n64, n72, n80, n96, n128, n256}

OPTIONAL

}                                                                               OPTIONAL,

-- R1 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP - separate

harqACK-separateMultiDCI-MultiTRP-r16       SEQUENCE {

maxNumberLongPUCCHs-r16                         ENUMERATED {longAndLong, longAndShort, shortAndShort}    OPTIONAL

}                                                                               OPTIONAL,

-- R1 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP - joint

harqACK-jointMultiDCI-MultiTRP-r16          ENUMERATED {supported}              OPTIONAL,

-- R4 9-1: BWP switching on multiple CCs RRM requirements

bwp-SwitchingMultiCCs-r16                   CHOICE {

type1-r16                                   ENUMERATED {us100, us200},

type2-r16                                   ENUMERATED {us200, us400, us800, us1000}

}                                                                               OPTIONAL

]],

[[

targetSMTC-SCG-r16                          ENUMERATED {supported}              OPTIONAL,

supportRepetitionZeroOffsetRV-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 11-12: in-order CBG-based re-transmission

cbg-TransInOrderPUSCH-UL-r16                ENUMERATED {supported}              OPTIONAL

]],

[[

-- R4 6-3: Dormant BWP switching on multiple CCs RRM requirements

bwp-SwitchingMultiDormancyCCs-r16           CHOICE {

type1-r16                                   ENUMERATED {us100, us200},

type2-r16                                   ENUMERATED {us200, us400, us800, us1000}

}                                                                               OPTIONAL,

-- R1 16-2a-8: Indicates that retransmission scheduled by a different CORESETPoolIndex for multi-DCI multi-TRP is not supported.

supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16               ENUMERATED {notSupported}          OPTIONAL,

-- R1 22-10: Support of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier scheduling with different SCSs

pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16   ENUMERATED {mode2, mode3}          OPTIONAL

]],

[[

-- R1 16-1j-1: Support of 2 port CSI-RS for new beam identification

newBeamIdentifications2PortCSI-RS-r16       ENUMERATED {supported}              OPTIONAL,

-- R1 16-1j-2: Support of 2 port CSI-RS for pathloss estimation

pathlossEstimation2PortCSI-RS-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16       ENUMERATED {supported}              OPTIONAL

]],

[[

-- R1 31-1: Support of Desired Guard Symbol reporting and provided guard symbol reception.

guardSymbolReportReception-IAB-r17          ENUMERATED {supported}              OPTIONAL,

-- R1 31-2: support of restricted IAB-DU beam reception

restricted-IAB-DU-BeamReception-r17         ENUMERATED {supported}              OPTIONAL,

-- R1 31-3: support of recommended IAB-MT beam transmission for DL and UL beam

recommended-IAB-MT-BeamTransmission-r17     ENUMERATED {supported}              OPTIONAL,

-- R1 31-4: support of case 6 timing alignment indication reception

case6-TimingAlignmentReception-IAB-r17      ENUMERATED {supported}              OPTIONAL,

-- R1 31-5: support of case 7 timing offset indication reception and case 7 timing at parent-node indication reception

case7-TimingAlignmentReception-IAB-r17      ENUMERATED {supported}              OPTIONAL,

-- R1 31-6: support of desired DL Tx power adjustment reporting and DL Tx power adjustment reception

dl-tx-PowerAdjustment-IAB-r17               ENUMERATED {supported}              OPTIONAL,

-- R1 31-7: support of desired IAB-MT PSD range reporting

desired-ul-tx-PowerAdjustment-r17           ENUMERATED {supported}              OPTIONAL,

-- R1 31-8: support of monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node

fdm-SoftResourceAvailability-DynamicIndication-r17  ENUMERATED{supported}       OPTIONAL,

-- R1 31-10: Support of updated T_delta range reception

updated-T-DeltaRangeReception-r17           ENUMERATED{supported}               OPTIONAL,

-- R1 30-5: Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4

slotBasedDynamicPUCCH-Rep-r17               ENUMERATED {supported}              OPTIONAL,

-- R1 25-1: Support of HARQ-ACK deferral in case of TDD collision

sps-HARQ-ACK-Deferral-r17                   SEQUENCE {

non-SharedSpectrumChAccess-r17              ENUMERATED {supported}          OPTIONAL,

sharedSpectrumChAccess-r17                  ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

-- R1 23-1-1k Maximum number of configured CC lists (per UE)

unifiedJointTCI-commonUpdate-r17            INTEGER (1..4)                      OPTIONAL,

-- R1 23-2-1c PDCCH repetition with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot

mTRP-PDCCH-singleSpan-r17                   ENUMERATED {supported}              OPTIONAL,

-- R1 27-23: Support of more than one activated PRS processing windows across all active DL BWPs

supportedActivatedPRS-ProcessingWindow-r17  ENUMERATED {n2, n3, n4}             OPTIONAL,

cg-TimeDomainAllocationExtension-r17        ENUMERATED {supported}              OPTIONAL

]],

[[

-- R1 25-20: Propagation delay compensation based on Rel-15 TA procedure for TN and licensed

ta-BasedPDC-TN-NonSharedSpectrumChAccess-r17 ENUMERATED {supported}             OPTIONAL,

-- R1 31-11: Directional Collision Handling in DC operation

directionalCollisionDC-IAB-r17              ENUMERATED {supported}              OPTIONAL

]],

[[

dummy1                                      ENUMERATED {supported}              OPTIONAL,

dummy2                                      ENUMERATED {supported}              OPTIONAL,

dummy3                                      ENUMERATED {supported}              OPTIONAL,

dummy4                                      ENUMERATED {supported}              OPTIONAL,

srs-AdditionalRepetition-r17                ENUMERATED {supported}              OPTIONAL,

pusch-Repetition-CG-SDT-r17                 ENUMERATED {supported}              OPTIONAL

]],

[[

multiPDSCH-PerSlotType1-CB-Support-r17      ENUMERATED {supported}              OPTIONAL

]],

[[

supportedCSI-RS-ReportSettingList-r18       SupportedCSI-RS-ReportSettingList-r18                     OPTIONAL,

-- R1 43-3: Aperiodic beam indication for access link

ncr-AperiodicBeamInd-AccessLink-r18                     SEQUENCE {

scs-15kHz-r18                                           INTEGER (0..1)                            OPTIONAL,

scs-30kHz-r18                                           INTEGER (0..1)                            OPTIONAL,

scs-60kHz-r18                                           INTEGER (0..2)                            OPTIONAL,

scs-120kHz-r18                                          INTEGER (0..2)                            OPTIONAL

}                                                                                                     OPTIONAL,

-- R1 43-4: Semi-persistent beam indication for access link

ncr-Semi-PersistentBeamInd-AccessLink-r18               ENUMERATED {supported}                        OPTIONAL,

-- R1 43-5: Simulatenous UL transmission of backhaul link and C-Link

ncr-SimultaneousUL-BackhaulAndC-Link-r18                ENUMERATED {supported}                        OPTIONAL,

-- R1 43-6: Dedicated signalling for backhaul link beam indication

ncr-BackhaulBeamInd-r18                                 ENUMERATED {nonUnifiedTCI, unifiedTCI, both}  OPTIONAL,

-- R1 43-8: Adaptive beam for NCR backhaul link/C-link

ncr-AdaptiveBeamBackhaulAndC-Link-r18                   ENUMERATED {nonUnifiedTCI, unifiedTCI, both}  OPTIONAL,

-- R1 49-4a: Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3

nominalRBG-SizeOfConfig-3-FDRA-Type-0-DCI-1-3-r18       ENUMERATED {supported}                        OPTIONAL,

-- R1 49-4b: Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3

nominalRBG-SizeOfConfig-3-FDRA-Type-0-DCI-0-3-r18       ENUMERATED {supported}                        OPTIONAL,

-- R1 49-4c: Configurable Type-1A fields for DCI format 0_3/1_3

configurableType-1A-FieldsForDCI-0-3-And-1-3-r18        ENUMERATED {supported}                        OPTIONAL,

-- R1 49-4d: FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3

fdra-Type-1-Gty-2-4-8-16-RBs-RIV-DCI-1-3-And-0-3-r18    ENUMERATED {supported}                        OPTIONAL,

-- R1 51-3: Support 5 MHz channel bandwidth with 20 PRB CORESET0

support-5MHz-ChannelBW-20PRB-CORESET0-r18               ENUMERATED {supported}                        OPTIONAL,

-- R1 55-1: Additional SR periodicities

additionalSR-Periodicities-r18                          SEQUENCE {

scs-30kHz-r18                                           ENUMERATED {supported}                    OPTIONAL,

scs-120kHz-r18                                          ENUMERATED {supported}                    OPTIONAL

}                                                                                                     OPTIONAL,

-- R1 55-5: Enable MAC CE based pathloss RS updates for Type 1 CG-PUSCH

pathlossRS-UpdateForType1CG-PUSCH-r18                   ENUMERATED {supported}                        OPTIONAL

]]

}

Phy-ParametersCommon-v16a0 ::=                  SEQUENCE {

srs-PeriodicityAndOffsetExt-r16                 ENUMERATED {supported}          OPTIONAL

}

Phy-ParametersXDD-Diff ::=          SEQUENCE {

dynamicSFI                          ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH          ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH          ENUMERATED {supported}                      OPTIONAL,

...,

[[

dl-SchedulingOffset-PDSCH-TypeA     ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB     ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                 ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFRX-Diff ::=                  SEQUENCE {

dynamicSFI                                  ENUMERATED {supported}                      OPTIONAL,

dummy1                                      BIT STRING (SIZE (2))                       OPTIONAL,

twoFL-DMRS                                  BIT STRING (SIZE (2))                       OPTIONAL,

dummy2                                      BIT STRING (SIZE (2))                       OPTIONAL,

dummy3                                      BIT STRING (SIZE (2))                       OPTIONAL,

supportedDMRS-TypeDL                        ENUMERATED {type1, type1And2}               OPTIONAL,

supportedDMRS-TypeUL                        ENUMERATED {type1, type1And2}               OPTIONAL,

semiOpenLoopCSI                             ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutPMI                        ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutCQI                        ENUMERATED {supported}                      OPTIONAL,

onePortsPTRS                                BIT STRING (SIZE (2))                       OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols                 ENUMERATED {supported}                      OPTIONAL,

pucch-F2-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F3-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F4-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F0-2WithoutFH                         ENUMERATED {notSupported}                   OPTIONAL,

pucch-F1-3-4WithoutFH                       ENUMERATED {notSupported}                   OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot      ENUMERATED {supported}                      OPTIONAL,

uci-CodeBlockSegmentation                   ENUMERATED {supported}                      OPTIONAL,

onePUCCH-LongAndShortFormat                 ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-AnyOthersInSlot                    ENUMERATED {supported}                      OPTIONAL,

intraSlotFreqHopping-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-LBRM                                  ENUMERATED {supported}                      OPTIONAL,

pdcch-BlindDetectionCA                      INTEGER (4..16)                             OPTIONAL,

tpc-PUSCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-PUCCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-SRS-RNTI                                ENUMERATED {supported}                      OPTIONAL,

absoluteTPC-Command                         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-HalfPi-BPSK                           ENUMERATED {supported}                      OPTIONAL,

pucch-F3-4-HalfPi-BPSK                      ENUMERATED {supported}                      OPTIONAL,

almostContiguousCP-OFDM-UL                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-RS                                   ENUMERATED {supported}                      OPTIONAL,

sp-CSI-IM                                   ENUMERATED {supported}                      OPTIONAL,

tdd-MultiDL-UL-SwitchPerSlot                ENUMERATED {supported}                      OPTIONAL,

multipleCORESET                             ENUMERATED {supported}                      OPTIONAL,

...,

[[

csi-RS-IM-ReceptionForFeedback              CSI-RS-IM-ReceptionForFeedback              OPTIONAL,

csi-RS-ProcFrameworkForSRS                  CSI-RS-ProcFrameworkForSRS                  OPTIONAL,

csi-ReportFramework                         CSI-ReportFramework                         OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot       SEQUENCE {

sameSymbol                                  ENUMERATED {supported}                      OPTIONAL,

diffSymbol                                  ENUMERATED {supported}                      OPTIONAL

}                                                                                       OPTIONAL,

mux-SR-HARQ-ACK-PUCCH                       ENUMERATED {supported}                      OPTIONAL,

mux-MultipleGroupCtrlCH-Overlap             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB             ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                         ENUMERATED {supported}                      OPTIONAL,

dl-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

ul-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

cqi-TableAlt                                ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-UL           ENUMERATED {supported}                      OPTIONAL

]],

[[

pdcch-BlindDetectionNRDC                SEQUENCE {

pdcch-BlindDetectionMCG-UE              INTEGER (1..15),

pdcch-BlindDetectionSCG-UE              INTEGER (1..15)

}                                                                                       OPTIONAL,

mux-HARQ-ACK-PUSCH-DiffSymbol               ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 11-1b: Type 1 HARQ-ACK codebook support for relative TDRA for DL

type1-HARQ-ACK-Codebook-r16                 ENUMERATED {supported}                      OPTIONAL,

-- R1 11-8: Enhanced UL power control scheme

enhancedPowerControl-r16                    ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1b-1: TCI state activation across multiple CCs

simultaneousTCI-ActMultipleCC-r16           ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1b-2: Spatial relation update across multiple CCs

simultaneousSpatialRelationMultipleCC-r16   ENUMERATED {supported}                      OPTIONAL,

cli-RSSI-FDM-DL-r16                         ENUMERATED {supported}                      OPTIONAL,

cli-SRS-RSRP-FDM-DL-r16                     ENUMERATED {supported}                      OPTIONAL,

-- R1 19-3: Maximum MIMO Layer Adaptation

maxLayersMIMO-Adaptation-r16                ENUMERATED {supported}                      OPTIONAL,

-- R1 12-5: Configuration of aggregation factor per SPS configuration

aggregationFactorSPS-DL-r16                 ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1g: Resources for beam management, pathloss measurement, BFD, RLM and new beam identification

maxTotalResourcesForOneFreqRange-r16        SEQUENCE {

maxNumberResWithinSlotAcrossCC-OneFR-r16    ENUMERATED {n2, n4, n8, n12, n16, n32, n64, n128}    OPTIONAL,

maxNumberResAcrossCC-OneFR-r16              ENUMERATED {n2, n4, n8, n12, n16, n32, n40, n48, n64, n72, n80, n96, n128, n256}

OPTIONAL

}                                           OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings

csi-ReportFrameworkExt-r16                  CSI-ReportFrameworkExt-r16                  OPTIONAL

]],

[[

twoTCI-Act-servingCellInCC-List-r16         ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 22-11: Support of 'cri-RI-CQI' report without non-PMI-PortIndication

cri-RI-CQI-WithoutNon-PMI-PortInd-r16       ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 25-11: 4-bits subband CQI for TN and licensed

cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17  ENUMERATED {supported}               OPTIONAL

]],

[[

multipleCORESET-RedCap-r17                  ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFR1 ::=                       SEQUENCE {

pdcch-MonitoringSingleOccasion              ENUMERATED {supported}                      OPTIONAL,

scs-60kHz                                   ENUMERATED {supported}                      OPTIONAL,

pdsch-256QAM-FR1                            ENUMERATED {supported}                      OPTIONAL,

pdsch-RE-MappingFR1-PerSymbol               ENUMERATED {n10, n20}                       OPTIONAL,

...,

[[

pdsch-RE-MappingFR1-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}         OPTIONAL

]],

[[

-- R1 22-12: PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a

-- slot

pdcch-MonitoringSingleSpanFirst4Sym-r16     ENUMERATED {supported}                      OPTIONAL

]],

[[

-- similar to NTN R1 26-10: K1 range extension defined for ATG as well

k1-RangeExtensionATG-r18                    ENUMERATED {supported}                      OPTIONAL,

-- similar to NTN R1 26-5: Increasing the number of HARQ processes defined for ATG as well

maxHARQ-ProcessNumberATG-r18                ENUMERATED {u16d32, u32d16, u32d32}         OPTIONAL,

-- similar to NTN R1 26-1: Uplink Time and Frequency pre-compensation and timing relationship enhancements defined for ATG as well

uplinkPreCompensationATG-r18                ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFR2 ::=                       SEQUENCE {

dummy                                       ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSymbol               ENUMERATED {n6, n20}                                    OPTIONAL,

...,

[[

pCell-FR2                                   ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}     OPTIONAL

]],

[[

-- R1 16-1c: Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH

defaultSpatialRelationPathlossRS-r16        ENUMERATED {supported}                                  OPTIONAL,

-- R1 16-1d: Support of spatial relation update for AP-SRS via MAC CE

spatialRelationUpdateAP-SRS-r16             ENUMERATED {supported}                                  OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16  ENUMERATED {n0, n1, n2, n4, n8, n16}           OPTIONAL

]]

}

-- TAG-PHY-PARAMETERS-STOP

-- ASN1STOP

Phy-ParametersFRX-Diff field descriptions

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework These fields are optionally present in fr1-fr2-Add-UE-NR-Capabilities in UE-NR-Capability. They shall not be set in any other instance of the IE Phy-ParametersFRX-Diff. If the network configures the UE with serving cells on both FR1 and FR2 bands, these parameters, if present, limit the corresponding parameters in MIMO-ParametersPerBand.

#### –	Phy-ParametersMRDC

The IE Phy-ParametersMRDC is used to convey physical layer capabilities for MR-DC.

Phy-ParametersMRDC information element

-- ASN1START

-- TAG-PHY-PARAMETERSMRDC-START

Phy-ParametersMRDC ::=              SEQUENCE {

naics-Capability-List               SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry         OPTIONAL,

...,

[[

spCellPlacement                     CarrierAggregationVariant                                                   OPTIONAL

]],

[[

-- R1 18-3b: Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of TDD PCell

tdd-PCellUL-TX-AllUL-Subframe-r16   ENUMERATED {supported}                                                      OPTIONAL,

-- R1 18-3a: Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of FDD PCell

fdd-PCellUL-TX-AllUL-Subframe-r16   ENUMERATED {supported}                                                      OPTIONAL

]]

}

NAICS-Capability-Entry ::=          SEQUENCE {

numberOfNAICS-CapableCC             INTEGER(1..5),

numberOfAggregatedPRB               ENUMERATED {n50, n75, n100, n125, n150, n175, n200, n225,

n250, n275, n300, n350, n400, n450, n500, spare},

...

}

-- TAG-PHY-PARAMETERSMRDC-STOP

-- ASN1STOP

PHY-ParametersMRDC field descriptions

naics-Capability-List Indicates that UE in MR-DC supports NAICS as defined in TS 36.331 [10].

#### –	Phy-ParametersSharedSpectrumChAccess

The IE Phy-ParametersSharedSpectrumChAccess is used to convey the physical layer capabilities specific for shared spectrum channel access.

Phy-ParametersSharedSpectrumChAccess information element

-- ASN1START

-- TAG-PHY-PARAMETERSSHAREDSPECTRUMCHACCESS-START

Phy-ParametersSharedSpectrumChAccess-r16 ::=    SEQUENCE {

-- 10-32 (1-2): SS block based SINR measurement (SS-SINR) for unlicensed spectrum

ss-SINR-Meas-r16                                ENUMERATED {supported}                      OPTIONAL,

-- 10-33 (2-32a): Semi-persistent CSI report on PUCCH for unlicensed spectrum

sp-CSI-ReportPUCCH-r16                          ENUMERATED {supported}                      OPTIONAL,

-- 10-33a (2-32b): Semi-persistent CSI report on PUSCH for unlicensed spectrum

sp-CSI-ReportPUSCH-r16                          ENUMERATED {supported}                      OPTIONAL,

-- 10-34 (3-6): Dynamic SFI monitoring for unlicensed spectrum

dynamicSFI-r16                                  ENUMERATED {supported}                      OPTIONAL,

-- 10-35c (4-19c): SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-

-- ACK/CSI are supposed to be sent with different starting symbols in a slot for unlicensed spectrum

-- 10-35 (4-19): SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-

-- ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot for unlicensed spectrum

mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot-r16       SEQUENCE {

sameSymbol-r16                                  ENUMERATED {supported}                  OPTIONAL,

diffSymbol-r16                                  ENUMERATED {supported}                  OPTIONAL

}                                                                                           OPTIONAL,

-- 10-35a (4-19a): Overlapping PUCCH resources have different starting symbols in a slot for unlicensed spectrum

mux-SR-HARQ-ACK-PUCCH-r16                       ENUMERATED {supported}                      OPTIONAL,

-- 10-35b (4-19b): SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when

-- SR/HARQ ACK/CSI are supposed to be sent with the same or different starting symbol in a slot for unlicensed spectrum

mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot-r16      ENUMERATED {supported}                      OPTIONAL,

-- 10-36 (4-28): HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols for unlicensed spectrum

mux-HARQ-ACK-PUSCH-DiffSymbol-r16               ENUMERATED {supported}                      OPTIONAL,

-- 10-37 (4-23): Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 for unlicensed spectrum

pucch-Repetition-F1-3-4-r16                     ENUMERATED {supported}                      OPTIONAL,

-- 10-38 (5-14): Type 1 configured PUSCH repetitions over multiple slots for unlicensed spectrum

type1-PUSCH-RepetitionMultiSlots-r16            ENUMERATED {supported}                      OPTIONAL,

-- 10-39 (5-16): Type 2 configured PUSCH repetitions over multiple slots for unlicensed spectrum

type2-PUSCH-RepetitionMultiSlots-r16            ENUMERATED {supported}                      OPTIONAL,

-- 10-40 (5-17): PUSCH repetitions over multiple slots for unlicensed spectrum

pusch-RepetitionMultiSlots-r16                  ENUMERATED {supported}                      OPTIONAL,

-- 10-40a (5-17a): PDSCH repetitions over multiple slots for unlicensed spectrum

pdsch-RepetitionMultiSlots-r16                  ENUMERATED {supported}                      OPTIONAL,

-- 10-41 (5-18): DL SPS

downlinkSPS-r16                                 ENUMERATED {supported}                      OPTIONAL,

-- 10-42 (5-19): Type 1 Configured UL grant

configuredUL-GrantType1-r16                     ENUMERATED {supported}                      OPTIONAL,

-- 10-43 (5-20): Type 2 Configured UL grant

configuredUL-GrantType2-r16                     ENUMERATED {supported}                      OPTIONAL,

-- 10-44 (5-21): Pre-emption indication for DL

pre-EmptIndication-DL-r16                       ENUMERATED {supported}                      OPTIONAL,

...

}

-- TAG-PHY-PARAMETERSSHAREDSPECTRUMCHACCESS-STOP

-- ASN1STOP

#### –	PosSRS-RRC-Inactive-OutsideInitialUL-BWP

The IE PosSRS-RRC-Inactive-OutsideInitialUL-BWP is used to convey the capabilities supported by the UE for SRS for Positioning transmission in RRC_INACTIVE state configured outside initial UL BWP.

PosSRS-RRC-Inactive-OutsideInitialUL-BWP information element

-- ASN1START

-- TAG-POSSRS-RRC-INACTIVE-OUTSIDEINITIALUL-BWP-START

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17::= SEQUENCE {

-- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

maxSRSposBandwidthForEachSCS-withinCC-FR1-r17   ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40,

mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100}             OPTIONAL,

maxSRSposBandwidthForEachSCS-withinCC-FR2-r17   ENUMERATED {mhz50, mhz100, mhz200, mhz400}                   OPTIONAL,

maxNumOfSRSposResourceSets-r17                  ENUMERATED {n1, n2, n4, n8, n12, n16}                         OPTIONAL,

maxNumOfPeriodicSRSposResources-r17             ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                    OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17      ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}        OPTIONAL,

differentNumerologyBetweenSRSposAndInitialBWP-r17  ENUMERATED {supported}                                     OPTIONAL,

srsPosWithoutRestrictionOnBWP-r17               ENUMERATED {supported}                                        OPTIONAL,

maxNumOfPeriodicAndSemipersistentSRSposResources-r17  ENUMERATED {n1, n2, n4, n8, n16, n32, n64}              OPTIONAL,

maxNumOfPeriodicAndSemipersistentSRSposResourcesPerSlot-r17  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}  OPTIONAL,

differentCenterFreqBetweenSRSposAndInitialBWP-r17  ENUMERATED {supported}                                     OPTIONAL,

switchingTimeSRS-TX-OtherTX-r17                 ENUMERATED {us100, us140, us200, us300, us500}                OPTIONAL,

-- R1 27-15c: Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS

maxNumOfSemiPersistentSRSposResources-r17       ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                    OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}      OPTIONAL,

...

}

-- TAG-POSSRS-RRC-INACTIVE-OUTSIDEINITIALUL-BWP-STOP

-- ASN1STOP

#### –	PowSav-Parameters

The IE PowSav-Parameters is used to convey the capabilities supported by the UE for the power saving preferences.

PowSav-Parameters information element

-- ASN1START

-- TAG-POWSAV-PARAMETERS-START

PowSav-Parameters-r16 ::=         SEQUENCE {

powSav-ParametersCommon-r16               PowSav-ParametersCommon-r16                                        OPTIONAL,

powSav-ParametersFRX-Diff-r16             PowSav-ParametersFRX-Diff-r16                                      OPTIONAL,

...

}

PowSav-Parameters-v1700 ::=     SEQUENCE {

powSav-ParametersFR2-2-r17      PowSav-ParametersFR2-2-r17                                                   OPTIONAL,

...

}

PowSav-ParametersCommon-r16 ::=    SEQUENCE {

drx-Preference-r16                        ENUMERATED {supported}                                             OPTIONAL,

maxCC-Preference-r16                      ENUMERATED {supported}                                             OPTIONAL,

releasePreference-r16                     ENUMERATED {supported}                                             OPTIONAL,

-- R1 19-4a: UE assistance information

minSchedulingOffsetPreference-r16         ENUMERATED {supported}                                             OPTIONAL,

...

}

PowSav-ParametersFRX-Diff-r16 ::=    SEQUENCE {

maxBW-Preference-r16                      ENUMERATED {supported}                                             OPTIONAL,

maxMIMO-LayerPreference-r16               ENUMERATED {supported}                                             OPTIONAL,

...

}

PowSav-ParametersFR2-2-r17 ::=      SEQUENCE {

maxBW-Preference-r17                      ENUMERATED {supported}                                             OPTIONAL,

maxMIMO-LayerPreference-r17               ENUMERATED {supported}                                             OPTIONAL,

...

}

-- TAG-POWSAV-PARAMETERS-STOP

-- ASN1STOP

#### –	ProcessingParameters

The IE ProcessingParameters is used to indicate PDSCH/PUSCH processing capabilities supported by the UE.

ProcessingParameters information element

-- ASN1START

-- TAG-PROCESSINGPARAMETERS-START

ProcessingParameters ::=        SEQUENCE {

fallback                        ENUMERATED {sc, cap1-only},

differentTB-PerSlot              SEQUENCE {

upto1                          NumberOfCarriers                    OPTIONAL,

upto2                          NumberOfCarriers                    OPTIONAL,

upto4                          NumberOfCarriers                    OPTIONAL,

upto7                          NumberOfCarriers                    OPTIONAL

}                                                                 OPTIONAL

}

NumberOfCarriers ::=    INTEGER (1..16)

-- TAG-PROCESSINGPARAMETERS-STOP

-- ASN1STOP

#### –	PRS-ProcessingCapabilityOutsideMGinPPWperType

The IE PRS-ProcessingCapabilityOutsideMGinPPWperType is used to indicate DL PRS Processing Capability outside MG capabilities supported by the UE.

PRS-ProcessingCapabilityOutsideMGinPPWperType information element

-- ASN1START

-- TAG-PRS-PROCESSINGCAPABILITYOUTSIDEMGINPPWPERType-START

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17                                 ENUMERATED {type1A, type1B, type2},

ppw-dl-PRS-BufferType-r17                             ENUMERATED {type1, type2, ...},

ppw-durationOfPRS-Processing-r17                      CHOICE {

ppw-durationOfPRS-Processing1-r17                     SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17              ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

ppw-durationOfPRS-ProcessingSymbolsT-r17              ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280}

},

ppw-durationOfPRS-Processing2-r17                     SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17             ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12},

ppw-durationOfPRS-ProcessingSymbolsT2-r17             ENUMERATED {ms4, ms5, ms6, ms8}

}

}                                                                                                                          OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17            SEQUENCE {

scs15-r17                                             ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs30-r17                                             ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs60-r17                                             ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs120-r17                                            ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

...

},

ppw-maxNumOfDL-Bandwidth-r17                          CHOICE {

fr1-r17                                               ENUMERATED {mhz5, mhz10, mhz20, mhz40,	mhz50, mhz80, mhz100},

fr2-r17                                               ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}                                                                                                                          OPTIONAL

}

-- TAG-PRS-PROCESSINGCAPABILITYOUTSIDEMGINPPWPERType-STOP

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, eutra, utra-fdd-v1610, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	RedCapParameters

The IE RedCapParameters is used to indicate the UE capabilities supported by RedCap UEs.

RedCapParameters information element

-- ASN1START

-- TAG-REDCAPPARAMETERS-START

RedCapParameters-r17::=                   SEQUENCE {

-- R1 28-1: RedCap UE

supportOfRedCap-r17                       ENUMERATED {supported}                                      OPTIONAL,

supportOf16DRB-RedCap-r17                 ENUMERATED {supported}                                      OPTIONAL

}

RedCapParameters-v1740::=                 SEQUENCE {

ncd-SSB-ForRedCapInitialBWP-SDT-r17       ENUMERATED {supported}                                      OPTIONAL

}

-- TAG-REDCAPPARAMETERS-STOP

-- ASN1STOP

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                                   SEQUENCE {

supportedBandListNR                                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList                        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter                           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540                  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested                          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550                  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560                  BandCombinationList-v1560                   OPTIONAL

]],

[[

supportedBandCombinationList-v1610                  BandCombinationList-v1610                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-r16    BandCombinationListSidelinkEUTRA-NR-r16     OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16     BandCombinationList-UplinkTxSwitch-r16      OPTIONAL

]],

[[

supportedBandCombinationList-v1630                  BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-v1630  BandCombinationListSidelinkEUTRA-NR-v1630   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1630   BandCombinationList-UplinkTxSwitch-v1630    OPTIONAL

]],

[[

supportedBandCombinationList-v1640                  BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1640   BandCombinationList-UplinkTxSwitch-v1640    OPTIONAL

]],

[[

supportedBandCombinationList-v1650                  BandCombinationList-v1650                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1650   BandCombinationList-UplinkTxSwitch-v1650    OPTIONAL

]],

[[

extendedBand-n77-r16                                ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-UplinkTxSwitch-v1670   BandCombinationList-UplinkTxSwitch-v1670    OPTIONAL

]],

[[

supportedBandCombinationList-v1680                  BandCombinationList-v1680                   OPTIONAL

]],

[[

supportedBandCombinationList-v1690                  BandCombinationList-v1690                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1690   BandCombinationList-UplinkTxSwitch-v1690    OPTIONAL

]],

[[

supportedBandCombinationList-v1700                  BandCombinationList-v1700                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1700   BandCombinationList-UplinkTxSwitch-v1700    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-r17   OCTET STRING                                OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSL-NonRelayDiscovery-r17 OCTET STRING                               OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSidelinkEUTRA-NR-v1710  BandCombinationListSidelinkEUTRA-NR-v1710   OPTIONAL,

sidelinkRequested-r17                               ENUMERATED {true}                           OPTIONAL,

extendedBand-n77-2-r17                              ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-v1720                  BandCombinationList-v1720                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1720   BandCombinationList-UplinkTxSwitch-v1720    OPTIONAL

]],

[[

supportedBandCombinationList-v1730                  BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1730   BandCombinationList-UplinkTxSwitch-v1730    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-v1730 BandCombinationListSL-Discovery-r17         OPTIONAL,

supportedBandCombinationListSL-NonRelayDiscovery-v1730 BandCombinationListSL-Discovery-r17      OPTIONAL

]],

[[

supportedBandCombinationList-v1740                  BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1740   BandCombinationList-UplinkTxSwitch-v1740    OPTIONAL

]],

[[

supportedBandCombinationList-v1760                  BandCombinationList-v1760                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1760   BandCombinationList-UplinkTxSwitch-v1760    OPTIONAL

]],

[[

supportedBandCombinationList-v1770                  BandCombinationList-v1770                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1770   BandCombinationList-UplinkTxSwitch-v1770    OPTIONAL

]],

[[

supportedBandCombinationList-v1800                  BandCombinationList-v1800                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1800   BandCombinationList-UplinkTxSwitch-v1800    OPTIONAL,

supportedBandCombinationListSL-U2U-Relay-r18        SEQUENCE {

supportedBandCombinationListSL-U2U-RelayDiscovery-r18  OCTET STRING             OPTIONAL,  -- Contains PC5

-- BandCombinationListSidelinkNR-r16

supportedBandCombinationListSL-U2U-DiscoveryExt BandCombinationListSL-Discovery-r17         OPTIONAL

}                                                                                               OPTIONAL

]]

}

RF-Parameters-v15g0 ::=                   SEQUENCE {

supportedBandCombinationList-v15g0        BandCombinationList-v15g0                   OPTIONAL

}

RF-Parameters-v16a0 ::=                            SEQUENCE {

supportedBandCombinationList-v16a0                 BandCombinationList-v16a0                    OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v16a0  BandCombinationList-UplinkTxSwitch-v16a0     OPTIONAL

}

RF-Parameters-v16c0 ::=                            SEQUENCE {

supportedBandListNR-v16c0                          SEQUENCE (SIZE (1..maxBands)) OF BandNR-v16c0

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]],

[[

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

]],

[[

asymmetricBandwidthCombinationSet     BIT STRING (SIZE (1..32))           OPTIONAL

]],

[[

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-r16 SharedSpectrumChAccessParamsPerBand-r16 OPTIONAL,

-- R1 11-7b: Independent cancellation of the overlapping PUSCHs in an intra-band UL CA

cancelOverlappingPUSCH-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-1: Multiple LTE-CRS rate matching patterns

multipleRateMatchingEUTRA-CRS-r16       SEQUENCE {

maxNumberPatterns-r16               INTEGER (2..6),

maxNumberNon-OverlapPatterns-r16    INTEGER (1..3)

}                                                                               OPTIONAL,

-- R1 14-1a: Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier

overlapRateMatchingEUTRA-CRS-r16        ENUMERATED {supported}                  OPTIONAL,

-- R1 14-2: PDSCH Type B mapping of length 9 and 10 OFDM symbols

pdsch-MappingTypeB-Alt-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-3: One slot periodic TRS configuration for FR1

oneSlotPeriodicTRS-r16                  ENUMERATED {supported}                  OPTIONAL,

olpc-SRS-Pos-r16                        OLPC-SRS-Pos-r16                        OPTIONAL,

spatialRelationsSRS-Pos-r16             SpatialRelationsSRS-Pos-r16             OPTIONAL,

simulSRS-MIMO-TransWithinBand-r16       ENUMERATED {n2}                         OPTIONAL,

channelBW-DL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                          SEQUENCE {

scs-60kHz                           ENUMERATED {supported}              OPTIONAL,

scs-120kHz                          ENUMERATED {supported}              OPTIONAL

}

}                                                                               OPTIONAL,

channelBW-UL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                              SEQUENCE {

scs-60kHz                               ENUMERATED {supported}          OPTIONAL,

scs-120kHz                              ENUMERATED {supported}          OPTIONAL

}

}                                                                               OPTIONAL,

rasterShift7dot5-IAB-r16                ENUMERATED {supported}                  OPTIONAL,

ue-PowerClass-v1610                     ENUMERATED {pc1dot5}                    OPTIONAL,

condHandover-r16                        ENUMERATED {supported}                  OPTIONAL,

condHandoverFailure-r16                 ENUMERATED {supported}                  OPTIONAL,

condHandoverTwoTriggerEvents-r16        ENUMERATED {supported}                  OPTIONAL,

condPSCellChange-r16                    ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeTwoTriggerEvents-r16    ENUMERATED {supported}                  OPTIONAL,

mpr-PowerBoost-FR2-r16                  ENUMERATED {supported}                  OPTIONAL,

-- R1 11-9: Multiple active configured grant configurations for a BWP of a serving cell

activeConfiguredGrant-r16               SEQUENCE {

maxNumberConfigsPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n12},

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 11-9a: Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell

jointReleaseConfiguredGrantType2-r16    ENUMERATED {supported}                  OPTIONAL,

-- R1 12-2: Multiple SPS configurations

sps-r16                                 SEQUENCE {

maxNumberConfigsPerBWP-r16                  INTEGER (1..8),

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 12-2a: Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell

jointReleaseSPS-r16                     ENUMERATED {supported}                  OPTIONAL,

-- R1 13-19: Simultaneous positioning SRS and MIMO SRS transmission within a band across multiple CCs

simulSRS-TransWithinBand-r16            ENUMERATED {n2}                         OPTIONAL,

trs-AdditionalBandwidth-r16             ENUMERATED {trs-AddBW-Set1, trs-AddBW-Set2}  OPTIONAL,

handoverIntraF-IAB-r16                  ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R1 22-5a: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for intra-band UL CA

-- R1 22-5c: Simultaneous transmission of SRS for antenna switching and SRS for antenna switching for intra-band UL CA

simulTX-SRS-AntSwitchingIntraBandUL-CA-r16  SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-v1630   SharedSpectrumChAccessParamsPerBand-v1630   OPTIONAL

]],

[[

handoverUTRA-FDD-r16                      ENUMERATED {supported}                       OPTIONAL,

-- R4 7-4: Report the shorter transient capability supported by the UE: 2, 4 or 7us

enhancedUL-TransientPeriod-r16            ENUMERATED {us2, us4, us7}                   OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1640 SharedSpectrumChAccessParamsPerBand-v1640    OPTIONAL

]],

[[

type1-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

type2-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

pusch-RepetitionMultiSlots-v1650          ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType1-v1650             ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType2-v1650             ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1650 SharedSpectrumChAccessParamsPerBand-v1650    OPTIONAL

]],

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

-- R1 36-1: Support of 1024QAM for PDSCH for FR1

pdsch-1024QAM-FR1-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R4 22-1 support of FR2 HST operation

ue-PowerClass-v1700                       ENUMERATED {pc5, pc6, pc7}                   OPTIONAL,

-- R1 24: NR extension to 71GHz (FR2-2)

fr2-2-AccessParamsPerBand-r17             FR2-2-AccessParamsPerBand-r17                OPTIONAL,

rlm-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

bfd-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

cg-SDT-r17                                ENUMERATED {supported}                       OPTIONAL,

locationBasedCondHandover-r17             ENUMERATED {supported}                       OPTIONAL,

timeBasedCondHandover-r17                 ENUMERATED {supported}                       OPTIONAL,

eventA4BasedCondHandover-r17              ENUMERATED {supported}                       OPTIONAL,

mn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

sn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3a: PDCCH skipping

pdcch-SkippingWithoutSSSG-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3b: 2 search space sets group switching

sssg-Switching-1BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3c: 3 search space sets group switching

sssg-Switching-2BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3d: 2 search space sets group switching with PDCCH skipping

pdcch-SkippingWithSSSG-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3e: Support Search space set group switching capability 2 for FR1

searchSpaceSetGrp-switchCap2-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 26-1: Uplink Time and Frequency pre-compensation and timing relationship enhancements

uplinkPreCompensation-r17                 ENUMERATED {supported}                       OPTIONAL,

-- R1 26-4: UE reporting of information related to TA pre-compensation

uplink-TA-Reporting-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-5: Increasing the number of HARQ processes

max-HARQ-ProcessNumber-r17                ENUMERATED {u16d32, u32d16, u32d32}          OPTIONAL,

-- R1 26-6: Type-2 HARQ codebook enhancement

type2-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6a: Type-1 HARQ codebook enhancement

type1-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6b: Type-3 HARQ codebook enhancement

type3-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-9: UE-specific K_offset

ue-specific-K-Offset-r17                  ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1f: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1

multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1g: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1

multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 14-4: Parallel PRS measurements in RRC_INACTIVE state, FR1/FR2 diff

parallelPRS-MeasRRC-Inactive-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 27-1-2: Support of UE-TxTEGs for UL TDOA

nr-UE-TxTEG-ID-MaxSupport-r17             ENUMERATED {n1, n2, n3, n4, n6, n8}          OPTIONAL,

-- R1 27-17: PRS processing in RRC_INACTIVE

prs-ProcessingRRC-Inactive-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-3-2: DL PRS measurement outside MG and in a PRS processing window

prs-ProcessingWindowType1A-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType1B-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType2-r17             ENUMERATED {option1, option2, option3}       OPTIONAL,

-- R1 27-15: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP

srs-AllPosResourcesRRC-Inactive-r17       SRS-AllPosResourcesRRC-Inactive-r17          OPTIONAL,

-- R1 27-16: OLPC for positioning SRS in RRC_INACTIVE state - gNB

olpc-SRS-PosRRC-Inactive-r17              OLPC-SRS-Pos-r16                             OPTIONAL,

-- R1 27-19: Spatial relation for positioning SRS in RRC_INACTIVE state - gNB

spatialRelationsSRS-PosRRC-Inactive-r17   SpatialRelationsSRS-Pos-r16                  OPTIONAL,

-- R1 30-1: Increased maximum number of PUSCH Type A repetitions

maxNumberPUSCH-TypeA-Repetition-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-2: PUSCH Type A repetitions based on available slots

puschTypeA-RepetitionsAvailSlot-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3: TB processing over multi-slot PUSCH

tb-ProcessingMultiSlotPUSCH-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3a: Repetition of TB processing over multi-slot PUSCH

tb-ProcessingRepMultiSlotPUSCH-r17        ENUMERATED {supported}                       OPTIONAL,

-- R1 30-4: The maximum duration for DM-RS bundling

maxDurationDMRS-Bundling-r17              SEQUENCE {

fdd-r17                                   ENUMERATED {n4, n8, n16, n32}            OPTIONAL,

tdd-r17                                   ENUMERATED {n2, n4, n8, n16}             OPTIONAL

}                                                                                      OPTIONAL,

-- R1 30-6: Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

pusch-RepetitionMsg3-r17                  ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1710 SharedSpectrumChAccessParamsPerBand-v1710    OPTIONAL,

-- R4 25-2: Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions

-- on normal operations with the serving cell

parallelMeasurementWithoutRestriction-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 25-5: Parallel measurements on multiple NGSO satellites within a SMTC

maxNumber-NGSO-SatellitesWithinOneSMTC-r17 ENUMERATED {n1, n2, n3, n4}                 OPTIONAL,

-- R1 26-10: K1 range extension

k1-RangeExtension-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R1 35-1: Aperiodic CSI-RS for tracking for fast SCell activation

aperiodicCSI-RS-FastScellActivation-r17   SEQUENCE {

maxNumberAperiodicCSI-RS-PerCC-r17        ENUMERATED {n8, n16, n32, n48, n64, n128, n255},

maxNumberAperiodicCSI-RS-AcrossCCs-r17    ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024}

}                                                                                      OPTIONAL,

-- R1 35-2: Aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth

aperiodicCSI-RS-AdditionalBandwidth-r17   ENUMERATED {addBW-Set1, addBW-Set2}          OPTIONAL,

-- R1 28-1a: RRC-configured DL BWP without CD-SSB or NCD-SSB

bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 28-3: Half-duplex FDD operation type A for (e)RedCap UE

halfDuplexFDD-TypeA-RedCap-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 OPTIONAL,

-- R4 15-3 UE support of CBW for 480kHz SCS

channelBWs-DL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 15-4 UE support of CBW for 960kHz SCS

channelBWs-DL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 17-1 UL gap for Tx power management

ul-GapFR2-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 25-4: One-shot HARQ ACK feedback triggered by DCI format 1_2

oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 25-5: PHY priority handling for one-shot HARQ ACK feedback

oneShotHARQ-feedbackPhy-Priority-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 25-6: Enhanced type 3 HARQ-ACK codebook feedback

enhancedType3-HARQ-CodebookFeedback-r17   SEQUENCE {

enhancedType3-HARQ-Codebooks-r17          ENUMERATED {n1, n2, n4, n8},

maxNumberPUCCH-Transmissions-r17          ENUMERATED {n1, n2, n3, n4, n5, n6, n7}

}                                                                                      OPTIONAL,

-- R1 25-7: Triggered HARQ-ACK codebook re-transmission

triggeredHARQ-CodebookRetx-r17              SEQUENCE {

minHARQ-Retx-Offset-r17                     ENUMERATED {n-7, n-5, n-3, n-1, n1},

maxHARQ-Retx-Offset-r17                     ENUMERATED {n4, n6, n8, n10, n12, n14, n16, n18, n20, n22, n24}

}                                                                                      OPTIONAL

]],

[[

-- R4 22-2 support of one shot large UL timing adjustment

ue-OneShotUL-TimingAdj-r17                        ENUMERATED {supported}               OPTIONAL,

-- R1 25-2: Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8

pucch-Repetition-F0-2-r17                         ENUMERATED {supported}               OPTIONAL,

-- R1 25-11a: 4-bits subband CQI for NTN and unlicensed

cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17   ENUMERATED {supported}               OPTIONAL,

-- R1 25-16: HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH

mux-HARQ-ACK-DiffPriorities-r17                   ENUMERATED {supported}               OPTIONAL,

-- R1 25-20a: Propagation delay compensation based on Rel-15 TA procedure for NTN and unlicensed

ta-BasedPDC-NTN-SharedSpectrumChAccess-r17        ENUMERATED {supported}               OPTIONAL,

-- R1 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17  ENUMERATED {supported}               OPTIONAL,

-- R1 33-2e: Multiple G-RNTIs for group-common PDSCHs

maxNumberG-RNTI-r17                               INTEGER (2..8)                       OPTIONAL,

-- R1 33-2f: Dynamic multicast with DCI format 4_2

dynamicMulticastDCI-Format4-2-r17                 ENUMERATED {supported}               OPTIONAL,

-- R1 33-2i: Supported maximal modulation order for multicast PDSCH

maxModulationOrderForMulticast-r17                CHOICE {

fr1-r17                                           ENUMERATED {qam256, qam1024},

fr2-r17                                           ENUMERATED {qam64, qam256}

}                                                                                                                          OPTIONAL,

-- R1 33-3-1: Dynamic Slot-level repetition for group-common PDSCH for TN and licensed

dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17  ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-3-1a: Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed

dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-4-1: DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast

nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17                ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

maxNumberG-CS-RNTI-r17                                          INTEGER (2..8)                                             OPTIONAL,

-- R1 33-10: Support group-common PDSCH RE-level rate matching for multicast

re-LevelRateMatchingForMulticast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 36-1a: Support of 1024QAM for PDSCH with maximum 2 MIMO layers for FR1

pdsch-1024QAM-2MIMO-FR1-r17                                     ENUMERATED {supported}                                     OPTIONAL,

-- R4 14-3 PRS measurement without MG

prs-MeasurementWithoutMG-r17                                    ENUMERATED {cpLength, quarterSymbol, halfSymbol, halfSlot} OPTIONAL,

-- R4 25-7: The number of target LEO satellites the UE can monitor per carrier

maxNumber-LEO-SatellitesPerCarrier-r17                          INTEGER (3..4)                                             OPTIONAL,

-- R1 27-3-3 DL PRS Processing Capability outside MG - buffering capability

prs-ProcessingCapabilityOutsideMGinPPW-r17    SEQUENCE (SIZE(1..3)) OF PRS-ProcessingCapabilityOutsideMGinPPWperType-r17   OPTIONAL,

-- R1 27-15a: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS

srs-SemiPersistent-PosResourcesRRC-Inactive-r17                 SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17                       ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}                                                                                                                          OPTIONAL,

-- R2: UE support of CBW for 120kHz SCS

channelBWs-DL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL,

channelBWs-UL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL

]],

[[

-- R1 30-4a: DM-RS bundling for PUSCH repetition type A

dmrs-BundlingPUSCH-RepTypeA-r17                                 ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4b: DM-RS bundling for PUSCH repetition type B

dmrs-BundlingPUSCH-RepTypeB-r17                                 ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4c: DM-RS bundling for TB processing over multi-slot PUSCH

dmrs-BundlingPUSCH-multiSlot-r17                                ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4d: DMRS bundling for PUCCH repetitions

dmrs-BundlingPUCCH-Rep-r17                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4e: Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH

interSlotFreqHopInterSlotBundlingPUSCH-r17                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4f: Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling

interSlotFreqHopPUCCH-r17                                       ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4g: Restart DM-RS bundling

dmrs-BundlingRestart-r17                                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4h: DM-RS bundling for non-back-to-back transmission

dmrs-BundlingNonBackToBackTX-r17                                ENUMERATED {supported}                                     OPTIONAL

]],

[[

-- R1 33-5-1e: Dynamic Slot-level repetition for SPS group-common PDSCH for multicast

maxDynamicSlotRepetitionForSPS-Multicast-r17                    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-5-1g: DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1i: Multicast SPS scheduling with DCI format 4_2

sps-MulticastDCI-Format4-2-r17                                  ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-2: Multiple SPS group-common PDSCH configuration on PCell

sps-MulticastMultiConfig-r17                                    INTEGER (1..8)                                             OPTIONAL,

-- R1 33-6-1: DL priority indication for multicast in DCI

priorityIndicatorInDCI-Multicast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-1a: DL priority configuration for SPS multicast

priorityIndicatorInDCI-SPS-Multicast-r17                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-2: Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities

-- for unicast and multicast at a UE

twoHARQ-ACK-CodebookForUnicastAndMulticast-r17                  ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-3: More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-9: Supporting unicast PDCCH to release SPS group-common PDSCH

releaseSPS-MulticastWithCS-RNTI-r17                             ENUMERATED {supported}                                     OPTIONAL

]],

[[

-- R1 41-3-1a  UE automomous TA adjustment when cell-reselection happens

posUE-TA-AutoAdjustment-r18                                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 41-4-6a   support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers

-- for SRS bandwidth aggregation in RRC_CONNECTED state

posJointTriggerBySingleDCI-RRC-Connected-r18                    ENUMERATED {supported}                                     OPTIONAL,

-- R1 41-5-1a PRS measurement with Rx frequency hopping in RRC_INACTIVE for RedCap UEs

dl-PRS-MeasurementWithRxFH-RRC-InactiveforRedCap-r18            ENUMERATED {supported}                                     OPTIONAL,

-- R1 41-5-1b PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs

dl-PRS-MeasurementWithRxFH-RRC-IdleforRedCap-r18                ENUMERATED {supported}                                     OPTIONAL,

-- R1 42-4: Cell DTX and/or DRX operation based on RRC configuration

nes-CellDTX-DRX-r18                                             ENUMERATED {cellDTXonly, cellDRXonly, both}                OPTIONAL,

-- R1 42-5: Cell DTX/DRX operation triggered by DCI format 2_9

nes-CellDTX-DRX-DCI2-9-r18                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 45-5: RACH-based early TA acquisition

rach-EarlyTA-Measurement-r18                                    INTEGER (1..8)                                             OPTIONAL,

-- R1 45-6: UE-based TA measurement

ue-TA-Measurement-r18                                           INTEGER (1..8)                                             OPTIONAL,

-- R1 45-7: TA indication in cell switch command

ta-IndicationCellSwitch-r18                                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 50-1: Multi-PUSCHs for Configured Grant

multiPUSCH-CG-r18                                               ENUMERATED {n16, n32}                                      OPTIONAL,

-- R1 50-1a: Multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell

multiPUSCH-ActiveConfiguredGrant-r18                            SEQUENCE {

maxNumberConfigsPerBWP                                          ENUMERATED {n1, n2, n4, n8, n12},

maxNumberConfigsAllCC-FR1                                       INTEGER (2..32),

maxNumberConfigsAllCC-FR2                                       INTEGER (2..32)

}                                                                                                                          OPTIONAL,

-- R1 50-2: UCI indication of unused CG-PUSCH transmission occasions

cg-PUSCH-UTO-UCI-Ind-r18                                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 50-3: PDCCH monitoring resumption after UL NACK

pdcch-MonitoringResumptionAfterUL-NACK-r18                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 51-1: support for 3MHz channel bandwidth

support-3MHz-ChannelBW-r18                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 51-2: support 12 PRB CORESET0

support-12PRB-CORESET0-r18                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 52-1: Reception of NR PDCCH candidates overlapping with LTE CRS REs

nr-PDCCH-OverlapLTE-CRS-RE-r18                                  SEQUENCE {

overlapInRE-r18                                                 ENUMERATED {oneSymbolNoOverlap, someOrAllSymOverlap},

overlapInSymbol-r18                                             ENUMERATED {symbol2,symbol1And2}

}                                                                                                                          OPTIONAL,

-- Editor's Note: someOrAllSymOverlap considers to be supported in overlapInRE-r18 only if RAN4 performance requirements for

-- someOrAllSymOverlap are not defined

-- R1 52-1a: Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns

nr-PDCCH-OverlapLTE-CRS-RE-MultiPatterns-r18                    ENUMERATED {supported}                                     OPTIONAL,

-- R1 52-1b: NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the

-- first 4 OFDM symbols in a slot

nr-PDCCH-OverlapLTE-CRS-RE-Span-3-4-r18                         ENUMERATED {supported}                                     OPTIONAL,

-- R1 52-2: Two LTE-CRS overlapping rate matching patterns within NR 15 kHz carrier overlapping with LTE carrier (regardless of

-- support or configuration of multi-TRP)

twoRateMatchingEUTRA-CRS-patterns-3-4-r18                       SEQUENCE {

maxNumberPatterns-r18                                           INTEGER (2..6),

maxNumberNon-OverlapPatterns-r18                                INTEGER (1..3)

}                                                                                                                          OPTIONAL,

-- R1 52-2a: Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within NR 15 kHz carrier

-- overlapping with LTE carrier

overlapRateMatchingEUTRA-CRS-Patterns-3-4-Diff-CS-Pool-r18      ENUMERATED {supported}                                     OPTIONAL,

-- R1 53-3: Support RLM/BM/BFD measurements based on NCD-SSB within active BWP

ncd-SSB-BWP-Wor-r18                                             ENUMERATED {supported}                                     OPTIONAL,

-- R1 53-4: Support Support RLM/BM/BFD measurements based on CSI-RS when CD-SSB is outside active BWP

rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18                          ENUMERATED {supported}                                     OPTIONAL,

-- R1 54-1: PRACH coverage enhancements

prach-CoverageEnh-r18                                          ENUMERATED {supported}                                     OPTIONAL,

-- R1 54-3: Dynamic waveform switching

dynamicWaveformSwitch-r18                                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 55-3: Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1

multiPUSCH-SingleDCI-NonConsSlots-r18                           ENUMERATED {supported}                                     OPTIONAL,

-- R1 55-2d: single-symbol DL-PRS used in RTT-based Propagation delay compensation

pdc-maxNumberPRS-ResourceProcessedPerSlot-r18                   SEQUENCE {

fr1-r18 SEQUENCE {

scs-15kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL,

scs-30kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL,

scs-60kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL

},

fr2-r18 SEQUENCE {

scs-60kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL,

scs-120kHz-r18                                  ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL

}

}                                                                                                                         OPTIONAL,

-- R4 27-2: LowerMSD for inter-band NR CA and EN-DC

lowerMSD-r18                                                    SEQUENCE (SIZE (1..maxLowerMSD-r18)) OF LowerMSD-r18      OPTIONAL,

-- R4 31-2 Beam sweeping factor reduction for FR2 unknown SCell activation

beamSweepingFactorReduction-r18                                 SEQUENCE {

reduceForCellDetection                                          ENUMERATED {n1, n2, n4, n6},

reduceForSSB-L1-RSRP-Meas                                       INTEGER (0..7)

}                                                                                                                         OPTIONAL,

-- R4 35-2: the requirements defined for ATG UE with antenna array or omni-direction antenna requirements.

antennaArrayType-r18                                           ENUMERATED {supported}                                     OPTIONAL,

locationBasedCondHandoverATG-r18                               ENUMERATED {supported}                                     OPTIONAL,

-- R4 35-3: rated maximum output power value range from 23dBm to 40dBm with 1dB as granularity at maximum modulation order and full

-- PRB configurations.

maxOutputPowerATG-r18                                          INTEGER (1..18)                                            OPTIONAL,

eventA4BasedCondHandoverNES-r18                                ENUMERATED {supported}                                     OPTIONAL,

nesBasedCondHandoverWithDCI-r18                                ENUMERATED {supported}                                     OPTIONAL,

rachLessHandoverNTN-r18                                        ENUMERATED {supported}                                     OPTIONAL,

locationBasedCondHandoverEMC-r18                               ENUMERATED {supported}                                     OPTIONAL,

mt-CG-SDT-r18                                                  ENUMERATED {supported}                                     OPTIONAL,

posSRS-RRC-InactiveInitialUL-BWP-r18                           ENUMERATED {supported}                                     OPTIONAL,

posSRS-RRC-InactiveOutsideInitialUL-BWP-r18                    ENUMERATED {supported}                                     OPTIONAL,

cg-SDT-PeriodicityExt-r18                                      ENUMERATED {supported}                                     OPTIONAL

]]

}

BandNR-v16c0 ::=                                                SEQUENCE {

pusch-RepetitionTypeA-v16c0                                     ENUMERATED {supported}                                     OPTIONAL,

...

}

LowerMSD-r18 ::=           SEQUENCE {

aggressorband1-r18         FreqBandIndicatorNR,

aggressorband2-r18         FreqBandIndicatorNR                                                                             OPTIONAL,

msd-Information-r18        SEQUENCE (SIZE (1..maxLowerMSDInfo-r18)) OF MSD-Information-r18

}

MSD-Information-r18 ::=    SEQUENCE {

msd-Type-r18               ENUMERATED {harmonic, harmonicMixing, crossBandIsolation, imd2, imd3, imd4, imd5, all, spare8, spare7,

spare6, spare5,spare4, spare3, spare2, spare1},

msd-PowerClass-r18         ENUMERATED {pc1dot5, pc2, pc3},

msd-Class-r18              ENUMERATED {classI, classII, classIII, classIV, classV, classVI, classVII, classVIII }

}

-- Editor note: The power class related part can be updated further pending RAN4 discussion.

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any, as described in clause 5.6.1.4. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationList A list of band combinations that the UE supports for NR (and NR-DC, if requested). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationListSidelinkEUTRA-NR A list of band combinations that the UE supports for NR sidelink communication only, for joint NR sidelink communication and V2X sidelink communication, or for V2X sidelink communication only. The UE does not include this field if the UE capability is requested by E-UTRAN (see TS 36.331[10]) and the network request includes the field eutra-nr-only.

supportedBandCombinationListSL-NonRelayDiscovery A list of band combinations that the UE supports for NR sidelink non-relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationListSL-RelayDiscovery A list of band combinations that the UE supports for NR sidelink relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationListSL-U2U-DiscoveryExt This field indicates the band parameter in BandCombinationListSL-Discovery-r17 that the UE supports for NR U2U sidelink relay discovery in a band included in supportedBandCombinationListSL-U2U-RelayDiscovery.

supportedBandCombinationListSL-U2U-RelayDiscovery A list of band combinations that the UE supports for NR U2U sidelink relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationList-UplinkTxSwitch A list of band combinations that the UE supports dynamic uplink Tx switching for NR UL CA and SUL. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandListNR A list of NR bands supported by the UE. If supportedBandListNR-v16c0 is included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandListNR (without suffix).

#### –	RF-ParametersMRDC

The IE RF-ParametersMRDC is used to convey RF related capabilities for MR-DC.

RF-ParametersMRDC information element

-- ASN1START

-- TAG-RF-PARAMETERSMRDC-START

RF-ParametersMRDC ::=                   SEQUENCE {

supportedBandCombinationList            BandCombinationList                             OPTIONAL,

appliedFreqBandListFilter               FreqBandList                                    OPTIONAL,

...,

[[

srs-SwitchingTimeRequested              ENUMERATED {true}                               OPTIONAL,

supportedBandCombinationList-v1540      BandCombinationList-v1540                       OPTIONAL

]],

[[

supportedBandCombinationList-v1550      BandCombinationList-v1550                       OPTIONAL

]],

[[

supportedBandCombinationList-v1560      BandCombinationList-v1560                       OPTIONAL,

supportedBandCombinationListNEDC-Only   BandCombinationList                             OPTIONAL

]],

[[

supportedBandCombinationList-v1570      BandCombinationList-v1570                       OPTIONAL

]],

[[

supportedBandCombinationList-v1580      BandCombinationList-v1580                       OPTIONAL

]],

[[

supportedBandCombinationList-v1590      BandCombinationList-v1590                       OPTIONAL

]],

[[

supportedBandCombinationListNEDC-Only-v15a0    SEQUENCE {

supportedBandCombinationList-v1540      BandCombinationList-v1540                   OPTIONAL,

supportedBandCombinationList-v1560      BandCombinationList-v1560                   OPTIONAL,

supportedBandCombinationList-v1570      BandCombinationList-v1570                   OPTIONAL,

supportedBandCombinationList-v1580      BandCombinationList-v1580                   OPTIONAL,

supportedBandCombinationList-v1590      BandCombinationList-v1590                   OPTIONAL

}                                                                                       OPTIONAL

]],

[[

supportedBandCombinationList-v1610      BandCombinationList-v1610                       OPTIONAL,

supportedBandCombinationListNEDC-Only-v1610   BandCombinationList-v1610                 OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16 BandCombinationList-UplinkTxSwitch-r16  OPTIONAL

]],

[[

supportedBandCombinationList-v1630                  BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1630         BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1630   BandCombinationList-UplinkTxSwitch-v1630    OPTIONAL

]],

[[

supportedBandCombinationList-v1640                  BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1640         BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1640   BandCombinationList-UplinkTxSwitch-v1640    OPTIONAL

]],

[[

supportedBandCombinationList-UplinkTxSwitch-v1670   BandCombinationList-UplinkTxSwitch-v1670    OPTIONAL

]],

[[

supportedBandCombinationList-v1700                  BandCombinationList-v1700                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1700   BandCombinationList-UplinkTxSwitch-v1700    OPTIONAL

]],

[[

supportedBandCombinationList-v1720                  BandCombinationList-v1720                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1720         SEQUENCE {

supportedBandCombinationList-v1700                  BandCombinationList-v1700               OPTIONAL,

supportedBandCombinationList-v1720                  BandCombinationList-v1720               OPTIONAL

}                                                                                               OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1720   BandCombinationList-UplinkTxSwitch-v1720    OPTIONAL

]],

[[

supportedBandCombinationList-v1730                  BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1730         BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1730   BandCombinationList-UplinkTxSwitch-v1730    OPTIONAL

]],

[[

supportedBandCombinationList-v1740                  BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1740         BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1740   BandCombinationList-UplinkTxSwitch-v1740    OPTIONAL

]],

[[

supportedBandCombinationList-v1770                  BandCombinationList-v1770                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1770   BandCombinationList-UplinkTxSwitch-v1770    OPTIONAL

]],

[[

supportedBandCombinationList-v1800                  BandCombinationList-v1800                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1800   BandCombinationList-UplinkTxSwitch-v1800                   OPTIONAL

]]

}

RF-ParametersMRDC-v15g0 ::=                    SEQUENCE {

supportedBandCombinationList-v15g0             BandCombinationList-v15g0        OPTIONAL,

supportedBandCombinationListNEDC-Only-v15g0    BandCombinationList-v15g0        OPTIONAL

}

RF-ParametersMRDC-v15n0 ::=                     SEQUENCE {

supportedBandCombinationList-v15n0                  BandCombinationList-v15n0                       OPTIONAL

}

RF-ParametersMRDC-v16e0 ::=                     SEQUENCE {

supportedBandCombinationList-UplinkTxSwitch-v16e0   BandCombinationList-UplinkTxSwitch-v16e0        OPTIONAL

}

-- TAG-RF-PARAMETERSMRDC-STOP

-- ASN1STOP

RF-ParametersMRDC field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for (NG)EN-DC, or both (NG)EN-DC and NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationListNEDC-Only, supportedBandCombinationListNEDC-Only-v1610 A list of band combinations that the UE supports only for NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationList-UplinkTxSwitch A list of band combinations that the UE supports dynamic UL Tx switching for (NG)EN-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

#### –	RLC-Parameters

The IE RLC-Parameters is used to convey capabilities related to RLC.

RLC-Parameters information element

-- ASN1START

-- TAG-RLC-PARAMETERS-START

RLC-Parameters ::= SEQUENCE {

am-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithLongSN                   ENUMERATED {supported}  OPTIONAL,

...,

[[

extendedT-PollRetransmit-r16    ENUMERATED {supported}  OPTIONAL,

extendedT-StatusProhibit-r16    ENUMERATED {supported}  OPTIONAL

]],

[[

am-WithLongSN-RedCap-r17        ENUMERATED {supported}  OPTIONAL

]],

[[

am-WithLongSN-NCR-r18           ENUMERATED {supported}  OPTIONAL

]]

}

-- TAG-RLC-PARAMETERS-STOP

-- ASN1STOP

#### –	SDAP-Parameters

The IE SDAP-Parameters is used to convey capabilities related to SDAP.

SDAP-Parameters information element

-- ASN1START

-- TAG-SDAP-PARAMETERS-START

SDAP-Parameters ::= SEQUENCE {

as-ReflectiveQoS              ENUMERATED {true}            OPTIONAL,

...,

[[

sdap-QOS-IAB-r16              ENUMERATED {supported}       OPTIONAL,

sdapHeaderIAB-r16             ENUMERATED {supported}       OPTIONAL

]],

[[

sdap-QOS-NCR-r18              ENUMERATED {supported}       OPTIONAL,

sdap-HeaderNCR-r18            ENUMERATED {supported}       OPTIONAL

]]

}

-- TAG-SDAP-PARAMETERS-STOP

-- ASN1STOP

#### –	SharedSpectrumChAccessParamsPerBand

The IE SharedSpectrumChAccessParamsPerBand is used to convey shared channel access related parameters specific for a certain frequency band (not per feature set or band combination).

SharedSpectrumChAccessParamsPerBand information element

-- ASN1START

-- TAG-SHAREDSPECTRUMCHACCESSPARAMSPERBAND-START

SharedSpectrumChAccessParamsPerBand-r16 ::=           SEQUENCE {

-- R1 10-1: UL channel access for dynamic channel access mode

ul-DynamicChAccess-r16                              ENUMERATED {supported}            OPTIONAL,

-- R1 10-1a: UL channel access for semi-static channel access mode

ul-Semi-StaticChAccess-r16                          ENUMERATED {supported}            OPTIONAL,

-- R1 10-2: SSB-based RRM for dynamic channel access mode

ssb-RRM-DynamicChAccess-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-2a: SSB-based RRM for semi-static channel access mode

ssb-RRM-Semi-StaticChAccess-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 10-2b: MIB reading on unlicensed cell

mib-Acquisition-r16                                 ENUMERATED {supported}            OPTIONAL,

-- R1 10-2c: SSB-based RLM for dynamic channel access mode

ssb-RLM-DynamicChAccess-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-2d: SSB-based RLM for semi-static channel access mode

ssb-RLM-Semi-StaticChAccess-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 10-2e: SIB1 reception on unlicensed cell

sib1-Acquisition-r16                                ENUMERATED {supported}            OPTIONAL,

-- R1 10-2f: Support monitoring of extended RAR window

extRA-ResponseWindow-r16                            ENUMERATED {supported}            OPTIONAL,

-- R1 10-2g: SSB-based BFD/CBD for dynamic channel access mode

ssb-BFD-CBD-dynamicChannelAccess-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 10-2h: SSB-based BFD/CBD for semi-static channel access mode

ssb-BFD-CBD-semi-staticChannelAccess-r16            ENUMERATED {supported}            OPTIONAL,

-- R1 10-2i: CSI-RS-based BFD/CBD for NR-U

csi-RS-BFD-CBD-r16                                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-7: UL channel access for 10 MHz SCell

ul-ChannelBW-SCell-10mhz-r16                        ENUMERATED {supported}            OPTIONAL,

-- R1 10-10: RSSI and channel occupancy measurement and reporting

rssi-ChannelOccupancyReporting-r16                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-11:SRS starting position at any OFDM symbol in a slot

srs-StartAnyOFDM-Symbol-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-20: Support search space set configuration with freqMonitorLocation-r16

searchSpaceFreqMonitorLocation-r16                  INTEGER (1..5)                    OPTIONAL,

-- R1 10-20a: Support coreset configuration with rb-Offset

coreset-RB-Offset-r16                               ENUMERATED {supported}            OPTIONAL,

-- R1 10-23:CGI reading on unlicensed cell for ANR functionality

cgi-Acquisition-r16                                 ENUMERATED {supported}            OPTIONAL,

-- R1 10-25: Enable configured UL transmissions when DCI 2_0 is configured but not detected

configuredUL-Tx-r16                                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-27: Wideband PRACH

prach-Wideband-r16                                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-29: Support available RB set indicator field in DCI 2_0

dci-AvailableRB-Set-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-30: Support channel occupancy duration indicator field in DCI 2_0

dci-ChOccupancyDuration-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-8: Type B PDSCH length {3, 5, 6, 8, 9, 10, 11, 12, 13} without DMRS shift due to CRS collision

typeB-PDSCH-length-r16                              ENUMERATED {supported}            OPTIONAL,

-- R1 10-9: Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring

searchSpaceSwitchWithDCI-r16                        ENUMERATED {supported}            OPTIONAL,

-- R1 10-9b: Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring

searchSpaceSwitchWithoutDCI-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 10-9d: Support Search space set group switching capability 2

searchSpaceSwitchCapability2-r16                    ENUMERATED {supported}            OPTIONAL,

-- R1 10-14: Non-numerical PDSCH to HARQ-ACK timing

non-numericalPDSCH-HARQ-timing-r16                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-15: Enhanced dynamic HARQ codebook

enhancedDynamicHARQ-codebook-r16                    ENUMERATED {supported}            OPTIONAL,

-- R1 10-16: One-shot HARQ ACK feedback

oneShotHARQ-feedback-r16                            ENUMERATED {supported}            OPTIONAL,

-- R1 10-17: Multi-PUSCH UL grant

multiPUSCH-UL-grant-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-26: CSI-RS based RLM for NR-U

csi-RS-RLM-r16                                      ENUMERATED {supported}            OPTIONAL,

dummy                                              ENUMERATED {supported}            OPTIONAL,

-- R1 10-31: Support of P/SP-CSI-RS reception with CSI-RS-ValidationWith-DCI-r16 configured

periodicAndSemi-PersistentCSI-RS-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 10-3: PRB interlace mapping for PUSCH

pusch-PRB-interlace-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-3a: PRB interlace mapping for PUCCH

pucch-F0-F1-PRB-Interlace-r16                       ENUMERATED {supported}            OPTIONAL,

-- R1 10-12: OCC for PRB interlace mapping for PF2 and PF3

occ-PRB-PF2-PF3-r16                                 ENUMERATED {supported}            OPTIONAL,

-- R1 10-13a: Extended CP range of more than one symbol for CG-PUSCH

extCP-rangeCG-PUSCH-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-18: Configured grant with retransmission in CG resources

configuredGrantWithReTx-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-21a: Support using ED threshold given by gNB for UL to DL COT sharing

ed-Threshold-r16                                    ENUMERATED {supported}            OPTIONAL,

-- R1 10-21b: Support UL to DL COT sharing

ul-DL-COT-Sharing-r16                               ENUMERATED {supported}            OPTIONAL,

-- R1 10-24: CG-UCI multiplexing with HARQ ACK

mux-CG-UCI-HARQ-ACK-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-28: Configured grant with Rel-16 enhanced resource configuration

cg-resourceConfig-r16                               ENUMERATED {supported}            OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1630 ::=       SEQUENCE {

-- R4 4-1: DL reception in intra-carrier guardband

dl-ReceptionIntraCellGuardband-r16                  ENUMERATED {supported}            OPTIONAL,

-- R4 4-2: DL reception when gNB does not transmit on all RB sets of a carrier as a result of LBT

dl-ReceptionLBT-subsetRB-r16                        ENUMERATED {supported}            OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1640 ::=       SEQUENCE {

-- 10-26b(1-4): CSI-RS based RRM measurement with associated SS-block

csi-RSRP-AndRSRQ-MeasWithSSB-r16                     ENUMERATED {supported}              OPTIONAL,

-- 10-26c(1-5): CSI-RS based RRM measurement without associated SS-block

csi-RSRP-AndRSRQ-MeasWithoutSSB-r16                 ENUMERATED {supported}               OPTIONAL,

-- 10-26d(1-6): CSI-RS based RS-SINR measurement

csi-SINR-Meas-r16                                      ENUMERATED {supported}               OPTIONAL,

-- 10-26e(1-8): RLM based on a mix of SS block and CSI-RS signals within active BWP

ssb-AndCSI-RS-RLM-r16                                 ENUMERATED {supported}               OPTIONAL,

-- 10-26f(1-9): CSI-RS based contention free RA for HO

csi-RS-CFRA-ForHO-r16                                 ENUMERATED {supported}               OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1650 ::=       SEQUENCE {

-- Extension of R1 10-9 capability to configure up to 16 instead of 4 cells or cell groups, respectively

extendedSearchSpaceSwitchWithDCI-r16                ENUMERATED {supported}               OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1710 ::=    SEQUENCE {

-- R1 25-12: UE initiated semi-static channel occupancy with dependent configurations

ul-Semi-StaticChAccessDependentConfig-r17        ENUMERATED {supported}            OPTIONAL,

-- R1 25-13: UE initiated semi-static channel occupancy with independent configurations

ul-Semi-StaticChAccessIndependentConfig-r17      ENUMERATED {supported}            OPTIONAL

}

-- TAG-SHAREDSPECTRUMCHACCESSPARAMSPERBAND-STOP

-- ASN1STOP

#### –	SharedSpectrumChAccessParamsSidelinkPerBand

The IE SharedSpectrumChAccessParamsSidelinkPerBand is used to convey shared channel access related parameters related to NR sidelink communication, specific for a certain frequency band (not per feature set or band combination).

SharedSpectrumChAccessParamsSidelinkPerBand information element

-- ASN1START

-- TAG-SHAREDSPECTRUMCHACCESSPARAMSSIDELINKPERBAND-START

SharedSpectrumChAccessParamsSidleinkPerBand-r18 ::=           SEQUENCE {

-- 47-k6: Type1 LBT blocking Option 1

sl-LBT-Option1-r18                                     ENUMERATED {supported}            OPTIONAL,

-- 47-k7: Type1 LBT blocking Option 2

sl-LBT-Option2-r18                                     ENUMERATED {supported}            OPTIONAL,

-- 47-m1: Interlace RB-based SL transmission/reception

sl-interlace-RB-TxRx-r18                               ENUMERATED {supported}            OPTIONAL

}

-- TAG-SHAREDSPECTRUMCHACCESSPARAMSSIDELINKPERBAND-STOP

-- ASN1STOP

#### –	SidelinkParameters

The IE SidelinkParameters is used to convey capabilities related to NR and V2X sidelink communications.

SidelinkParameters information element

-- ASN1START

-- TAG-SIDELINKPARAMETERS-START

SidelinkParameters-r16 ::=    SEQUENCE {

sidelinkParametersNR-r16                  SidelinkParametersNR-r16                                                  OPTIONAL,

sidelinkParametersEUTRA-r16               SidelinkParametersEUTRA-r16                                               OPTIONAL

}

SidelinkParametersNR-r16 ::= SEQUENCE {

rlc-ParametersSidelink-r16                RLC-ParametersSidelink-r16                                                OPTIONAL,

mac-ParametersSidelink-r16                MAC-ParametersSidelink-r16                                                OPTIONAL,

fdd-Add-UE-Sidelink-Capabilities-r16      UE-SidelinkCapabilityAddXDD-Mode-r16                                      OPTIONAL,

tdd-Add-UE-Sidelink-Capabilities-r16      UE-SidelinkCapabilityAddXDD-Mode-r16                                      OPTIONAL,

supportedBandListSidelink-r16             SEQUENCE (SIZE (1..maxBands)) OF BandSidelink-r16                         OPTIONAL,

...,

[[

relayParameters-r17                       RelayParameters-r17                                                       OPTIONAL

]],

[[

-- R1 32-x: Use of new P0 parameters for open loop power control

p0-OLPC-Sidelink-r17                      ENUMERATED {supported}                                                    OPTIONAL

]],

[[

pdcp-ParametersSidelink-r18               PDCP-ParametersSidelink-r18                                               OPTIONAL

]]

}

SidelinkParametersEUTRA-r16 ::= SEQUENCE {

sl-ParametersEUTRA1-r16                   OCTET STRING                                                              OPTIONAL,

sl-ParametersEUTRA2-r16                   OCTET STRING                                                              OPTIONAL,

sl-ParametersEUTRA3-r16                   OCTET STRING                                                              OPTIONAL,

supportedBandListSidelinkEUTRA-r16        SEQUENCE (SIZE (1..maxBandsEUTRA)) OF BandSidelinkEUTRA-r16               OPTIONAL,

...

}

RLC-ParametersSidelink-r16 ::= SEQUENCE {

am-WithLongSN-Sidelink-r16                ENUMERATED {supported}                                                    OPTIONAL,

um-WithLongSN-Sidelink-r16                ENUMERATED {supported}                                                    OPTIONAL,

...

}

MAC-ParametersSidelink-r16 ::= SEQUENCE {

mac-ParametersSidelinkCommon-r16          MAC-ParametersSidelinkCommon-r16                                          OPTIONAL,

mac-ParametersSidelinkXDD-Diff-r16        MAC-ParametersSidelinkXDD-Diff-r16                                        OPTIONAL,

...

}

UE-SidelinkCapabilityAddXDD-Mode-r16 ::=  SEQUENCE {

mac-ParametersSidelinkXDD-Diff-r16        MAC-ParametersSidelinkXDD-Diff-r16                                        OPTIONAL

}

MAC-ParametersSidelinkCommon-r16 ::= SEQUENCE {

lcp-RestrictionSidelink-r16               ENUMERATED {supported}                                                    OPTIONAL,

multipleConfiguredGrantsSidelink-r16      ENUMERATED {supported}                                                    OPTIONAL,

...,

[[

drx-OnSidelink-r17                        ENUMERATED {supported}                                                    OPTIONAL

]],

[[

sl-LBT-FailureDectectionRecovery-r18      ENUMERATED {supported}                                                    OPTIONAL

]]

}

MAC-ParametersSidelinkXDD-Diff-r16 ::=  SEQUENCE {

multipleSR-ConfigurationsSidelink-r16     ENUMERATED {supported}                                                    OPTIONAL,

logicalChannelSR-DelayTimerSidelink-r16   ENUMERATED {supported}                                                    OPTIONAL,

...

}

BandSidelinkEUTRA-r16 ::=               SEQUENCE {

freqBandSidelinkEUTRA-r16               FreqBandIndicatorEUTRA,

-- R1 15-7: Transmitting LTE sidelink mode 3 scheduled by NR Uu

gnb-ScheduledMode3SidelinkEUTRA-r16     SEQUENCE {

gnb-ScheduledMode3DelaySidelinkEUTRA-r16 ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1,

ms1dot25, ms1dot5, ms1dot75, ms2, ms2dot5, ms3, ms4,

ms5, ms6, ms8, ms10, ms20}

}                                                                                                                   OPTIONAL,

-- R1 15-9: Transmitting LTE sidelink mode 4 configured by NR Uu

gnb-ScheduledMode4SidelinkEUTRA-r16     ENUMERATED {supported}                                                      OPTIONAL

}

BandSidelink-r16 ::=  SEQUENCE {

freqBandSidelink-r16                          FreqBandIndicatorNR,

--15-1

sl-Reception-r16                              SEQUENCE {

harq-RxProcessSidelink-r16                    ENUMERATED {n16, n24, n32, n48, n64},

pscch-RxSidelink-r16                          ENUMERATED {value1, value2},

scs-CP-PatternRxSidelink-r16                  CHOICE {

fr1-r16                                       SEQUENCE {

scs-15kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r16                                       SEQUENCE {

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r16                                BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-RxSidelink-r16                     ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-2

sl-TransmissionMode1-r16                      SEQUENCE {

harq-TxProcessModeOneSidelink-r16             ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeOne-r16           CHOICE {

fr1-r16                                       SEQUENCE {

scs-15kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r16                                       SEQUENCE {

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r16                                BIT STRING (SIZE (16))                OPTIONAL

}

},

extendedCP-TxSidelink-r16                     ENUMERATED {supported}                        OPTIONAL,

harq-ReportOnPUCCH-r16                        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-4

sync-Sidelink-r16                             SEQUENCE {

gNB-Sync-r16                                  ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNB-ENB-r16     ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNSS-r16        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-10

sl-Tx-256QAM-r16                              ENUMERATED {supported}                            OPTIONAL,

--15-11

psfch-FormatZeroSidelink-r16                  SEQUENCE {

psfch-RxNumber                                ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64},

psfch-TxNumber                                ENUMERATED {n4, n8, n16}

}                                                                                               OPTIONAL,

--15-12

lowSE-64QAM-MCS-TableSidelink-r16             ENUMERATED {supported}                            OPTIONAL,

--15-15

enb-sync-Sidelink-r16                         ENUMERATED {supported}                            OPTIONAL,

...,

[[

--15-3

sl-TransmissionMode2-r16                      SEQUENCE {

harq-TxProcessModeTwoSidelink-r16              ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r16           ENUMERATED {supported}                        OPTIONAL,

dl-openLoopPC-Sidelink-r16                    ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-5

congestionControlSidelink-r16                 SEQUENCE {

cbr-ReportSidelink-r16                        ENUMERATED {supported}                        OPTIONAL,

cbr-CR-TimeLimitSidelink-r16                  ENUMERATED {time1, time2}

}                                                                                               OPTIONAL,

--15-22

fewerSymbolSlotSidelink-r16                   ENUMERATED {supported}                            OPTIONAL,

--15-23

sl-openLoopPC-RSRP-ReportSidelink-r16         ENUMERATED {supported}                            OPTIONAL,

--13-1

sl-Rx-256QAM-r16                              ENUMERATED {supported}                            OPTIONAL

]],

[[

ue-PowerClassSidelink-r16                         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

OPTIONAL

]],

[[

--32-4a

sl-TransmissionMode2-RandomResourceSelection-r17 SEQUENCE {

harq-TxProcessModeTwoSidelink-r17             ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17               CHOICE {

fr1-r17                                           SEQUENCE {

scs-15kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-30kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-60kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL

},

fr2-r17                                           SEQUENCE {

scs-60kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-120kHz-r17                                    BIT STRING (SIZE (16))            OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-Mode2Random-r17                    ENUMERATED {supported}                        OPTIONAL,

dl-openLoopPC-Sidelink-r17                    ENUMERATED {supported}                            OPTIONAL

}                                                                                               OPTIONAL,

--32-4b

sync-Sidelink-v1710                           SEQUENCE {

sync-GNSS-r17                                 ENUMERATED {supported}                        OPTIONAL,

gNB-Sync-r17                                  ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNB-ENB-r17     ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNSS-r17        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--32-4c

enb-sync-Sidelink-v1710                       ENUMERATED {supported}                            OPTIONAL,

--32-5a-2

rx-IUC-Scheme1-PreferredMode2Sidelink-r17     ENUMERATED {supported}                            OPTIONAL,

--32-5a-3

rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17  ENUMERATED {supported}                            OPTIONAL,

--32-5b-2

rx-IUC-Scheme2-Mode2Sidelink-r17              ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-6-1

rx-IUC-Scheme1-SCI-r17                        ENUMERATED {supported}                            OPTIONAL,

--32-6-2

rx-IUC-Scheme1-SCI-ExplicitReq-r17            ENUMERATED {supported}                            OPTIONAL

]],

[[

sharedSpectrumChAccessParamsSidelinkPerBand-r18 SharedSpectrumChAccessParamsSidelinkPerBand-r18 OPTIONAL,

-- R1 47-s1: Transmission/Reception using dynamic resource pool sharing

sl-DynamicSharingTxRx-r18                       ENUMERATED {supported}                            OPTIONAL,

-- R4 45-2: SL reception in intra-carrier guard band

sl-ReceptionIntraCarrierGuardBand-r18         ENUMERATED {supported}                            OPTIONAL

]]

}

RelayParameters-r17 ::= SEQUENCE {

relayUE-Operation-L2-r17                         ENUMERATED {supported}                         OPTIONAL,

remoteUE-Operation-L2-r17                        ENUMERATED {supported}                         OPTIONAL,

remoteUE-PathSwitchToIdleInactiveRelay-r17       ENUMERATED {supported}                         OPTIONAL,

...,

[[

relayUE-U2U-OperationL2-r18                             ENUMERATED {supported}                  OPTIONAL,

remoteUE-U2U-OperationL2-r18                            ENUMERATED {supported}                  OPTIONAL,

remoteUE-U2N-PathSwitchOperationL2-r18                  ENUMERATED {supported}                  OPTIONAL,

multipathRemoteUE-PC5L2-r18                             ENUMERATED {supported}                  OPTIONAL,

multipathRelayUE-PC5L2-r18                              ENUMERATED {supported}                  OPTIONAL,

multipathRelayUE-N3C-r18                                ENUMERATED {supported}                  OPTIONAL,

multipathRemoteUE-N3C-r18                               ENUMERATED {supported}                  OPTIONAL,

remoteUE-IndirectPathAddChangeToIdleInactiveRelay-r18   ENUMERATED {supported}                  OPTIONAL,

pdcp-DuplicationMoreThanOneUuRLC-r18                    ENUMERATED {supported}                  OPTIONAL

]]

}

PDCP-ParametersSidelink-r18 ::=   SEQUENCE {

pdcp-DuplicationSRB-sidelink-r18                        ENUMERATED {supported}                  OPTIONAL,

pdcp-DuplicationDRB-sidelink-r18                        ENUMERATED {supported}                  OPTIONAL,

...

}

-- TAG-SIDELINKPARAMETERS-STOP

-- ASN1STOP

SidelinkParametersEUTRA field descriptions

sl-ParametersEUTRA1, sl-ParametersEUTRA2, sl-ParametersEUTRA3 This field includes IE of SL-Parameters-v1430 (where v2x-eNB-Scheduled-r14 and V2X-SupportedBandCombination-r14 shall not be included), SL-Parameters-v1530 (where V2X-SupportedBandCombination-r1530 shall not be included) and SL-Parameters-v1540 respectively defined in 36.331 [10]. It is used for reporting the per-UE capability for V2X sidelink communication.

#### –	SimultaneousRxTxPerBandPair

The IE SimultaneousRxTxPerBandPair contains the simultaneous Rx/Tx UE capability for each band pair in a band combination.

SimultaneousRxTxPerBandPair information element

-- ASN1START

-- TAG-SIMULTANEOUSRXTXPERBANDPAIR-START

SimultaneousRxTxPerBandPair ::=             BIT STRING (SIZE (3..496))

-- TAG-SIMULTANEOUSRXTXPERBANDPAIR-STOP

-- ASN1STOP

#### –	SON-Parameters

The IE SON-Parameters contains SON related parameters.

SON-Parameters information element

-- ASN1START

-- TAG-SON-PARAMETERS-START

SON-Parameters-r16 ::= SEQUENCE {

rach-Report-r16        ENUMERATED {supported}    OPTIONAL,

...,

[[

rlfReportCHO-r17       ENUMERATED {supported}    OPTIONAL,

rlfReportDAPS-r17      ENUMERATED {supported}    OPTIONAL,

success-HO-Report-r17  ENUMERATED {supported}    OPTIONAL,

twoStepRACH-Report-r17 ENUMERATED {supported}    OPTIONAL,

pscell-MHI-Report-r17  ENUMERATED {supported}    OPTIONAL,

onDemandSI-Report-r17  ENUMERATED {supported}    OPTIONAL

]],

[[

spr-Report-r18            ENUMERATED {supported} OPTIONAL,

successIRAT-HO-Report-r18 ENUMERATED {supported} OPTIONAL

]]

}

-- TAG-SON-PARAMETERS-STOP

-- ASN1STOP

#### –	SpatialRelationsSRS-Pos

The IE SpatialRelationsSRS-Pos is used to convey spatial relation for SRS for positioning related parameters.

SpatialRelationsSRS-Pos information element

-- ASN1START

-- TAG-SPATIALRELATIONSSRS-POS-START

SpatialRelationsSRS-Pos-r16 ::=                    SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16      ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16   ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16      ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16              ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16        ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16        ENUMERATED {supported}                OPTIONAL

}

--TAG-SPATIALRELATIONSSRS-POS-STOP

-- ASN1STOP

#### –	SRS-AllPosResourcesRRC-Inactive

The IE SRS-AllPosResourcesRRC-Inactive is used to convey SRS positioning related parameters specific for a certain band.

SRS-AllPosResourcesRRC-Inactive information element

-- ASN1START

-- TAG-SRS-ALLPOSRESOURCESRRC-INACTIVE-START

SRS-AllPosResourcesRRC-Inactive-r17 ::=             SEQUENCE {

srs-PosResourcesRRC-Inactive-r17                    SEQUENCE {

-- R1 27-15: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP

maxNumberSRS-PosResourceSetPerBWP-r17               ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r17                 ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSRS-ResourcesPerBWP-PerSlot-r17            ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maxNumberPeriodicSRS-PosResourcesPerBWP-r17         ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r17 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

dummy1                                              ENUMERATED {n1, n2, n4, n8, n16, n32, n64 },

dummy2                                              ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

}

-- TAG-SRS-ALLPOSRESOURCESRRC-INACTIVE-STOP

-- ASN1STOP

SRS-AllPosResourcesRRC-Inactive field descriptions

dummy1, dummy2 The fields are not used in the specification and the network ignores the received values.

#### –	SRS-SwitchingTimeNR

The IE SRS-SwitchingTimeNR is used to indicate the SRS carrier switching time supported by the UE for one NR band pair.

SRS-SwitchingTimeNR information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMENR-START

SRS-SwitchingTimeNR ::= SEQUENCE {

switchingTimeDL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL,

switchingTimeUL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMENR-STOP

-- ASN1STOP

#### –	SRS-SwitchingTimeEUTRA

The IE SRS-SwitchingTimeEUTRA is used to indicate the SRS carrier switching time supported by the UE for one E-UTRA band pair.

SRS-SwitchingTimeEUTRA information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMEEUTRA-START

SRS-SwitchingTimeEUTRA ::= SEQUENCE {

switchingTimeDL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL,

switchingTimeUL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMEEUTRA-STOP

-- ASN1STOP

#### –	SupportedBandwidth

The IE SupportedBandwidth is used to indicate the channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth ::=      CHOICE {

fr1                         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2                         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::= CHOICE {

fr1-r17    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17    ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

#### –	UE-BasedPerfMeas-Parameters

The IE UE-BasedPerfMeas-Parameters contains UE-based performance measurement parameters.

UE-BasedPerfMeas-Parameters information element

-- ASN1START

-- TAG-UE-BASEDPERFMEAS-PARAMETERS-START

UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE {

barometerMeasReport-r16      ENUMERATED {supported}        OPTIONAL,

immMeasBT-r16                ENUMERATED {supported}        OPTIONAL,

immMeasWLAN-r16              ENUMERATED {supported}        OPTIONAL,

loggedMeasBT-r16             ENUMERATED {supported}        OPTIONAL,

loggedMeasurements-r16       ENUMERATED {supported}        OPTIONAL,

loggedMeasWLAN-r16           ENUMERATED {supported}        OPTIONAL,

orientationMeasReport-r16    ENUMERATED {supported}        OPTIONAL,

speedMeasReport-r16          ENUMERATED {supported}        OPTIONAL,

gnss-Location-r16            ENUMERATED {supported}        OPTIONAL,

ulPDCP-Delay-r16             ENUMERATED {supported}        OPTIONAL,

...,

[[

sigBasedLogMDT-OverrideProtect-r17 ENUMERATED {supported}  OPTIONAL,

multipleCEF-Report-r17             ENUMERATED {supported}  OPTIONAL,

excessPacketDelay-r17              ENUMERATED {supported}  OPTIONAL,

earlyMeasLog-r17                   ENUMERATED {supported}  OPTIONAL

]],

[[

loggedMDT-PNI-NPN-r18              ENUMERATED {supported}  OPTIONAL,

loggedMDT-SNPN-r18                 ENUMERATED {supported}  OPTIONAL

]]

}

-- TAG-UE-BASEDPERFMEAS-PARAMETERS-STOP

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of radio access technology specific capability containers.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-START

UE-CapabilityRAT-ContainerList ::=    SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::=        SEQUENCE {

rat-Type                              RAT-Type,

ue-CapabilityRAT-Container            OCTET STRING

}

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For rat-Type set to nr: the encoding of UE capabilities is defined in UE-NR-Capability. For rat-Type set to eutra-nr: the encoding of UE capabilities is defined in UE-MRDC-Capability. For rat-Type set to eutra: the encoding of UE capabilities is defined in UE-EUTRA-Capability specified in TS 36.331 [10]. For rat-Type set to utra-fdd: the octet string contains the INTER RAT HANDOVER INFO message defined in TS 25.331 [45].

#### –	UE-CapabilityRAT-RequestList

The IE UE-CapabilityRAT-RequestList is used to request UE capabilities for one or more RATs from the UE.

UE-CapabilityRAT-RequestList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-START

UE-CapabilityRAT-RequestList ::=        SEQUENCE (SIZE (1..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request

UE-CapabilityRAT-Request ::=            SEQUENCE {

rat-Type                                RAT-Type,

capabilityRequestFilter                 OCTET STRING                    OPTIONAL,   -- Need N

...

}

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-Request field descriptions

capabilityRequestFilter Information by which the network requests the UE to filter the UE capabilities. For rat-Type set to nr or eutra-nr: the encoding of the capabilityRequestFilter is defined in UE-CapabilityRequestFilterNR. For rat-Type set to eutra: the encoding of the capabilityRequestFilter is defined by UECapabilityEnquiry message defined in TS36.331 [10], in which RAT-Type in UE-CapabilityRequest includes only 'eutra'.

rat-Type The RAT type for which the NW requests UE capabilities.

#### –	UE-CapabilityRequestFilterCommon

The IE UE-CapabilityRequestFilterCommon is used to request filtered UE capabilities. The filter is common for all capability containers that are requested.

UE-CapabilityRequestFilterCommon information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-START

UE-CapabilityRequestFilterCommon ::=            SEQUENCE {

mrdc-Request                                SEQUENCE {

omitEN-DC                                   ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNR-DC                                ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNE-DC                                ENUMERATED {true}                      OPTIONAL     -- Need N

}                                                                                  OPTIONAL,        -- Need N

...,

[[

codebookTypeRequest-r16        SEQUENCE {

type1-SinglePanel-r16          ENUMERATED {true}                                    OPTIONAL,    -- Need N

type1-MultiPanel-r16           ENUMERATED {true}                                    OPTIONAL,    -- Need N

type2-r16                      ENUMERATED {true}                                    OPTIONAL,    -- Need N

type2-PortSelection-r16        ENUMERATED {true}                                    OPTIONAL     -- Need N

}                                                                                   OPTIONAL,    -- Need N

uplinkTxSwitchRequest-r16      ENUMERATED {true}                                    OPTIONAL     -- Need N

]],

[[

requestedCellGrouping-r16      SEQUENCE (SIZE (1..maxCellGroupings-r16)) OF CellGrouping-r16    OPTIONAL    -- Cond NRDC

]],

[[

fallbackGroupFiveRequest-r17   ENUMERATED {true}                                    OPTIONAL     -- Need N

]],

[[

lowerMSDRequest-r18            SEQUENCE {

pc1dot5-r18                    ENUMERATED {true}                                    OPTIONAL,    -- Need N

pc2-r18                        ENUMERATED {true}                                    OPTIONAL,    -- Need N

pc3-r18                        ENUMERATED {true}                                    OPTIONAL     -- Need N

}                                                                                       OPTIONAL     -- Need N

]]

}

CellGrouping-r16 ::=    SEQUENCE {

mcg-r16                 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR,

scg-r16                 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR,

mode-r16                ENUMERATED {sync, async}

}

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-STOP

-- ASN1STOP

-- Editor Note: The power class related part can be updated further pending RAN4 discussion.

UE-CapabilityRequestFilterCommon field descriptions

codebookTypeRequest Only if this field is present, the UE includes SupportedCSI-RS-Resource supported for the codebook type(s) requested within this field (i.e. type I single/multi-panel, type II and type II port selection) into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC. If this field is present and none of the codebook types is requested within this field (i.e. empty field), the UE includes SupportedCSI-RS-Resource supported for all codebook types into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC.

fallbackGroupFiveRequest Only if this field is present, the UE supporting FR2 CA bandwidth class from fallback group 5 shall include band combinations with FR2 CA bandwidth class from fallback group 5, and shall omit band combinations with FR2 CA bandwidth class from fallback group 2 or 3 (see TS 38.101-2 [39]) with same or lower capabilities.

includeNE-DC Only if this field is present, the UE supporting NE-DC shall indicate support for NE-DC in band combinations and include feature set combinations which are applicable to NE-DC. Band combinations supporting both NE-DC and (NG)EN-DC shall be included in supportedBandCombinationList, band combinations supporting only NE-DC shall be included in supportedBandCombinationListNEDC-Only.

includeNR-DC Only if this field is present, the UE supporting NR-DC shall indicate support for NR-DC in band combinations and include feature set combinations which are applicable to NR-DC.

lowerMSDRequest Only if this field is present, the UE supporting lower MSD shall indicate the lower MSD capability or the requested power class if supported. Otherwise, the UE supporting lower MSD shall indicate the lower MSD capability for the highest supported power class of the band combination including victim band and aggressor band(s).

mode The mode of NR-DC operation that the NW is interested in for this cell grouping. The value sync means that the UE only indicates NR-DC support for band combinations for which it supports synchronous NR-DC with the requested cell grouping. The value async means that the UE only indicates NR-DC support for band combinations for which it supports asynchronous NR-DC with the requested cell grouping.

omitEN-DC Only if this field is present, the UE shall omit band combinations and feature set combinations which are only applicable to (NG)EN-DC.

requestedCellGrouping The NR-DC cell groupings that the NW is interested in, i.e., the bands that it might use in an MCG and the bands that it might use in an SCG. Only if this field is present, the UE indicates NR-DC support for band combinations for which it supports the requested cell grouping, i.e., in which it supports at least one of the mcg bands on MCG and at least one of the scg bands on the SCG. In its supportedBandCombinationList, the UE indicates which of its NR-DC band combinations supports which of the requested cell groupings. The first element in this list is referred to by ID#0, the second by ID#1 and so on. If this field is absent, the UE only includes band combinations for which it supports NR-DC with only FR1 bands in MCG and only FR2 bands in SCG. Example 1: requestedCellGrouping is set to mcg=[n1, n7, n41, n66] and scg=[n78, n261]. This assumes that the NW would always use CA among n1, n7, n41 and n66 (depending on which are deployed on a given site) whereas with n78 and/or n261 the NW may need to use DC. With this filter a UE may report a band combination n1A-n7A-n78A for NR-DC only if it supports that serving cells for n1 and n7 are in the MCG and a serving cell for n78 is in the SCG. The UE may also report a band combination n41C-n261M for NR-DC provided that it supports a serving cell for n41 in the MCG and a serving cell for n261 in the SCG. Example 2: One requestedCellGrouping is set to mcg=[n1, n7, n41, n66] and scg=[n78, n261] and another requestedCellGrouping is set to mcg=[n1, n7, n66] and scg=[ n41, n78, n261]. This assumes that the NW uses sometimes CA among n1, n7, n41 and n66 (as in example 1) and sometimes CA among n1, n7 and n66 but DC towards one or several of n41, n78, n261. If a UE supports n1A-n41A-n78A only if n41A and n78A are in the same cell group, this UE may only indicate cell grouping ID#1 (not #0) in its BC.

uplinkTxSwitchRequest Only if this field is present, the UE supporting dynamic UL Tx switching shall indicate support for UL Tx switching in band combinations which are applicable to inter-band UL CA, SUL and (NG)EN-DC.

| Conditional Presence | Explanation |
| --- | --- |
| NRDC | The field is optionally present, Need N, if includeNR-DC is included. It is absent otherwise. |

#### –	UE-CapabilityRequestFilterNR

The IE UE-CapabilityRequestFilterNR is used to request filtered UE capabilities.

UE-CapabilityRequestFilterNR information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERNR-START

UE-CapabilityRequestFilterNR ::=            SEQUENCE {

frequencyBandListFilter                     FreqBandList                          OPTIONAL,   -- Need N

nonCriticalExtension                        UE-CapabilityRequestFilterNR-v1540    OPTIONAL

}

UE-CapabilityRequestFilterNR-v1540 ::=      SEQUENCE {

srs-SwitchingTimeRequest                    ENUMERATED {true}                     OPTIONAL,  -- Need N

nonCriticalExtension                        UE-CapabilityRequestFilterNR-v1710    OPTIONAL

}

UE-CapabilityRequestFilterNR-v1710 ::=      SEQUENCE {

sidelinkRequest-r17                         ENUMERATED {true}                     OPTIONAL,  -- Need N

nonCriticalExtension                        SEQUENCE {}                           OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERNR-STOP

-- ASN1STOP

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [26].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=              SEQUENCE {

measAndMobParametersMRDC            MeasAndMobParametersMRDC                                                        OPTIONAL,

phy-ParametersMRDC-v1530            Phy-ParametersMRDC                                                              OPTIONAL,

rf-ParametersMRDC                   RF-ParametersMRDC,

generalParametersMRDC               GeneralParametersMRDC-XDD-Diff                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

tdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

fr1-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

fr2-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

featureSetCombinations              SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

pdcp-ParametersMRDC-v1530           PDCP-ParametersMRDC                                                             OPTIONAL,

lateNonCriticalExtension            OCTET STRING (CONTAINING UE-MRDC-Capability-v15g0)                              OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1560                                                        OPTIONAL

}

-- Regular non-critical extensions:

UE-MRDC-Capability-v1560 ::=        SEQUENCE {

receivedFilters                     OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                         OPTIONAL,

measAndMobParametersMRDC-v1560      MeasAndMobParametersMRDC-v1560                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

tdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1610                                                        OPTIONAL

}

UE-MRDC-Capability-v1610 ::=        SEQUENCE {

measAndMobParametersMRDC-v1610      MeasAndMobParametersMRDC-v1610                                                  OPTIONAL,

generalParametersMRDC-v1610         GeneralParametersMRDC-v1610                                                     OPTIONAL,

pdcp-ParametersMRDC-v1610           PDCP-ParametersMRDC-v1610                                                       OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1700                                                        OPTIONAL

}

UE-MRDC-Capability-v1700 ::=        SEQUENCE {

measAndMobParametersMRDC-v1700      MeasAndMobParametersMRDC-v1700,

nonCriticalExtension                UE-MRDC-Capability-v1730                                                        OPTIONAL

}

UE-MRDC-Capability-v1730 ::=        SEQUENCE {

measAndMobParametersMRDC-v1730      MeasAndMobParametersMRDC-v1730                                                  OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1800                                                        OPTIONAL

}

UE-MRDC-Capability-v1800 ::=        SEQUENCE {

-- R4 33-2: Support network control of requirementnetwork applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16

requirementTypeIndication-r18       ENUMERATED {supported}                                                          OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

-- Late non-critical extensions:

UE-MRDC-Capability-v15g0 ::=        SEQUENCE {

rf-ParametersMRDC-v15g0             RF-ParametersMRDC-v15g0                                                         OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v15n0                                                        OPTIONAL

}

UE-MRDC-Capability-v15n0 ::=        SEQUENCE {

rf-ParametersMRDC-v15n0             RF-ParametersMRDC-v15n0                                                         OPTIONAL,

-- Following field is only for REL-15 late non-critical extensions

lateNonCriticalExtension            OCTET STRING                                                                    OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v16e0                                                        OPTIONAL

}

UE-MRDC-Capability-v16e0 ::=        SEQUENCE {

rf-ParametersMRDC-v16e0             RF-ParametersMRDC-v16e0                                                         OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff                                           OPTIONAL,

generalParametersMRDC-XDD-Diff          GeneralParametersMRDC-XDD-Diff                                              OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode-v1560 ::=    SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560                                  OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path             ENUMERATED {supported}                                                          OPTIONAL,

splitDRB-withUL-Both-MCG-SCG        ENUMERATED {supported}                                                          OPTIONAL,

srb3                                ENUMERATED {supported}                                                          OPTIONAL,

dummy                               ENUMERATED {supported}                                                          OPTIONAL,

...

}

GeneralParametersMRDC-v1610 ::= SEQUENCE {

f1c-OverEUTRA-r16                   ENUMERATED {supported}                                                          OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList and supportedBandCombinationListNEDC-Only in UE-MRDC-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)                      OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

-- Regular non-critical Rel-15 extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1610                                        OPTIONAL

}

-- Late non-critical Rel-15 extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0                                        OPTIONAL,

partialFR2-FallbackRX-Req                ENUMERATED {true}                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15g0                                       OPTIONAL

}

UE-NR-Capability-v15g0 ::=               SEQUENCE {

rf-Parameters-v15g0                      RF-Parameters-v15g0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15j0                                       OPTIONAL

}

UE-NR-Capability-v15j0 ::=               SEQUENCE {

-- Following field is only for REL-15 late non-critical extensions

lateNonCriticalExtension                 OCTET STRING                                                 OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16a0                                       OPTIONAL

}

-- Regular non-critical Rel-16 extensions:

UE-NR-Capability-v1610 ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v1610                   NRDC-Parameters-v1610                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

bh-RLF-Indication-r16                   ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16       ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16                      BAP-Parameters-r16                                            OPTIONAL,

referenceTimeProvision-r16              ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16                  SidelinkParameters-r16                                        OPTIONAL,

highSpeedParameters-r16                 HighSpeedParameters-r16                                       OPTIONAL,

mac-Parameters-v1610                    MAC-Parameters-v1610                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16                      SON-Parameters-r16                                            OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1640                                        OPTIONAL

}

UE-NR-Capability-v1640 ::=               SEQUENCE {

redirectAtResumeByNAS-r16               ENUMERATED {supported}                                        OPTIONAL,

phy-ParametersSharedSpectrumChAccess-r16  Phy-ParametersSharedSpectrumChAccess-r16                    OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1650                                        OPTIONAL

}

UE-NR-Capability-v1650 ::=               SEQUENCE {

mpsPriorityIndication-r16                ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1650                HighSpeedParameters-v1650                                    OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1690                                       OPTIONAL

}

UE-NR-Capability-v1690 ::=               SEQUENCE {

ul-RRC-Segmentation-r16                  ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1700                                       OPTIONAL

}

-- Late non-critical extensions from Rel-16 onwards:

UE-NR-Capability-v16a0 ::=               SEQUENCE {

phy-Parameters-v16a0                     Phy-Parameters-v16a0                                         OPTIONAL,

rf-Parameters-v16a0                      RF-Parameters-v16a0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16c0                                       OPTIONAL

}

UE-NR-Capability-v16c0 ::=               SEQUENCE {

rf-Parameters-v16c0                      RF-Parameters-v16c0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16d0                                       OPTIONAL

}

UE-NR-Capability-v16d0 ::=               SEQUENCE {

featureSets-v16d0                        FeatureSets-v16d0                                            OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical Rel-17 extensions:

UE-NR-Capability-v1700 ::=               SEQUENCE {

inactiveStatePO-Determination-r17        ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1700                HighSpeedParameters-v1700                                    OPTIONAL,

powSav-Parameters-v1700                  PowSav-Parameters-v1700                                      OPTIONAL,

mac-Parameters-v1700                     MAC-Parameters-v1700                                         OPTIONAL,

ims-Parameters-v1700                     IMS-Parameters-v1700                                         OPTIONAL,

measAndMobParameters-v1700               MeasAndMobParameters-v1700,

appLayerMeasParameters-r17               AppLayerMeasParameters-r17                                   OPTIONAL,

redCapParameters-r17                     RedCapParameters-r17                                         OPTIONAL,

ra-SDT-r17                               ENUMERATED {supported}                                       OPTIONAL,

srb-SDT-r17                              ENUMERATED {supported}                                       OPTIONAL,

gNB-SideRTT-BasedPDC-r17                 ENUMERATED {supported}                                       OPTIONAL,

bh-RLF-DetectionRecovery-Indication-r17  ENUMERATED {supported}                                       OPTIONAL,

nrdc-Parameters-v1700                    NRDC-Parameters-v1700                                        OPTIONAL,

bap-Parameters-v1700                     BAP-Parameters-v1700                                         OPTIONAL,

musim-GapPreference-r17                  ENUMERATED {supported}                                       OPTIONAL,

musimLeaveConnected-r17                  ENUMERATED {supported}                                       OPTIONAL,

mbs-Parameters-r17                       MBS-Parameters-r17,

nonTerrestrialNetwork-r17                ENUMERATED {supported}                                       OPTIONAL,

ntn-ScenarioSupport-r17                  ENUMERATED {gso, ngso}                                       OPTIONAL,

sliceInfoforCellReselection-r17          ENUMERATED {supported}                                       OPTIONAL,

ue-RadioPagingInfo-r17                   UE-RadioPagingInfo-r17                                       OPTIONAL,

-- R4 17-2 UL gap pattern for Tx power management

ul-GapFR2-Pattern-r17                    BIT STRING (SIZE (4))                                        OPTIONAL,

ntn-Parameters-r17                       NTN-Parameters-r17                                           OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1740                                       OPTIONAL

}

UE-NR-Capability-v1740 ::=               SEQUENCE {

redCapParameters-v1740                   RedCapParameters-v1740,

nonCriticalExtension                     UE-NR-Capability-v1750                                       OPTIONAL

}

UE-NR-Capability-v1750 ::=               SEQUENCE {

crossCarrierSchedulingConfigurationRelease-r17  ENUMERATED {supported}                                OPTIONAL,

nonCriticalExtension                            UE-NR-Capability-v1800                                OPTIONAL

}

-- Regular non-critical Rel-18 extensions:

UE-NR-Capability-v1800 ::=               SEQUENCE {

airToGroundNetwork-r18                   ENUMERATED {supported}                                       OPTIONAL,

eRedCapParameters-r18                    ERedCapParameters-r18                                        OPTIONAL,

ncr-Parameters-r18                       NCR-Parameters-r18                                           OPTIONAL,

softSatelliteSwitchResyncNTN-r18         ENUMERATED {supported}                                       OPTIONAL,

hardSatelliteSwitchResyncNTN-r18         ENUMERATED {supported}                                       OPTIONAL,

mt-SDT-r18                               ENUMERATED {supported}                                       OPTIONAL,

mt-SDT-NTN-r18                           ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndAutonomousDenial-r18      ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndFDM-r18                   ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndTDM-r18                   ENUMERATED {supported}                                       OPTIONAL,

musim-GapPriorityPreference-r18          ENUMERATED {supported}                                       OPTIONAL,

musim-CapabilityRestriction-r18          ENUMERATED {supported}                                       OPTIONAL,

multiRx-FR2-Preference-r18               ENUMERATED {supported}                                       OPTIONAL,

ra-InsteadCG-SDT-r18                     ENUMERATED {supported}                                       OPTIONAL,

resumeAfterSDT-Release-r18               ENUMERATED {supported}                                       OPTIONAL,

additionalBSR-Table-r18                  ENUMERATED {supported}                                       OPTIONAL,

delayStatusReport-r18                    ENUMERATED {supported}                                       OPTIONAL,

disableCG-RetransmissionMonitoring-r18   ENUMERATED {supported}                                       OPTIONAL,

enhancedDRX-r18                          ENUMERATED {supported}                                       OPTIONAL,

pdu-SetDiscard-r18                       ENUMERATED {supported}                                       OPTIONAL,

psi-BasedDiscard-r18                     ENUMERATED {supported}                                       OPTIONAL,

ul-TrafficInfo-r18                       ENUMERATED {supported}                                       OPTIONAL,

aerialParameters-r18                     AerialParameters-r18                                         OPTIONAL,

nonCriticalExtension                     SEQUENCE{}                                                   OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=          SEQUENCE {

phy-ParametersXDD-Diff                   Phy-ParametersXDD-Diff                                       OPTIONAL,

mac-ParametersXDD-Diff                   MAC-ParametersXDD-Diff                                       OPTIONAL,

measAndMobParametersXDD-Diff             MeasAndMobParametersXDD-Diff                                 OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::=          SEQUENCE {

phy-ParametersFRX-Diff                   Phy-ParametersFRX-Diff                                       OPTIONAL,

measAndMobParametersFRX-Diff             MeasAndMobParametersFRX-Diff                                 OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1610 ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16       ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16          ENUMERATED {supported}                                       OPTIONAL

}

BAP-Parameters-v1700 ::=                 SEQUENCE {

bapHeaderRewriting-Rerouting-r17         ENUMERATED {supported}                                       OPTIONAL,

bapHeaderRewriting-Routing-r17           ENUMERATED {supported}                                       OPTIONAL

}

MBS-Parameters-r17 ::=                   SEQUENCE {

maxMRB-Add-r17                           INTEGER (1..16)                                              OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

UE-NR-Capability-v1540 field descriptions

fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.

#### –	UE-RadioPagingInfo

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

-- TAG-UE-RADIOPAGINGINFO-START

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

-- TAG-UE-RADIOPAGINGINFO-STOP

-- ASN1STOP

END OF 1st CHANGE

START OF 2nd CHANGE

### 6.6.2	Message definitions

---------------------------------------------------------------------------------------------------------unrelated part omitted-------------------------------------------------------------------------------------------------------

#### –	UECapabilityInformationSidelink

The UECapabilityInformationSidelink message is used to transfer UE radio access capabilities. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

UECapabilityInformationSidelink message

-- ASN1START

-- TAG-UECAPABILITYINFORMATIONSIDELINK-START

UECapabilityInformationSidelink ::=         SEQUENCE {

rrc-TransactionIdentifier-r16               RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

ueCapabilityInformationSidelink-r16         UECapabilityInformationSidelink-r16-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

UECapabilityInformationSidelink-r16-IEs ::= SEQUENCE {

accessStratumReleaseSidelink-r16            AccessStratumReleaseSidelink-r16,

pdcp-ParametersSidelink-r16                 PDCP-ParametersSidelink-r16                                             OPTIONAL,

rlc-ParametersSidelink-r16                  RLC-ParametersSidelink-r16                                              OPTIONAL,

supportedBandCombinationListSidelinkNR-r16  BandCombinationListSidelinkNR-r16                                       OPTIONAL,

supportedBandListSidelink-r16               SEQUENCE (SIZE (1..maxBands)) OF BandSidelinkPC5-r16                    OPTIONAL,

appliedFreqBandListFilter-r16               FreqBandList                                                            OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        UECapabilityInformationSidelink-v1700-IEs                               OPTIONAL

}

UECapabilityInformationSidelink-v1700-IEs ::= SEQUENCE {

mac-ParametersSidelink-r17                    MAC-ParametersSidelink-r17                                            OPTIONAL,

supportedBandCombinationListSidelinkNR-v1710  BandCombinationListSidelinkNR-v1710                                   OPTIONAL,

nonCriticalExtension                          UECapabilityInformationSidelink-v1800-IEs                             OPTIONAL

}

UECapabilityInformationSidelink-v1800-IEs ::= SEQUENCE {

sfn-DFN-OffsetSupported-r18                   ENUMERATED { supported }                                              OPTIONAL,

posSIB-ForwardingSupported-r18                ENUMERATED { supported }                                              OPTIONAL,

pdcp-ParametersSidelink-r18                   PDCP-ParametersSidelink-r18                                           OPTIONAL,

nonCriticalExtension                          SEQUENCE {}                                                           OPTIONAL

}

PDCP-ParametersSidelink-r18 ::=               SEQUENCE {

pdcp-DuplicationSRB-sidelink-r18              ENUMERATED {supported}                                                OPTIONAL,

pdcp-DuplicationDRB-sidelink-r18              ENUMERATED {supported}                                                OPTIONAL,

...

}

MAC-ParametersSidelink-r17 ::= SEQUENCE {

drx-OnSidelink-r17                          ENUMERATED {supported}                                                  OPTIONAL,

...

}

AccessStratumReleaseSidelink-r16 ::= ENUMERATED { rel16, rel17, rel18, spare5, spare4, spare3, spare2, spare1, ... }

PDCP-ParametersSidelink-r16 ::= SEQUENCE {

outOfOrderDeliverySidelink-r16              ENUMERATED {supported}      OPTIONAL,

...

}

BandCombinationListSidelinkNR-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkNR-r16

BandCombinationListSidelinkNR-v1710 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkNR-v1710

BandCombinationParametersSidelinkNR-r16 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelink-r16

BandCombinationParametersSidelinkNR-v1710 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelink-v1710

BandParametersSidelink-v1710 ::=    SEQUENCE {

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17    ENUMERATED {supported}                      OPTIONAL,

--32-5b-1

tx-IUC-Scheme2-Mode2Sidelink-r17    ENUMERATED {n4, n8, n16}                    OPTIONAL

}

BandSidelinkPC5-r16 ::=           SEQUENCE {

freqBandSidelink-r16              FreqBandIndicatorNR,

--15-1

sl-Reception-r16                  SEQUENCE {

harq-RxProcessSidelink-r16        ENUMERATED {n16, n24, n32, n64},

pscch-RxSidelink-r16              ENUMERATED {value1, value2},

scs-CP-PatternRxSidelink-r16      CHOICE {

fr1-r16                           SEQUENCE {

scs-15kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-30kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-60kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL

},

fr2-r16                           SEQUENCE {

scs-60kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-120kHz-r16                    BIT STRING (SIZE (16))                            OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-RxSidelink-r16         ENUMERATED {supported}                                    OPTIONAL

}                                                                                               OPTIONAL,

--15-10

sl-Tx-256QAM-r16                  ENUMERATED {supported}                                        OPTIONAL,

--15-12

lowSE-64QAM-MCS-TableSidelink-r16 ENUMERATED {supported}                                        OPTIONAL,

...,

[[

--15-14

csi-ReportSidelink-r16                SEQUENCE {

csi-RS-PortsSidelink-r16              ENUMERATED {p1, p2}

}                                                                                               OPTIONAL,

--15-19

rankTwoReception-r16                  ENUMERATED {supported}                                    OPTIONAL,

--15-23

sl-openLoopPC-RSRP-ReportSidelink-r16 ENUMERATED {supported}                                    OPTIONAL,

--13-1

sl-Rx-256QAM-r16                      ENUMERATED {supported}                                    OPTIONAL

]],

[[

--32-5a-2

rx-IUC-Scheme1-PreferredMode2Sidelink-r17     ENUMERATED {supported}                           OPTIONAL,

--32-5a-3

rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17  ENUMERATED {supported}                           OPTIONAL,

--32-5b-2

rx-IUC-Scheme2-Mode2Sidelink-r17               ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-6-1

rx-IUC-Scheme1-SCI-r17                         ENUMERATED {supported}                           OPTIONAL,

--32-6-2

rx-IUC-Scheme1-SCI-ExplicitReq-r17             ENUMERATED {supported}                           OPTIONAL,

--32-7

scheme2-ConflictDeterminationRSRP-r17          ENUMERATED {supported}                           OPTIONAL

]],

[[

-- R4 45-2: SL reception in intra-carrier guard band

sl-ReceptionIntraCarrierGuardBand-r18         ENUMERATED {supported}                            OPTIONAL

]]

}

-- TAG-UECAPABILITYINFORMATIONSIDELINK-STOP

-- ASN1STOP

END OF 2nd CHANGE

START OF 3rd CHANGE

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAdditionalRACH-r17                   INTEGER ::= 256     -- Maximum number of additional RACH configurations.

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxCandidateBandIndex-r18               INTEGER ::= 8       -- Maximum number of band entry index for MUSIM capability

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxTwoPUCCH-Grp-ConfigList-r17          INTEGER ::= 16      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config} for PUCCH cell switching

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum number of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellATG-r18                          INTEGER ::= 8       -- Maximum number of ATG neighbour cells for which assistance information is

-- provided

maxCellExcluded                         INTEGER ::= 16      -- Maximum number of NR exclude-listed cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited PCells reported

maxPSCellHistory-r17                    INTEGER ::= 16      -- Maximum number of visited PSCells across all reported PCells

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellNTN-r17                          INTEGER ::= 4       -- Maximum number of NTN neighbour cells for which assistance information is

-- provided

maxCarrierTypePairList-r16              INTEGER ::= 16      -- Maximum number of supported carrier type pair of (carrier type on which

-- CSI measurement is performed, carrier type on which CSI reporting is

-- performed) for CSI reporting cross PUCCH group

maxCellAllowed                          INTEGER ::= 16      -- Maximum number of NR allow-listed cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellExcluded                   INTEGER ::= 16      -- Maximum number of E-UTRA exclude-listed physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFeatureCombPreamblesPerRACHResource-r17 INTEGER ::= 256  -- Maximum number of feature combination preambles.

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCells + SCells) minus 1

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofAggregatedCellsPerCellGroupMinus4-r16 INTEGER ::= 12

maxNrofAperiodicFwdTimeResource-r18     INTEGER ::= 112     -- Max number of aperiodic fowarding time resources for NCR

maxNrofAperiodicFwdTimeResource-1-r18   INTEGER ::= 111     -- Max number of aperiodic fowarding time resources for NCR minus 1

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAppLayerMeas-r17                 INTEGER ::= 16      -- Max number of simultaneous application layer measurements

maxNrofAppLayerMeas-1-r17               INTEGER ::= 15      -- Max number of simultaneous application layer measurements minus 1

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-1-r16 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofIABResourceConfig-r17            INTEGER ::= 65536   -- Max number of IAB-ResourceConfigID used in MAC CE

maxNrofIABResourceConfig-1-r17          INTEGER ::= 65535   -- Max number of IAB-ResourceConfigID used in MAC CE minus 1

maxNrofPeriodicFwdResourceSet-r18       INTEGER ::= 32      -- Max number of periodic fowarding resource sets for NCR

maxNrofPeriodicFwdResourceSet-1-r18     INTEGER ::= 31      -- Max number of periodic fowarding resource sets for NCR minus 1

maxNrofPeriodicFwdResource-r18          INTEGER ::= 1024    -- Max number of periodic fowarding resources for NCR

maxNrofPeriodicFwdResource-1-r18        INTEGER ::= 1023    -- Max number of periodic fowarding resources for NCR minus 1

maxNrofSemiPersistentFwdResourceSet-r18 INTEGER ::= 32      -- Max number of semi-persistent fowarding resource sets for NCR

maxNrofSemiPersistentFwdResourceSet-1-r18 INTEGER ::= 31    -- Max number of semi-persistent fowarding resource sets for NCR minus 1

maxNrofSemiPersistentFwdResource-r18    INTEGER ::= 128     -- Max number of semi-persistent fowarding resources for NCR

maxNrofSemiPersistentFwdResource-1-r18  INTEGER ::= 127     -- Max number of semi-persistent fowarding resources for NCR minus 1

maxNrofSCellActRS-r17                   INTEGER ::= 255     -- Max number of RS configurations per SCell for SCell activation

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement object

maxNrofCRS-IM-InterfCell-r17            INTEGER ::= 8       -- Maximum number of LTE interference cells for CRS-IM per UE

maxNrofRelayMeas-r17                    INTEGER ::= 32      -- Maximum number of L2 U2N Relay UEs to measure for each measurement object

-- on sidelink frequency

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of sidelink configured grant

maxNrofCG-SL-1-r16                      INTEGER ::= 7       -- Max number of sidelink configured grant minus 1

maxSL-GC-BC-DRX-QoS-r17                 INTEGER ::= 16      -- Max number of sidelink DRX configurations for NR

-- sidelink groupcast/broadcast communication

maxNrofSL-RxInfoSet-r17                 INTEGER ::= 4       -- Max number of sidelink DRX configuration sets in sidelink DRX assistant

-- information

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCondCells-1-r17                  INTEGER ::= 7       -- Max number of conditional candidate SpCells minus 1

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofDL-AllocationsExt-r17            INTEGER ::= 64      -- Maximum number of PDSCH time domain resource allocations for multi-PDSCH

-- scheduling

maxNrofDL-Allocations-1-r18             INTEGER ::= 15      -- Maximum number of PDSCH time domain resource allocations minus 1

maxNrofPDU-Sessions-r17                 INTEGER ::= 256     -- Maximum number of PDU Sessions

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxNrofLCGs-r18                         INTEGER ::= 8       -- Maximum number of LCGs

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLCG-ID-IAB-r17                       INTEGER ::= 255     -- Maximum value of LCG ID for IAB-MT

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= 65855   -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets              INTEGER ::= 12      -- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxNrofSearchSpacesLinks-1-r17          INTEGER ::= 39      -- Max number of Search Space links minus 1

maxNrofBFDResourcePerSet-r17            INTEGER ::= 64      -- Max number of reference signal in one BFD set

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxIAB-IP-Address-r16                   INTEGER ::= 32      -- Max number of assigned IP addresses

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourcesPerSet-1-r18 INTEGER ::= 63      -- Maximum number of NZP CSI-RS resources per resource set minus 1

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resource sets per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resource sets per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofCSI-SSB-ResourceSetsPerConfigExt INTEGER ::= 2       -- Maximum number of CSI SSB resource sets per resource configuration

-- extended

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFailureDetectionResources-1-r17  INTEGER ::= 63      -- Maximum number of the enhanced failure detection resources minus 1

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequency for NR sidelink communication

maxNrofFreqSL-1-r18                     INTEGER ::= 7       -- Maximum number of carrier frequency for NR sidelink communication minus 1

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for NR sidelink communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequency for NR sidelink communication

maxNrofSL-MeasId-r16                    INTEGER ::= 64      -- Maximum number of sidelink measurement identity (RSRP) per destination

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP) per destination

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP) per destination

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resource pool for NR sidelink measurement to measure for

-- each measurement object (for CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequency for NR sidelink communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establishment

maxTAC-r17                              INTEGER ::= 12      -- Maximum number of Tracking Area Codes to which a cell belongs to

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

-- minus 1.

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication and discovery

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication and discovery

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE without duplication

maxSL-LCID-Plus1-r18                    INTEGER ::= 513     -- Maximum number of RLC bearer for NR sidelink communication per UE without duplication plus 1

maxSL-LCID-r18                          INTEGER ::= 1024    -- Maximum number of RLC bearer for NR sidelink communication per UE with duplication

maxSL-NonAnchorRBsets                   INTEGER ::= 4       -- Maximum number of non-anchor RB sets

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource pool for NR sidelink communication and

-- discovery

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resource pool for NR sidelink communication and

-- discovery

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication and

-- discovery

maxNrofSRS-PathlossReferenceRS-r16      INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for SRS power control.

maxNrofSRS-PathlossReferenceRS-1-r16    INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for SRS power control

-- minus 1.

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxSimultaneousBands-2-r18              INTEGER ::= 30      -- Maximum number of simultaneously aggregated bands minus 2.

maxULTxSwitchingBandPairs               INTEGER ::= 32      -- Maximum number of band pairs supporting dynamic UL Tx switching in a band

-- combination.

maxULTxSwitchingBetweenBandPairs-r18    INTEGER ::= 32      -- Maximum number of combinations of a band pair and another band pair/band

-- between which dynamic UL Tx switching requires additional switching

-- period.

maxSchedulingBandCombination-r18        INTEGER ::= 32      -- Maximum number of combinations of scheduling cell and co-scheduled cells

-- have same or different carrier type.

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r17 INTEGER ::= 7       -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1.

maxNrofPUCCH-PathlossReferenceRSsDiff-r16 INTEGER ::= 60    -- Difference between the extended maximum and the non-extended maximum

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= 128     -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofPowerControlSetInfos-r17         INTEGER ::= 8       -- Maximum number of PUCCH power control set infos

maxNrofMultiplePUSCHs-r16               INTEGER ::= 8       -- Maximum number of multiple PUSCHs in PUSCH TDRA list

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see TS 38.213 [13], clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see TS 38.213 [13], clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended minus 1

maxNrofPUSCH-PathlossReferenceRSsDiff-r16  INTEGER ::= 60   -- Difference between maxNrofPUSCH-PathlossReferenceRSs-r16 and

-- maxNrofPUSCH-PathlossReferenceRSs

maxNrofPathlossReferenceRSs-r17         INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH, PUCCH, SRS

-- power control for unified TCI state operation

maxNrofPathlossReferenceRSs-1-r17       INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH, PUCCH, SRS

-- power control for unified TCI state operation minus 1

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModList).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqLayers                           INTEGER ::= 4       -- Max number of frequency layers.

maxFreqPlus1                            INTEGER ::= 9       -- Max number of frequencies for Slicing.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 48      -- Max number of PRACH-ResourceDedicatedBFR in the CandidateBeamRSListExt

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximum number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 256

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxUL-TCI-r17                           INTEGER ::= 64      -- Maximum number of TCI states.

maxUL-TCI-1-r17                         INTEGER ::= 63      -- Maximum number of TCI states minus 1.

maxNrofAdditionalPCI-r17                INTEGER ::= 7       -- Maximum number of additional PCI

maxNrofAdditionalPRACHConfigs-r18       INTEGER ::= 7       -- Maximum number of additional PRACH configurations for 2TA

maxNrofdelayD-r18                       INTEGER ::= 4       -- Maximum number of delayD values.

maxMPE-Resources-r17                    INTEGER ::= 64      -- Maximum number of pooled MPE resources

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks supported by the UE

maxNrofCSI-RS-ResourcesExt-r16          INTEGER ::= 16      -- Maximum number of codebook resources supported by the UE for eType2/Codebook combo

maxNrofCSI-RS-ResourcesExt-r17          INTEGER ::= 8       -- Maximum number of codebook resources for fetype2R1 and fetype2R2

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16          INTEGER ::= 512     -- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16        INTEGER ::= 511     -- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxSIB-MessagePlus1-r17                 INTEGER::= 33       -- Maximum number of SIB messages plus 1

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of access control parameter sets

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identities in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxGIN-r17                              INTEGER ::= 24      -- Maximum number of broadcast GINs

maxHRNN-Len-r16                         INTEGER ::= 48      -- Maximum length of HRNNs

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxSNPN-ConfigCellId-r18                INTEGER ::= 32      -- Maximum number of Cell ID subject for SNPNS for MDT scope

maxSNPN-ConfigID-r18                    INTEGER ::= 16      -- Maximum number of SNPNs in the MDT SNPN list

maxSNPN-ConfigTAI-r18                   INTEGER ::= 8       -- Maximum number of TA subject for MDT scope

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxK0-SchedulingOffset-r17              INTEGER ::= 64      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r17              INTEGER ::= 64      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-7-Size-r17                     INTEGER ::= 43      -- Maximum size of DCI format 2-7

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxDCI-2-9-Size-1-r18                   INTEGER ::= 139     -- Maximum DCI format 2-9 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofUL-Allocations-1-r18             INTEGER ::= 63      -- Maximum number of PUSCH time domain resource allocations minus 1

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxOnDemandSIB-r16                      INTEGER ::= 8       -- Maximum number of SIB(s) that can be requested on-demand

maxOnDemandPosSIB-r16                   INTEGER ::= 32      -- Maximum number of posSIB(s) that can be requested on-demand

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-1-r16             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxUu-RelayRLC-ChannelID-r17            INTEGER ::= 32      -- Maximum value of Uu Relay RLC channel ID

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the RA report

maxTxConfig-r16                         INTEGER ::= 64      -- Maximum number of sidelink transmission parameters configurations

maxTxConfig-1-r16                       INTEGER ::= 63      -- Maximum number of sidelink transmission parameters configurations minus 1

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-1-r16         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofCLI-SRS-Resources-r16            INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofCC-Group-r17                     INTEGER ::= 16      -- Maximum number of CC groups for DC location report

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-1-r16      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofCG-Type2DeactivationState        INTEGER ::= 16      -- Maximum number of deactivation state for type 2 configured grants per BWP

maxNrofConfiguredGrantConfigMAC-1-r16   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18 INTEGER ::= 8 -- Maximum number of CSI report subconfigurations per CSI report

-- configuration

maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-1-r18 INTEGER ::= 7 -- Maximum number of CSI report subconfigurations per CSI report

-- configuration minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofPPW-Config-r17                   INTEGER ::= 4       -- Maximum number of Preconfigured PRS processing windows per DL BWP

maxNrofPPW-ID-1-r17                     INTEGER ::= 15      -- Maximum number of Preconfigured PRS processing windows minus 1

maxNrOfTxTEGReport-r17                  INTEGER ::= 256     -- Maximum number of UE Tx Timing Error Group Report

maxNrOfTxTEG-ID-1-r17                   INTEGER ::= 7       -- Maximum number of UE Tx Timing Error Group ID minus 1

maxNrofPagingSubgroups-r17              INTEGER ::= 8       -- Maximum number of paging subgroups per paging occasion

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3

maxNrofReqComDC-Location-r17            INTEGER ::= 128     -- Maximum number of requested carriers/BWPs combinations for DC location

-- report

maxNrofServingCellsTCI-r16              INTEGER ::= 32      -- Maximum number of serving cells in simultaneousTCI-UpdateList

maxNrofTxDC-TwoCarrier-r16              INTEGER ::= 64      -- Maximum number of UL Tx DC locations reported by the UE for 2CC uplink CA

maxNrofRB-SetGroups-r17                 INTEGER ::= 8       -- Maximum number of RB set groups

maxNrofRB-Sets-r17                      INTEGER ::= 8       -- Maximum number of RB sets

maxNrofEnhType3HARQ-ACK-r17             INTEGER ::= 8       -- Maximum number of enhanced type 3 HARQ-ACK codebook

maxNrofEnhType3HARQ-ACK-1-r17           INTEGER ::= 7       -- Maximum number of enhanced type 3 HARQ-ACK codebook minus 1

maxNrofPRS-ResourcesPerSet-r17          INTEGER ::= 64      -- Maximum number of PRS resources for one set

maxNrofPRS-ResourcesPerSet-1-r17        INTEGER ::= 63      -- Maximum number of PRS resources for one set minus 1

maxNrofPRS-ResourceOffsetValue-1-r17    INTEGER ::= 511

maxNrofGapId-r17                        INTEGER ::= 8       -- Maximum number of measurement gap ID

maxNrofPreConfigPosGapId-r17            INTEGER ::= 16      -- Maximum number of preconfigured positioning measurement gap

maxNrOfGapPri-r17                       INTEGER ::= 16      -- Maximum number of gap priority level

maxCEFReport-r17                        INTEGER ::= 4       -- Maximum number of CEF reports by the UE

maxNrofMultiplePDSCHs-r17               INTEGER ::= 8       -- Maximum number of PDSCHs in PDSCH TDRA list

maxSliceInfo-r17                        INTEGER ::= 8       -- Maximum number of NSAGs

maxCellSlice-r17                        INTEGER ::= 16      -- Maximum number of cells supporting the NSAG

maxNrofTRS-ResourceSets-r17             INTEGER ::= 64      -- Maximum number of TRS resource sets

maxNrofSearchSpaceGroups-1-r17          INTEGER ::= 2       -- Maximum number of search space groups minus 1

maxNrofRemoteUE-r17                     INTEGER ::= 32      -- Maximum number of connected L2 U2N Remote UEs

maxDCI-4-2-Size-r17                     INTEGER ::= 140     -- Maximum size of DCI format 4-2

maxFreqMBS-r17                          INTEGER ::= 16      -- Maximum number of MBS frequencies reported in MBSInterestIndication

maxNrofDRX-ConfigPTM-r17                INTEGER ::= 64      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell

maxNrofDRX-ConfigPTM-1-r17              INTEGER ::= 63      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell minus 1

maxNrofMBS-ServiceListPerUE-r17         INTEGER ::= 16      -- Maximum number of services which the UE can include in the  MBS interest

-- indication

maxNrofMBS-Session-r17                  INTEGER ::= 1024    -- Maximum number of MBS sessions provided in MBS broadcast in a cell

maxNrofMTCH-SSB-MappingWindow-r17       INTEGER ::= 16      -- Maximum number of MTCH to SSB beam mapping pattern

maxNrofMTCH-SSB-MappingWindow-1-r17     INTEGER ::= 15      -- Maximum number of MTCH to SSB beam mapping pattern minus 1

maxNrofMRB-Broadcast-r17                INTEGER ::= 4       -- Maximum number of broadcast MRBs configured for one MBS broadcast service

maxNrofPageGroup-r17                    INTEGER ::= 32      -- Maximum number of paging groups in a paging message

maxNrofPDSCH-ConfigPTM-r17              INTEGER ::= 16      -- Maximum number of PDSCH configuration groups for PTM

maxNrofPDSCH-ConfigPTM-1-r17            INTEGER ::= 15      -- Maximum number of PDSCH configuration groups for PTM minus 1

maxG-RNTI-r17                           INTEGER ::= 16      -- Maximum number of G-RNTI that can be configured for a UE.

maxG-RNTI-1-r17                         INTEGER ::= 15      -- Maximum number of G-RNTI that can be configured for a UE minus 1.

maxG-CS-RNTI-r17                        INTEGER ::= 8       -- Maximum number of G-CS-RNTI that can be configured for a UE.

maxG-CS-RNTI-1-r17                      INTEGER ::= 7       -- Maximum number of G-CS-RNTI that can be configured for a UE minus 1.

maxMRB-r17                              INTEGER ::= 32      -- Maximum number of multicast MRBs (that can be added in MRB-ToAddModLIst)

maxFSAI-MBS-r17                         INTEGER ::= 64      -- Maximum number of MBS frequency selection area identities

maxNeighCellMBS-r17                     INTEGER ::= 8       -- Maximum number of MBS broadcast neighbour cells

maxNrofPdcch-BlindDetectionMixed-1-r16  INTEGER ::= 7       -- Maximum number of combinations of mixed Rel-16 and Rel-15 PDCCH

-- monitoring capabilities minus 1

maxNrofPdcch-BlindDetection-r17         INTEGER ::= 16      -- Maximum number of combinations of PDCCH blind detection monitoring

-- capabilities

maxNrofAltitudeRanges-r18               INTEGER ::= 8       -- Maximum number of altitude ranges for altitude-based measurement configurations

maxWayPoint-r18                         INTEGER ::= 20      -- Maximum number of flight path information waypoints

maxAltitude-r18                         INTEGER ::= 10000   -- Maximum altitude in meters

minAltitude-r18                         INTEGER ::= -420    -- Minimum altitude in meters

maxMeasSequence-r18                     INTEGER ::= 64      -- Maximum number of configured sequence for measurement

maxNrofHops-r18-1                       INTEGER ::= 5       -- Maximum number of Hops that can be configured for Positioning SRS Transmission

maxNrOfCellsInVA-r18                    INTEGER ::= 16      -- Maximum number of cells in validity area for Positioning SRS is FFS

maxNrOfLinkedSRS-PosResourceSet-r18     INTEGER ::= 3       -- Value is FFS Maximum number of SRSPosResourceSets that can be aggregated across CCs

maxCBR-ConfigDedSL-PRS-1-r18            INTEGER ::= 7       -- Maximum number of CBR ranges for dedicated SL PRS resource pool

maxCBR-LevelDedSL-PRS-1-r18             INTEGER ::= 15      -- Maximum number of CBR levels for dedicated SL PRS resource pool

maxNrofSL-PRS-TxPool-r18                INTEGER ::= 8       -- Maximum number of Tx dedicated SL-PRS resource pool for NR sidelink positioning is FFS

maxNrofSL-PRS-TxConfig-r18              INTEGER ::= 64      -- Maximum number of SL PRS transmission parameter configurations

maxNrOfVA-r18                           INTEGER ::= 16      -- Maximum number of validity area is FFS

maxNrofLTM-Configs-r18                  INTEGER ::= 8       -- Maximum number of LTM candidate cells

maxNrofLTM-Configs-r18-plus-1           INTEGER ::= 9       -- Maximum number of LTM candidate cells plus 1

maxNrofLTM-CSI-ReportConfigurations-r18     INTEGER ::= 48     -- Maximum number of LTM CSI reporting configurations

maxNrofLTM-CSI-ReportConfigurations-1-r18   INTEGER ::= 47     -- Maximum number of LTM CSI reporting configurations minus 1

maxNrofLTM-CSI-SSB-ResourcesPerSet-r18      INTEGER ::= 512    -- Maximum number of LTM CSI SSB resource per set

maxNrofLTM-CSI-ResourceConfigurations-r18   INTEGER ::= 112    -- Maximum number of LTM CSI resource configurations

maxNrofLTM-CSI-ResourceConfigurations-r18-1 INTEGER ::= 111    -- Maximum number of LTM CSI resource configurations minus 1

maxNrofCandidateTCI-State-r18           INTEGER ::= 128     -- Maximum number of LTM TCI states

maxNrofCandidateTCI-State-r18-1         INTEGER ::= 127     -- Maximum number of LTM TCI states minus 1

maxNrofCandidateUL-TCI-r18              INTEGER ::= 64      -- Maximum number of LTM UL TCI states

maxNrofCandidateUL-TCI-r18-1            INTEGER ::= 63      -- Maximum number of LTM UL TCI states minus 1

maxSecurityCellSet-r18                  INTEGER ::= 9       -- Maximum number of cell sets for subsequent CPAC.

maxSK-Counter-r18                       INTEGER ::= 8       -- Maximum number of SK-counters configured for a cell set for subsequent CPAC.

maxNrofThresholdMBS-r18                 INTEGER ::= 8       -- Max number of thresholds of MBS sessions for RRC connection resume for a

-- UE receiving multicast in RRC_INACTIVE

maxTN-AreaInfo-r18                      INTEGER ::= 32      -- Maximum number of TN coverage areas for which assistance info is

-- provided in an NTN cell

maxNrofSetsOfCells-r18                  INTEGER ::= 4       -- Maximum number of sets of cells for multi-cell PDSCH/PUSCH scheduling

maxNrofSetsOfCells-1-r18                INTEGER ::= 3       -- Maximum number of sets of cells for multi-cell PDSCH/PUSCH scheduling

-- minus 1

maxNrofCellsInSet-r18                   INTEGER ::= 4       -- Maximum number of cells configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling

maxNrofCellsInSet-1-r18                 INTEGER ::= 3       -- Maximum number of cells configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling minus 1

maxNrofCellCombos-r18                   INTEGER ::= 16      -- Maximum number of combinations of co-scheduled cells for multi-cell

-- PDSCH/PUSCH scheduling

maxNrofBWPsInSetOfCells-r18             INTEGER ::= 16      -- Maximum number of BWPs configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling

maxLowerMSD-r18                         INTEGER ::= 256     -- Maximum number of lower MSD capability sets for a victim band

maxLowerMSDInfo-r18                     INTEGER ::= 64      -- Maximum number of lower MSD capability sets for a band combination

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

END OF 3rd CHANGE