3GPP TSG-RAN WG2 Meeting #125	 R2-2400398

Athens, Greece, 26, Feb - 1, Mar, 2024

Agenda Item:	7.15.2

Source:	Xiaomi

Title:	[X020] Correction on SL carrier addition/release/modification triggered SUI

Document for:	Discussion

Introduction

In this contribution, we propose [X020] corrections on SL carrier addition/release/modificaiton triggered SUI.

Discussion

In SUI, sidelink TX UE in RRC_CONNECTED would indicate the transmission frequency of the associated destination as following spec shows. In legacy, the frequency is provided by upper layer.

#### 5.8.3.3 Actions related to transmission of SidelinkUEInformationNR message The UE shall set the contents of the SidelinkUEInformationNR message as follows: … 5> set sl-InterestedFreqList to indicate the frequency of the associated destination for NR sidelink communication transmission;

In SL CA, TX UE can add/modify/release the sidelink transmission carrier(s) by sidelink RRC reconfiguration procedure. The sidelink transmission carrier(s) addition/modification/release would apply upon reception of RRCReconfigurationCompleteSidelink message. In such case, the transmission carrier(s) is chagned. TX UE in RRC shall transmit SUI to update the transmission frequency of the associated destination. Otherwise, gNB may not be able to provide transmission resource on the sidelink transmission carrier(s).

Proposal 1: UE shall trigger SUI transmission if sidelink carrier(s) is changed upon reception of RRCReconfigurationCompleteSidelink.

We propose to add another bullet to trigger SUI transmission if sidelink carrier(s) is changed. Corresponding TP is provided in Annex.

Proposal 2: Agree the TP in Annex.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: UE shall trigger SUI transmission if sidelink carrier(s) is changed upon reception of RRCReconfigurationCompleteSidelink.

Proposal 2: Agree the TP in Annex.

TP

##### 5.8.9.1.9	Reception of an RRCReconfigurationCompleteSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink:

1>	stop timer T400 for the destination, if running;

1>	consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied.

2>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

3>	consider no sidelink DRX to be applied for the corresponding sidelink unicast communication.

- If UE is in RRC_CONNECTED and sl-CarrierToAddModList or sl-CarrierToReleaseList was included in the transmitted RRCReconfigurationSidelink:

2> perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;