3GPP TSG-RAN WG2 Meeting #125	R2-2400400

Athens, Greece, 26, Feb - 1, Mar, 2024

Agenda Item:	7.9.2

Source:	Xiaomi

Title:	Correction to 38.300 on Relay enhancement

Document for:	Discussion

Introduction

In this contribution, we propose corrections to 38.300 on Relay enhancement.

Discussion

In U2U, E2E SRB(s) is mapped to a specified PC5 Relay RLC channel. The mapping of E2E DRB(s) to PC5 Relay RLC channel is configurable. However, the E2E SRB(s) and DRB(s) can’t be mapped to the same PC5 Relay RLC channel. In current spec, it’s unclear whether E2E SRB(s) and DRB(s) can be mapped to the same PC5 Relay RLC channel. We propose to add a sentence to clarify an end-to-end DRB and an end-to-end SRB can’t be mapped into the same PC5 Relay RLC channel.

Proposal 1: add a sentence to clarify an end-to-end DRB and an end-to-end SRB can’t be mapped into the same PC5 Relay RLC channel.

#### 16.12.6.2 Switching from direct to indirect path … For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from direct to indirect path via a L2 U2N Relay UE in RRC_CONNECTED under another gNB: Figure 16.12.6.2-2: Procedure for L2 U2N Remote UE inter-gNB switching from direct to indirect path via a L2 U2N Relay UE in RRC_CONNECTED … 3. The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to one cell.

In step 3 of inter-gNB direct to indirect path procedure, source gNB send HANDOVER REQUEST to target gNB, which includes candidate target relay UE IDs. However, the candidate target relay UE(s) can belong to different cell as long as these cells belong to the target gNB. So, it’s not mandatory to require all candidate target relay UE belong to one cell. It’s proposed to change ‘one cell’ to ‘target gNB’. Similar issue exists in step 3 of inter-gNB indirect to indirect path procedure

Proposal 2: Change ‘one cell’ to ‘target gNB’ in step 3 of inter-gNB direct/indirect to indirect path procedure

- 16.12.6.3 Switching from indirect to indirect path … For service continuity of L2 U2N Remote UE between inter-gNBs, the following procedure is used, in case of the L2 U2N Remote UE, which is connected to indirect path, switching to another indirect path via a target L2 U2N Relay UE in RRC_CONNECTED under another gNB: Figure 16.12.6.3-2: Procedure for L2 U2N Remote UE inter-gNB switching from indirect to indirect path via a target L2 U2N Relay UE in RRC_CONNECTED 1. The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and Uu measurements to the source gNB, after it measures/discovers the candidate L2 U2N Relay UE(s): - The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria; - The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE's serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

In step 1 of inter-gNB indirect to indirect path procedure, remote UE reports the candidate relay UE along with associated sidelink measurement result. Remote UE may not be able to obtain Uu measurements due to OOC. It should not be mandatory to report Uu measurement results in step 1.

Proposal 3: Change ‘Uu measurements’ to ‘PC5 measurement results’ in step 1 of inter-gNB indirect to indirect path procedure

### 16.12.7 Control plane procedures for L2 U2U Relay The L2 U2U Remote UE needs to establish end-to-end SL-SRB/DRBs with the peer L2 U2U Remote UE before user plane data transmission. The following high level connection establishment procedure in Figure 16.12.7-1 applies to a L2 U2U Relay UE and L2 U2U Remote UE: Figure 16.12.7-1: Procedure for L2 U2U Remote UE connection establishment 1. The L2 U2U Remote UE, L2 U2U Relay UE, and peer L2 U2U Remote UE perform discovery procedure or integrated discovery procedure. 2a. The L2 U2U Remote UE establishes/modifies a PC5-RRC connection with the selected L2 U2U Relay UE (i.e., as specified in TS 23.304 [48]). 2b. The L2 U2U Relay UE establishes/modifies a PC5-RRC connection with the peer L2 U2U Remote UE (i.e., as specified in TS 23.304 [48]). 3. The L2 U2U Relay UE allocates two local IDs and it is delivered via RRCReconfigurationSidelink message to each of the L2 U2U Remote UEs: one local ID to identify the L2 U2U Remote UE, the other local ID to identify the peer L2 U2U Remote UE. When the local ID is delivered, an L2 ID of the peer L2 U2U Remote UE is also delivered to the U2U Remote UE for making the association between the local ID and the L2 ID of the peer U2U Remote UE. 4. The L2 U2U Remote UE establishes end-to-end PC5-RRC connection with the peer L2 U2U Remote UE via the L2 U2U Relay UE. For the end-to-end connection establishment, fixed indexes (i.e., 0/1/2/3) are defined for end-to-end SL-SRB 0/1/2/3 respectively, and specified PC5 Relay RLC Channel configuration is used on each hop. The sidelink UE capability is exchanged between the L2 U2U Remote UEs via PC5-RRC (e.g., SL-SRB3) message. 5. The L2 U2U Remote UE sends to the L2 U2U Relay UE all the QoS profiles for the end-to-end QoS flows via PC5-RRC. 6. The L2 U2U Relay UE performs QoS split only for PDB. NOTE: It is up to L2 U2U Relay UE implementation on how to split PDB. 7. The L2 U2U Relay UE sends the split QoS value (i.e., PDB) via PC5-RRC message to the L2 U2U Remote UE.8. The L2 U2U Remote UE or the serving gNB of the L2 U2U Remote UE derives the PDCP and SDAP configuration for end-to-end SL-DRB and provides the portion of the configuration related to reception to the peer L2 U2U Remote UE using end-to-end RRCReconfigurationSidelink messages. The end-to-end bearer IDs for SL-SRB and SL-DRB are used as input for the L2 U2U Relay ciphering and deciphering at PDCP. 9a. The L2 U2U Remote UE or the serving gNB of the L2 U2U Remote UE derives the first hop configuration (e.g. PC5 Relay RLC Channel configuration) for SL-DRB and provides to the L2 U2U Relay UE of the configuration related to receiving on the first hop (i.e., Rx by the relay UE), using per-hop RRCReconfigurationSidelink message. 9b. The L2 U2U Relay UE or the serving gNB of the L2 U2U Relay UE derives the second hop configuration (e.g. PC5 Relay RLC Channel configuration) for each SL-DRB and provides to the peer L2 U2U Remote UE of the configuration related to receiving data packets on the second hop (i.e., RX by the peer remote UE), using per-hop RRCReconfigurationSidelink message. 10. The L2 U2U Remote UE and the peer L2 U2U Remote UE transmit or receive data via L2 U2U Relay UE.

In step 8 of control plane procedure for L2 U2U relay, Remote UE shall send the PDCP/SDAP configuration to peer remote UE using E2E RRCReconfigurationSidelink message. Apparently, serving gNB of the remote UE can’t send RRCReconfigurationSidelink message to peer remote UE. Similar issue exists in step /9a/9b.

Proposal 4: Remove ‘or the serving gNB of the L2 U2U Remote UE’ from step 8/9a/9b from control plane procedure for L2 U2U relay.

#### 16.21.3.1 Path Management … Figure 16.21.3.1-3 describes the procedures for the direct path addition on top of the indirect path for the L2 MP Relay operation. This procedure is only applicable to the L2 MP Remote UE using SL indirect path. Figure 16.21.3.1-3: Procedure for direct path addition on top of indirect path 0. The L2 MP Remote UE performs data transmission and reception by using indirect path via PC5 link. 1. The L2 MP Remote UE perform measurements based on measurement configuration. When the measurement reporting is triggered, the L2 MP Remote UE reports at least the Uu signal strength of the serving cell and neighbour cells with the cell IDs (i.e., NCGI/NCI).

In step 1 of procedure for direct path addition on top of indirect path, L2 MP remote UE reports the Uu measurement results. In this step, remote UE only connects to relay UE and direct path is not added yet. There is no serving cell yet. So, remote UE should not be required to report the Uu signal strength of the serving cell. On the other hand, remote UE shall report the PC5 measurement result of the serving relay UE.

Proposal 5: Remove the ‘Uu signal strength of the serving cell’ and add ‘PC5 signal strength of the serving relay UE’ in step 1 of procedure for direct path addition on top of indirect path.

Corresponding TP is provided in Section 4.

Proposal 6: Agree the TP in Section 4.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: add a sentence to clarify an end-to-end DRB and an end-to-end SRB can’t be mapped into the same PC5 Relay RLC channel.

Proposal 2: Change ‘one cell’ to ‘target gNB’ in step 3 of inter-gNB direct/indirect to indirect path procedure

Proposal 3: Change ‘Uu measurements’ to ‘PC5 measurement results’ in step 1 of inter-gNB indirect to indirect path procedure

Proposal 4: Remove ‘or the serving gNB of the L2 U2U Remote UE’ from step 8/9a/9b from control plane procedure for L2 U2U relay.

Proposal 5: Remove the ‘Uu signal strength of the serving cell’ and add ‘PC5 signal strength of the serving relay UE’ in step 1 of procedure for direct path addition on top of indirect path.

Proposal 6: Agree the TP in Section 4.

TP

Start of change

#### 16.12.2.2	L2 UE-to-UE Relay

The protocol stacks for the user plane and the control plane of the L2 U2U Relay architecture are illustrated in Figure 16.12.2.2-1 and Figure 16.12.2.2-2. The SRAP sublayer is placed above the RLC sublayer for both CP and UP at both PC5 interfaces. The sidelink SDAP, PDCP and RRC are terminated between two L2 U2U Remote UEs (i.e., end-to-end), while SRAP, RLC, MAC and PHY are terminated in each hop of PC5 link.

Figure 16.12.2.2-1: User plane protocol stack for L2 UE-to-UE Relay

Figure 16.12.2.2-2: Control plane protocol stack for L2 UE-to-UE Relay

For L2 UE-to-UE Relay, the SRAP sublayer at L2 U2U Remote UE:

-	The SRAP sublayer at L2 U2U Remote UE performs bearer mapping between end-to-end PC5 Radio Bearers (SL-SRBs or SL-DRBs) of the L2 U2U Remote UE and at each hop of PC5 Relay RLC Channel between the L2 U2U Remote UE and the L2 U2U Relay UE.

-	For the traffic transmitted from an L2 U2U Remote UE to an L2 U2U Relay UE, the different end-to-end PC5 Radio Bearers (SL-SRBs or SL-DRBs) towards the same peer L2 U2U Remote UE and/or different peer L2 U2U Remote UEs can be multiplexed to the same PC5 Relay RLC channel, which is between the L2 U2U Remote UE(s) and the L2 U2U Relay UE. An end-to-end DRB and an end-to-end SRB can’t be multiplexed to the same PC5 Relay RLC channel.

-	For the traffic received at L2 U2U Remote UE, the same PC5 Relay RLC channel from one L2 U2U Relay UE can be de-multiplexed to the different end-to-end PC5 Radio Bearers (SL-SRBs or SL-DRBs) of the same peer L2 U2U Remote UE and/or different peer L2 U2U Remote UEs.

-	The SRAP sublayer at L2 U2U Remote UE supports identification of the peer L2 U2U Remote UE and itself. The local IDs are assigned by L2 U2U Relay UE to both L2 U2U Remote UEs for identification. For the two local IDs, one of them identifies L2 U2U Remote UE and the other identifies the peer L2 U2U Remote UE. The local ID of the peer L2 U2U Remote UE and the local ID of L2 U2U Remote UE are delivered by L2 U2U Relay UE to the L2 U2U Remote UEs along with the corresponding L2 ID of the peer L2 U2U Remote UE. The identity information of the end-to-end PC5 Radio Bearer and two local IDs are included in the SRAP header in order for the peer L2 U2U Remote UE to correlate the received packets for the specific PDCP entity associated with the right end-to-end PC5 Radio Bearer of the L2 U2U Remote UEs.

For L2 UE-to-UE Relay, the SRAP sublayer at L2 U2U Relay UE:

-	The SRAP sublayer at L2 U2U Relay UE determines the egress PC5 Relay RLC Channel based on the mapping of the end-to-end PC5 Radio Bearer and egress PC5 Relay RLC Channel for a particular pair between the L2 U2U Remote UE and the peer L2 U2U Remote UE.

-	For the ingress traffic received from an/multiple L2 U2U Remote UE(s) at L2 U2U Relay UE, the different end-to-end PC5 Radio Bearers (SL-SRBs or SL-DRBs) of the same L2 U2U Remote UE and/or the same/different end-to-end PC5 Radio Bearers (SL-SRBs or SL-DRBs) of L2 U2U Remote UEs can be multiplexed to the same egress PC5 Relay RLC channel, which is in between the L2 U2U Relay UE and the peer L2 U2U Remote UE. An end-to-end DRB and an end-to-end SRB can’t be multiplexed to the same PC5 Relay RLC channel.

Next change

- 16.12.6.2	Switching from direct to indirect path

…

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from direct to indirect path via a L2 U2N Relay UE in RRC_CONNECTED under another gNB:

Figure 16.12.6.2-2: Procedure for L2 U2N Remote UE inter-gNB switching from direct to indirect path via a L2 U2N Relay UE in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and measurements to the source gNB, after it measures/discovers the candidate L2 U2N Relay UE(s):

-	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

-	The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE's serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

2.	The source gNB decides to trigger path switch for the L2 U2N Remote UE onto indirect path of the target gNB, based on MeasurementReport and RRM information.

3.	The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to target gNB.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB selects one target Relay UE from the list of candidate Relay UEs provided by the source gNB, and sends the RRCReconfiguration message to L2 U2N Relay UE for relaying configuration, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu Relay RLC channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

6.	The target gNB sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB, which contains new RRC configuration for L2 U2N Remote UE.

7.	The source gNB sends the RRCReconfiguration message to the L2 U2N Remote UE, which includes at least the L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated Uu end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the source gNB.

8.	The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of the L2 U2N Remote UE's DRBs for which PDCP status preservation applies (i.e. for RLC AM).

9.	The L2 U2N Remote UE establishes PC5 connection with L2 U2N Relay UE.

10.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to target gNB via the L2 U2N Relay UE.

11.	The data path is switched from direct path to indirect path between the L2 U2N Remote UE and the target gNB via the target L2 U2N Relay UE.

12.	The target gNB sends the UE CONTEXT RELEASE message to inform the source gNB about the success of the path switch.

Next change

- 16.12.6.3	Switching from indirect to indirect path

…

For service continuity of L2 U2N Remote UE between inter-gNBs, the following procedure is used, in case of the L2 U2N Remote UE, which is connected to indirect path, switching to another indirect path via a target L2 U2N Relay UE in RRC_CONNECTED under another gNB:

Figure 16.12.6.3-2: Procedure for L2 U2N Remote UE inter-gNB switching from indirect to indirect path via a target L2 U2N Relay UE in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and PC5 measurement results to the source gNB, after it measures/discovers the candidate L2 U2N Relay UE(s):

-	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

-	The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE's serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

2.	The source gNB decides to trigger the L2 U2N Remote UE to switch to an indirect path of another gNB.

3.	The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to target gNB.

NOTE:	In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data has been acknowledged by the source L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB during the data forwarding procedure.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB selects one target Relay UE from the list of candidate Relay UEs provided by the source gNB, sends the RRCReconfiguration message to the L2 U2N Relay UE for relaying configuration, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu Relay RLC channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

6.	The target gNB sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB, which contains new RRC configuration for L2 U2N Remote UE.

7.	The source gNB sends the RRCReconfiguration message to the L2 U2N Remote UE, which includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control plane transmission over the (source) indirect path after reception of the RRCReconfiguration message from the source gNB.

8.	The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of the L2 U2N Remote UE's DRBs for which PDCP status preservation applies (i.e. for RLC AM).

9.	The L2 U2N Remote UE establishes PC5 connection to the target L2 U2N Relay UE.

10.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the target gNB via the target L2 U2N Relay UE.

11.	The data path is switched from indirect path to indirect path between the L2 U2N Remote UE and the target gNB via the target L2 U2N Relay UE.

12.	The target gNB sends the UE CONTEXT RELEASE message to inform the source gNB about the success of the path switch.

13.	The source gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the source gNB. The RRCReconfiguration message to the source L2 U2N Relay UE can be sent any time after step 7 based on source gNB implementation (e.g., to release Uu Relay RLC channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

14.	Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer indicates upper layer to release PC5 unicast link after receiving the RRCReconfiguration message from the source gNB. The timing to execute link release is up to UE implementation.

Next change

### 16.12.7	Control plane procedures for L2 U2U Relay

The L2 U2U Remote UE needs to establish end-to-end SL-SRB/DRBs with the peer L2 U2U Remote UE before user plane data transmission.

The following high level connection establishment procedure in Figure 16.12.7-1 applies to a L2 U2U Relay UE and L2 U2U Remote UE:

Figure 16.12.7-1: Procedure for L2 U2U Remote UE connection establishment

1.	The L2 U2U Remote UE, L2 U2U Relay UE, and peer L2 U2U Remote UE perform discovery procedure or integrated discovery procedure.

2a.	The L2 U2U Remote UE establishes/modifies a PC5-RRC connection with the selected L2 U2U Relay UE (i.e., as specified in TS 23.304 [48]).

2b.	The L2 U2U Relay UE establishes/modifies a PC5-RRC connection with the peer L2 U2U Remote UE (i.e., as specified in TS 23.304 [48]).

3.	The L2 U2U Relay UE allocates two local IDs and it is delivered via RRCReconfigurationSidelink message to each of the L2 U2U Remote UEs: one local ID to identify the L2 U2U Remote UE, the other local ID to identify the peer L2 U2U Remote UE. When the local ID is delivered, an L2 ID of the peer L2 U2U Remote UE is also delivered to the U2U Remote UE for making the association between the local ID and the L2 ID of the peer U2U Remote UE.

4.	The L2 U2U Remote UE establishes end-to-end PC5-RRC connection with the peer L2 U2U Remote UE via the L2 U2U Relay UE. For the end-to-end connection establishment, fixed indexes (i.e., 0/1/2/3) are defined for end-to-end SL-SRB 0/1/2/3 respectively, and specified PC5 Relay RLC Channel configuration is used on each hop. The sidelink UE capability is exchanged between the L2 U2U Remote UEs via PC5-RRC (e.g., SL-SRB3) message.

5.	The L2 U2U Remote UE sends to the L2 U2U Relay UE all the QoS profiles for the end-to-end QoS flows via PC5-RRC.

6.	The L2 U2U Relay UE performs QoS split only for PDB.

NOTE:	It is up to L2 U2U Relay UE implementation on how to split PDB.

7.	The L2 U2U Relay UE sends the split QoS value (i.e., PDB) via PC5-RRC message to the L2 U2U Remote UE.8.	The L2 U2U Remote UE derives the PDCP and SDAP configuration for end-to-end SL-DRB and provides the portion of the configuration related to reception to the peer L2 U2U Remote UE using end-to-end RRCReconfigurationSidelink messages. The end-to-end bearer IDs for SL-SRB and SL-DRB are used as input for the L2 U2U Relay ciphering and deciphering at PDCP.

9a.	The L2 U2U Remote UE derives the first hop configuration (e.g. PC5 Relay RLC Channel configuration) for SL-DRB and provides to the L2 U2U Relay UE of the configuration related to receiving on the first hop (i.e., Rx by the relay UE), using per-hop RRCReconfigurationSidelink message.

9b.	The L2 U2U Relay UE derives the second hop configuration (e.g. PC5 Relay RLC Channel configuration) for each SL-DRB and provides to the peer L2 U2U Remote UE of the configuration related to receiving data packets on the second hop (i.e., RX by the peer remote UE), using per-hop RRCReconfigurationSidelink message.

10.	The L2 U2U Remote UE and the peer L2 U2U Remote UE transmit or receive data via L2 U2U Relay UE.

Next change

#### 16.21.3.1	Path Management

…

Figure 16.21.3.1-3 describes the procedures for the direct path addition on top of the indirect path for the L2 MP Relay operation. This procedure is only applicable to the L2 MP Remote UE using SL indirect path.

Figure 16.21.3.1-3: Procedure for direct path addition on top of indirect path

0.	The L2 MP Remote UE performs data transmission and reception by using indirect path via PC5 link.

1.	The L2 MP Remote UE perform measurements based on measurement configuration. When the measurement reporting is triggered, the L2 MP Remote UE reports at least the PC5 signal strength of the serving relay UE and neighbour cells with the cell IDs (i.e., NCGI/NCI).

2.	The gNB decides to add the direct path for the L2 MP Remote UE. The cell serving the direct path and the cell serving the L2 MP Relay UE on the indirect path belong to the same gNB but can be same or different.

3.	The gNB sends an RRCReconfiguration message to the L2 MP Relay UE to update the indirect path configuration, if necessary.

4.	The gNB sends the RRCReconfiguration message to the L2 MP Remote UE via the L2 MP Relay UE. The contents in the RRCReconfiguration message includes at least a target cell within direct path addition configuration.

5.	The L2 MP Remote UE synchronizes to DL of the target cell serving the direct path and performs random access procedure towards the cell serving the direct path. The L2 MP Remote UE configures the target cell as PCell.

6a.	The L2 MP Remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path addition procedure.

6b.	If a split SRB1 with duplication is configured, the L2 MP Remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the L2 MP Relay UE.

7.	The L2 MP Remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a L2 MP Relay UE.

End of change