3GPP TSG-RAN WG2 Meeting #125	R2-2400468

Athens, Greece, 26 Feb - 1 Mar, 2024

Agenda Item		:	7.4.1.3.1

Source		:	LG Electronics Inc.

Title		:	Discussion on LTM remaining issues

Document for		:	Discussion and Decision

1.	Introduction

The rapporteur summarizes the open issue R18 mobility as follows:

- Issue 1. Coexistence of LTM with other features is addressed during the ASN.1 review or maintenance.

- Issue 2. FFS how the L2 reset (PDCP re-establishment, PDCP recovery, and RLC re-establishment) is indicated by the network in case of subsequent CPAC.

- Issue 3. Objective 7 on EMR.

- Issue 4. Handling of key stream re-used in case of fast LTM recovery.

In this paper, we discuss the issue 1 and other issues on LTM.

2.	Discussion

2.1.	[Issue 1] Coexistence of LTM with conditional reconfiguration

Based on the chair note [1], the following items are noted for feature and procedure coexsitence:

- LTM and Conditional reconfiguration (e.g. CHO, CPAC, S-CPAC)

- LTM and CHO fast recovery race condition

- …

The network may want to configure the UE with CHO/CPAC for inter-CU candidate cells while the network configures the UE with LTM for intra-CU candidate cells. For example, some companies think LTM is for capacity and CHO is for robustness, and consequently there are reasons to support the co-existence. In our view, it seems that RRC signalling with the current specification can support such co-existence. Thus, we propose to support the coexistence of LTM and conditional reconfiguration (e.g. CHO, CPAC, and S-CPAC).

Proposal 1. Support the coexistence of LTM and conditional reconfiguration (e.g., CHO, CPAC, and S-CPAC).

In the LTM-CHO coexistence scenario, both LTM and CHO may be configured for a specific candidate cell. In this case, the UE may select a cell configured with both LTM and CHO for fast recovery when a failure condition (e.g. RLF, handover failure, etc.) occurs, which may lead to LTM and CHO fast recovery race condition. In other words, it is unclear for the UE to perform which procedure (i.e. LTM-based recovery or CHO-based recovery) for fast recovery. Moreover, we do not see any benefit of configuring a candidate cell for both LTM and CHO. Thus, we propose that the network does not simultaneously configure LTM and conditional reconfiguration for a specific candidate cell.

Proposal 2. The network does not simultaneously configure LTM and conditional reconfiguration for a specific candidate cell.

Based on the current RRC specification [2], the UE keeps the stored configuration for CHO when LTM based recovery is performed. In this case, the UE may perform CHO based fast recovery after LTM based recovery is failed. CHO based recovery after the failure of LTM based recovery may lead to increase of data interruption because the configuration for CHO may be invalid upon CHO based recovery.

If the UE performs LTM based failure recovery by executing LTM to the selected cell during the failure recovery, it discards the stored conditional reconfiguration. As a result of the discarding, if LTM based recovery fails (i.e., LTM execution to the selected cell during the recovery is not successful), UE does not re-attempt another failure recovery with CHO configuration but performs legacy re-establishment by initiating cell selection procedure and then transmitting the RRCReestablishmentRequest message to the selected cell.

Proposal 3. The UE removes all the entries within the MCG and the SCG VarConditionalReconfig if LTM based recovery is performed.

Based on Proposal 3, we suggest that RAN2 adopt the text proposal in Annex 5.1.

Proposal 4. If Proposal 3 is agreed, RAN2 adopt the TP in Annex 5.1.

2.2.	[B106] Whether UE-based TA can be used in LTM based fast recovery

Some companies may think UE-based TA can be used for LTM based fast recovery, i.e., during T311 is running the UE performs RACH-less LTM execution with UE-based TA to the selected cell if the selected cell is an LTM candidate cell and UE-based TA of the candidate cell has been obtained. However, in addition to TA information, beam information (i.e. beam indication) is also required for RACH-less LTM cell switch. In the case of LTM cell switch triggered by fast recovery, the UE probably does not have the beam information of the selected cell.

Moreover, the recovery target cell (i.e. the selected cell during T311 is running) does not aware of UE arrival because it is not an LTM cell switch triggered by the network. That is, at the time of failure recovery, the recovery target cell may not reserve the UL resources (i.e. CG or CFRA resource) for the UE to perform fast recovery based on the the corresponding candidate cell configuration.

Concludingly, the failure rate of LTM-based recovery may be higher if RACH-less LTM cell switch is attempted for LTM-based recovery due to any of the followings: DG failure due to, e.g., use of a wrong beam for PDCCH monitoring or target cell not providing DG, CG failure due to network not listening to the CG or use of a wrong beam to send/received the CG. Even for CFRA-based LTM cell switch for LTM-based recovery, the recovery may fail because CFRA resource is not timely reserved at the recovery target cell at the moment of failure recovery. Thus, CBRA-based LTM cell switch is safe for LTM-based recovery.

Proposal 5. UE performs CBRA-based LTM cell switch for LTM-based fast recovery regardless of whether UE-based TA is obtained or not.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Proposal 1. Support the coexistence of LTM and conditional reconfiguration (e.g., CHO, CPAC, and S-CPAC).

Proposal 2. The network does not simultaneously configure LTM and conditional reconfiguration for a specific candidate cell.

Proposal 3. The UE removes all the entries within the MCG and the SCG VarConditionalReconfig if LTM based recovery is performed.

Proposal 4. If Proposal 3 is agreed, RAN2 adopt the TP in Annex 5.1.

Proposal 5. UE performs CBRA-based LTM cell switch for LTM-based fast recovery regardless of whether UE-based TA is obtained or not.

4.	References

[1] RAN2#124 Report from breakout session on feMob, mIAB, LP-WUS, Session Chair (Mediatek), Johan

[2] 3GPP TS 38.331 V18.0.0 Radio Resource Control (RRC) protocol specification (Release 18)

5.	Annex

5.1.	Text proposal based on [2]

5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA or CPC), or upon execution of an LTM cell switch:

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3; or

1> if the RRCReconfiguration is applied due to a LTM cell switch execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

[…below is skipped…]