3GPP TSG-RAN WG2 Meeting #125	R2-2400490

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.5.3.4

Source:	OPPO

Title:	Open issues on the CG enhancement

Document for:	Discussion, Decision

Introduction

This paper will discuss the open issues on the configured grant enhancement.

Discussion

In the legacy spec, if uplink skipping is enabled and no data is available for a CG PUSCH, the MAC does not generate a MAC PDU if there is no legacy UCI (e.g. HARQ-ACK/CSI) to be multiplexed in that CG PUSCH. In other words, the MAC always needs to generate a dummy TB to multiplex the legacy UCI even if there is no data to transmit.

The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

As another type of UCI, i.e. UTO-UCI is introduced in Rel-18, the issue here, suggested to be handled in RAN2[4], is how to handle the case of UL skipping in combination with UTO-UCI.

Issue#3-2: Uplink skipping Moderator’s summary: OPPO raised an issue related to UL skipping operation in combination with UTO-UCI. Moderator’s observation: During the WI, early on it was commonly understood in RAN1 that UL skipping related issues should be discussed in RAN2. It is not clear why RAN1 should discuss 38.321 specifications and inform RAN2 of changes, where RAN2 is responsible for specification of this feature too. The issue can be raised in RAN2. Moderator’s recommendation: No need to discuss this topic in RAN1.

We believe UTO-UCI should not be an extra factor in judging the MAC PDU generation. Suppose the MAC entity is required to consider UTO-UCI in the UL skipping operation. In that case, no matter whether there is available data or not, the MAC entity needs to generate the MAC PDU for each CG PUSCH occasion as the UTO-UCI is supposed to be multiplexed in all CG PUSCH transmissions. Consequently, the benefit of having UL skipping operation and the UTO-UCI indication is lost. Thus, we would like to keep the first “if” condition in “step 2>” of the UL skipping case being applicable to the legacy UCI only. Otherwise, the MAC spec would also contradict the following conclusions achieved in RAN1.

Agreement - Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112) - FFS details Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Update the MAC spec to explicitly exclude UTO-UCI from the uplink skipping operation, i.e. if uplink skipping is enabled and no data is available for a CG PUSCH configured with UTO-UCI transmission, the MAC does not generate a MAC PDU for this CG PUSCH in the case of no other UCI to be multiplexed on this CG PUSCH.

Conclusion

We have the following proposals:

Proposal 1	Update the MAC spec to explicitly exclude UTO-UCI from the uplink skipping operation, i.e. if uplink skipping is enabled and no data is available for a CG PUSCH configured with UTO-UCI transmission, the MAC does not generate a MAC PDU for this CG PUSCH in the case of no other UCI to be multiplexed on this CG PUSCH.

Text Proposal

##### 5.4.3.1.3	Allocation of resources

[omitted]

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI other than UTO-UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

[omitted]

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] R1-2312386 Moderator Summary#2 – Maintenance of XR Enhancements