3GPP TSG-RAN WG2 #125	R2-2400514

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	7.15.4

Source:	Ericsson

Title:	Discussion on issues for 38.300

Document for:	Discussion, Decision

# 1	Introduction

We discuss some open MAC issues and propose corresponding corrections in this paper.

# 2	Discussion

## 2.1 Issue 1 – MCSt and LCP

#### In clause 16.9.9.5, resource allocation for SL-U is described.

#### 16.9.9.5	Resource Allocation

A UE using mode 2 resource allocation may support resource selection enhancements to avoid LBT blocking and increase COT resource utilization. If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation, the UE may be (pre)configured to prioritize/select resource(s) in the slot(s) for transmission. To avoid blocking, a UE may avoid selection of a number of consecutive resources (up to UE implementation) before a reserved resource. Furthermore, a UE may avoid selection of a number of consecutive resources (up to UE implementation) after a reserved resource.

A UE using mode 2 resource allocation supports resource selection for multiple consecutive slot transmission (MCSt). A UE autonomously determines whether to use MCSt, and the number of consecutive slots in an MCSt up to the maximum COT duration for a specific SL-CAPC as defined in TS 37.213 [37]. MCSt can be used for transmission of a single TB or multiple TBs. For each TB transmitted in an MCSt, the UE triggers resource (re)selection only when LBT failure is detected on the resources for the initial transmission and all retransmissions of the TB. When performing LCP procedure for transmission of multiple TBs in an MCSt, a UE may perform an enhanced LCP procedure as specified in TS 38.321 [6] based on the SL-CAPC used for LBT of the first TB.

For the above highlighted text, it needs to be deleted since RAN2 has agreed to withdraw the previous RAN2 agreement

- RAN2 to withdraw below RAN2 agreement (For the subsequent slots in MCSt, LCP procedure for COT initiating UE is enhanced: the LCHs with lower or equal CAPC than the CAPC value used for LBT check for the first TB.).

Therefore, we suggest that

To remote texts on application of the enhanced LCP procedure for MCSt.

Based on the above proposal, we have also prepared the text proposals.

Adopt the text proposal captured in the Annex.

# 3 Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To remote texts on application of the enhanced LCP procedure for MCSt.

Proposal 2	Adopt the text proposal captured in the Annex.

# 4 Annex – text proposals

## 4.1 Changes to TS 38.300 Rel-18

< changes to Issue 1>

#### 16.9.9.5	Resource Allocation

A UE using mode 2 resource allocation may support resource selection enhancements to avoid LBT blocking and increase COT resource utilization. If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation, the UE may be (pre)configured to prioritize/select resource(s) in the slot(s) for transmission. To avoid blocking, a UE may avoid selection of a number of consecutive resources (up to UE implementation) before a reserved resource. Furthermore, a UE may avoid selection of a number of consecutive resources (up to UE implementation) after a reserved resource.

A UE using mode 2 resource allocation supports resource selection for multiple consecutive slot transmission (MCSt). A UE autonomously determines whether to use MCSt, and the number of consecutive slots in an MCSt up to the maximum COT duration for a specific SL-CAPC as defined in TS 37.213 [37]. MCSt can be used for transmission of a single TB or multiple TBs. For each TB transmitted in an MCSt, the UE triggers resource (re)selection only when LBT failure is detected on the resources for the initial transmission and all retransmissions of the TB.