3GPP TSG-RAN WG2 Meeting #125	R2-2400587

Athens, Greece, 26th February – 1st March, 2024

Agenda Item:	7.7.5

Source:	ETRI

Title:		Discussion on the measurement rules for cell re-selection

Document for:	Discussion and Decision

1	Introduction

In R18 NR NTN, the location-based measurement initiation for quasi-Earth-fixed system was reused for Earth-moving system [1]. However, the moving nature of the Earth-moving system was not fully considered in the reuse.

In this paper, we discuss unnecessary measurement initiation in Earth-moving system and propose TP for the measurement rules for cell re-selection in 38.304.

2	Discussion

2.1	Unnecessary measurement initiation in Earth-moving system

According to the current R18 spec. [2] for NTN Earth-moving system, the measurement for cell re-selection is required for the UEs whose distances from the serving cell reference location are greater than distanceThresh, even if they are in good signal conditions.

However, not all the UEs whose distances from the serving cell reference location are greater than distanceThresh require the measurement.

Figure 1. The location-based measurement initiations in an Earth-moving system by the current spec. (left) and the proposed TP (right).

The UEs that are in good signal conditions and getting closer to the serving cell reference location as the satellite moves may choose not to perform the measurement, even if their distances from the serving cell reference location are greater than distanceThresh.

This is justified by the fact that the downlink signal strength becomes stronger as the UEs are getting closer to the serving cell reference location due to the movement of the satellite and thus re-selections are less likely to be needed.

In NTN Earth-moving system, the UEs that are in good signal conditions and getting closer to the serving cell reference location as the satellite moves may choose not to perform the measurement, even if their distances from the serving cell reference location are greater than distanceThresh.

By allowing not to perform the measurement, more power saving can be achieved for the UEs.

2.2	TP for 38.304

From Observation 1, we propose the TP for 38.304 [2] as follows:

#### 5.2.4.2 Measurement rules for cell re-selection Following rules are used by the UE to limit needed measurements: - If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements; - else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information: - if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh : or, - if the distance between UE's location and the serving cell reference location derived based on movingReferenceLocation is expected to decrease: - the UE may not perform intra-frequency measurements; - else, the UE shall perform intra-frequency measurements; - Else, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements. - The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1: - For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8]. - For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency: - If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its UE location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8]; - else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information: - if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh : or, - if the distance between UE's location and the serving cell reference location derived based on movingReferenceLocation is expected to decrease: - the UE may not perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8]; - Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8]. - If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9. - For UE camping on NTN cell, if the UE supports skipping TN measurement, and the UE has obtained its location information, and if coverageAreaInfoList and tn-AreaIdList are broadcast in system information, the UE may not perform measurements of a TN frequency when UE is not in the coverage of that frequency provided via tn-AreaIdList, regardless of the frequency priority. If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service). NOTE 1: When evaluating the distance between UE and the serving cell reference location, it's up to UE implementation to obtain UE location information. NOTE 2: In the Earth-moving system, it's up to UE implementation to maintain a valid serving cell reference location, which is derived based on the serving satellite ephemeris, epochTime and movingReferenceLocation.

Adopt the TP for TS 38.304

3	Conclusions

In this paper, the following observation is made:

Observation 1	In NTN Earth-moving system, the UEs that are in good signal conditions and getting closer to the serving cell reference location as the satellite moves may choose not to perform the measurement, even if their distances from the serving cell reference location are greater than distanceThresh.

In this paper, we propose the following:

Proposal 1	Adapt the TP for TS 38.304

References

R2-2313774, Agreed 38.304 CR for R18 NR NTN, Introduction of NR NTN enhancements in 38.304, ZTE Corporation, Sanechips, RAN2 #124.

TS 38.304, v18.0.0 (2023-12).