3GPP TSG-RAN WG2 #125	Tdoc R2-2400589

Athens, February-March 2024

Agenda Item: 7.8.2

Source:	Ericsson

Title: H746 H747 J064 E048 J061 E144

Document for:	Discussion, Decision

# H746 H747 Height-dependent configuration for reporting measurements

In RAN2#121bis, the following agreements were made:

Agreements

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

In RAN2#122, the following agreements were made for enhancements to UAV measurement reports:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

Based on current running RRC CR, the height dependent numberoftriggeringcells is not implemented such that there would be height range in which UE applies certain numberOfTriggeringCells. When UE is configured with an event which is a combination of RSRP threshold and H threshold, it is true that that event may have certain numberOfTriggeringCells configured, and UE may be configured with another similar event but another height threshold and numberOfTriggeringCells. Therefore, this way of implementing the height-dependent numberoftriggeringcelss increases network complexity but also consumes multiple MeasIds and hence limits the possibility to configure other type of measurements for the UE.

The current implementation of height-dependent numberoftriggeringcells in RRC spec consumes multiple MeasIds and hence limits the possibility to configure other type of measurements for the UE.

- For height dependent numberoftriggeringcells, RAN2 to implement the agreement such that in certain height range, there can be a certain configured numberOfTriggeringCells.

# J064 Combined event

Sharp raised an issue that UE’s behaviour for leaving condition for the combined event is not captured:

[RIL]: J064 [Delegate]: Sharp (LIU Lei) [WI]: UAV [Class]: 1 [Status]: ToDo [TDoc]: None [Proposed Conclusion]: v030

[Description]: UE behaviour is missing for event AxHy when only H leaving condition is fulfilled.

[Proposed Change]: For event AxHy, when only H leaving condition is fulfilled, the leaving condition applicable for the event is not related to cells and does not need measurement or layer 3 filtering. The description in this bullet and following sub-bullets is not applicable for this case.

Suggest to add description in the following pharagraph as below:

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventH1 or eventH2 or eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, eventA5H2 and if the leaving condition applicable for this event is fulfilled for the associated VarMeasReport within the VarMeasReportList for this measId during timeToTrigger defined within the VarMeasConfig for this event and if the leaving condition applicable for this event is fulfilled only due to Aerial UE’s altitude change for eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, and eventA5H2:

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

In our view, a combined event has entering condition RSRP and H, and leaving condition RSRP or H. Since entering condition is per cell, it seems the event is per cell and even the possible leaving of height H is per UE and not per cell, the event itself is per cell due to the entering condition.

Further, since the leaving condition is already defined in two parts with or operation, there is no need to try to capture the following in the procedural part “and if the leaving condition applicable for this event is fulfilled only due to Aerial UE’s altitude change for eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, and eventA5H2:”

- For J064 we agree with the first change but not with the second change since leaving condition is defined RSRP or H related.

# E048 J061 additionalPmax

In E048 and J061 aspects for the below procedural text is raised:

3>	else:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UEs, (e)RedCap-specific initial uplink BWP, if configured, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink or, for a (e)RedCap UE, of the (e)RedCap-specific initial uplink BWP if configured;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UEs, (e)RedCap-specific initial downlink BWP, if configured, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a (e)RedCap UE, of the (e)RedCap-specific initial downlink BWP if configured;

4>	if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAeria, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxListAerial, if present:

5>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxListAerial;

4>	else:

5>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for RedCap UEs if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

[RIL]: E048 [Delegate]: Ericsson (Helka-Liina) [WI]: UAV [Class]:1 [Status]: PropReject [TDoc]: [Proposed Conclusion]: v188

[Description]: the condition is specific to the defined frequency bands in that said list, the additional condition on aerial UE is redundant and misleading. UE needs to do the following procedures if certain Freq is supported.

This is in multiple places: 5.2.2.4.2, 5.2.2.4.3, 5.2.2.4.5

[Proposed Change]: Remove "if the UE ia aerial UE"

[Comments]: [QC v188 as CR rapp] There is no guarantee that frequencyBandListAerial cannot include a frequency used for terrestrial (non-Aerial) UE. In terms of just supporting "one additionalSpectrumEmission values in nr-NS-PmaxListAerial" this could be true for non-Aerial UE also, however "7> apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxListAerial within frequencyBandListAerial;" should only be done by an Aerial UE. Therefore suggesting PropReject.

For yellow part, our understanding is that it is not about what frequency, but about which list the frequency is included in. That is if, there is freq that can be used also by non-aerial UEs that frequency would also be in the non-aerial list frequency for those UEs to use. And non-aerial UE should not check an aerial list for that list possibly having a frequency it supports.

For blue part, our understanding is that it is not possible to nonaerial UE to support values in aerial list.

For pink part, our understanding is that if UE supports a frequency in this list frequencyBandListAerial we can remove the “is aerial UE.”

As a summary, in our view the list frequencyBandListAerial is defined such that only aerial UEs will use and attempt to read the list and therefore the grey highlighted part of the procedural text is not needed. It actually implies that non-aerial UEs would use the list as well.

The list frequencyBandListAerial is defined such that only aerial UEs will use and attempt to read the list.

- Clarify in 38.331 that frequencyBandListAerial is only for aerial UEs and Agree RIL E048.

Then, Sharp raises a good point that if the aerial UE NS value is not present the UE should use the other NS value.

[RIL]: J061 [Delegate]: Sharp (LIU Lei) [WI]: UAV [Class]: 1 [Status]: ToDo [TDoc]: None [Proposed Conclusion]: v030

[Description]: Selection of the first frequency band in frequencyBandList is incorrect when nr-NS-PmaxListAerial is absent.

[Proposed Change]: Based on field description, if nr-NS-PmaxListAerial is absent, nr-NS-PmaxList is used for the corresponding NR frequency band number. In this case, UE should still select the first frequency band in frequencyBandListAerial not in frequencyBandList.

Suggest to add the following description before this “else” to cover nr-NS-PmaxListAerial absent case:

4>	else if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present:

5>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList;

[Comments]:

- Agree RIL J061

Finally, if network does broadcast Aerial NS values, but the aerial UE does not understand or does not support the NS values, the UE should barr the call.

- If a UE, after reading the frequencyBandListAerial and finding aerial NS values, finds that it does not support those values, the UE should consider the cell barred.

One example to implement the above proposals:

*************************start TP*************************************

3>	else:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UEs, (e)RedCap-specific initial uplink BWP, if configured, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink or, for a (e)RedCap UE, of the (e)RedCap-specific initial uplink BWP if configured;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UEs, (e)RedCap-specific initial downlink BWP, if configured, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a (e)RedCap UE, of the (e)RedCap-specific initial downlink BWP if configured;

4>	if the UE supports at least one frequency band in the frequencyBandListAeria, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink,

5>If for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxListAerial, if present:

6>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxListAerial;

5>elseif which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present::

6>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList;

5>else:

6> UE considers the cell barred

4>	else:

5>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for RedCap UEs if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

*************************end TP*************************************

# E144 UAV capabilities

In earlier RAN2 discussions, there have been some proposals about defining an Aerial UE capability which the UE can use to indicate that it is an aerial UE, as follows:

| aerialUE-Capability-r18 Indicates whether the UE supports aerial UE enhancements. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| altitudeMeas-r18 Indicates whether the UE supports altitude based measurement reporting as specified in TS 38.331 [9]. It is mandatory if the UE supports aerialUE-Capability-r18. | UE | CY | No | No |
| multipleCellsMeasExtension-r18 Indicates whether the UE supports measurement reporting triggered based on a number of cells for eventA3, eventA4, and eventA5 as specified in TS 38.331 [9]. It is mandatory if the UE supports aerialUE-Capability-r18. | UE | CY | No | No |

RAN4 definition of an Aerial UE:

Aerial UE: A UE supporting UAS (Uncrewed Aircraft Systems) as indicated by the capability [NR-UAV-support] [15] and that has an aerial subscription as described in TS 23.256 [17]. The UE is considered to have access to UAS services after the UE ha performed a successful authentication and authorization with the USS as described in TS 23.256 [17].

Further, from the definition of the generic aerial UE capability cited above, it remains unclear what the aerial UE enhancements consist of. The network does not obtain any information upon only receiving the aerialUE-Capability-r18. If this is used to indicate that the UE has an aerial UE subscription, then this is not correct as described in the RAN4 definition above, the UE is considered to have an aerial subscription only after successful authentication and authorization procedure i.e., subscription information cannot be inferred from access stratum signaling. As it cannot be tied to the subscription, there is no further information that can be deduced based on this capability.

Furthermore, the definition of altitudeMeas-r18 and multipleCellsMeasExtension-r18 are conditional mandatory on this standalone capability. However, the standalone capability itself is optional hence, there is no clear directive on which field should be included in the AerialParameters-r18 IE to represent the capability of an “Aerial UE” i.e., as the standalone capability is optional, the UE can send an empty list of AerialParameteres-r18 to the network. This should be avoided.

As a result, our preferred option would be to remove this standalone aerial UE capability.

- RAN2 to remove the standalone aerial UE capability to indicate whether a UE is an aerial UE.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The current implementation of height-dependent numberoftriggeringcells in RRC spec consumes multiple MeasIds and hence limits the possibility to configure other type of measurements for the UE.

Observation 2	The list frequencyBandListAerial is defined such that only aerial UEs will use and attempt to read the list.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For height dependent numberoftriggeringcells, RAN2 to implement the agreement such that in certain height range, there can be a certain configured numberOfTriggeringCells.

Sharp raised an issue that UE’s behaviour for leaving condition for the combined event is not captured:

Proposal 2	For J064 we agree with the first change but not with the second change since leaving condition is defined RSRP or H related.

Proposal 3	Clarify in 38.331 that frequencyBandListAerial is only for aerial UEs and Agree RIL E048.

Proposal 4	Agree RIL J061

Proposal 5	If a UE, after reading the frequencyBandListAerial and finding aerial NS values, finds that it does not support those values, the UE should consider the cell barred.

Proposal 6	RAN2 to remove the standalone aerial UE capability to indicate whether a UE is an aerial UE.