3GPP TSG-RAN WG2 Meeting #125                                         R2-2400677

Athens, Greece, Feb 26th – Mar 1st, 2024

Source:         ZTE Corporation

Title:           Discussion on LSs of LMF involvement in SL positioning

Agenda item:    7.2.1

Document for:	Discussion and Decision

- Introduction

This discussion paper discusses the LMF involvement in SL positioning, triggered by the LSs from RAN1 and RAN3.

- Discussion

In RAN2#123[1] meeting, the following agreement is made:

Agreement: When aperiodic/one-shot SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, design a new MAC CE for the UE to send to the gNB for SL-PRS resource request. (12/14) FFS when LMF is involved. At least when periodic SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, the UE sends an RRC message to the gNB for providing the assistance information for CG configuration. (13/14) FFS when the LMF is involved.

UE can provide the assistance information for CG configuration only when the UE knows the QoS of the sidelink positioning session, since the assistance information contains the approximate volume of needed SL-PRS transmission to satisfy the QoS. However for SL-MT-LR, the QoS of sidelink positioning is firstly known by the LMF. Further for LMF-based positioning, LMF gathers the measurement and calculates the location estimation. Under this case, it is redundant that LMF firstly tell QoS of the session to UE, then the UE make UL request to gNB for CG configuration. Instead, LMF can directly request gNB for configuring suitable SL-PRS configuration which best satisfies the QoS.

Observation 1: For SL-MT-LR and LMF based SL positioning, it is redundant that LMF firstly tell QoS of the session to UE, then the UE make UL request to gNB for CG configuration. Instead, LMF can directly request gNB for configuring suitable SL-PRS configuration which best satisfies the QoS.

RAN3 has sent the LS[2] to RAN2, asking whether LMF should be involved in the SL-PRS resource allocation:

# 1 Overall description RAN3 discussed the signalling design in RAN3 on SL-PRS resource allocation for scheme 1. RAN3 would like to ask RAN2 whether LMF is involved in the SL-PRS resource allocation, and if yes, whether RAN2 expects NRPPa impacts. # 2 Actions To RAN2 ACTION: RAN3 kindly asks RAN2 to provide feedback on the above questions to RAN3.

RAN1 also sends the LS[3] to RAN2, indicating that both LMF and UE should indicate some of the SL PRS parameters to the gNB:

# 1. Overall Description: The following agreement was reached in RAN1 related to the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration for scheme 1 Agreement Send an LS to RAN2 and RAN3 with the following: - From RAN1 perspective, for scheme 1, it is important for the following request to be specified: - a gNB is able to receive a request from either LMF or UE for SL-PRS bandwidth - Action to RAN2 and RAN3 to consider how to specify support for such request, if not already specified.. # 2. Actions To RAN2 and RAN3: RAN1 respectfully requests RAN2 and RAN3 to consider how to specify support for such request, if not already specified.

These two LSs further prove that LMF recommend gNB is feasible and should be considered.

SL-PRS configured per Tx pool, i.e. dedicated pool or shared pool. The dedicated pool SL-PRS configuration and shared pool SL-PRS configuration are given by RAN1 in the following agreements:

Agreement For a dedicated resource pool, explicit (pre-)configuration of SL PRS resources in a slot includes: - SL PRS Resource ID, (M, N) pattern, starting symbol, comb offset. - FFS: constraints to the (pre-)configuration to address potential AGC issues Conclusion For a dedicated resource pool, only the case where SL PRS bandwidth is the same as resource pool bandwidth is supported in Rel-18. Working assumption For a shared resource pool, - Explicit (pre-)configuration of SL PRS resources in a slot, applicable for an indicated frequency domain allocation, includes: - SL PRS Resource ID, (M, N) pattern, comb offset. Agreement For Scheme 2, in a dedicated resource pool, - with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point. Agreement In resource allocation in scheme 1, for a dedicated resource pool: - For configured grant type 1 resource allocation, - RRC is used for indicating at least the following: - Info-1: the periodicity,

The SL-PRS characteristic that may impact the positioning accuracy towards a given QoS are: SL-PRS transmission periodicity(resource reservation period), SL-PRS resource symbol number and SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number. LMF can recommend these information to gNB, to help gNB make better scheduling when configuring the Tx pool and associated SL-PRS configuration to UE.

Proposal 1: Support LMF to recommend SL-PRS configuration to the gNB for resource allocation, the SL-PRS configuration includes SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number.

RAN1#114[4] also reaches the following agreements:

Agreement Support to include SL PRS resource ID in sidelink positioning measurement report. Note: RAN1 will not further discuss how LMF/UE could use reported resource ID.

According to the agreement, LMF will know the SL PRS resource ID in the measurement report. However, it is useless if LMF only knows the SL PRS resource ID. LMF should also know the SL-PRS configuration/characteristic of this SL-PRS resource ID. To achieve this, we think it is natural to allow gNB to report the SL-PRS configuration (associated with the pool configuration) to the LMF in advance.

The SL-PRS configuration can include at least one of SL-PRS resource ID, SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number.

Proposal 2: Allow gNB to report the SL-PRS configuration to the LMF.

- The SL-PRS configuration can at least include SL-PRS resource ID, SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number

So a reply to RAN3 can include these two aspects, i.e., LMF’s recommendation to gNB, and gNB’s report to LMF. A draft response to RAN3 is given in Annex.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: For SL-MT-LR and LMF based SL positioning, it is redundant that LMF firstly tell QoS of the session to UE, then the UE make UL request to gNB for CG configuration. Instead, LMF can directly request gNB for configuring suitable SL-PRS configuration which best satisfies the QoS.

Proposal 1: Support LMF to recommend SL-PRS configuration to the gNB for resource allocation, the SL-PRS configuration includes SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number.

Proposal 2: Allow gNB to report the SL-PRS configuration to the LMF.

- The SL-PRS configuration can at least include SL-PRS resource ID, SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number

- References

- Report of 3GPP TSG RAN WG2 meeting #123, Toulouse, France, 2023-08

- R3-237860, LS on LMF involvement in SL-PRS resource allocation, RAN3, LS in, 2023-11

- R1-2312630, LS on the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration, RAN1, LS in, 2023-11

- Annex

3GPP TSG-RAN2 Meeting #125	R2-24xxxxx

# Athens, Greece, Feb 26 – Mar 1, 2024

Title:	Reply LS on LMF involvement in SL-PRS resource allocation

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN2

To:	RAN3

Cc:	RAN1, SA2

Contact person:	Yu Pan

pan.yu24@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- 1	Overall description

RAN2 thanks RAN3’s LS and discussed the necessary signalling design that have NRPPa impact on SL-PRS resource allocation for scheme 1:

- Support LMF to recommend SL-PRS configuration to the gNB for resource allocation, the SL-PRS configuration includes SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number.

- Support gNB to report the SL-PRS configuration to the LMF. The SL-PRS configuration includes SL-PRS resource ID, SL-PRS transmission periodicity, SL-PRS resource symbol number, SL-PRS comb size (M,N pattern), SL-PRS bandwidth, SL-PRS retransmission number.

- 2	Actions

To RAN3

ACTION: 	RAN2 kindly asks RAN3 to take the above into account and design corresponding NRPPa signaling to support SL-PRS resource allocation for scheme 1.

- 3	Dates of next RAN2 meetings

RAN2 Meeting #125-bis        		15 Apr -20 Apr 2024	 	Changsha, CN

RAN3 Meeting #126                20 May -25 May 2024		Fukuoka City, JP