3GPP TSG-RAN WG2 Meeting #125	R2-2400792

Athens, Greece, Feb 26- March 1, 2024

Agenda item:	    7.3.3

Source: 		Qualcomm Incorporated

Title: 	Control Plane Open Issues for NES

Document for:	Discussion

# Introduction

In RAN2 #124, the Rel-18 NES WI was closed from a RAN2 POV. The Control Plane rapporteur have identified the following open issues for further handling in the maintenance phase:

Issue 1: Relation of NES cell barring with cell DTX/DRX UE capability. RAN1 designed the cell DTX/DRX UE capability as 3 options: {cell DTX only, cell DRX only, both}; this implementation does not work well with 1-bit NES cell DTX/DRX barring we have in RAN2. The relation of NES cell barring with cell DTX/DRX UE capability needs to be resolved.

In this contribution we provide some views on the current open control plane issues

# Which UEs are allowed to bypass MIB Cell Barring

RAN2 has agreed that UEs with cell DTX/DRX capability would bypass the cellBarred field in MIB if set to {barred} to obtain possible NES-specific barring information if the cell is using this barring field to support Rel-18 Cell DTX/DRX which is not compatible with legacy. However, as identified by the WI rapporteur above, RAN1 has introduced three options for capability: {cell DTX only, cell DRX only, both}.

Given the reason Cell DTX/DRX may not be compatible with legacy are related to L1 group activation and given that the cell DRX aspects of the system (Cell DRX: Do not transmit SR, CG or trigger SR-RACH, P/SP SRS (except positioning), P/SP CSI report) are all UE-specific, the NW can hide the cell DRX aspects from a UE with {cell DTX only capability}.

Furthermore, since RAN1 has decided to breakdown this capability on DTX/DRX/Both granularities, we think if cell barring bypass was to only apply for UE that support both cell DTX/DRX mandatorily, the {cell DTX only} capability would be undermines as there would be no incentive to support it, which is clearly not RAN1 intention.

Observation 1:  A UE that supports {Cell DTX only} can operate on a cell applying both cell DTX and cell DRX by proper NW configuration of UL resources.

Observation 2: RAN1 intention of breaking down the UE capability granularity to {cell DTX only, cell DRX only, both} if support for both cell DTX and cell DRX were mandatory to access a NES-cell.

Proposal 1: UEs with Cell DTX/DRX capability of “Cell DTX only” or “Both” can access a NES cell, i.e., bypass the cellBarred field in MIB if set to {barred} to obtain the NES cell barring information from SIB1, as outlined in 38.304.

# Capturing Acquiring MIB clause

The current ASN.1 of clause 5.2.2.4.1 (Actions upon reception on the MIB) is captured as follows:

1> if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running: 2> if the access is not for NTN or the UE is not capable of NTN or the UE is not capable of NES cell DTX/DRX; and 2> if the access is not for ATG or the UE is not capable of ATG; and 2> if the cellBarred in the acquired MIB is set to barred: 3> if the UE is an (e)RedCap UE and ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]): 4> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

As outlined in RIL List shared by rapporteur [1] before the meeting issue [J60] have pointed out a problem in this text. The problem with the current capture is that “or” should be an “and”, so as not to make the NTN condition useless. Furthermore, given the following agreement from last meeting [2]:

Agreements: - RAN2 will not optimize or study NES capable UE and NTN

We have a slight preference for breaking down this condition for readability to capture the behaviour like this:

1> if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running: 2> if the access is not for NTN or the UE is not capable of NTN; and 2> if the UE is not capable of NES cell DTX/DRX: 3> if the access is not for ATG or the UE is not capable of ATG; and 3> if the cellBarred in the acquired MIB is set to barred: 4> if the UE is an (e)RedCap UE and ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]): 5> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];\ 2> else: 3> apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

Proposal 2: RAN2 to discuss how best to capture the actions of the UE acquiring MIB with respect to NES and NTN special behaviours.

# Conclusion

Observation 1:  A UE that supports {Cell DTX only} can operate on a cell applying both cell DTX and cell DRX by proper NW configuration of UL resources.

Observation 2: RAN1 intention of breaking down the UE capability granularity to {cell DTX only, cell DRX only, both} if support for both cell DTX and cell DRX were mandatory to access a NES-cell.

Proposal 1: UEs with Cell DTX/DRX capability of “Cell DTX only” or “Both” can access a NES cell, i.e., bypass the cellBarred field in MIB if set to {barred} to obtain the NES cell barring information from SIB1, as outlined in 38.304.

Proposal 2: RAN2 to discuss how best to capture the actions of the UE acquiring MIB with respect to NES and NTN special behaviours.

# References

- RIL List-v177 NES v00 rapp.xlsx

- RAN2 #124 Chair notes