3GPP TSG-RAN2 Meeting #125	R2-2400911

# Athens, Greece, 26th Feb 2024 – 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4568 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Latency bound requirement of NR SL CSI report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is mirror CR of R2-2400794. If sl-LatencyBoundCSI-Report is configured with a value smaller than minimum UE’s processing time (TSLproc, 1) defined in TS 38.214, the SL-CSI report will always be cancelled based on MAC spec, section 5.22.1.7. 2> if the sl-CSI-ReportTimer for the triggered SL-CSI reporting expires: 3> cancel the triggered SL-CSI reporting. For example: when µSL is 1, TSLproc, 1 is 5 slots. If sl-LatencyBoundCSI-Report is configured with 3, which is smaller than minimum physical layer processing time TSLproc, 1. From MAC layer point of view, the sl-CSI-ReportTimer will always expires and cancel the SL-CSI report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference to RAN1 spec (TS 38.214) in field description of sl-LatencyBoundCSI-Report. A UE is not configured with a value smaller than minimum UE’s processing time. Impact analysis Impacted functionality: NR SL CSI reporting feature. Inter-operability: If the UE is implemented according to this CR and the network is not, the UE may be configured with unexpected NR SL CSI reporting parameters. If the network is implemented according to this CR, and the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR and the other UE are not, the UE may be configured with unexpected NR SL CSI reporting parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR SL CSI reporting feature might become useless under improper network configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2 – RRCReconfigurationSidelink |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.6.2	Message definitions

#### –	RRCReconfigurationSidelink

The RRCReconfigurationSidelink message is the command to AS configuration of the PC5 RRC connection. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

RRCReconfigurationSidelink message

-- ASN1START

-- TAG-RRCRECONFIGURATIONSIDELINK-START

RRCReconfigurationSidelink ::=          SEQUENCE {

rrc-TransactionIdentifier-r16           RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfigurationSidelink-r16          RRCReconfigurationSidelink-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfigurationSidelink-r16-IEs ::=  SEQUENCE {

slrb-ConfigToAddModList-r16             SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16             OPTIONAL, -- Need N

slrb-ConfigToReleaseList-r16            SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-PC5-ConfigIndex-r16    OPTIONAL, -- Need N

sl-MeasConfig-r16                       SetupRelease {SL-MeasConfig-r16}                                    OPTIONAL, -- Need M

sl-CSI-RS-Config-r16                    SetupRelease {SL-CSI-RS-Config-r16}                                 OPTIONAL, -- Need M

sl-ResetConfig-r16                      ENUMERATED {true}                                                   OPTIONAL, -- Need N

sl-LatencyBoundCSI-Report-r16           INTEGER (3..160)                                                    OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCReconfigurationSidelink-v1700-IEs                                OPTIONAL

}

RRCReconfigurationSidelink-v1700-IEs ::= SEQUENCE {

sl-DRX-ConfigUC-PC5-r17                 SetupRelease { SL-DRX-ConfigUC-r17 }                                OPTIONAL, -- Need M

sl-LatencyBoundIUC-Report-r17           SetupRelease { SL-LatencyBoundIUC-Report-r17 }                      OPTIONAL, -- Need M

sl-RLC-ChannelToReleaseListPC5-r17      SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17         OPTIONAL, -- Need N

sl-RLC-ChannelToAddModListPC5-r17       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfigPC5-r17  OPTIONAL, -- Need N

nonCriticalExtension                    RRCReconfigurationSidelink-v1800-IEs                                OPTIONAL

}

RRCReconfigurationSidelink-v1800-IEs ::= SEQUENCE {

sl-SFN-DFN-Offset-r18                   SetupRelease { SL-SFN-DFN-Offset-r18 }                                OPTIONAL, -- Need M

sl-CarrierToAddModList-r18              SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierConfig-r18      OPTIONAL, -- Need N

sl-CarrierToReleaseList-r18             SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierId-r18          OPTIONAL, -- Need N

sl-RLC-BearerToAddModList-r18           SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfig-r18        OPTIONAL, -- Need N

sl-RLC-BearerToReleaseList-r18          SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfigIndex-r18   OPTIONAL, -- Need N

sl-LocalID-PairToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N

sl-LocalID-PairToAddModList-r18         SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SRAP-ConfigPC5-r18      OPTIONAL, -- Need N

nonCriticalExtension                    SEQUENCE {}                                                           OPTIONAL

}

SL-CarrierConfig-r18 ::= SEQUENCE {

sl-Carrier-Id-r18                       SL-CarrierId-r18,

sl-OffsetToCarrier-r18                  INTEGER (0..2199),

sl-AbsoluteFrequencyPointA-r18          ARFCN-ValueNR                                                       OPTIONAL  -- Need R

}

SL-CarrierId-r18 ::=                    INTEGER (0..maxNrofFreqSL-1-r18)

SL-RLC-BearerConfig-r18 ::=         CHOICE {

srb                                 SEQUENCE {

sl-SRB-IdentityWithDuplication      INTEGER (1..3),

sL-RLC-BearerConfigIndex-r18        SL-RLC-BearerConfigIndex-r18,

...

},

drb                                 SEQUENCE {

slrb-PC5-ConfigIndex-r18            SLRB-PC5-ConfigIndex-r16,

sL-RLC-BearerConfigIndex-r18        SL-RLC-BearerConfigIndex-r18,

sl-RLC-ConfigPC5-r18                SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r18  SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

}

SL-RLC-BearerConfigIndex-r18 ::=        INTEGER (1..maxSL-LCID-r16)

SL-LatencyBoundIUC-Report-r17::=            INTEGER (3..160)

SLRB-Config-r16::=                      SEQUENCE {

slrb-PC5-ConfigIndex-r16                SLRB-PC5-ConfigIndex-r16,

sl-SDAP-ConfigPC5-r16                   SL-SDAP-ConfigPC5-r16                                               OPTIONAL, -- Need M

sl-PDCP-ConfigPC5-r16                   SL-PDCP-ConfigPC5-r16                                               OPTIONAL, -- Need M

sl-RLC-ConfigPC5-r16                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r16      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

SLRB-PC5-ConfigIndex-r16 ::=            INTEGER (1..maxNrofSLRB-r16)

SL-SDAP-ConfigPC5-r16 ::=               SEQUENCE {

sl-MappedQoS-FlowsToAddList-r16         SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16      OPTIONAL, -- Need N

sl-MappedQoS-FlowsToReleaseList-r16     SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16      OPTIONAL, -- Need N

sl-SDAP-Header-r16                      ENUMERATED {present, absent},

...

}

SL-PDCP-ConfigPC5-r16 ::=               SEQUENCE {

sl-PDCP-SN-Size-r16                     ENUMERATED {len12bits, len18bits}                                   OPTIONAL, -- Need M

sl-OutOfOrderDelivery-r16               ENUMERATED { true }                                                 OPTIONAL,  -- Need R

...

}

SL-RLC-ConfigPC5-r16 ::=                CHOICE {

sl-AM-RLC-r16                           SEQUENCE {

sl-SN-FieldLengthAM-r16                 SN-FieldLengthAM                                                OPTIONAL, -- Need M

...

},

sl-UM-Bi-Directional-RLC-r16            SEQUENCE {

sl-SN-FieldLengthUM-r16                 SN-FieldLengthUM                                                OPTIONAL, -- Need M

...

},

sl-UM-Uni-Directional-RLC-r16           SEQUENCE {

sl-SN-FieldLengthUM-r16                 SN-FieldLengthUM                                                OPTIONAL, -- Need M

...

}

}

SL-LogicalChannelConfigPC5-r16 ::=      SEQUENCE {

sl-LogicalChannelIdentity-r16           LogicalChannelIdentity,

...,

[[

sl-LogicalChannelIdentity-v1800         INTEGER (33..38)

]]

}

SL-PQFI-r16 ::=                         INTEGER (1..64)

SL-CSI-RS-Config-r16 ::=                SEQUENCE {

sl-CSI-RS-FreqAllocation-r16            CHOICE {

sl-OneAntennaPort-r16                   BIT STRING (SIZE (12)),

sl-TwoAntennaPort-r16                   BIT STRING (SIZE (6))

}                                                                                                           OPTIONAL, -- Need M

sl-CSI-RS-FirstSymbol-r16               INTEGER (3..12)                                                     OPTIONAL, -- Need M

...

}

SL-RLC-ChannelConfigPC5-r17::=          SEQUENCE {

sl-RLC-ChannelID-PC5-r17                SL-RLC-ChannelID-r17,

sl-RLC-ConfigPC5-r17                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r17      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

SL-SFN-DFN-Offset-r18 ::=               SEQUENCE {

sl-FrameOffset-r18                      INTEGER (0..1023),

sl-SubframeOffset-r18                   INTEGER (0..9),

sl-SlotOffset-r18                       INTEGER (0..31)

}

SL-SRAP-ConfigPC5-r18 ::=               SEQUENCE {

sl-PeerRemoteUE-L2Identity-r18          SL-DestinationIdentity-r16                                          OPTIONAL, -- Need M

sl-PeerRemoteUE-LocalIdentity-r18       INTEGER (0..255)                                                    OPTIONAL, -- Need M

sl-RemoteUE-L2Identity-r18              SL-SourceIdentity-r17                                               OPTIONAL, -- Need M

sl-RemoteUE-LocalIdentity-r18           INTEGER (0..255)                                                    OPTIONAL, -- Need M

...

}

-- TAG-RRCRECONFIGURATIONSIDELINK-STOP

-- ASN1STOP

Editor's note: Whether the field sl-AbsoluteFrequencyPointA, together with sl-OffsetToCarrier, is sufficient for Rx UE to understand the carrier to add/modify/release from Rx UE perspective.

RRCReconfigurationSidelink field descriptions

sl-AbsoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A.

sl-CarrierToAddModList Indicate the carrier(s) to be added/modified for transmission by UE transmitting RRCReconfigurationSidelink message

sl-CarrierToReleaseList Indicate the carrier(s) to be released for the transmission by UE transmitting RRCReconfigurationSidelink message.

sl-CSI-RS-FreqAllocation Indicates the frequency domain position for sidelink CSI-RS.

sl-CSI-RS-FirstSymbol Indicates the position of first symbol of sidelink CSI-RS.

sl-DRX-ConfigUC-PC5 Indicates the NR sidelink DRX configuration for unicast communication, as specified in TS 38.321 [3]

sl-LatencyBoundCSI-Report Indicates the latency bound of SL CSI report from the associated SL CSI triggering in terms of number of slots. The UE is not configured with a value smaller than Tproc,1 defined in TS 38.214 [19] clause 8.1.4.

sl-LatencyBoundIUC-Report Indicates the latency bound of SL Inter-UE coordination report from the associated SL Inter-UE coordination explicit request triggering in terms of number of slots.

sl-LogicalChannelIdentity Indicates the identity of the sidelink logical channel.

sl-MappedQoS-FlowsToAddList Indicate the QoS flows to be mapped to the configured sidelink DRB. Each entry is indicated by the SL-PQFI, which is used between UEs, as defined in TS 23.287 [55].

sl-MappedQoS-FlowsToReleaseList Indicate the QoS flows to be released from the configured sidelink DRB. Each entry is indicated by the SL-PQFI, which is used between UEs, as defined in TS 23.287 [55].

sl-MeasConfig Indicates the sidelink measurement configuration for the unicast destination.

sl-OffsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

sl-OutOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the sidelink radio bearer is established.

sl-PDCP-SN-Size Indicates the PDCP SN size of the configured sidelink DRB.

sl-Resetconfig Indicates that the full configuration should be applicable for the RRCReconfigurationSidelink message.

sl-RLC-BearerToAddModList Indicate the additional Sidelink RLC bearer to be added / modified for the configured sidelink DRB.

sl-RLC-BearerToReleaseList Indicate the additional Sidelink RLC bearer to be released for the configured sidelink DRB.

sl-SDAP-Header Indicates whether or not a SDAP header is present on this sidelink DRB.

sl-SFN-DFN-Offset Indicates the SFN-DFN offset to be used for determining the SFN timeline based on the DFN timeline.

sl-SRB-IdentityWithDuplication Indicate the sidelink SRB for which duplication is configured.

slrb-PC5-ConfigIndex Indicates the identity of the configuration of a sidelink DRB. In case of L2 U2U relay, only value 4-31 can be signaled for end-to-end sidelink DRB, and all other values are reserved.

SL-SRAP-ConfigPC5 field descriptions

sl-RemoteUE-LocalIdentity Indicates the local UE ID of the L2 U2U Remote UE used in SRAP as specified in TS 38.351 [66].

sl-RemoteUE-L2Identity Indicates the Source L2 ID of the L2 U2U Remote UE as specified in TS 23.304 [65].

sl-PeerRemoteUE-LocalIdentity Indicates the local UE ID of the peer L2 U2U Remote UE used in SRAP as specified in TS 38.351 [66].

sl-PeerRemoteUE-L2Identity Indicates the Source L2 ID of the peer L2 U2U Remote UE as specified in TS 23.304 [65].

END OF CHANGE