# 3GPP TSG RAN WG2 Meeting #125      	                           R2-2400949
Athen, Greece, 26th Feb. – 1st March, 2024

Agenda item:	7.9.1

Source:	Apple

Title:	Discussion on Reply LS on L2ID and User Info for L2 U2U

WID/SID:	NR_SL_relay_enh – Release 18

Document for:	Discussion and Decision

# 1 Introduction

RAN2 has sent a LS [1] to check the feasibility of a working assumption which RAN2 has proposed to ensure the Source remote UE could understand the ”allocated local ID and L2 ID of the target remote UE” are belonging to which “User Info” the Source remote UE intends to reach via the L2 U2U relay UE. However, it appears that SA2 has some concerns about this working assumption. SA2 has also suggested the alternative option in the reply LS [2] which requires AS layer to exchange “User Info” in a container which is enclosed in PC5-RRC signaling.

In this paper, we discuss the issue in lieu of the information/solutions provided by SA2 reply LS and proposed a way forward.

# 2 Discussion

Form the LS[2], there are several concerns raised about the working assumption proposed by RAN2:

- Need add PC5-S procedure to exchange (target) L2 ID in ProSe layer;

- L2 ID collisions issue beyond 1-hop; and

- L2 ID to associate Local ID may also affect other procedures, e.g. Link identifier update procedure.

First, if analyzed carefully, it is obvious that the L2 ID collision should not be a blocking issue because Source Remote UE shall always put this “L2 ID of Target remote UE” reported by AS layer in the context of “L2 U2U relay UE”. So, this L2 ID can be still examined in one-hop context. Therefore, even if there are two Target Remote UE’s with different “User Info ID” allocated the same L2 ID, the Source remote UE will be able to discern that and there shall be no concern.

Observation 1	L2 ID collisions concern can be solved by associating the target remote UE’s L2 ID with the reporting L2 U2U relay.

Then, regarding the other two concerns, I think this is mainly because SA2 does not want to recognize the “L2 ID of target end UE” in this early stage of U2U relay setup.

According to SA2 specification [4] for Layer 2 UE-to-UE relay, the Source remote UE does not know the Target remote UE’s L2 ID until the e2e PC5 link has been established during the end-to-end PC5-S procedure for direct link establishment. Only after that, the PC5 link is officially recognized with a “Link Identifier” and the L2 ID(s) and link identifier are passed to AS layer.

This can be seen from the diagram below (which is derived from Figure 6.7.2-1 in TS 23.304 [4]):

Figure 1: High-level ProSe Communication procedure via L2 U2U relay and how Source End UE establish the context about link to target End UE

As can be seen from Figure 1 above, when AS layer allocates local ID, the Source remote UE is not aware of the L2 ID used by the target remote UE, the Source remote UE’s ProSe layer does not really have a “Link Identifier” context. That is why SA2 has some doubt about how ProSe layer to associate and use this “local ID”, which corresponds to an end-to-end PC5 link not established yet.

Observation 2	Implementing Option 2 requires SA2 to add new PC5-S procedure to exchange Target L2 ID and established a “Link” context earlier than what has been currently specified in ProSe layer.

However, based on SA2 reply LS, SA2 seems fine with associating the local ID with “User Info ID” in ProSe layer w/o L2 ID association, which was discussed as Option 1 in R2-2311275 [3]. However, during the RAN2 discussion, the majority of RAN2 companies has concern to carry User Info ID in PC5-RRC signaling because User Info ID is a pure upper layer identity and its contents are out of 3GPP scope. Also, as the reply LS[2] indicated,

(Option 1) does not require change on the existing SA2 defined procedures but requires some interaction between ProSe layer and AS layer which means SA2 specification change is required to support this option. However, some companies have concern about the inter layer interaction at UE-to-UE Relay and Remote UEs and providing User Info ID to the AS layer.

As we can see, the extra cross-layer change of sharing “User Info ID” with AS layer does incur additional specification efforts in SA2 and some SA2 companies have concern on that.

Observation 3	If Option 1 requires inter-layer exchange of User Info ID, there are some concerns in both RAN2 and SA2 about the specification impact.

Based on the above observations, we think the best way-forward is to find a way to implement Option 1 without inter-layer exchange of “User Info ID”. If the PC5-RRC signaling assigning local ID contains neither L2 ID nor User Info ID, then this solves the concerns for both Option 1 and Option 2. It also has very minimum impact.

This approach can work because according to current TS 23.304 [4], there is very likely to have only one end-to-end ProSe link via the same Layer 2 UE-to-UE relay pending for local ID assignment. As shown in Figure 2 below, when Source Remote UE first trigger the DCR for established the 1st hop PC5 link, it will not be able to initiate new “PROSE DIRECT LINK MODIFICAITON REQUEST” message to request the relay UE to setup a new Target remote UE because the DCA message has not been received yet.

Figure 2: Procedure for Source Remote UE to Trigger End-to-End with Two Target Remote UEs via L2 U2U

Between the time that ProSe layer of Source Remote UE receives DCA message back from the L2 U2U relay UE and time point when PorSe layer receives the “local ID association” from AS layer, there are just a few PC5-RRC messages exchanged, which is quite a short period of time. As long as the Source Remote UE abstain from initiating the new link modification request to the same relay UE, there is no any ambiguity for the ProSe Layer of source remote UE to associate the local ID (from AS layer) with the target Remote UE's User Info ID. After that association is done, the remote UE is then free to initiate PC5-S signaling to add another target remote UE via the same relay UE. The same “short” abstaining period applies to every new target remote UE multiplexed in the same first hop.

Observation 4	As long as ProSe layer abstain from initiating link modification request (to add new target remote UE via the same relay) in a very short time period, there is no need for inter-layer exchange of “User Info ID”.  .

Hence, we think this is a feasible way-forward to address the concerns for both Option 1 and Option 2, with this approach, there are the following advantages:

- No need of new procedure to exchange L2 ID in PC5-S protocol.

- No need of inter-layer exchange of User Info ID

- carrying L2 ID with local ID in PC5-RRC “RRCReconfigurartionSidelink” becomes Optional because there is no need for upper layer to bind L2 ID with User Info ID

Therefore, this approach actually reduce the signaling overhead and specification impact in both upper layer and AS layer thereby minimizing the efforts in both RAN2 and SA2. We suggest RAN2 to agree this as the way-forward for concluding this issue.

Proposal 1	RAN2 agrees the way forward to implementation Option 1 without inter-layer User Info ID exchange with the assumption that ProSe layer can abstain from initiating new link modification request before the local ID is assigned/associated with a pending Target User Info.

As SA2 request RAN2 feedback in [2], RAN2 can include this way-forward and check if this can be accepted by SA2.

Proposal 2: 	RAN2 send reply LS to SA2 to seek confirmation of the feasibility of this way-forward.

# 3 Conclusion

In this contribution, we discuss the SA2 reply LS on L2 ID and User Info and have the following observations:

Observation 1	L2 ID collisions concern can be solved by associating the target remote UE’s L2 ID with the reporting L2 U2U relay.

Observation 2	Implementing Option 2 requires SA2 to add new PC5-S procedure to exchange Target L2 ID and established a “Link” context earlier than what has been currently specified in ProSe layer.

Observation 3	If Option 1 requires inter-layer exchange of User Info ID, there are some concerns in both RAN2 and SA2 about the specification impact.

Observation 4	As long as ProSe layer abstain from initiating link modification request (to add new target remote UE via the same relay) in a very short time period, there is no need for inter-layer exchange of “User Info ID”.  .

Then, we have the following proposals:

Proposal 1	RAN2 agrees the way forward to implementation Option 1 without inter-layer User Info ID change with the assumption that ProSe layer can abstain from initiating new link modification request before the local ID is assigned and associated with a pending Target User Info.

Proposal 2: 	RAN2 send reply LS to SA2 to seek confirmation of the feasibility of this way-forward.

# 4 References

[1] R2-2311566, LS on L2 ID and User Info for L2 based U2U.

[2] S2-2313796, Reply LS on L2 ID and User Info for L2 based U2U

[3] R2-2311275 Report from session on positioning and sidelink relay (RAN2#123bis)

[4] 3GPP TS 23.304