3GPP TSG-RAN WG2 Meeting #125	R2-2401012

Athens, Greece, 26 February – 01 March 2023

Agenda item:	7.12.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issue list for mobile IAB UE capabilities

WID/SID:	NR_mobile_IAB-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution captures the open issues for mobile IAB-related UE capabilities.

# 2	Open issue list

-	Issue 1: Whether we introduce a mobileIAB-r18 capability (i.e. to identify an IAB-MT as mobile IAB-capable)

-	RAN2 earlier assumed that capability signalling was needed to a) inform the donor CU that the IAB-MT is a mobile IAB-MT, and b) inform the target CU that the IAB-MT is a mobile IAB-MT during handover.

-	Problem a) appears to have been addressed by the addition of the mobileIAB-Node-Indication-r18 to the RRCSetupComplete message in TS 38.331.

-	Problem b) appears to have been addressed by the addition of the "Mobile IAB Authorization Status" IE to the HANDOVER REQUEST message in TS 38.423.

-	Companies should discuss whether a mobileIAB-r18 capability is still needed.

-	Issue 2:  Whether to differentiate the mobile IAB RACH-less HO capability (/capabilities) from NTN RACH-less

-	At RAN2#124, it was agreed:

-	For mobile IAB: introduce separate RACH-less UE capabilities to differentiate UE support for the configured grant and dynamic grant

-	For NTN: introduce one RACH-less UE capability, rachLessHandoverNTN-r18, supporting CG and DG

-	Companies should discuss whether NTN and mobile IAB need to use different RACH-less capabilities.

NOTE:	There is a common agenda item 7.0.4 at RAN2#125 to discuss whether independent RACH-less handover configurations/procedures need to be defined for NTN and mobile IAB or if they can be shared. Issue 2 can also be discussed under AI 7.0.4.