3GPP TSG-RAN2 Meeting #125	R2-2401053

Athens, Greece, 26th Feb. – 1st Mar 2024

Agenda item:	7.19.3

Source:	Qualcomm Incorporated

Title:	Discussion on SA2 LS regarding the RedCap and eRedCap capabilities

WID/SID:	NR_redcap_enh-Core – Release 18

Document for:	Discussion and Decision

# Introduction

SA2 sent LS [1] to reply RAN3 and asked additional questions to RAN2.

Answer 2: SA2 considers it useful to include an eREDCAP indication the NGAP INITIAL UE MESSAGE for use for e.g. subscription based access restrictions (for which SA2 has approved the attached CRs), etc. SA2 understands that the access stratum can differentiate the different types of UEs, i.e., RedCap vs. eRedCap and the UE radio capabilities are independently defined for RedCap UEs and eRedCap UEs. As the radio capabilities are independently defined for a RedCap UE and an eRedCap UE the NAS Mobility Registration Update with UE Radio Capability Update as described in TS 23.501 clause. 5.4.4.1, would be triggered if a change occurred. SA2 would like to understand whether it is possible that a UE can have both REDCAP and eREDCAP capabilities and can change from a REDCAP UE to an eREDCAP UE (and vice versa)? If the UE can change from a REDCAP UE to an eREDCAP UE (and vice versa), what is the expected behaviour of a UE with regards to REDCAP/eREDCAP indication stated above in access stratum and provided during the RRC connection establishment procedure?

In this paper, we would like to discuss this issue and provide our view.

# Discussion

In Rel-17/18 spec, the RedCap and eRedCap UE have been defined with separate capabilities, and the spec allows UE to indicate one capability at one time, i.e., either RedCap or eRedCap to network. However, there is no restriction to limit a UE to be implemented with both RedCap and eRedCap capabilities. It should be up to UE implementation to have different capabilities within one UE, i.e., RedCap, eRedCap or non-(e)RedCap.

Observation: It is possible that UE can have both RedCap and eRedCap capabilities.

If UE is implemented with both RedCap and eRedCap capabilities, it is possible that UE would change its capability from RedCap to eRedCap (and vice versa) in some cases.

For example, for a RedCap UE (UE has already reported RedCap capability to network), if radio conditions of this RedCap UE become worse or the battery level of this RedCap UE is low, this RedCap UE would change its capabilities to eRedCap (assuming this UE also has eRedCap capability) with working on 5MHz DL/UL data channel and also working on lower peak data rate for processing time extension and further reducing the UE power consumption. The similar requirement on the opposite also exists. For another example, an eRedCap UE may want to do the capability update to RedCap, if there is good enough battery life to support UE working on 20MHz with a relative higher data rate. The legacy UE capability update procedure may help to achieve this target. From the procedure perspective, the UE capability update procedure has already been defined in the spec as SA2 LS pointed out, i.e., the NAS Mobility Registration Update with UE Radio Capability Update would be triggered if a change occurs.

Thus, if UE can have both RedCap and eRedCap capabilities, it is possible that the change from a RedCap UE to an eRedCap UE (and vice versa) is required.

Proposal 1: If UE can have both RedCap and eRedCap capabilities, it is possible that the change from a RedCap UE to an eRedCap UE (and vice versa) is required.

When UE intends to change its capability, i.e., from RedCap to eRedCap (or vice versa), the existing procedure does not prevent the (e)RedCap UEs from changing its capability, i.e., performing the NAS Registration procedure with Mobility Registration Update and including the UE Radio Capability Update.

Then it seems the SA2 question is about what the RedCap or eRedCap indication is provided by UE to RAN during the RRC connection establishment. When UE decides to change its capability, UE should already be informed by system information on whether the camped cell supports RedCap and/or eRedCap or not. UE performs the RRC connection establishment and during the initial access UE should determine to indicate the corresponding RedCap or eRedCap early indication based on the intended UE capability, i.e., RedCap or eRedCap. The whole procedure of initial access with the RedCap or eRedCap indication has already been well defined in the RAN2 spec. Thus, we have following proposal.

Proposal 2: If UE changes from a RedCap to an eRedCap UE (or vice versa), UE should use the RedCap or eRedCap early indication for the capability which the UE intends to use during the RRC connection establishment procedure that is used for sending the Registration Request that indicates to AMF the intention of the UE to change the radio capabilities.

Based on the discussion above, a draft reply LS is provided in the Annex.

Proposal 3: RAN2 to agree the draft reply LS provided in the Annex.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation: It is possible that UE can have both RedCap and eRedCap capabilities.

Proposal 1: If UE can have both RedCap and eRedCap capabilities, it is possible that the change from a RedCap UE to an eRedCap UE (and vice versa) is required.

Proposal 2: If UE changes from a RedCap to an eRedCap UE (or vice versa), UE should use the RedCap or eRedCap early indication for the capability which the UE intends to use during the RRC connection establishment procedure that is used for sending the Registration Request that indicates to AMF the intention of the UE to change the radio capabilities.

Proposal 3: RAN2 to agree the draft reply LS provided in the Annex.

# References

- S2-2401530, Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502, Huawei.

# Annex (draft LS)

3GPP TSG-RAN WG2#125                                                   R2-24xxxxx

Athens, Greece, 26th Feb. – 1st Mar 2024

Title:	[DRAFT] Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502

Response to:	R2-24xxxxx (S2-2401530)

Release:	Rel-18

Work Item:	       NR_redcap_enh-Core

Source:	RAN2

To:	SA2, RAN3

Cc:                 CT1, CT4

Contact Person:

#### Name:	Ruiming Zheng

E-mail Address:	rzheng@qti.qualcomm.com

Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS regarding the issue of UE changing the capability of RedCap to eRedCap (and vice versa).

RAN2 discussed the issues raised in SA2 LS. From RAN2 perspective, it is possible that UE can have both RedCap and eRedCap capabilities and UE can change from RedCap to eRedCap capability (and vice versa). If UE changes from a RedCap to an eRedCap UE (or vice versa), UE should use the RedCap or eRedCap early indication for the capability which the UE intends to use during the RRC connection establishment procedure that is used for sending the Registration Request that indicates to AMF the intention of the UE to change the radio capabilities.

2. Actions:

To RAN3:

ACTION: 	RAN2 kindly asks SA2 to take above response into account for SA2 work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2#125-bis                   15 – 19 April 2024		China (Changsha)

TSG-RAN WG2#126	20 – 24 May 2024			Japan (Fukuoka)