3GPP TSG-RAN WG2 Meeting #125	R2-2401145

Athens, Greece, 26 Feb - 1 Mar, 2024

Agenda item:	7.24.2.2

Source:	CMCC, China Unicom, Huawei, ZTE, CATT

Title:	Introduction of PDCCH CCE Usage for gNB Layer 2 measurement

Document for:	Discussion and Decision

1	Introduction

In this contribution, we first analysis the absence and necessity of evaluating the usage of resources of PDCCH, and propose to introduce a new measurement.

2	Discussion

2.1 Justification of PDCCH usage

Currently, the following KPIs for PRB Usage for MIMO have been defined in TS 38.314 v17.4.0, which are applicable for PDSCH and PUSCH only:

4.2.1.7.1  PDSCH PRB Usage for MIMO in the DL per cell

4.2.1.7.2  PUSCH PRB Usage for MIMO in the DL per cell

4.2.1.7.3  PDSCH PRB Usage based on the statistical MIMO layer in the DL per cell

4.2.1.7.4  PUSCH PRB Usage based on the statistical MIMO layer in the UL per cell

4.2.1.7.5  Enhanced PDSCH PRB Usage for MIMO in the DL per cell

4.2.1.7.6  Enhanced PUSCH PRB Usage for MIMO in the UL per cell

Observation 1: The current defined KPIs for PRB Usage for MIMO are only applicable for PDSCH/PUSCH. However, there are no KPIs specified to reflect the usage of resources of PDCCH.

There are mainly following 3 motivations for introducing PDCCH CCE usage in TS 38.314:

Motivation 1: OAM performance observability

In real-life deployment, it is observed that the number of users is large while PDSCH/PUSCH PRB usage is relatively low. The user experience is bad due to limited PDCCH resources. Under the circumstance in which the PDCCH is the bottleneck constraint channel, i.e. the usage of PDCCH is high but the PRB usage is relatively low, the performance of the whole cell is affected. However, operators have no metric to evaluate and detect this situation. Therefore, it is necessary to introduce the PDCCH usage metric which is accessible for OAM, and operators can perform network maintenance to improve user experience and cell performance. As shown in Figure1 a) and b), when the number of UEs in the cell increases, the average UL/DL UE throughput gets deteriorated with a relatively low PRB usage. (The KPIs, including mean number of RRC connections, average UL/DL UE throughput, and UL/DL total PRB usage, have been specified in TS 28.552.).

- b)

Figure 1. An example of a cell with relatively low PRB Usage but bad UE Experience

Motivation 2: inter-cell load balancing

The PDCCH CCE usage can also be utilized as the essential metric for network to perform load balancing appropriately. For example, if one cell has high PDCCH usage (as shown in Figure 1a) but the neighbour cells have low PDCCH usage, the network can handover UEs to cells with low load for better performance.

Observation 2: Introducing the usage of PDCCH is essential for OAM performance observability and inter-cell load balancing.

Motivation 3: to reflect the space division multiplexing and MIMO layers in the metric

Similar to PDSCH/PUSCH usage metric, multiple MIMO layers of PDCCH can be scheduled to the UE. And several different users can share the same PDCCH CCE at the same time with the deployment of MIMO. If MIMO layer is not taken into account, the calculation of the usage of resources of PDCCH is always higher than the reality for Massive MIMO scenario. Therefore, we propose to calculate the usage of resources of PDCCH by the SUM of PDCCH CCE allocated for each UE, by taking MIMO layer into consideration. Meanwhile, to unify the measurement, the MIMO layer can be set to 1 for scenario of non-MIMO.

Observation 3: The space division multiplexing and MIMO layer need to be taken into consideration to reflect the usage of resources of PDCCH precisely.

2.2 Discussion on the definition of PDCCH usage

Regarding the definition of PDCCH usage, firstly, we think PDCCH CCE usage can be considered. In TS 38.211 v18.1.0, PDCCH Control-channel element (CCE) has been defined in section 7.3.2.1, and the PDCCH CCE usage can be used to indicate the usage of resources of PDCCH. Secondly, PDCCH CCE usage should take MIMO layer into account, which is similar to the existing KPIs for PRB usage.

As below, we provide a definition of the new measurement (for TS 38.314):

---------------------------------------------Start of Proposed New Measurements------------------------------------------------

#### 4.2.1.x	PDCCH CCE Usage

##### 4.2.1.x.1	PDCCH CCE Usage per cell

This measurement provides the total usage (in percentage) of PDCCH control-channel elements (CCEs) per cell. The objective of the measurement is to measure usage of time, frequency and space resources.

Protocol Layer: MAC, PHY

Table 4.2.x.1.1-1: Definition for PDCCH CCE Usage per cell

| Definition | PDCCH CCE Usage per cell is calculated in the time-frequency-space domain. Detailed Definition: MT=∀i∀j{M1ij(T)*Lij(T)}∀j{Pj(T)}*Alpha*100, where explanations can be found in the table 4.2.x.1.1-2 below. |
| --- | --- |

Table 4.2.x.1.1-2: Parameter description for PDCCH CCE Usage per cell

| M(T) | Total PDCCH CCE usage per cell which is percentage of CCEs used for MIMO and non-MIMO, averaged during time period T with integer value range: 0-100. |
| --- | --- |
| M1ij(T) | For MIMO, a count of PDCCH CCEs used for control information transmission for UE i on single MIMO layer per cell at sampling occasion j; For non-MIMO, a count of PDCCH CCEs used for control information transmission for UE i per cell at sampling occasion j. |
| Lij(T) | For MIMO, the number of MIMO layers scheduled for UE i at sampling occasion j; For non-MIMO, this parameter should be set to 1. |
| i | A UE i that is scheduled during time period 𝑇. |
| j | Sampling occasion during time period 𝑇. A sampling occasion is 1 symbol. |
| Pj(T) | For MIMO, total number of PDCCH CCEs available for sampling occasion j on single MIMO layer per cell. For non-MIMO, total number of PDCCH CCEs available for sampling occasion j per cell. |
| T | Time period during which the measurement is performed to calculate MT, e.g., 15min, 1 hour, etc. |
| Alpha | A constant value configured by OAM during time period 𝑇 with float value range: 1.00-100.00. For MIMO, this parameter should be set to 1.00-100.00; For non-MIMO, this parameter should be set to 1.00. With this parameter, M(T) should not be larger than 100. |

------------------------------------------------------End of New Measurements-----------------------------------------------------

Proposal 1: Introduce a new measurement of PDCCH CCE Usage in TS 38.314.

Proposal 2: RAN2 to agree on the CR provided in Annex.

3	Conclusion

In this contribution, we discuss the motivation of introducing a new measurement of PDCCH CCE Usage. Here are the observations and proposal:

Observation 1: The current defined KPIs for PRB Usage for MIMO are only about PDSCH/PUSCH PRB Usage. However, there are no KPIs defined in RAN2 to reflect the usage of resources of PDCCH.

Observation 2: Introducing the usage of PDCCH is essential for OAM performance observability and inter-cell load balancing.

Observation 3: The space division multiplexing and MIMO layer need to be taken into consideration to reflect the usage of resources of PDCCH precisely.

Proposal 1: Introduce a new measurement of PDCCH CCE Usage in TS 38.314.

Proposal 2: RAN2 to agree on the CR provided in Annex.

4	Annex (CR to TS 38.314)

3GPP TSG-RAN WG2 Meeting # 125	R2-240xxxx

# Athens, Greece, 26 Feb - 1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | xxxx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PDCCH CCE Usage for gNB Layer 2 measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, China Unicom, Huawei, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current specification, the performance metric PRB Usage for MIMO has been defined (see subclause 4.2.1.7), which can measure the usage of resources of PDSCH/PUSCH. In real-life deployment, it is observed that the number of users is large while PDSCH/PUSCH PRB usage is relatively low. In such case, the PDCCH is the bottleneck channel. Introducing the usage of PDCCH is essential for both OAM performance observability and inter-cell load balancing. - Similar to PDSCH/PUSCH usage metric, multiple MIMO layers of PDCCH can be scheduled to the UE. And several different users can share the same PDCCH CCE at the same time with the deployment of MIMO. If MIMO layer is not taken into account, the calculation of the usage of resources of PDCCH is always higher than the reality for Massive MIMO scenario. Therefore, the introduction of PDCCH CCE Usage need to take the space division multiplexing into consideration to reflect the usage of resources of PDCCH precisely. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new measurement of PDCCH CCE Usage is introduced in TS 38.314 to reflect the usage of resources of PDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is impossible to precisely reflect the usage of PDCCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

# 4	Layer 2 measurements

### 4.2.1	Measurements valid for all gNB deployment scenarios

<< Partially omitted >>

#### 4.2.1.x	PDCCH CCE Usage

##### 4.2.1.x.1	PDCCH CCE Usage per cell

This measurement provides the total usage (in percentage) of PDCCH control-channel elements (CCEs) per cell. The objective of the measurement is to measure usage of time, frequency and space resources.

Protocol Layer: MAC, PHY

Table 4.2.1.x.1-1: Definition for PDCCH CCE Usage per cell

| Definition | PDCCH CCE Usage per cell is calculated in the time-frequency-space domain. Detailed Definition: MT=∀i∀j{M1ij(T)*Lij(T)}∀j{Pj(T)}*Alpha*100, where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for PDCCH CCE Usage per cell

| M(T) | Total PDCCH CCE usage per cell which is percentage of CCEs used for MIMO and non-MIMO, averaged during time period T with integer value range: 0-100. |
| --- | --- |
| M1ij(T) | For MIMO, a count of PDCCH CCEs used for control information transmission for UE i on single MIMO layer per cell at sampling occasion j; For non-MIMO, a count of PDCCH CCEs used for control information transmission for UE i per cell at sampling occasion j. |
| Lij(T) | For MIMO, the number of MIMO layers scheduled for UE i at sampling occasion j; For non-MIMO, this parameter should be set to 1. |
| i | A UE i that is scheduled during time period 𝑇. |
| j | Sampling occasion during time period 𝑇. A sampling occasion is 1 symbol. |
| Pj(T) | For MIMO, total number of PDCCH CCEs available for sampling occasion j on single MIMO layer per cell. For non-MIMO, total number of PDCCH CCEs available for sampling occasion j per cell. |
| T | Time period during which the measurement is performed to calculate MT, e.g., 15min, 1 hour, etc. |
| Alpha | A constant value configured by OAM during time period 𝑇 with float value range: 1.00-100.00. For MIMO, this parameter should be set to 1.00-100.00; For non-MIMO, this parameter should be set to 1.00. With this parameter, M(T) should not be larger than 100. |

## << End of change >>