3GPP TSG-RAN WG2 Meeting #125	R2-2401246

Athens, Greece, February 26 – March 1, 2024

Agenda item:	7.2.3

Source: 	Qualcomm Incorporated

Title: 	[RILs Q003, Q005, Q012] Various SLPP Corrections

Document for:		Discussion and Decision

# 1.	SL-AoA Request/Provide Location Information

By comparing the RAN1 parameter list and agreements with the IEs SL-AoA-RequestLocationInformation/SL-AoA-ProvideLocationInformation, we observe the following issues:

- All measurements and attributes are listed in the IE SL-AoA-MeasElement. However, there is no structure/grouping of the elements. For example, the current IE implies that it can be possible to report AoA and ZoA in different coordinate systems.

(2)	The measurement request cannot indicate a request for AoA and ZoA individually.

(3)	The SL-AoA-AdditionalPathList can include separate LCS-GCS Translation information, SL-PRS Resource ID, or ARP ID. However, multi-path measurements are made from the same SL-PRS reception (analogous to Uu positioning).

(4)	A UE can request multiple measurements and attributes. However, there are currently no capabilities indicated.

Proposal 1: Agree the TP in the Annex A of this contribution.

# 2.	SL-TDOA/SL-TOA Request/Provide Assistance Data

In the current SLPP, a UE can request RTD info from another endpoint:

sl-RTD-InfoRequest This field indicates the SL RTD information requested.

The response would be a list of RTDs:

SL-RTD-Info ::= SEQUENCE {

referenceRTD-Info    ReferenceRTD-Info,

rtd-InfoList         RTD-InfoList

}

RTD-InfoList ::= SEQUENCE (SIZE (1.. maxNrOfSLTxUEs)) OF RTD-InfoListPerTxUE

However, the request is just a flag. RTD is at least between two UEs and these must be UEs participating in the session. The UE IDs of the source (UE which is requesting the RTD) and of the target (UEs for which the RTD is requested) is currently missing.

Further, the SL-TDOA-ProvideAssistanceData currently includes two nested SEQUENCE's, which is unncessary. The RTD info is labeled as "Position Calculation Assistance". Although, RTDs are needed for position calculation, they can also be used – for example -- to determine "expected RSTDs" for measurements.

Proposal 2: Agree the TP in the Annex B of this contribution.

# 3.	Range and Direction in CommonIEsProvideLocationInformation

The range and direction are defined in TS 23.023, clause 5.10 as follows:

The "range and direction" from a point A to a point B is characterised by three components comprising a range from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B as shown in Figure 3d. The range provides a straight-line distance from point A to point B. The azimuth provides a direction to point B from point A in a horizontal plane through point A and as measured clockwise from North. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a horizontal plane through point A. The range, azimuth and elevation can be each independently included or excluded in a "range and direction" and each has an uncertainty and a confidence.

SLPP encodes the above information currently as follows:

RangeAndOrDirection ::= SEQUENCE {

range                 Range       OPTIONAL,

azimuth               Azimuth     OPTIONAL,

elevation             Elevation   OPTIONAL

}

Range ::= SEQUENCE {

rangeResult                  INTEGER (0..999),

uncertainty                  INTEGER (0..127),

confidence                   INTEGER (0..100)             OPTIONAL

}

Azimuth ::= SEQUENCE {

azimuthResult                INTEGER (0..89),

uncertainty                  INTEGER (0..127),

confidence                   INTEGER (0..100)             OPTIONAL

}

Elevation ::= SEQUENCE {

elevationResult              INTEGER (0..89),

uncertainty                  INTEGER (0..63),

confidence                   INTEGER (0..100)             OPTIONAL

}

From the above, we observe the following issues:

(1)	range is encoded as INTEGER (0..999), but without units and scale factor.

(2)	uncertainty is encoded as INTEGER (0..127) for both, range and azimuth, but without units and scale factor.

(3)	Elevation uncertainty is encoded as INTEGER (0..63), but without units and scale factor.

(4) azimuth and elevation are encoded as INTEGER (0..89), but without units and scale factor.

Range encoding:

The range could in principle be encoded similar to deltaLatitude, deltaLongitude, and deltaHeight in IE RelativeLocation by providing a units field together with "fine" and "coarse" values.  However, the value range for the RelativeLocation in LPP was designed to cover TRP coordinates in a rather large area. For sidelink, a 20-bit value (i.e., INTEGER(0..1023) for both, "coarse" and "fine" values could cover > 1-km with 1-mm resolution.

Range Uncertainty encoding:

The range uncertainty is currently encoded as INTEGER (0..127). This may mean the Uncertainty Encoding as defined in clause 6.2 of TS 23.032. However, assuming a Range encoding as proposed above, such an uncertainty would be too coarse. Instead, the "High Accuracy Uncertainty" as defined in clause 6.2a of TS 23.032 is proposed for the Range Uncertainty.

Azimuth/Elevation encoding:

The azimuth and elevation should be encoded as in SL-AoA-ProvideLocationInformation (see Annex A above):

SL-AoA-Measurement ::= SEQUENCE {

sl-AzimuthAoA                         INTEGER (0..3599)         OPTIONAL,

sl-AzimuthAoA-Quality                 MeasurementAngleQuality   OPTIONAL,

sl-ZenithAoA                          INTEGER (0..1800)         OPTIONAL,

sl-ZenithAoA-Quality                  MeasurementAngleQuality   OPTIONAL,

...

}

I.e., if a AoA measurement can be provided with 0.1-degree resolution, the azimuth/elevation in RangeAndOrDirection should have the same resolution.

Similar, the angle uncertainty from SL-AoA-ProvideLocationInformation can be re-used:

MeasurementAngleQuality ::= SEQUENCE {

resolution                  ENUMERATED {deg0dot1, ...},

angleMeasurementQuality     INTEGER (0..255)

}

Proposal 3: Agree the TP in the Annex C of this contribution.

# Annex A: TP for TS 38.355

#### –	SL-AoA-RequestLocationInformation

-- ASN1START

-- TAG-SL-AOA-REQUESTLOCATIONINFORMATION-START

SL-AoA-RequestLocationInformation ::= SEQUENCE {

sl-AoA-Request                        ENUMERATED { aoa, zoa, both },

sl-ARP-InfoRequest                    ENUMERATED { true }    OPTIONAL,

sl-LOS-NLOS-IndicatorRequest          ENUMERATED { true }    OPTIONAL,

sl-PRS-RSRP-Request                   ENUMERATED { true }    OPTIONAL,

sl-FirstPathRSRPP-Request             ENUMERATED { true }    OPTIONAL,

sl-AdditionalPathsRequest             ENUMERATED { true }    OPTIONAL,

...

}

-- TAG-SL-AOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-AoA-RequestLocationInformation field descriptions

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-AoA-AdditionalPathList.

sl-AoA-Request This field, if present, indicates that the UE is requested to provide sl-AzimuthAoA, sl-ZenithAoA, or both in SL-AoA-Measurement.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-FirstPathRSRPP-Request This field, if present, indicates that the UE is requested to provide sl-FirstPathRSRPP-Result.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide los-NLOS-Indicator.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

#### –	SL-AoA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-START

SL-AoA-ProvideLocationInformation ::= SEQUENCE {

sl-AoA-SignalMeasurementInformation                   SL-AoA-SignalMeasurementInformation    OPTIONAL,

...

}

SL-AoA-SignalMeasurementInformation ::= SEQUENCE {

sl-AoA-MeasList                         SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-AoA-MeasElement,

...

}

SL-AoA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING,

sl-AoA-Measurement                    SL-AoA-Measurement,

lcs-to-gcs-translation                LCS-GCS-Translation       OPTIONAL,

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-AoA-AdditionalPathList             SL-AoA-AdditionalPathList OPTIONAL,

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-FirstPathRSRPP-Result          INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

...

}

SL-AoA-Measurement ::= SEQUENCE {

sl-AzimuthAoA                         INTEGER (0..3599)         OPTIONAL,

sl-AzimuthAoA-Quality                 MeasurementAngleQuality   OPTIONAL,

sl-ZenithAoA                          INTEGER (0..1800)         OPTIONAL,

sl-ZenithAoA-Quality                  MeasurementAngleQuality   OPTIONAL,

...

}

SL-AoA-AdditionalPathList ::= SEQUENCE (SIZE(1..2)) OF SL-AoA-AdditionalPath

SL-AoA-AdditionalPath  ::= SEQUENCE {

sl-AoA-Measurement                         SL-AoA-Measurement,

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)          OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

...

}

MeasurementAngleQuality ::= SEQUENCE {

resolution                  ENUMERATED {deg0dot1, ...},

angleMeasurementQuality     INTEGER (0..255)

}

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

SL-AoA-ProvideLocationInformation field descriptions

lcs-to-gcs-translation This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8]. If absent, the azimuth and/or zenith AoA are provided in GCS.

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements.

sl-AzimuthAoA-Quality This field specifies the quality of the Azimuth Angle-of-Arrival measurement.

sl-AoA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-AzimuthAoA This field specifies the SL-Azimuth AoA measurement.

sl-POS-ARP-ID-Rx This field indicates the ARP ID of an ARP used for SL-PRS reception. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the PRS resource ID used for the measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-FirstPathRSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TimeStamp This field specifies the time instance at which the SL-AoA-Measurement, sl-PRS-RSRP-Result (if included), and sl-PRS-FirstPathRSRPP-Result (if included) measurements are performed.

sl-ZenithAoA This field specifies the SL-Zenith AoA measurement.

sl-ZenithAoA-Quality This field specifies the quality of the Zenith Angle-of-Arrival measurement.

#### –	SL-AoA-ProvideCapabilities

The IE SL-AOA-ProvideCapabilities is used to indicate the support of SL-AOA and to provide SL-AOA positioning capabilities.

-- ASN1START

-- TAG-SL-AOA-PROVIDECAPABILITIES-START

SL-AoA-ProvideCapabilities ::= SEQUENCE {

applicationLayerID                        OCTET STRING,

positioningModes                          PositioningModes,

tenMsUnitResponseTime                     PositioningModes                      OPTIONAL,

periodicalReporting                       PositioningModes                      OPTIONAL,

Sl-AoA-Request-Sup                        ENUMERATED { aoaSup, zoaSup, both },

sl-ARP-InfoRequest-Sup                    ENUMERATED { supported }              OPTIONAL,

sl-LOS-NLOS-IndicatorRequest-Sup          ENUMERATED { supported }              OPTIONAL,

sl-PRS-RSRP-Request-Sup                   ENUMERATED { supported }              OPTIONAL,

sl-FirstPathRSRPP-Request-Sup             ENUMERATED { supported }              OPTIONAL,

sl-AdditionalPathsRequest-Sup             ENUMERATED { supported }              OPTIONAL,

...

}

-- TAG-SL-AOA-PROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-AoA-ProvideCapabilities field descriptions

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support periodicalReporting in CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-AoA mode(s) supported by the UE.

tenMsUnitResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero value means not supported. If this field is absent, the UE does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

sl-AdditionalPathsRequest-Sup This field, if present, indicates that the UE supports sl-AoA-AdditionalPathList in IE SL-AoA-ProvideLocationInformation.

Sl-AoA-Request-Sup This field, if present, indicates that the UE supports sl-AzimuthAoA, sl-ZenithAoA, or both in SL-AoA-Measurement in IE SL-AoA-ProvideLocationInformation.

sl-ARP-InfoRequest-Sup This field, if present, indicates that the UE supports sl-POS-ARP-ID-Rx in IE SL-AoA-ProvideLocationInformation.

sl-FirstPathRSRPP-Request-Sup This field, if present, indicates that the UE supports sl-FirstPathRSRPP-Result in IE SL-AoA-ProvideLocationInformation.

sl-LOS-NLOS-IndicatorRequest-Sup This field, if present, indicates that the UE supports los-NLOS-Indicator in IE SL-AoA-ProvideLocationInformation.

sl-PRS-RSRP-Request-Sup This field, if present, indicates that the UE supports sl-PRS-RSRP-Result in IE SL-AoA-ProvideLocationInformation.

# Annex B: TP for TS 38.355

#### –	SL-TDOA-RequestAssistanceData

-- ASN1START

-- TAG-SL-TDOA-REQUESTASSISTANCEDATA-START

SL-TDOA-RequestAssistanceData ::= SEQUENCE {

applicationLayerID                 OCTET STRING,

sl-RTD-InfoRequest                 ENUMERATED { true}                                          OPTIONAL,

sl-RTD-InfoRequestPerUE            SEQUENCE (SIZE (1..maxNrOfUEs)) OF RTD-InfoRequestPerUE     OPTIONAL,

...

}

RTD-InfoRequestPerUE ::= SEQUENCE {

applicationLayerID-Req                OCTET STRING,

...

}

-- TAG-SL-TDOA-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

SL-TDOA-RequestAssistanceData field descriptions

applicationLayerID This field indicates the application layer ID of the UE which is requesting the assistance data.

applicationLayerID-Req This field indicates the application layer ID of the UE for which the RTD information is requested.

sl-RTD-InfoRequest This field, if present, indicates that SL RTD information is requested.

sl-RTD-InfoRequestPerUE This field, if present, indicates the UE IDs for which the SL RTD information is requested. This field may only be present if sl-RTD-InfoRequest is present.

#### –	SL-TDOA-ProvideAssistanceData

-- ASN1START

-- TAG-SL-TDOA-PROVIDEASSISTANCEDATA-START

SL-TDOA-ProvideAssistanceData ::= SEQUENCE {

sl-RTD-Info                             SL-RTD-Info                             OPTIONAL,

...

}

-- TAG-SL-TDOA-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

SL-TDOA-ProvideAssistanceData field descriptions

sl-RTD-Info This field provides synchronization information of anchor UEs.

# Annex C: TP for TS 38.355

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information SLPP message Type.

-- ASN1START

-- TAG-COMMONIESPROVIDELOCATIONINFORMATION-START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate                        LocationCoordinates    OPTIONAL, -- [locationTargetUe-sl-pos]

rangeAndOrDirection                     RangeAndOrDirection    OPTIONAL,

velocityEstimate                        Velocity               OPTIONAL,

locationError                           LocationError          OPTIONAL,

...

}

[...]

RangeAndOrDirection ::= SEQUENCE {

range                 Range       OPTIONAL,

azimuth               Azimuth     OPTIONAL,

elevation             Elevation   OPTIONAL

}

Range ::= SEQUENCE {

rangeResult                  INTEGER (0..1048575),

uncertainty                  INTEGER (0..255),

confidence                   INTEGER (0..100)             OPTIONAL

}

Azimuth ::= SEQUENCE {

azimuthResult                INTEGER (0..3599),

uncertainty                  MeasurementAngleQuality,

confidence                   INTEGER (0..100)             OPTIONAL

}

Elevation ::= SEQUENCE {

elevationResult              INTEGER (0..1800),

uncertainty                  MeasurementAngleQuality,

confidence                   INTEGER (0..100)             OPTIONAL

}

[...]

-- TAG-COMMONIESPROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

CommonIEsProvideLocationInformation field descriptions

locationError This field shall be included if and only if a location estimate and measurements are not included in the SLPP PDU. The field includes information concerning the reason for the lack of location information. The LocationFailureCause 'periodicLocationMeasurementsNotAvailable' shall be used by the UE if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [7]. Coding of the values of the various fields internal to each geographic shape follow the rules in TS 23.032 [7]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message.

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [7]. Coding of the values of the various fields internal to each velocity shape follow the rules in TS 23.032 [7].

uncertainty This field provides the uncertainty of the rangeResult. Coding of the value is according to the "High Accuracy Uncertainty" defined in TS 23.032 [7].

rangeResult This field povides the range between two points, as defined in TS 23.032 [7] for the "Range and Direction". Scale factor is 1 milli-metres.

azimuthResult This field provides the azimuth angle between two points, as defined in TS 23.032 [7] for the "Range and Direction". Scale factor is 0.1 degrees.

elevationResult This field provides the elevation angle between two points, as defined in TS 23.032 [7] for the "Range and Direction". Scale factor is 0.1 degrees.

confidence This field provides the confidence with which the rangeResult, azimuthResult, and elevationResult are known as defined in TS 23.032 [7].