3GPP TSG-RAN WG2 Meeting #125	R2-2401250

Athens, Greece, February 26 – March 1, 2024

Agenda item:	7.2.4

Source: 	Qualcomm Incorporated

Title: 	[RILs Q019, Q024, Q028] Clarification of field description for aggregated and hopping measurement results

Document for:		Discussion and Decision

# 1.	Discussion

## 1.1	nr-RSTD-BasedOnAggregatedResources

According to the following RAN1 agreement

Agreement If the UE/gNB reports aggregated timing measurement, the single reported RSRP/RSRPP (if reported) is based on aggregated PRS/SRS resources across aggregated PFLs/carriers. - Note1: it is up to RAN4 whether to define a corresponding requirement - Note2: for UL, measured SRS signals refer to aggregated SRS resources. For DL, measured PRS signals refer to aggregated PRS resources.

the RSRP and RSRPP measurements (if reported) are also based on aggregated DL-PRS Resources.  However, the current LPP specification implies that only RSTD/RxTx-Time difference is based on aggregated resources; e.g.:

[...]

[[

nr-RSTD-BasedOnAggregatedResources-r18		ENUMERATED {true}					OPTIONAL,

nr-AggregatedDL-PRS-ResourceSetID-List-r18	SEQUENCE (SIZE (2.. 3)) OF

NR-AggregatedDL-PRS-ResourceSetID-Element-r18		OPTIONAL,

[...]

nr-RSTD-BasedOnAggregatedResources This field indicates whether the measurement is based on aggregation across PFLs for DL-TDOA.

## 1.2	nr-DL-PRS-RxHoppingRequest

The current field description for the nr-DL-PRS-RxHoppingRequest seems to imply that all DL-TDOA/Multi-RTT measurements can be performed using DL-PRS Rx hopping:

nr-DL-PRS-RxHoppingRequest This field, if present, indicates that the target device is requested to perform DL PRS Rx hopping measurements and reporting.

However, carrier phase measurements cannot be performed using DL-PRS Rx hopping.

# Annex: TP for TS 37.355

#### 6.5.10.4	NR DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR DL-TDOA measurements to the location server.

NOTE 1:	The dl-PRS-ReferenceInfo defines the "RSTD reference" TRP. The nr-RSTD's and nr-RSTD-ResultDiff's in nr-DL-TDOA-MeasList are provided relative to the "RSTD reference" TRP.

NOTE 2:	The "RSTD reference" TRP may or may not be the same as the "assistance data reference" TRP provided by nr-DL-PRS-ReferenceInfo in IE NR-DL-PRS-AssistanceData.

NOTE 3:	The target device includes a value of zero for the nr-RSTD and nr-RSTD-ResultDiff of the "RSTD reference" TRP in nr-DL-TDOA-MeasList.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...,

kMinus1-r18					INTEGER (0..3940097),

kMinus2-r18					INTEGER (0..7880193)

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-MeasBasedOnAggregatedResources-r18		ENUMERATED {true}					OPTIONAL,

nr-AggregatedDL-PRS-ResourceSetID-List-r18	SEQUENCE (SIZE (2.. 3)) OF

NR-AggregatedDL-PRS-ResourceSetID-Element-r18		OPTIONAL,

nr-RSCPD-r18								INTEGER (0..61565)					OPTIONAL,

nr-PhaseQuality-r18							NR-PhaseQuality-r18					OPTIONAL,

nr-RSCPD-AddSampleMeasurements-r18		SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF

NR-RSCPD-AdditionalMeasurementElement-r18					OPTIONAL,

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18

ENUMERATED { singleHop, multipleHop }						OPTIONAL

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...,

kMinus1-r18					INTEGER (0..16382),

kMinus2-r18					INTEGER (0..32764)

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]],

[[

nr-MeasBasedOnAggregatedResources-r18		ENUMERATED {true}					OPTIONAL,

nr-AggregatedDL-PRS-ResourceSetID-List-r18	SEQUENCE (SIZE (2.. 3)) OF

NR-AggregatedDL-PRS-ResourceSetID-Element-r18		OPTIONAL,

nr-RSCPD-r18								INTEGER (0..61565)					OPTIONAL,

nr-PhaseQuality-r18							NR-PhaseQuality-r18					OPTIONAL,

nr-RSCPD-AdditionalMeasurementsAddSample-r18	SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF

NR-RSCPD-AdditionalMeasurementElement-r18					OPTIONAL,

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18

ENUMERATED { singleHop, multipleHop }						OPTIONAL

]]

}

NR-RSCPD-AdditionalMeasurementElement-r18 ::= SEQUENCE {

nr-RSCPD-Result-r18						INTEGER (0..61565)						OPTIONAL,

nr-PhaseQuality-r18						NR-PhaseQuality-r18						OPTIONAL,

nr-TimeStamp-r18						NR-TimeStamp-r16						OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UERxTEG | The field is optionally present, need OP, if the field nr-UE-Rx-TEG-ID is present; otherwise it is not present. |

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-DL-TDOA-SignalMeasurementInformation. If the nr-UE-Rx-TEG-ID is present and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the TOA, RSCP (if included) and DL PRS-RSRP/RSRPP (if included) measurement is performed. The nr-SFN, nr-Slot and nr-Symbol (if included) in IE NR-TimeStamp correspond to the TRP provided in dl-PRS-ReferenceInfo as specified in TS 38.214 [45]. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-DL-TDOA-AdditionalMeasurements This field provides up to 3 additional RSTD measurements per pair of TRPs, with each measurement between a different pair of DL-PRS Resources or DL-PRS Resource Sets of the DL-PRS for those TRPs [45]. If this field is present, the field nr-DL-TDOA-AdditionalMeasurementsExt shall be absent.

nr-UE-Rx-TEG-ID This field provides the ID of the UE Rx TEG associated with the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. When different UE Rx TEGs for RSTD measurements are requested, the maximum number of reported RSTD measurements associated with different DL-PRS Resources per UE Rx TEG per target TRP is 4.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. This field also applies to specify the target device's best estimate of the LOS or NLOS of the RSCP measurement for the TRP or resource. Note, the RSCP measurement refers to the RSCP of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSCPD or nr-RSCPD-ResultDiff. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-DL-TDOA-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-DL-TDOA-MeasElement, provides TOA measurements of up to 4 DL-PRS Resources of a TRP with different UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 TOA measurement results with respect to different Rx TEGs. If this field is present, the field nr-DL-TDOA-AdditionalMeasurements shall be absent.

nr-MeasBasedOnAggregatedResources This field indicates whether the RSTD measurement, RSRP measurement (if included), and RSRPP measurement (if included) is based on aggregated DL-PRS Resources.

nr-AggregatedDL-PRS-ResourceSetID-List This field provides the PRS resource set IDs and the PRS resource IDs for the aggregated measurement which are used for timing measurement results. If the field is present, the field nr-DL-PRS-ResourceID and nr-DL-PRS-ResourceSetID should not be included.

nr-RSCPD This field specifies the NR DL reference signal carrier phase difference measurement, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46]. The target and the reference TRP are in the same PFL.

nr-PhaseQuality This field specifies the target device′s best estimate of the quality of the RSCPD measurement.

nr-RSCPD-AddSampleMeasurements This field, in addition to the measurements provided in NR-DL-TDOA-MeasElement, provides up to 3 RSCPD measurements associated with the nr-RSTD in NR-DL-TDOA-MeasElement.

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx This field indicates that the reported measurement is based on receiving single or multiple hops of DL PRS.

nr-RSTD-ResultDiff This field provides the additional DL RSTD measurement result relative to nr-RSTD. The RSTD value of this measurement is obtained by adding the value of this field to the value of the nr-RSTD field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

nr-RSCPD-AdditionalMeasurements This field, provides up to 4 RSCPD measurements associated with the TOA measurement in NR-DL-TDOA-AdditionalMeasurementElement.

nr-RSCPD-ResultDiff This field provides the additional RSCPD measurement result relative to nr-RSCPD. The RSCPD value of this measurement is obtained by adding the value of this field to the value of the nr-RSCPD field.

[…]

#### 6.5.10.5	NR DL-TDOA Location Information Request

#### –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1),

jointMeasurementsReq-r18 (2)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-JointMeasurementRequested-r18		SEQUENCE (SIZE (2..3)) OF

INTEGER (0..nrMaxFreqLayers-1-r16)	OPTIONAL, -- Need ON

nr-DL-PRS-RxHoppingRequest-r18				ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-DL-PRS-RxHoppingTotalBandwidth-r18		CHOICE {

fr1											ENUMERATED {mhz40, mhz50, mhz80, mhz100},

fr2											ENUMERATED {mhz100, mhz200, mhz400}

}																			OPTIONAL, -- Need ON

nr-DL-PRS-RSCPD-Request-r18					ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRP-Pair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }		OPTIONAL  -- Need ON

]],

[[

timingReportingGranularityFactorExt-r18		INTEGER (6..7)					OPTIONAL, -- Need ON

nr-DL-PRS-MeasurementTimeWindowsConfig-r18

NR-DL-PRS-MeasurementTimeWindowsConfig-r18		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements. The jointMeasurementsReq means that the target device is requested to perform joint measurement across aggregated PFLs for DL-TDOA.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTEG-Request This field, if present, indicates that the target device is requested to provide the nr-UE-Rx-TEG-ID in IE NR-DL-TDOA-SignalMeasurementInformation.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-TDOA-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in fields nr-AdditionalPathList or nr-AdditionalPathListExt.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-DL-TDOA-SignalMeasurementInstances (in the case of UE-assisted mode is requested) or nr-DL-TDOA-LocationInformationInstances (in the case of UE-based mode is requested) in IE NR-DL-TDOA-ProvideLocationInformation.

nr-DL-PRS-JointMeasurementRequested This field, if present, indicates the target device is requested to perform joint measurements on the indicated two or three PFLs. The field can be present if jointMeasurementsReq in nr-RequestedMeasurements is set to one-value. Otherwise, it is absent. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on.

nr-DL-PRS-RxHoppingRequest This field, if present, indicates that the target device is requested to perform the RSTD measurement, RSRP measurement (if requested) and RSRPP measurement (if requested) using DL-PRS Rx hopping. This field is not included when nr-DL-PRS-RSCPD-Request is included.

nr-DL-PRS-RxHoppingTotalBandwidth This field, if present, indicates the total bandwidth of all hops in MHz.

nr-DL-PRS-RSCPD-Request This field, if present, indicates that the target device is requested to provide the RSCPD measurement together with DL-PRS RSTD measurement.

maxDL-PRS-RSTD-MeasurementsPerTRP-Pair This field specifies the maximum number of DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers. When requested for aggregated measurements by the location server, this field specifies the maximum number of aggregated DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers.

timingReportingGranularityFactor, timingReportingGranularityFactorExt This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) and value (6..7) corresponds to (kMinus1..kMinus2) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff. The timingReportingGranularityFactorExt should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactor is included.The timingReportingGranularityFactor should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactorExt is included.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-RxTEG-Request should also be present. When the location server requests aggregated measurements, the target device is requested to measure the same aggregated DL-PRS Resources of a TRP with N different UE Rx TEGs.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46]. When requested for aggregated measurements by the location server, this field indicates processing of reduced number of samples for the aggregated measurements.

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability. When requested for aggregated measurements by the location server, this field indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability for the aggregated measurements.

NR-DL-PRS-MeasurementTimeWindowsConfig This field indicates DL-PRS resource set(s) occurring within time window(s) for performing measurements where the time window is indicated by a start time, periodicity, offset and duration.

[…]

#### 6.5.12.4	NR Multi-RTT Location Information Elements

#### –	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...,

kMinus1-r18					INTEGER (0..3940097),

kMinus2-r18					INTEGER (0..7880193)

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-MeasBasedOnAggregatedResources-r18				ENUMERATED {true}			OPTIONAL,

nr-AggregatedDL-PRS-ResourceSetID-List-r18	SEQUENCE (SIZE (2.. 3)) OF

NR-AggregatedDL-PRS-ResourceSetID-Element-r18		OPTIONAL,

nr-RSCP-r18											INTEGER (0..3600)			OPTIONAL,

nr-PhaseQuality-r18									NR-PhaseQuality-r18			OPTIONAL,

nr-RSCP-AddSampleMeasurements-r18

SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF NR-RSCP-AdditionalMeasurements-r18

OPTIONAL,

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18

ENUMERATED { singleHop, multipleHop }		OPTIONAL,

nr-NTN-UE-RxTxTimeDiff-r18			NR-NTN-UE-RxTxTimeDiff-r18					OPTIONAL

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16					OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...,

kMinus1-r18						INTEGER (0..16382),

kMinus2-r18						INTEGER (0..32764)

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]],

[[

nr-MeasBasedOnAggregatedResources-r18	ENUMERATED {true}				OPTIONAL,

nr-AggregatedDL-PRS-ResourceSetID-List-r18	SEQUENCE (SIZE (2.. 3)) OF

NR-AggregatedDL-PRS-ResourceSetID-Element-r18		OPTIONAL,

nr-RSCP-r18											INTEGER (0..3600)			OPTIONAL,

nr-PhaseQuality-r18									NR-PhaseQuality-r18			OPTIONAL,

nr-RSCP-AdditionalMeasurementsAddSample-r18

SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF NR-RSCP-AdditionalMeasurements-r18

OPTIONAL,

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18

ENUMERATED { singleHop, multipleHop }		OPTIONAL,

nr-NTN-UE-RxTxTimeDiff-r18			NR-NTN-UE-RxTxTimeDiff-r18					OPTIONAL

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

NR-RSCP-AdditionalMeasurements-r18 ::= SEQUENCE {

nr-RSCP-ResultDiff-r18					INTEGER (0..3600)							OPTIONAL,

nr-PhaseQuality-r18						NR-PhaseQuality-r18							OPTIONAL,

nr-TimeStamp-r18						NR-TimeStamp-r16							OPTIONAL,

...

}

NR-NTN-UE-RxTxTimeDiff-r18 ::= SEQUENCE {

nr-NTN-UE-RxTxTimeDiffSubframeOffset-r18	INTEGER (0..542),

nr-NTN-DL-TimingDrift-r18					INTEGER (-265..265)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Case2-3 | The field is mandatory present if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice case3. Otherwise it is not present. |
| TEGCase2-3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase1-2 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case1 and case2. Otherwise it is not present. |

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-NTA-Offset This field provides the NTAoffset used by the target device as specified in TS 38.133 [46], Table 7.1.2-2. Enumerated values nTA1, nTA2, nTA3, and nTA4 correspond to NTAoffset of 25600 Tc, 0 Tc, 39936 Tc, and 13792 Tc, respectively.

nr-SRS-TxTEG-Set This field provides the SRS for Positioning Resources associated with a particular UE Tx TEG and comprises the following subfields: - nr-TimeStamp specifies the start time for which the NR-SRS-TxTEG-Element is valid. If this field is absent, the nr-TimeStamp of this instance of the NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set is the same as the nr-TimeStamp of the previous instance of the NR-SRS-TxTEG-Element. If this field is also absent in the first NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set, all NR-SRS-TxTEG-Element's provided are valid for the measurement period of the NR-Multi-RTT-SignalMeasurementInformation. - nr-UE-Tx-TEG-ID specifies the ID of this UE Tx TEG. - carrierFreq specifies the frequency of the SRS for positioning resources. - srs-PosResourceList specifies the SRS for Positioning Resources belonging to this UE Tx TEG. For each UE Tx TEG, there may be up to 8 changes (different nr-TimeStamp) of the TEG-SRS association information provided in nr-SRS-TxTEG-Set, i.e., the maximum value for maxTxTEG-Sets is 64.

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case3 and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-TxTEG-TimingErrorMargin This field specifies the UE Tx TEG timing error margin value for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case2 or case3 and this field is absent, the receiver should consider the UE Tx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-RxTxTEG-TimingErrorMargin This field specifies the UE RxTx TEG timing error margin value for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case1 or case2 and this field is absent, the receiver should consider the UE RxTx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 38.215 [36].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimeStamp This field specifies the time instance for which the measurement is performed. If RSCP measurement is present, the timestamp applies to both RSCP and UE Rx–Tx time difference measurement.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the measurement.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-Multi-RTT-AdditionalMeasurements This field provides up to 3 additional UE Rx-Tx time difference measurements corresponding to a single configured SRS Resource or Resource Set for positioning. Each measurement corresponds to a single received DL-PRS Resource or DL-PRS Resource Set [45]. If this field is present, the field nr-Multi-RTT-AdditionalMeasurementsExt shall be absent.

nr-UE-RxTx-TEG-Info This field provides the ID(s) of the UE TEG associated with the nr-UE-RxTxTimeDiff or nr-UE-RxTxTimeDiffAdditional measurement. One of the following combinations of TEG IDs can be provided: - case1 provides the UE RxTx TEG ID; - case2 provides the UE RxTx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on; - case3 provides the UE Rx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the TRP or resource. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-Multi-RTT-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-Multi-RTT-MeasElement, provides UE Rx-Tx time difference measurements of up to 4 DL-PRS Resources of a TRP with different UE RxTx or UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 measurement results with respect to different UE RxTx or UE Rx TEGs. If this field is present, the field nr-Multi-RTT-AdditionalMeasurements shall be absent.

nr-MeasBasedOnAggregatedResources This field indicates whether the UE RxTx Time Difference measurement, RSRP measurement (if included), and RSRPP measurement (if included) is based on aggregated DL-PRS Resources.

nr-AggregatedDL-PRS-ResourceSetID-List This field provides the PRS resource set IDs and the PRS resource IDs for the aggregated measurement which are used for RSRP/RSRPP and/or timing measurement results. If the field is present, the field nr-DL-PRS-ResourceID and nr-DL-PRS-ResourceSetID should not be included.

nr-RSCP This field specifies the NR DL reference signal carrier phase measurement, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].

nr-PhaseQuality This field specifies the target device′s best estimate of the quality of the RSCP measurement.

nr-RSCP-AddSampleMeasurements This field, in addition to the measurements provided in NR-Multi-RTT-MeasElement, provides up to 3 RSCP measurements associated with the nr-UE-RxTxTimeDiff in NR-Multi-RTT-MeasElement.

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx This field indicates that the reported measurement is based on receiving single or multiple hops of DL PRS.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result. The mapping of this field is defined as in TS 38.133 [46].

nr-UE-RxTxTimeDiffAdditional This field provides the additional UE Rx-Tx Difference measurement result relative to nr-UE-RxTxTimeDiff. The UE Rx-Tx Difference value of this measurement is obtained by adding the value of this field to the value of the nr-UE-RxTxTimeDiff field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the resource. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

nr-RSCP-AdditionalMeasurements This field, provides up to 4 RSCP measurements associated with the UE Rx-Tx Time Difference measurement in NR-Multi-RTT-MeasElement.

nr-RSCP-ResultDiff This field provides the additional RSCP measurement result relative to nr-RSCP. The RSCP value of this measurement is obtained by adding the value of this field to the value of the nr-RSCP field.

nr-NTN-UE-RxTxTimeDiff This field provides the offset of the UE Rx–Tx time difference measurement for NTN and comprises the following subfields: - nr-NTN-UE-RxTxTimeDiffSubframeOffset specifies the UE Rx – Tx time difference subframe offset measurement in unit of subframe, as defined in TS 38.215 [36]. - nr-NTN-DL-TimingDrift specifies the DL timing drift measurement, as defined in TS 38.215 [36]. The granularity of nr-NTN-DL-TimingDrift is 0.1 ppm. Values are given in unit of corresponding granularity.

#### 6.5.12.5	NR Multi-RTT Location Information Request

#### –	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1),

jointMeasurementsReq-r18 (2)} (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17

ENUMERATED { requested, ... }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL, -- Need ON

multiMeasInSameReport-r17			ENUMERATED { requested }			OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { requested }			OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-RxHoppingRequest-r18				ENUMERATED { requested }	OPTIONAL,	-- Need ON

nr-DL-PRS-RxHoppingTotalBandwidth-r18		CHOICE {

fr1				ENUMERATED {mhz40, mhz50, mhz80, mhz100},

fr2											ENUMERATED {mhz100, mhz200, mhz400}

}																		OPTIONAL,	-- Need ON

timingReportingGranularityFactorExt-r18		INTEGER (6..7)				OPTIONAL,	-- Need ON

nr-DL-PRS-JointMeasurementRequested-r18		SEQUENCE (SIZE (2..3)) OF

INTEGER (0..nrMaxFreqLayers-1-r16)	OPTIONAL,	-- Need ON

nr-UE-RSCP-Request-r18						ENUMERATED { requested }	OPTIONAL,	-- Need ON

nr-DL-PRS-MeasurementTimeWindowsConfig-r18

NR-DL-PRS-MeasurementTimeWindowsConfig-r18	OPTIONAL	-- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16	INTEGER (0..5)					OPTIONAL  -- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiffMeasurementInfoRequest This field, if present, indicates that the target device is requested to report the DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) associated with the DL-PRS Resources(s) or the DL-PRS Resource Set(s) which are used in determining the UE Rx-Tx time difference measurements.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RxTxTimeDiffMeasPerTRP This field specifies the maximum number of UE-Rx-Tx time difference measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP.

timingReportingGranularityFactor, timingReportingGranularityFactorExt This field specifies the recommended reporting granularity for the UE Rx-Tx time difference measurements. Value (0..5) corresponds to (k0..k5) and value (6..7) corresponds to (kMinus1..kMinus2) used for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional in NR-Multi-RTT-MeasElement. The UE may select a different granularity value for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional. The timingReportingGranularityFactorExt should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactor is included. The timingReportingGranularityFactor should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactorExt is included.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTxTEG-Request This field, if present, indicates that the target device is requested to provide the NR-UE-RxTx-TEG-Info in IE NR-Multi-RTT-SignalMeasurementInformation. Enumerated value 'case1' indicates that the target device is requested to provide the case1 choice in NR-UE-RxTx-TEG-Info, enumerated value 'case2' indicates that the target device is requested to provide the case2 choice in NR-UE-RxTx-TEG-Info, and so on.

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE RxTx TEGs and with the same UE Tx TEG. Enumerated value 'n0' indicates that the number N of different UE RxTx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE RxTx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE RxTx TEGs, and so on. When the location server requests aggregated measurements, a request for configuring the target device to measure the same aggregated DL-PRS Resources of a TRP with N different UE RxTx TEGs. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTEGs should not be present.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. When the location server requests aggregated measurements, a request for configuring the UE to measure the same aggregated DL-PRS Resources of a TRP with N different UE Rx TEGs. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTxTEGs should not be present.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46]. When requested for aggregated measurements by the location server, this field indicates processing of reduced number of samples for the aggregated measurements.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-Multi-RTT-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in the field nr-AdditionalPathList or nr-AdditionalPathListExt.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-Multi-RTT-SignalMeasurementInstances in IE NR-Multi-RTT-ProvideLocationInformation.

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability. When requested for aggregated measurements by the LMF, this field indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability for the aggregated measurements.

nr-DL-PRS-RxHoppingRequest This field, if present, indicates that the target device is requested to perform the UE Rx-Tx Time Difference measurement, RSRP measurement (if requested) and RSRPP measurement (if requested) using DL-PRS Rx hopping. This field is not included when nr-UE-RSCP-Request is included.

nr-DL-PRS-RxHoppingTotalBandwidth This field, if present, indicates the total bandwidth of all hops in MHz.

nr-DL-PRS-JointMeasurementRequested This field, if present, indicates that the target device is requested to perform joint measurements on the indicated two or three PFLs that are linked for DL PRS BW aggregation. The field can be present if jointMeasurementsReq in nr-RequestedMeasurements is set to one-value. Otherwise, it is absent. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on.

nr-UE-RSCP-Request This field, if present, indicates that the device is requested to provide the DL RSCP measurement in IE NR-Multi-RTT-SignalMeasurementInformation together with nr-UE-RxTxTimeDiff.

nr-DL-PRS-MeasurementTimeWindowsConfig This field indicates DL-PRS resource set(s) occurring within time window(s) for performing measurements where the time window is indicated by a start time, periodicity, offset and duration.