3GPP TSG-RAN WG2 Meeting #125	R2-2401257

Athens, Greece, February 26 – March 1, 2024

Agenda item:	7.24.2

Source: 	Qualcomm Incorporated

Title: 	[RIL Q033] localOrigin-r18 definition is not in agreement with TS 23.032/29.572

Document for:		Discussion and Decision

# 1.	Discussion

SA2 added two local GAD shapes to TS 23.032 which are also supported in LPP:

- Local 2D point with uncertainty ellipse (Clause 5.8 in TS 23.032)

- Local 3D point with uncertainty ellipsoid (Clause 5.9 in TS 23.032)

The structure of local 2D point with uncertainty ellipse and local 3D point with uncertainty ellipsoid is defined in clause 6.1.6.2.38 in TS 29.572 and in clause 6.1.6.2.39 in TS 29.572, respectively, as follows:

Definition of type Local2dPointUncertaintyEllipse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE". |
| localOrigin | LocalOrigin | M | 1 | Indicates the local origin in the local Cartesian co-ordinates system configured by the PLMN operator. |
| point | RelativeCartesianLocation | M | 1 | Indicates a 2D-point (specified by "x" and "y" coordinates) relative to origin in refenrece system. |
| uncertaintyEllipse | UncertaintyEllipse | M | 1 | Indicates an uncertainty ellipse. |
| confidence | Confidence | M | 1 | Indicates the value of confidence. |

Definition of type Local3dPointUncertaintyEllipsoid

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID". |
| localOrigin | LocalOrigin | M | 1 | Indicates the local origin in the local Cartesian co-ordinates system configured by the PLMN operator. |
| point | RelativeCartesianLocation | M | 1 | Indicates a 3D-point (specified by "x", "y" and "z" coordinates) relative to origin in refenrece system. |
| uncertaintyEllipsoid | UncertaintyEllipsoid | M | 1 | Indicates the uncertainty ellipsoid |
| confidence | Confidence | M | 1 | Indicates the value of confidence. |

The above structure has been translated into ASN.1 for LPP in IEs Local2dPointWithUncertaintyEllipse and Local3dPointWithUncertaintyEllipsoid. For example:

Local2dPointWithUncertaintyEllipse-r18 ::= SEQUENCE {

localOrigin-r18								ReferencePoint-r16,

cartesianCoordinatesUnits-r18				ENUMERATED { mm, cm, dm, m, ...},

x-value-r18									X-Value-r18,

y-value-r18									Y-Value-r18,

uncertaintySemiMajor-r18					INTEGER (0..127),

uncertaintySemiMinor-r18					INTEGER (0..127),

orientationMajorAxis-r18					INTEGER (0..179),

confidence-r18								INTEGER (0..100)

}

The localOrigin-r18 above refers to the IE ReferencePoint-r16, which is defined as follows:

-- ASN1START

ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16		CHOICE {

location3D-r16			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...,

localOrigin-v1800		CoordinateID-r18

},

...

}

-- ASN1STOP

Coordinate ID is defined in LPP as follows:

-- ASN1START

CoordinateID-r18 ::= VisibleString (SIZE (1..256))

-- ASN1STOP

However, the localOrigin is defined in clause 6.1.6.2.36 of TS 29.572 as follows:

Definition of type LocalOrigin

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| coordinateId | string | M | 1 | This IE defines a known reference point which configured by the PLMN operator |
| point | GeographicalCoordinates | O | 0..1 | Indicates a geographic point represented by its longitude and latitude. |

Or as OpenAPI in YAML format:

LocalOrigin:

description: Indicates a Local origin in a reference system

type: object

properties:

coordinateId:

type: string

point:

$ref: '#/components/schemas/GeographicalCoordinates'

The "point" refers to the ellipsoid point (GeographicalCoordinates) defined in clause 6.1.6.2.4 of TS 29.572:

Definition of type GeographicalCoordinates

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| lon | number | M | 1 | Longitude (Double-precision float value): Format: double Minimum: -180 Maximum: 180 |
| lat | number | M | 1 | Latitude (Double-precision float value): Format: double Minimum: -90 Maximum: 90 |

Point:

description: Ellipsoid Point.

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

Therefore, the definition of "point" in the LPP localOrigin is missing.

# 2.	Proposed Solution

Change the IE CoordinateID-r18 into a LocalOrigin definition analogous to clause 6.1.6.2.36 of TS 29.572:

#### –	LocalOrigin

The IE LocalOrigin is used to indicate a reference point that defines the origin of a local Cartesian Coordinate System as defined in TS 23.032 [15].

-- ASN1START

LocalOrigin-r18 ::= SEQUENCE {

coordinateID-r18		VisibleString (SIZE (1..256)),

point-r18				Ellipsoid-Point						OPTIONAL,

...

}

-- ASN1STOP

#### –	ReferencePoint

The IE ReferencePoint provides a well-defined location relative to which other locations may be defined.

-- ASN1START

ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16		CHOICE {

location3D-r16			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...,

localOrigin-v1800		LocalOrigin-r18

},

...

}

-- ASN1STOP

A corresponding TP is provided in the Annex of this contribution.

# 3. Summary

This contribution discussed some misalignment between LPP and TS 23.032/29.572 with respect to the "local origin" definition.

Proposal: Agree the TP in the Annex of this contribution.

# Annex: TP for 37.355

### 6.4.3	Common NR Positioning Information Elements

#### –	AreaID-CellList

The IE AreaID-CellList provides the NR Cell-IDs of the TRPs belonging to a particular network area where the associated assistance data are valid. Each cell is included in only one area.

-- ASN1START

AreaID-CellList-r17 ::= SEQUENCE (SIZE(1..maxCellIDsPerArea-r17)) OF NR-Cell-IDs-r17

NR-Cell-IDs-r17 ::= SEQUENCE {

nr-CellGlobalID-r17				NCGI-r15					OPTIONAL,	-- Need ON

nr-PhysCellID-r17				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-ARFCN-r17					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

AreaID-CellList field descriptions

nr-CellGlobalID This field specifies the NR Cell Global ID of the TRP belonging to a particular network area where the associated assistance data are applicable.

nr-PhysCellID This field specifies the physical cell identity of the TRP belonging to a particular network area where the associated assistance data are applicable.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

#### –	DL-PRS-ID-Info

The IE DL-PRS-ID-Info provides the IDs of the reference TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-ID-Info-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-DL-PRS-ResourceID-List-r16	SEQUENCE (SIZE (1..nrMaxResourceIDs-r16)) OF

NR-DL-PRS-ResourceID-r16

OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16

OPTIONAL  -- Need ON

}

-- ASN1STOP

DL-PRS-ID-Info field descriptions

nr-DL-PRS-ResourceID-List This field provides a list of DL-PRS Resource IDs under the same DL-PRS Resource Set.

#### –	LCS-GCS-TranslationParameter

The IE LCS-GCS-TranslationParameter provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44].

-- ASN1START

LCS-GCS-TranslationParameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..359),

alpha-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

beta-r16						INTEGER (0..359),

beta-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

gamma-r16						INTEGER (0..359),

gamma-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| AzElFine | The field is mandatory present if the angles where this IE is used are provided with 0.1 degrees resolution; otherwise it is not present. |

LCS-GCS-TranslationParameter field descriptions

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 1 degree; range 0 to 359 degrees.

alpha-fine This field provides finer granularity for the alpha. The total bearing angle α is given by alpha + alpha-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

beta This field specifies the downtilt angle β for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 1 degree; range 0 to 359 degrees.

beta-fine This field provides finer granularity for the beta. The total downtilt angle β is given by beta + beta-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 1 degree; range 0 to 359 degrees.

gamma-fine This field provides finer granularity for the gamma. The total slant angle γ is given by gamma + gamma-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

#### –	LocalOrigin

The IE LocalOrigin is used to indicate a reference point that defines the origin of a local Cartesian Coordinate System as defined in TS 23.032 [15].

-- ASN1START

LocalOrigin-r18 ::= SEQUENCE {

coordinateID-r18		VisibleString (SIZE (1..256)),

point-r18				Ellipsoid-Point						OPTIONAL,

...

}

-- ASN1STOP

[…]

#### –	ReferencePoint

The IE ReferencePoint provides a well-defined location relative to which other locations may be defined.

-- ASN1START

ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16		CHOICE {

location3D-r16			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...,

localOrigin-v1800		LocalOrigin-r18

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic or local location of the reference point.

localOrigin This field provides an identifier for the reference point that defines the origin of a local Cartesian coordinate system [15].