3GPP TSG-RAN2 Meeting #125	R2-2401348

Athens, Greece, 26th February - 1st March 2024

Agenda Item:	6.1.3.1 NR RRC

Source: 	Ericsson

Title:  	Open issues RLM/BFD relaxation

Document for:	Discussion and Decision

# Introduction

The following open issues for RLM/BFD relaxation are discussed:

- RLM/BFR measurement relaxation state reporting

- Short DRX

# Discussion

## RLM/BFR measurement relaxation state reporting

Background

According to 38.331 the UE is allowed to relax RLM/BFD measurements when the UE is stationary and/or in good coverage. However, in 38.133 additional conditions are defined e.g. the DRX cycle has to be no longer than 80 ms and “no DRX is used” as defined in section 3.6.1 in 38.133.

The 38.133 requirements make relaxation dependent on the DRX state i.e. the UE is only allowed to relax when the UE is outside Active Time or during the OnDuration. When the measurements are relaxed, the UE is allowed to take longer time to do the required number of RLM/BFD measurements. But if the DRX state changes frequently, there is little power saving gain for the UE. Furthermore, there is a problem with the relaxation state reporting in RAN2, i.e. when the UE is only allowed to relax outside Active Time, then when the relaxation state changes to “relaxed” during DRX, this will trigger the UE to go to Active Time where relaxation is not allowed.

RAN2#124

Possible corrections in RAN2 [1,2] were postponed pending further progress in RAN4.

RAN4#109

RAN4 reached the following agreement (RRM session report):

Agreement:

Agree the CR, continue clarifying the meaning of “no DRX” in the RLM/BFD table and other relevant tables (if any) in NR specification. Further change on the change made by the CR is allowed if there will an agreement on the generic meaning of “no DRX” in TS 38.133 can be made in the future.

And the following corrections to 38.133 were agreed [3]:

<Start of Change 1>

The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The timer T310 is running.

-	No DRX is configured or configured DRX cycle is longer than 80ms

<End of Change 1>

<Start of Change 2>

The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

-	The timer beamFailureDetectionTimer is running.

-	No DRX is configured or configured DRX cycle is longer than 80ms

<End of Change 2>

Based on the RAN4 corrections the following observation can be made:

RLM and BFD measurement relaxation does not depend on the DRX state, but it depends on the DRX configuration.

In clause 5.7.13 in 38.331 it is specified, that dependent on the low mobility/good serving cell criterion configuration, the UE is allowed to relax according to requirements specified in TS 38.133 [14]:

In case both low mobility criterion and good serving cell criterion are configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when both relaxed measurement criterion for low mobility and relaxed measurement criterion for good serving cell quality are met.

In case only the good serving cell quality criterion is configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when the relaxed measurement criterion for good serving cell quality is met.

Thus no further clarifications are needed in RAN2 specification w.r.t. RLM/BFD relaxation state reporting:

No further clarifications are needed in RAN2 specification w.r.t. RLM/BFD relaxation state reporting.

## Short DRX

Short DRX is an optional feature, which enables the configuration of a longer sleeping time (longer drx-Longcycle) while keeping a quick response time while drx-ShortTimer is running and drx-Shortcycle is used. It is possible to configure a drx-Shortcycle shorter than 80 ms (relaxation allowed) and a drx-Longcycle longer than 80 ms (relaxation is not allowed because a DRX cycle of 80 ms already provides sufficient UE power saving gains):

From a UE power saving and relaxation state reporting perspective it is preferred that RLM/BFD measurement relaxation state does not depend on the DRX state (e.g. when short DRX is configured).

Furthermore with Short DRX, as the naming indicates, the UE is typically only in Short DRX cycle for a short time, and most of the time in Long DRX cycle. Therefore allowing the UE to relax during the Short DRX cycle gives minor power saving, and most of the power savings are anyways received when the Long DRX cycle is larger than 80 ms:

RAN2 assumes that “configured DRX cycle is longer than 80ms” in clause 8.1.1.1 and 8.5.1.1 in 38.133 refers to the “drx-Longcycle” when short DRX cycle is configured.

If proposal 2 is agreeable RAN2 should inform RAN4 about the RAN2 assumption.

If RAN4 does not see a problem with the RAN2 assumption, then this could be captured in chapter 11 UE power saving in 38.300:

UE power saving may be achieved by UE relaxing measurements for RLM/BFD. When configured, UE determines whether it is in low mobility state and/or whether its serving cell radio link quality is better than a threshold. The configuration for low mobility and good serving cell quality criterion is provided through dedicated RRC signalling.

RLM and BFD relaxation may be enabled/disabled separately through RRC Configuration. Additionally, RLM relaxation may be enabled/disabled on per Cell Group basis while BFD relaxation may be enabled/disabled on per serving cell basis.

The UE is only allowed to perform RLM and/or BFD relaxation when relaxed measurement criterion for low mobility and/or for good serving cell quality is met. If configured to do so, the UE shall trigger reporting of its RLM and/or BFD relaxation status through UE assistance information if the UE changes its respective RLM and/or BFD relaxation status while meeting the UE minimum requirements specified in TS 38.133 [13]. When short DRX is configured then the configured DRX cycle in clause 8.1.1.1 and 8.5.1.1 in TS 38.133 [13] refers to the long DRX cycle.

# Summary

RAN2 is kindly asked to discuss the open issues with RLM/BFD measurement relaxation:

Observation 1	RLM and BFD measurement relaxation does not depend on the DRX state, but it depends on the DRX configuration.

Observation 2	From a UE power saving and relaxation state reporting perspective it is preferred that RLM/BFD measurement relaxation state does not depend on the DRX state (e.g. when short DRX is configured).

Proposal 1	No further clarifications are needed in RAN2 specification w.r.t. RLM/BFD relaxation state reporting.

Proposal 2	RAN2 assumes that “configured DRX cycle is longer than 80ms” in clause 8.1.1.1 and 8.5.1.1 in 38.133 refers to the “drx-Longcycle” when short DRX cycle is configured.

# References

- R2-2312030, Correction on RLM/BFD relaxation state reporting, CATT, Nokia, CR 38.331, RAN2#124

- R2-2312958, RLM and BFD relaxation state reporting, Ericsson, DISC, RAN2#124

- R4-2321620, CR on maintaining RLM/BFD relaxation requirements R17, CR 38.133, Rel-17, Huawei et all, RAN4#109