3GPP TSG-RAN WG2 #125	Tdoc R2-2401360

Athens, Greece, 26 Feb – 1 Mar, 2024

Agenda Item:	7.25.1.7

Source:	Ericsson

Title:	Support of Enhanced channel raster for (e)RedCap

Document for:	Information, Discussion

# 1	Introduction

In R4-2401840 [1], it is proposed that support of the enhanced channel raster is mandated for all RedCap UEs from Rel-17. If agreed in RAN4, we can expect to receive an LS in RAN2. Due to the urgency to finalize this, we provide in this document text proposal to illustrate the possible impact to TS 38.306.

# 2	Discussion

RAN4 have earlier indicated the need to introduce UE capability for the Rel-18 WI Enhanced Channel Raster, see [2], [3], [4], [5].

But mandatory support with a UE capability introduced in Rel-18 signalling seems not desirable for Rel-17 RedCap UE, given that RedCap UE is also introduced in Rel-17. Instead, provided that RAN4 agrees on that support of the enhanced channel raster is mandated for all RedCap UEs from Rel-17, we propose the text in Annex is used to capture this requirement.

Use TP for TS 38.306 in annex to capture mandatory support of enhanced channel raster for Rel-17 RedCap UE.

Similar TP is introduced also for eRedCap UE in rel-18 TS 38.306.

If agreed in RAN4 and RAN2, Ericsson will provide the CRs to Rel-17/18 TS 38.306.

# Conclusion

Based on the discussion in this document we propose the following:

Proposal 1	Use TP for TS 38.306 in annex to capture mandatory support of enhanced channel raster for Rel-17 RedCap UE.

Proposal 2	Similar TP is introduced also for eRedCap UE in rel-18.

If agreed in RAN4 and RAN2, Ericsson will provide the CRs to Rel-17/18 TS 38.306.

References

[1]	R4-2401840 	(NR_channel_raster_enh-Core) Definition of the enhanced raster and support for RedCap from Rel-17, Ericsson

[2]	R2-2311758 	LS on a capability for channel raster enhancement (R4-2317773; contact: Ericsson)   RAN4

[3]	R2-2313624	LS on RAN4 UE feature list for Rel-18 (version 2) (R4-2321823; contact: CMCC)

[4]	R2-2400841 UE capability for Enhanced channel raster, Ericsson, CR, Rel-18 38.331

[5]	R2-2400842   UE capability for Enhanced channel raster       Ericsson, CR, Rel-18 38.306

### ANNEX A: TP to 38.306 v17.7.0

<Start of changes>

### 4.2.21	RedCap Parameters

#### 4.2.21.1	Definition of RedCap UE

RedCap UE is the UE with reduced capability:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	The maximum mandatory supported DRB number is 8;

-	The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

-	The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as other UEs, unless indicated otherwise.

RedCap UE supports the channel raster as specified in TS 38.101-1 [2], clause 5.4I, for all bands supported by the UE.

<End of changes>

### 4.2.22	eRedCap Parameters

#### 4.2.22.1	Definition of eRedCap UE

eRedCap UE is the UE with reduced peak data rate and, with or without reduced baseband bandwidth in FR1:

-	The maximum bandwidth is 20 MHz for FR1. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 are not supported by eRedCap UEs. eRedCap UEs do not support operation in FR2 and in FR1 60kHz SCS.

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the eRedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by eRedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for eRedCap UEs same as other UEs, unless indicated otherwise.

eRedCap UE supports Enhanced channel raster as specified in TS 38.101-1, TS 38.101-4 and TS 38.101-5 for all applicable bands supported by the UE.