3GPP TSG-RAN WG2 Meeting #125	R2-2401367

# Athens, Greece, 26 February – 1 March, 2024

Agenda Item:	7.5.3.3

Source:	Ericsson

Title:	Discussion on Delay-critical PDCP data

Document for:	Discussion, Decision

# 1	Introduction

In RAN2# 123-bis [1] details on the DSR was agreed:

Agreements on DSR

1. For triggering DSR, the shortest remaining-time left for the buffered data in UL is smaller than a configured threshold is used, if there is no pending DSR associated for that LCG.

2. One threshold per LCG for triggering purposes is enough for delay status report

3. The data volume calculation to be reported in the DSR will consider the at size of the full remaining PDUs in the PDU set (if any PDU within the PDU set is with remaining time below the threshold), if the PDU set discard is configured. FFS what to report for the case of not PDU set discard configured

4. Support single delay information per LCG as baseline for Rel-18 DSR. The remaining time (the shortest remaining time in the LCG) will be explicitly reported in the DSR.

This contribution raises an issue related to the implementation of the agreements and provide a solution for it.

# 2	Discussion on Delay-critical PDCP data

## 2.1	Issue with current calculation

In current specification (PDCP v18.0 [2]) the data volume reported in DSR is called delay-critical PDCP data volume and is only calculated based on the legacy discard timer (discardTimer). However, when PSI discarding is configured there is PDCP SDUs that will have another timer running (discardTimerForLowImportance). Thus, the low importance packets will be left out of the delay critical data volume calculation.

One reason for this solution was to avoid too frequent DSR reports when the PSI discarding is activated. However, there was some aspect that was overlooked with this solution. The most critical one is related to the situation when the low important data is still in the buffer queue and therefore will use the grant resources. For simplicity focus can be on the scenario where only one logical channel has data, but the issue is not limited to this scenario. In this case if the DSR only contains information about the high importance packets (as in the current solution) then the network will likely give grants corresponding to that size (assuming this is the logical channel with high priority and high data rate, which should be the case when configured with DSR). This will be a problem if the low importance packets are first in the queue since those will then use up the granted resources first from this logical channel and the high importance packets will have less chance to be transmitted on time. If network was provided with more precise information in the DSR it could potentially have given a larger grant that would fit both the low and high importance data from this logical channel.

- Current solution can lead to inaccurate scheduling and potential delays of high importance data.

## 2.2	Possible solutions to the issue

In essence there could be multiple solutions on the table to solve this interoperability issue. Following sections go over a few of the possibilities.

### 2.2.1 Redefining the definition of Delay-critical PDCP SDU

A simple solution that solves the issue with network not knowing how large the data size is in the delay critical threshold. In this way the DSR will always report all the data that is below the threshold, i.e. by redefining the definition of “Delay Critical PDCP SDU” in the PDCP specification to also account for Low PSI PDU Sets. Below we provide an example text:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer or discardTimerForLowImportance , expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is  configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer or discardTimerForLowImportance  expiry less than the remainingTimeThreshold.

One consequence with including the low importance data in the delay critical data volume calculation is that it will open an issue of what to report in the ‘remaining time’ field in the DSR. Considering that low importance PDU Sets can have a shorter “remaining time until discard” than other PDU Sets in the buffer. Again for network scheduling purpose it would be beneficial to have accurate information of this in the scheduler. Thus to solve this the definition of the ‘remaining time’ field in the DSR can be changed to the following:

Remaining Time: This field indicates the shortest remaining value of PDCP discardTimer or PDCP discardTimerForLowImportance (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs buffered for an LCG, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. The value r in this field indicates a remaining time within the range of (r, r + 1] msec

Reporting with this changed remaining time will however not match selection of data from the logical channel as that is done by the queue order. Going back to the main problematic scenario described in 2.1 it is when low importance data use up the grant resources and unnecessary extra delay may be introduced to the delivery of the high importance data. Essentially the scenario when the low importance PDU Set is first in the queue and higher importance PDU Set is later in the queue. To target this issue specifically, conditional reporting could be introduced.

- One possible solution is to redefine the definition of Delay-critical PDCP SDU and Remaining Time.

### 2.2.2 Conditional reporting

Instead of simply redefining the delay critical data and remaining time, there could be conditions when the low importance data should be included in the delay critical volume. Such condition could be that only when the low importance data is first in the queue of a logical channel (i.e. it will use up any grant resources that this logical channel can use) it will be incorporated in the delay critical volume calculation. Furthermore, when first in the queue the remaining time value provided can be calculated on the corresponding low importance timer, since this will give a truthful value of the remaining time when the first discarding will happen. In this way the handling will be same as the case without PSI discarding and there are multiple PDU Sets in the buffer, i.e. since DSR only reports one remaining time value corresponding only to the most time critical PDU Set will be reported and not any value for the following PDU Set. In the same way as the DSR would be helpful with multiple reported remaining time values this would also be helped by that.

- Another solution is to make it conditional when low importance data is included in the Delay-critical data volume.

### 2.2.3 Indicating inclusion of low importance data

A more flexible solution would be to include an explicit indication in the DSR if there is any low importance data included in the delay-critical data volume. This could be done by utilizing the R-bit in the DSR format. For example the R-bit could indicate if low importance is first in the queue for this logical channel, thus the network can take this into consideration when doing the scheduling.

- A more flexible solution is to include indication in the DSR what importance data is included in the report.

### 2.2.4 Summary of solutions

In summary there are multiple possible solutions to improve the current issue of the interoperability between the DSR and PSI discarding features. There are different benefits with each solution, redefining can be a simpler from standard point but doesn’t fully solve the problem of unclear information to the scheduler, while introducing conditions provide better information but can be more complex. It is important that RAN2 take these aspects into consideration when discussing which solution that is preferred.

RAN2 to decide on a solution that is preferred to solve the interoperability issue.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Current solution can lead to inaccurate scheduling and potential delays of high importance data.

Observation 2	One possible solution is to redefine the definition of Delay-critical PDCP SDU and Remaining Time.

Observation 3	Another solution is to make it conditional when low importance data is included in the Delay-critical data volume.

Observation 4	A more flexible solution is to include indication in the DSR what importance data is included in the report.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to decide on a solution that is preferred to solve the interoperability issue.

# 4 References

- RAN2#123-bis ChairNotes, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_123bis/Inbox/Chair_Notes/R2_123bis_ChairNotes_23-10-13_13-00_final.docx

- TS 38.323, https://www.3gpp.org/ftp/Specs/2023-12/Rel-18/38_series/38323-i00.zip