3GPP TSG-RAN WG2 Meeting #125	R2-2401451

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda item:	7.9.4

Source: 	Huawei, HiSilicon

Title: 	SRAP corrections on L2 U2U relay operation

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the changes that are needed for the SRAP specification TS 38.351 to clarify how the U2U remote UE and the U2U relay UE determine the egress RLC channel during L2 U2U relay operation and to align the description in SRAP specification with the description in RRC specification TS 38.331.

# Discussion

According to RRC specification

- A remote UE or a relay UE in RRC_CONNECTED state determines the egress RLC channel based on the SRAP configuration from gNB.

- A remote UE or a relay UE in RRC_IDLE/INACTIVE/OOC state determines the the egress RLC channel based on the mapping between SL-DRB and PC5 RLC channel that is derived by itself.

However, such description for U2U remote and U2U relay UE is missing in SRAP specification hence we propose to add this description in SRAP specification to align it with RRC specification.

Proposal 1 : It is proposed to clarify how the U2U remote UE and the U2U relay UE determine the egress RLC channel during L2 U2U relay operation and to align the description in SRAP specification with the description in RRC specification TS 38.331 as shown in the text proposal in the Annex

# 3	Conclusion

Proposal 1 : It is proposed to clarify how the U2U remote UE and the U2U relay UE determine the egress RLC channel during L2 U2U relay operation and to align the description in SRAP specification with the description in RRC specification TS 38.331 as shown in the text proposal in the Annex

# Annex

START OF CHANGES

### 5.3a.1	Transmitting operation of U2U Remote UE

#### 5.3a.1.1	General

The transmitting part of the SRAP entity on the PC5 interface of U2U Remote UE can receive SRAP SDU from upper layer and constructs U2U SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the UE ID fields and BEARER ID field in accordance with clause 5.3a.1.2;

-	Construct an U2U SRAP Data PDU with SRAP header, where the UE ID fields and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3a.1.3;

-	Submit this U2U SRAP Data PDU to the determined egress PC5 Relay RLC channel.

#### 5.3a.1.2	UE ID fields and BEARER ID field determination

For an U2U SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID (for SRC) field corresponding to sl-RemoteUE-LocalIdentity and UE ID (for DST) field corresponding to sl-PeerRemoteUE-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field for SL-SRBs as the fixed value (i.e., set 0/1/2/3 for SL-SRB0/1/2/3 respectively) or for SL-DRBs as the 5 LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure as specified in TS 38.331 [3].

#### 5.3a.1.3	Egress RLC channel determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the U2U SRAP Data PDU is for SRB (i.e., the BEARER ID field is 0/1/2/3):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3].

-	Else if the SRAP Data is for DRB:

-	If the mapping between DRB and PC5 RLC channel is configured in sl-MappingToAddMod-U2U-List as specified in TS 38.331 [3],

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned bearer as specified in TS 38.331 [3].

-	Else if the mapping between DRB and PC5 RLC channel is derived by U2U Remote UE itself based on SIB12/SidelinkPreconfigNR as specified in TS 38.331 [3],

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to the RLC channel associated with the concerned bearer as specified in TS 38.331 [3].

NEXT CHANGE

### 5.3a.3	Transmitting operation of U2U Relay UE

#### 5.3a.3.1	General

The transmitting part of the SRAP entity of U2U Relay UE on the PC5 interface between the U2U Relay UE and the peer U2U Remote UE can receive SRAP data packets from the receiving part of the SRAP entity of the same U2U Relay UE on the PC5 interface between the U2U Relay UE and the U2U Remote UE, and constructs U2U SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity of the U2U Relay UE has an U2U SRAP Data PDU to transmit on the PC5 interface between U2U Relay UE and the peer U2U Remote UE, the transmitting part of the SRAP entity of U2U Relay UE shall:

-	Determine the egress link in accordance with clause 5.3a.3.2;

-	Determine the egress RLC channel in accordance with clause 5.3a.3.3;

-	Submit this U2U SRAP Data PDU to the determined egress RLC channel of the determined egress link.

#### 5.3a.3.2	Egress link determination

For a U2U SRAP Data PDU to be transmitted, SRAP entity shall:

-	Determine the egress link on PC5 interface towards the peer U2U remote UE based on the UE ID fields in the U2U SRAP Data PDU.

#### 5.3a.3.3	Egress RLC channel determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the U2U SRAP Data PDU is for SRB (i.e., the BEARER ID field is 0/1/2/3):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3].

-	Else if the U2U SRAP Data PDU is for DRB:

-	If the mapping between DRB and PC5 RLC channel is configured in sl-MappingToAddMod-U2U-List as specified in TS 38.331 [3],

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned bearer as specified in TS 38.331 [3].

-	Else if the mapping between DRB and PC5 RLC channel is derived by U2U Relay UE itself based on SIB12/SidelinkPreconfigNR as specified in TS 38.331 [3],

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to the RLC channel associated with the concerned bearer as specified in TS 38.331 [3].

END OF CHANGE