3GPP TSG-RAN WG2 Meeting #125	R2-2401474

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.24.2.2

Source: 	OPPO

Title:  	Discussion on beam failure recovery for SDT

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to further discuss the open issues for SDT beam failure recovery, i.e., how to avoid frequent RA.

Discussion

In last RAN2 meeting, beam failure recovery for RA-SDT was discussed in R18 TEI agenda. It was proposed that a UE can trigger RACH procedure if the RSRP value of the current SSB is lower than a pre-configured threshold during an ongoing RA-SDT procedure, which is similar to CG-SDT.

While for the proposed solution, concerns were raised that the approach may cause excessive RA. As a conclusion from last meeting, further study is needed for SDT beam failure recovery solution without leading to frequent RA.

=> We will adopt a solution to deal with the beam failure recover in Rel-18, but wait for next meeting to determine how we need to deal with frequent RA

For the conditions to trigger beam failure recovery during RA-SDT, the beam quality of both current SSBs and other SSBs shall be taken into account:

- For current SSB: Whether the beam quality of current SSB is getting worse (e.g., the SS-RSRP is lower than an RSRP threshold) shall be checked.

- For other SSBs: The beam failure recovery shall be triggered when there is at least one of other SSBs getting better (e.g., the SS-RSRP is larger than an RSRP threshold), otherwise, a followed-up RA for BFR will be triggered if only current beam is considered to declare a beam failure.

UE triggers RACH for beam failure recovery during an ongoing RA-SDT if the following conditions are met:

      - The SS-RSRP of current SSB is lower than a threshold and lasts for a preconfigured duration; and,

      - There is at least one of the SSBs with SS-RSRP above the threshold

For the RSRP threshold, the configuration can be provided as follows:

- Option1: A new RSRP threshold in SDT configuration which is specific for beam failure recovery purpose.

- Option2: Reuse the rsrp-ThresholdSSB in RA-SDT configuration.

- Opton3: Reuse the rsrp-ThreholdSSB in non-SDT RA configuration.

For option3, additional discussions are required on the rsrp-ThresholdSSB from which non-SDT RA configuration is applied considering that there may be more than one set of non-SDT RA configurations, e.g., on NUL and SUL. Both Option1 and Option can work and we are fine with either.

For the RSRP threshold configuration, rsrp-ThresholdSSB in RA-SDT configuration can be reused, or, a new RSRP threshold can be introduced in SDT configuration in SIB.

Conclusion

Based on the discussion in section 2 we have following observations:

Proposal 1	UE triggers RACH for beam failure recovery during an ongoing RA-SDT if the following conditions are met:

	The SS-RSRP of current SSB is lower than a threshold and lasts for a preconfigured duration; and,

	There is at least one of the SSBs with SS-RSRP above the threshold

Proposal 2	For the RSRP threshold configuration, rsrp-ThresholdSSB in RA-SDT configuration can be reused, or, a new RSRP threshold can be introduced in SDT configuration in SIB.