3GPP TSG-RAN2 Meeting # 125	R2-2401485

# Athens, Greece, 26, Feb, - 1, Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0386 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE in any RRC state may obtain SL DRX configuration for groupcast and broadcast from SIB12. However, current spec only allows UE in RRC_IDLE and RRC_INACTIVE to obtain SL configuration for groupcast and broadcast from SIB12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove ‘in RRC_IDLE and RRC_INACTIVE state’ Impact analysis Impacted 5G architecture options: NR SA Impacted functionality Pre-configuration usage for NR Sidelink Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue because this CR is to correct the wrongly captured behavior without changing the agreement. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue because this CR is to correct the wrongly captured behavior without changing the agreement. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue because this CR is to correct the wrongly captured behavior without changing the agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE in RRC_CONNECTED can’t obtain broadcast/groupcast SL DRX configuration from SIB12. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 8.1	NR sidelink communication, and V2X sidelink communication, NR sidelink discovery, and ranging/ sidelink positioning

The UE may transmit or receive NR sidelink communication/discovery if it fulfils the condition(s) defined in TS 38.331 [3], clause 5.8.2. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform NR sidelink communication/discovery according to SIB12, and when out-of-coverage for sidelink, the UE may perform NR sidelink communication/discovery according to SL-PreconfigurationNR or according to SIB12 of the cell on the frequency which provides inter-carrier NR sidelink configuration, or according to SIB12 received from the connected L2 U2N Relay UE as specified in TS 38.331 [3]. The UE shall not perform NR sidelink communication/discovery according to SL-PreconfigurationNR if the UE detects a cell providing NR sidelink configuration or inter-carrier NR sidelink configuration for the frequency UE is interested to perform NR sidelink communication/discovery on, or if the UE is a L2 U2N Remote UE and has received SIB12 from the connected L2 U2N Relay UE.

The UE may transmit or receive V2X sidelink communication if it fulfills the condition(s) defined in TS 36.331[6], clause 5.10.1d. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform V2X sidelink communication according to SIB13/ SIB14 of the cell on an NR frequency.

The UE may transmit or receive SL-PRS for ranging/sidelink positioning if it fulfils the conditions defined in TS 38.331 [3].

The U2N Remote UE, the U2N Relay UE, the U2U Remote UE, or the U2U Relay UE may transmit NR sidelink relay discovery (i.e., as specified in TS 23.304 [22]) if it fulfills the condition(s) defined in TS 38.331 [3].

For NR sidelink broadcast and groupcast, the UE may obtain SL DRX configuration from SIB12 (for in-coverage UE, as defined in clause 8.2; or for out-of-coverage, UE as defined in clause 8.2, on the frequency which the UE is configured to perform NR sidelink communication/discovery and which is included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12) or SL-PreconfigurationNR (for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which the UE is configured to perform NR sidelink communication/discovery and which is not included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12).

For inter-UE coordination (IUC) information configuration, the UE may obtain it from SIB12 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state; or for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which UE is configured to perform NR sidelink communication and which is included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12) or SL-PreconfigurationNR (for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which UE is configured to perform NR sidelink communication and which is not included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12).

For ranging/sidelink positioning, the UE may obtain the configuration from SIB23 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state) or SL-PreconfigurationNR (for out-of-coverage UE, as defined in clause 8.2).

End of Change