3GPP TSG-RAN WG2 Meeting #125	R2-2401552

Athens, Greece 26 Feb – 01 Mar, 2024

Agenda Item:	7.17.2

Source:	Huawei, HiSilicon

Title:	On Issue#6 FFS on additional info on how the network set the content of MUSIM band list filter

Document for: Discussion and Decision

Introduction

In this document, we clarify the band(s) included in the MUSIM band list filter and provide the corresponding TP.

Discussion

Issue #6, FFS on additional info on how the network set the content of MUSIM band list filter. R2-2400112 Discussion on remaining open issues for MUSIM CATT discussion NR_DualTxRx_MUSIM-Core Proposal 6: no additional information on how the networks set the content of MUSIM band list filter is needed. Discussions based on the contribution(s) above: - HW think it is important the NW includes the bands that it intend to use for this UE. Samsung understand HW’s consideration is to include the serving frequency band. - Ericsson, ZTE, QC support HW’s intention. - vivo think it is also allowed that NW does not config a band in the band filter. [CB] will be concluded in the CB session, based on the majority’s view observed above.

Based on the online discussion, there are some supports of clarification for MUSIM band list filter. The intention is to avoid ping-pong issue for serving cell, e.g. if a serving cell in band A is released due to the reactive UAI reporting, the UE needs to further use proactive UAI to indicate whether the band A can be used again by the network for configuring serving cell. Thus, it is suggested to clarify that the MUSIM band list filter includes band(s) that the network intends to use for at least serving cells, it does not preclude other bands.

Proposal: To agree the TP below for clarifying the band(s) included in the MUSIM band list filter.

musim-CandidateBandList A list of candidate bands that the network intends to use for at least serving cells, and for which the UE is requested to provide information on temporary restricted capabilities for MUSIM operation as described in 5.7.4.3.

Conclusion

Proposal: To agree the TP below for clarifying the band(s) included in the MUSIM band list filter.

musim-CandidateBandList A list of candidate bands that the network intends to use for at least serving cells, and for which the UE is requested to provide information on temporary restricted capabilities for MUSIM operation as described in 5.7.4.3.