3GPP TSG-RAN WG2 Meeting #125          	          R2-2401672

Athens, Greece, 26 February - 1 March, 2024

Agenda Item:	7.13.2

Source: 	Huawei, HiSilicon

Title: 	Report of TPs for agreements for TS 36.331

Document for:	Discussion and decision

Introduction

This is the report of the following email discussion (based on [1]):

- [AT125][654][SONMDT] TS 36.331 (Huawei)

Scope: Develop TPs for agreements during the meeting; no need to discuss the actual CR, which will be approved by a post meeting email discussion

Intended outcome: Report in R2-24001672

Deadline:  Friday CB session

Discussion

During the online discussion, the following papers are discussed:

R2-2400658 WI RIL list for 36.331 for R18 SONMDT Huawei, HiSilicon discussion Rel-18 NR_ENDC_SON_MDT_enh2-Core - Noted Proposal 1: It is proposed to agree on RIL C300, N003, C301, C302, C306, E033, and C307. Huawei: C306 should be PropAgreed. Nokia: assume we will review the CR offline Chair: CRs will go for post meeting email approval and so people will have the chance to double check Huawei: for ToDo RILs we rely on discussion papers - RILs C300, N003, C301, C302, C306, E033,and C307 are Agreed LTE 36.331 RILs R2-2401491 [C304]Discussion on when to retrieve the NR RACH information in LTE spec CATT discussion Rel-18 NR_ENDC_SON_MDT_enh2-Core Late - Noted E///: the current implementation is by design ZTE, Xiaomi: disagree with the proposal CATT, Nokia: this should be fixed in RAN3 - C304 is not agree R2-2401492 [C305]Correction on RACH-Report in LTE spec CATT discussion Rel-18 NR_ENDC_SON_MDT_enh2-Core Late - Noted - Introduce a new IE, not a critical extension

In summary, for ToDo RILs C303, C304, and C305, the current status is:

- C303: the RIL paper R2-2401490 was not treated

- C304: not agree. So the status is PropReject

- C305: PropAgree

For the TPs, TP for C305 can be found below. TPs for other PropAgree RILs can be found in the Tdoc below:

R2-2400659	Corrections to TS 36.331 for R18 SONMDT	Huawei, HiSilicon	CR	Rel-18	36.331	18.0.0	4989	-	F	NR_ENDC_SON_MDT_enh2-Core

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse message

-- ASN1START

UEInformationResponse-r9	::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

ueInformationResponse-r9			UEInformationResponse-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationResponse-r9-IEs ::=		SEQUENCE {

rach-Report-r9							RACH-Report-r16		OPTIONAL,

rlf-Report-r9							RLF-Report-r9			OPTIONAL,

nonCriticalExtension					UEInformationResponse-v930-IEs			OPTIONAL

}

-- Late non critical extensions

UEInformationResponse-v9e0-IEs ::= SEQUENCE {

rlf-Report-v9e0						RLF-Report-v9e0					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- Regular non critical extensions

UEInformationResponse-v930-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING UEInformationResponse-v9e0-IEs)	OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1020-IEs		OPTIONAL

}

UEInformationResponse-v1020-IEs ::= SEQUENCE {

logMeasReport-r10					LogMeasReport-r10					OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1130-IEs		OPTIONAL

}

UEInformationResponse-v1130-IEs ::= SEQUENCE {

connEstFailReport-r11				ConnEstFailReport-r11				OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1250-IEs		OPTIONAL

}

UEInformationResponse-v1250-IEs ::= SEQUENCE {

mobilityHistoryReport-r12			MobilityHistoryReport-r12			OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1530-IEs		OPTIONAL

}

UEInformationResponse-v1530-IEs ::= SEQUENCE {

measResultListIdle-r15				MeasResultListIdle-r15			OPTIONAL,

flightPathInfoReport-r15			FlightPathInfoReport-r15		OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1610-IEs		OPTIONAL

}

UEInformationResponse-v1610-IEs ::= SEQUENCE {

rach-Report-v1610					RACH-Report-v1610				OPTIONAL,

measResultListExtIdle-r16			MeasResultListExtIdle-r16		OPTIONAL,

measResultListIdleNR-r16			MeasResultListIdleNR-r16		OPTIONAL,

nonCriticalExtension				UEInformationResponse-v1710-IEs	OPTIONAL

}

UEInformationResponse-v1710-IEs ::= SEQUENCE {

coarseLocationInfo-r17				OCTET STRING					OPTIONAL,	nonCriticalExtension				UEInformationResponse-v1800-IEs	OPTIONAL

}

UEInformationResponse-v1800-IEs ::= SEQUENCE {

rach-ReportNR-r18					RACH-ReportNR-r18				OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

RACH-Report-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16			NumberOfPreamblesSent-r11,

contentionDetected-r16				BOOLEAN

}

RACH-Report-v1610 ::=	SEQUENCE {

initialCEL-r16						INTEGER (0..3),

edt-Fallback-r16					BOOLEAN

}

RACH-ReportNR-r18 ::=	SEQUENCE {

rach-ReportListNR-r18				OCTET STRING,

cellIdListNR-r18					CellIdListNR-r18

}

CellIdListNR-r18 ::=	SEQUENCE (SIZE (1..maxCellRAReportNR-r18)) OF CellIdNR-r18

<Partially omitted>

If you have any comments, please add them in the table below.

| Company | RIL number | Comments |
| --- | --- | --- |

Conclusion

Due to limited time for companies to double check the changes, the email rapporteur suggest to use the TPs in this Tdoc for post-meeitng email discussion on TS 36.331 CR.

References

[1] R2_125 MBS and QoE session notes (Dawid) 2024-02-28 1030