3GPP TSG-RAN WG2 Meeting #125bis	R2-24xxxxx

Changsha, China,  April 15th – 19th, 2024

Source: 	Session Chair (MediaTek)

Title:	Report from session on positioning and sidelink relay

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.4	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

This Agenda Item will be handled by email.

Tdoc Limitation: 1 tdocs

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 3 tdocs in total for all sub agenda items NOTE: some agenda items have additional Tdoc limits.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treatee together), the sub-Ais below this

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Tdoc Limitation: 1 tdocs

R2-2402455	Correction to on-demand SIB request in RRC_CONNECTED for RTK	Huawei, HiSilicon	CR	Rel-16	38.331	16.16.0	4658	-	F	NR_pos-Core

- Revised in R2-2403700

R2-2403700	Correction to on-demand SIB request for RTK	Huawei, HiSilicon	CR	Rel-16	38.331	16.16.0	4658	1	F	NR_pos-Core

R2-2402456	Correction to on-demand SIB request for RTK in R17	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4659	-	A	NR_pos-Core

- Revised in R2-2403701

R2-2403701	Correction to on-demand SIB request for RTK	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4659	1	A	NR_pos-Core

R2-2402457	Correction to on-demand SIB request for RTK in R17	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4660	-	A	NR_pos-Core

- Revised in R2-2403702

R2-2403702	Correction to on-demand SIB request for RTK	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4660	1	A	NR_pos-Core

R2-2403559	RIL E138 SBAS-ID Field Description Correction	Ericsson, ZTE Corporation	CR	Rel-16	38.331	16.16.0	4756	-	F	NR_pos-Core

R2-2403528	SBAS-ID Field Description Correction	Ericsson, ZTE Corporation	CR	Rel-17	38.331	17.8.0	4753	-	A	NR_pos-Core

R2-2403527	RIL E138 SBAS-ID Field Description Correction	Ericsson, ZTE Corporation	CR	Rel-18	38.331	18.1.0	4752	-	A	NR_pos-Core

R2-2403553	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-16	38.306	16.16.0	1087	-	F	NR_pos-Core

R2-2403558	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-17	38.306	17.8.0	1088	-	A	NR_pos-Core

R2-2403524	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-18	38.306	18.1.0	1086	-	A	NR_pos-Core

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editiorials should only be submitted by spec rapporteurs.

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Tdoc Limitation: 1 tdoc

R2-2402513	Clarification on the Remote UE behaviour on short message monitoring	CATT	CR	Rel-17	38.331	17.8.0	4665	-	F	NR_SL_relay-Core

R2-2402514	Clarification on the Remote UE behaviour on short message monitoring	CATT	CR	Rel-18	38.331	18.1.0	4666	-	A	NR_SL_relay-Core

R2-2402678	Miscellaneous RRC corrections for Rel-17 SL relay	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4682	-	F	NR_SL_relay-Core

R2-2402679	Miscellaneous RRC corrections for SL relay	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4683	-	A	NR_SL_relay-Core

R2-2403309	SRAP-related corrections to 38.300	Samsung	CR	Rel-17	38.300	17.8.0	0787	1	F	NR_SL_relay-Core	R2-2400557

R2-2403652	SRAP-related corrections to 38.300	Samsung	CR	Rel-18	38.300	18.1.0	0788	2	A	NR_SL_relay-Core	R2-2400558

R2-2403398	Correction on SidelinkUEInformationNR	Philips International B.V.	CR	Rel-17	38.331	17.8.0	4731	-	F	NR_SL_relay-Core

R2-2403400	Correction on SidelinkUEInformationNR	Philips International B.V.	CR	Rel-18	38.331	18.1.0	4732	-	A	NR_SL_relay-Core

R2-2403474	Corrections for SL relay	ZTE, Sanechips	CR	Rel-17	38.331	17.8.0	4747	-	F	NR_SL_relay-Core

R2-2403475	Corrections for SL relay	ZTE, Sanechips	CR	Rel-18	38.331	18.1.0	4748	-	A	NR_SL_relay-Core

Not available/Withdrawn

R2-2403310	SRAP-related corrections to 38.300	Samsung	CR	Rel-18	38.300	18.1.0	0788	1	A	NR_SL_relay-Core	R2-2400558	Withdrawn

## 6.4	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Tdoc Limitation: 1 tdoc

R2-2402458	Correction on the UL TEG report	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4661	-	F	NR_pos_enh-Core

R2-2402459	Correction on the UL TEG report	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4662	-	A	NR_pos_enh-Core

R2-2403387	Correction on posSIB(s) acquisition [SI-SCHEDULING]	Philips International B.V., Ericsson	CR	Rel-17	38.331	17.8.0	4467	1	F	NR_pos_enh-Core	R2-2313100

R2-2403388	Correction on posSIB(s) acquisition [SI-SCHEDULING]	Philips International B.V., Ericsson	CR	Rel-18	38.331	18.1.0	4725	-	A	NR_pos_enh-Core

R2-2403525	Correction of when to cancel the triggered SR for positioning measurement gap activation/deactivation	Ericsson	CR	Rel-18	38.321	18.1.0	1825	-	A	NR_pos_enh-Core

R2-2403526	Correction of when to cancel the triggered SR for positioning measurement gap activation/deactivation	Ericsson	CR	Rel-17	38.321	17.8.0	1826	-	F	NR_pos_enh-Core

# 7	Rel-18

## 7.2	Expanded and improved NR positioning

(NR_pos_enh2; leading WG: RAN1; REL-18; WID: RP-232670)

Time budget: 0 TU

Tdoc Limitation: 4 tdocs

### 7.2.1	Organizational

Including incoming LSs and rapporteur inputs. CR rapporteurs are asked to continue maintaining an open issues list reflecting known issues to be handled during the maintenance phase.

Incoming LS with RAN2 in Cc:

R2-2402141	Reply LS on UE selection for Ranging_SL (S2-2403682; contact: Qualcomm)	SA2	LS in	Rel-18	Ranging_SL	To:CT1	Cc:RAN2

Incoming LSs with “take into account” actions and no draft reply

R2-2402106	Reply LS on MAC agreements for SL Positioning (R1-2401552; contact: Intel)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

R2-2402108	LS on bandwidth aggregation for positioning (R1-2401708; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2, RAN3

R2-2402121	LS on higher layer parameters for SL Positioning (R1-2401827; contact: Intel, Qualcomm)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

R2-2402127	Updates on measurement report mapping for Positioning Enhancements WI (R4-2403363; contact: Huawei)	RAN4	LS in	Rel-18	NR_pos_enh2	To:RAN2, RAN3	Cc:RAN1

Other incoming LSs and related inputs (note: RAN2 are in Cc: but have an action in R2-2402133)

R2-2402118	LS on the bandwidth used in measurements for positioning of RedCap UEs (R1-2401801; contact: Ericsson)	RAN1	LS in	Rel-18	NR_pos_enh2	To:RAN2,RAN4

R2-2403532	Bandwidth used in measurements for positioning of RedCap Ues	Ericsson	discussion	Rel-18

R2-2402133	LS on SRS and PRS bandwidth aggregation feature for positioning (R4-2403654; contact: Ericsson)	RAN4	LS in	Rel-18	NR_pos_enh2	To:RAN1	Cc:RAN2

R2-2403536	On SRS and PRS bandwidth aggregation feature for positioning	Ericsson	discussion	Rel-18

Draft LS out [measurement report issue to be discussed under LPP agenda item]

R2-2402258	Questions on PRS and SRS bandwidth aggregation	CATT	LS out	Rel-18	NR_pos_enh2	To:RAN1	Cc:RAN4

RIL/open issue lists [note: RRC RIL list is in AI 7.2.5 and SLPP open issue list is in AI 7.2.3]

R2-2402255	LPP RIL list for Rel-18 Positioning	CATT	discussion	Rel-18	NR_pos_enh2

- Revised in R2-2403721

R2-2403721	LPP RIL list for Rel-18 Positioning	CATT	discussion	Rel-18	NR_pos_enh2

R2-2403533	Open issues list For RRC Positioning	Ericsson	discussion	Rel-18

R2-2402257	Leftover Issues on LPP	CATT	discussion	Rel-18	NR_pos_enh2

Proposal 1: Stage 3 impact is not specified any more to align PRS to fixed (e)DRX.

Proposal 2: Keep the existing IE structure of the request for location+measurements for LocationInformationType in CommonIEsRequestLocationInformation.

Proposal 3: The PRU shall not support types in LocationCoordinates including Ellipsoid-Point, Polygon and EllipsoidPointWithAltitude which don't support uncertainty.

Rapporteur CRs

R2-2402256	Corrections to TS 37.355	CATT	CR	Rel-18	37.355	18.1.0	0500	-	F	NR_pos_enh2-Core

R2-2403537	Miscellaneous RRC Positioning Corrections	Ericsson	CR	Rel-18	38.331	18.1.0	4759	-	F	NR_pos_enh2

### 7.2.2	Stage 2

Impact to 38.300, 37.340, and 38.305. For each specification, a single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

This agenda item may be handled at lower priority.

R2-2402469	Correction to TS 38300 for R18 SL positioning	Huawei, HiSilicon	CR	Rel-18	38.300	18.1.0	0835	-	F	NR_pos_enh2	Revised

- Revised in R2-2403625

R2-2403625	Correction to TS 38300 for SL positioning	Huawei, HiSilicon	CR	Rel-18	38.300	18.1.0	0835	1	F	NR_pos_enh2	R2-2402469

R2-2402470	Correction to TS 38305 for R18 positioning	Huawei, HiSilicon	CR	Rel-18	38.305	18.1.0	0162	-	F	NR_pos_enh2

R2-2402646	Discussion on remaining corrections in stage-2	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2403188	Miscellaneous Stage 2 Corrections	Qualcomm Incorporated	CR	Rel-18	38.305	18.1.0	0163	-	F	NR_pos_enh2

R2-2403500	Further clarifications for Positioning in RRC_INACTIVE state	Nokia	discussion	Rel-18	38.305	NR_pos_enh2-Core

R2-2403535	Text Proposal for Stage2 TS 38.305	Ericsson	discussion	Rel-18

### 7.2.3	SLPP corrections

Impact to 38.355. A single CR with miscellaneous corrections is requested from the spec rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Open issue list

R2-2402414	[Post125][407][POS] 38.355 updated Open Issue list	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

Rapporteur CR

R2-2402416	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.1.0	0003	-	F	NR_pos_enh2-Core

Contributions including RILs identified in R2-2402414 (S101, A006, H016, ZTE004, Q004, Rapp002)

R2-2403261	[H016][ZTE004][A006] SLPP corrections	Nokia	discussion	Rel-18

R2-2403189	Remaining issues for SLPP	Qualcomm Incorporated	discussion

R2-2402415	Further considerations on SLPP open issues	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

R2-2402647	Discussion on remaining corrections in SLPP	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

Other contributions

R2-2402465	Discussion on the remaining issues for R18 SLPP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2	Late

R2-2402517	Discussion on the necessity of including the server UE positioning method in the discovery message	OPPO, LG	discussion	Rel-18	NR_pos_enh2	R2-2401464

R2-2402555	Correction on the maximum number of SL-PRS resource ID in ARP info	vivo	draftCR	Rel-18	38.355	18.1.0	F	FS_NR_pos_enh2

R2-2402707	Discussion on SLPP open issues	Xiaomi	discussion	Rel-18	NR_pos_enh2

R2-2402792	Discussion on error messaging in SLPP	Lenovo	discussion	Rel-18	NR_pos_enh2

R2-2402937	Further discussion on anchor UE selection	LG Electronics Inc.	discussion	Rel-18

R2-2403231	Inclusion of the Server UE Positioning Method in the Discovery Message	CEWiT	discussion

R2-2403424	Remaining issues on SLPP	Samsung	discussion	Rel-18	NR_pos_enh2

R2-2403534	Discussion on SLPP RIL issues	Ericsson	discussion	Rel-18	Late

R2-2403541	SLPP RIL Issue	Ericsson	discussion	Rel-18

Not available/Withdrawn

R2-2402899	Miscellaneous SLPP corrections	Apple	discussion	Rel-19	NR_pos_enh2	Withdrawn

### 7.2.4	LPP corrections

Impact to 37.355. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Prioritised RIL ToDo items: Q014, Q019, Q024, Q028 [note H006 and N013 are waiting for input from RAN1]

R2-2403190	[RILs Q014 Q019, Q024, Q028] LPP Corrections	Qualcomm Incorporated	discussion

Proposal 1:	Regarding Q014 (IntegrityLocationBounds), agree the Text Proposal in section 2.1.2. [Chair’s note: description of subfields in field description table]

Proposal2:	Regarding Q019, Q028 (nr-RSTD-BasedOnAggregatedResources, nr-UE-RxTxTimeDiffBasedOnAggregatedResources), agree the Text Proposal in section 2.2.2. [Chair’s note: clarification of which measurements are based on aggregation]

Proposal 3:	Regarding Q024 (nr-DL-PRS-RxHoppingRequest), agree the Text Proposal in section 2.3.2. [Chair’s note: indicating for which measurements the hopping request is applicable]

Proposal 4:	Regarding nr-DL-PRS-RSCPD-Request/nr-DL-PRS-RSCP-Request, agree the Text Proposal in section 3.1.2. [Chair’s note: new issue on signalling format for RSCPD request]

Proposal 5:	Regarding IE description for SpatialDelta in IE GNSS-LOS-NLOS-GridPoints, agree the Text Proposal in section 3.2.2. [Chair’s note: actually a TEI18 issue, on an IE description for SpatialDelta that should be included in a field description instead]

Identified open issues: PRU terminology (P1); BWA capability alignment (P2) if time permits

R2-2402466	Discussion on the remaining issues for R18 LPP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2	Late

Proposal1: Specify PRU in the LPP spec as a feature that a UE supports, not as a new UE type/LPP signaling entity. Adopts the following changes [Chair’s note: changes to describe “supporting PRU” in field description of locationInformationType]

Proposal2: Move the two fields posSRS-BWA-RRC-Connected-r18 and posSRS-BWA-IndependentCA-RRC-Connected-r18 from IE SRS-CapabilityPerBand-r16 to SRS-PosResourcesPerBand-r16. Adopt the TP in Annex A.

Identified open issues: PRS-DRX alignment (P1-P3) if not resolved under open issues document; CPP measurement window (P4) if time permits

R2-2402648	Discussion on remaining corrections in LPP	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

Proposal 1: In LPP on-demand PRS request message, support UE to request one or more PRS time offset(s) associated with each requested PRS periodicity, to better align the actual paging location.

Proposal 2: In LPP on-demand PRS request message, support UE to request separate PRS periodicities per PFL in order to align with the paging cycle inside-PTW and outside-PTW, respectively.

Proposal 3: Support LMF to include UE’s requested PRS periodicities or PRS time offsets in the NRPPa PRS CONFIGURATION REQUEST message. Send LS to RAN3 to inform the NRPPa impact.

Proposal 4: In NR-DL-PRS-MeasurementTimeWindowsConfig, change need ON to need OR for the IE nr-PeriodicityAndSlotOffsetTimeWindow-r18, and add a slot offset IE dedicated for one-shot window configuration. Take the corresponding TP.

Potential LS to RAN1 on bandwidth aggregation

R2-2402259	Discussion on measurement report for the bandwidth aggregation	CATT	discussion	Rel-18	NR_pos_enh2

Proposal 1: The used PRS resource for measurement within each aggregated PRS resource set can be indicated to LMF within the measurement report.

Proposal 2: LS to RAN1 to ask them whether UE also need to indicate the used PRS resource for measurement within each aggregated PRS resource set when reporting the measurement results to LMF.

Other contributions (all with ASN.1 impact)

R2-2402556	Correction on RSCP measurement info in PRU DL info	vivo	draftCR	Rel-18	37.355	18.1.0	F	FS_NR_pos_enh2

R2-2402998	LPP Stage 3 Open Issue - CPP	Lenovo	discussion	Rel-18

R2-2403191	LPP Open Issues: PRU Operation and DL-PRS–DRX Alignment	Qualcomm Incorporated	discussion

R2-2403501	Corrections to NR-PRU-DL-Info IE	Nokia	discussion	Rel-18	37.355	NR_pos_enh2-Core

R2-2403502	Request for carrier phase measurement or joint measurement and clarification for time window configuration	Nokia	discussion	Rel-18	37.355	NR_pos_enh2-Core

R2-2403540	LPP RIL issue	Ericsson	discussion	Rel-18

### 7.2.5	RRC corrections

Impact to 38.331, except for UE capabilities. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

RIL list

R2-2403530	RRC Positioning RIL List	Ericsson	discussion	Rel-18	Late

- Revised in R2-2403791

R2-2403791	RRC Positioning RIL List	Ericsson	discussion	Rel-18

SRS activation and release

R2-2402261	Discussion on the release of SRS configuration	CATT, Samsung, LG Electronics Inc, Xiaomi	discussion	Rel-18	NR_pos_enh2	R2-2400202

R2-2402260	Activation of SP SRS When Configured with Validity Area	CATT	discussion	Rel-18	NR_pos_enh2

R2-2402832	Discussion on the remaining issues for the SRS with validity area	Xiaomi	discussion

R2-2403416	[S208][S209][S210] Remaining issues on RRC	Samsung	discussion	Rel-18	NR_pos_enh2

ToDo RIL issues

R2-2402468	Discussion on the remaining issues for R18 RRC	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2	Late

SUI and UAI contents

R2-2402417	[I166-I171] Further considerations on parameters in SUI and UAI	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

R2-2402333	Discussion on remaining RRC issues on the positioning	OPPO	discussion	NR_pos_enh2

R2-2402557	Discussion on SUI Content for Sidelink Positioning	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2402649	Discussion on remaining corrections in RRC	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

Other contributions

R2-2402558	RRC correction for UE not supporting sidelink positioning in limited service state in 5GS	vivo	draftCR	Rel-18	38.331	18.1.0	F	FS_NR_pos_enh2

R2-2403194	Remaining issues for pre-configured SRS	Qualcomm Incorporated	discussion

R2-2403718	[X041] Correction on SL-PRS-QoS-Info	Beijing Xiaomi Mobile Software	discussion	Rel-18

### 7.2.6	MAC corrections

Impact to 38.321. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR

R2-2402467	Rapporteur MAC CR for R18 positioning	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1794	-	F	NR_pos_enh2

Other contributions

R2-2402650	Discussion on remaining corrections in MAC	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2402262	Discussion on the remaining issues on bandwidth aggregation for SRS	CATT	discussion	Rel-18	NR_pos_enh2

R2-2402334	Discussion on RAN1 reply LS on SL-PRS transmission	OPPO	discussion	NR_pos_enh2

R2-2402471	Discussion on the remaining issues for MAC for R18 positioning	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2402577	Discussion on SL-PRS	ASUSTeK	discussion	Rel-18	38.321	NR_pos_enh2

R2-2402706	Discussion on SL positioning MAC open issues	Xiaomi	discussion	Rel-18	NR_pos_enh2

R2-2403201	MAC CE for activation/deactivation of aggregated SP SRS for positioning	Qualcomm Incorporated	discussion

R2-2403341	Discussion on remaining MAC issues for SL positioning	InterDigital, Inc.	discussion	Rel-18	NR_pos_enh2

R2-2403417	Remaining issues on MAC	Samsung	discussion	Rel-18	NR_pos_enh2

R2-2403531	Addressing MAC open issues	Ericsson	discussion	Rel-18

### 7.2.7	UE capabilities

Impact to 38.306 and capability-related impact to 38.331. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2402321	Compatibility between Redcap positioning feature and other R18 positioning features	CATT	discussion	Rel-18	NR_pos_enh2

R2-2402578	Correction on UE capability regarding SL PRS	ASUSTeK	discussion	Rel-18	38.306	NR_pos_enh2

### 7.2.8	Corrections to other specifications

Impact to any specifications not identified above.

## 7.9	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 0TU

Tdoc Limitation: 3 tdocs

### 7.9.1	Organizational

Including incoming LSs and rapporteur inputs. CR rapporteurs are asked to continue maintaining an open issues list reflecting known issues to be handled during the maintenance phase.

### 7.9.2	Stage 2 corrections

Impact to 38.300. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2402209	Discussion on stage-2 corrections	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2402428	Correction to 38.300 on Relay enhancement	Xiaomi	discussion

R2-2402721	LTM in L2 relay case	Lenovo	discussion	Rel-18

R2-2403311	Stage-2 Corrections for SL relay enhancements	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

### 7.9.3	RRC corrections

Impact to 38.331, except for capability-related issues (see agenda item 7.9.7). A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues where no clear conclusion was reached in [Post125][417] can be discussed based on contributions.

Including outcome of [Post125][417][Relay] Rel-18 relay RRC open issues (Huawei)

RIL list

R2-2403705	RIL list for Rel-18 SL relay enhancement	Huawei, HiSilicon	report	Rel-18	NR_SL_relay_enh-Core

Open issues list

R2-2402681	RRC open issue list for Rel-18 SL relay	Huawei, HiSilicon	report	Rel-18	NR_SL_relay_enh-Core

Email discussion report

R2-2402682	Report of [Post125][417][Relay] Rel-18 relay RRC open issues	Huawei, HiSilicon	report	Rel-18	NR_SL_relay_enh-Core

Rapporteur CR

R2-2402680	Rapp RRC CR for Rel-18 SL relay enhancement	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4684	-	F	NR_SL_relay_enh-Core	Late

Flow-to-bearer mapping [U2U]

R2-2402890	Discussion on remaining ASN.1 issues for Layer-2 UE-to-UE Relay	Apple	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2403552	discussion on flow-to-bearer mapping indication	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

ToDo RIL items with ASN.1 impact: B107, N122

R2-2402286	[B107] Discussion on IndirectPathFailureInformation message	Lenovo, Apple, China Telecom, Sharp, InterDigital, Kyocera	discussion	Rel-18	38.331	NR_SL_relay_enh-Core

R2-2403603	[N121][N122] RILs on sidelink relay	Nokia	discussion	NR_SL_relay_enh-Core

Other RILs with possible ASN.1 impact

R2-2402208	[O400-407, O421], [O425], [O418, O427, O428], [O419], [Q581] Discussion on ToDo RILs for R18 Relay	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2402504	[C262]Clarification on Indirect Path Failure for MP Scenario 1	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2402718	[B109] on sidelink RRC reconfiguration failure for U2U	Lenovo	discussion	Rel-18

R2-2402927	Discussion for the remaining issues for U2U relay	LG Electronics Inc.	discussion	Rel-18

R2-2403140	Remaining issues and corrections on RRC specification	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2403200	RSRP thresholds for U2N relay selection and re-selection	Nokia	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2403357	Discovery [O419] Open Issues [Post125][417] and [X033] [X251] PC5 trigger for U2U Relay UE selection	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_SL_relay_enh-Core

MCG terminology

R2-2403607	RILs on SL Relays	Ericsson	discussion	Rel-18	Late

Covered by email discussion

R2-2402785	[H659] Network support for non-3GPP multi-path relay	MediaTek Inc., OPPO, ZTE	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2403314	[H064][H686] Discussion for RIL issues on U2U relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

Other contributions

R2-2402427	[X028] Correction on SIB1 forwarding in multipath	Xiaomi	discussion

R2-2402505	[C261]Handling of Indirect Path When Remote UE Enters RRC_IDLE	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2402506	[C263]Clarification on U2U Remote UE Threshold Conditions	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2402600	[X029,030,031] Correction on the relay reselection	Xiaomi	discussion

R2-2402717	[B108] on unsolicited SIB1 forwarding in MP	Lenovo	discussion	Rel-18

R2-2402719	[B112] on maintained indirect path during direct path addition v1.0	Lenovo	discussion	Rel-18

R2-2402720	[B113] TP on T390 in MP scenario	Lenovo	discussion	Rel-18

R2-2403369	Remaining Open Issues in 38.331	Ericsson	discussion	Rel-18

R2-2403476	[Z756]SRAP configuration for non-RRC connected L2 U2U UEs	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

R2-2403477	Discussion on relay UE traffic pattern reporting in UAI	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

R2-2403719	[X260] [X262] U2U Relay UE selection	Beijing Xiaomi Mobile Software	discussion	Rel-18	38.331	NR_SL_relay_enh-Core

### 7.9.4	SRAP corrections

Impact to 38.351. A single CR with miscellaneous corrections is requested from the specification rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR

R2-2402206	Corrections for NR sidelink relay enhancements	OPPO	CR	Rel-18	38.351	18.1.0	0034	-	F	NR_SL_relay_enh-Core

Other contributions

R2-2403478	Discussion on SRAP corrections for U2U relay	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

R2-2402587	Clarification on UE ID pair allocation and determination	ASUSTeK	discussion	Rel-18	38.331	NR_SL_relay_enh-Core

### 7.9.5	MAC corrections

Impact to 38.321. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

### 7.9.6	RLC and PDCP corrections

Impact to 38.322 and 38.323. For each specification, a single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions

Rapporteur CR.

R2-2402816	Miscellaneous Rapporteur Corrections to 38.323 for SL Relay	InterDigital France R&D, SAS	CR	Rel-18	38.323	18.1.0	0135	-	F	NR_SL_relay_enh-Core

Deactivated path case (overlap with rapporteur CR)

R2-2402207	Discussion on duplicated PDU submitted to indirect path RLC	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

Data volume calculation

R2-2403479	Discussion on PDCP corrections for MP	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

R2-2403313	PDCP corrections on data volume calculation for multi-path relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2403412	Clarification on PDCP with multi-path	Nokia	discussion	Rel-18

### 7.9.7	UE capabilities

Impact to 38.306 and capability-related impact to 38.331. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2403139	UE capabilities on MP relay	Qualcomm Incorporated	discussion	NR_SL_relay_enh-Core

R2-2403370	Remaining Open Issues in 38.306	Ericsson	discussion	Rel-18

R2-2403312	UE capability corrections for multi-path operation	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

### 7.9.8	Idle mode corrections

Impact to 38.304. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2403602	Correction on 38.304 for SL Relays	Ericsson	CR	Rel-18	38.304	18.1.0	0400	-	F	NR_SL_relay_enh

## 7.24	TEI18

Specific items may be allocated to a breakout session for treatment.

Time budget: 1 TU

### 7.24.1	TEI proposals by Other Groups

Items initiated by other groups that is/has been communicated by LS, where the other group indicate this is TEI18. (Specific other-group-WIs should use the R18 Other Agenda Item below).

R2-2403538	Introduction of LCS User Plane	Ericsson, Intel Corporation, Huawei, HiSilicon, ZTE Corporation, vivo	CR	Rel-18	38.305	18.1.0	0159	1	B	TEI18	R2-2401320

### 7.24.2	TEI proposals by RAN2

Items initiated in RAN2 for NR and LTE.

No contributions should be submitted under 7.24.2.  They should be submitted under 7.24.x

Tdoc limitation: 1 tdoc, limitation applicable to new proposals.  No new Cat. B proposals expected for this meeting

#### 7.24.2.2	Other RAN2 TEI-18

Contributions should focus only critical issues/corrections for already agreed TEI-18 topics.   New TEI proposals should address critical issues that should be resolved by RAN2#125.  Co-sourcing of such proposals is encouraged.   Contributions on items that were explicitly downprioritized from Rel-18 WIs should not be brought as TEI18.  No new Cat. B proposals expected for this meeting

Including outcome of [POST125][022][RedCap emergency calls] Review CRs (Apple) and [POST125][612][TEI18] CR for MBS operation with eDRX/MICO (Nokia)

SUI for U2N relay

R2-2402210	Remove of AS condition checking of SUI for U2N Relay communication	OPPO, Apple	CR	Rel-18	38.331	18.1.0	4645	-	F	TEI18, NR_SL_relay_enh-Core

R2-2402211	Discussion on AS condition checking of SUI for U2N Relay communication	OPPO, Apple	discussion	Rel-18	TEI18, NR_SL_relay_enh-Core

MUSIM paging cause forwarding

R2-2402372	Corrections for MUSIM paging cause forwarding	Samsung Electronics Co., Ltd	discussion	Rel-18	TEI18

BT AoA/AoD

R2-2402418	Corrections for Bluetooth AoA/AoD [BT-AoA-AoD]	Intel Corporation	discussion	Rel-18	TEI18

LS out to RTCM

R2-2403358	LS to RTCM on GNSS positioning and integrity	Swift Navigation, Ericsson	discussion

Sub-1s periodicity in LPP

R2-2403605	LPP support for sub 1s location information reporting periodicity	Ericsson, AT&T, T-Mobile, Vivo, Deutsche Telekom	CR	Rel-18	37.355	18.1.0	0501	-	B	TEI18

PosL2RemoteUE [ASN.1 issue B021]

R2-2403792	[B021] Missing posSibType2-17a in list of posSIB types [PosL2RemoteUE]	MediaTek Inc.	CR	Rel-18	38.331	18.1.0	4767	-	F	TEI18	Late

Not available/Withdrawn

R2-2403539	LPP support for sub 1s location information reporting periodicity	Ericsson, AT&T, T-Mobile, Vivo, Deutsche Telekom	CR	Rel-18	38.305	18.1.0	0159	2	B	TEI18	R2-2401320	Withdrawn

## 7.25	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Clarification CRs should be discussed with spec rapporteurs of the topic prior to submission.

Time budget: 2 TU

Tdoc Limitation: -

### 7.25.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing, NTN self evaluation issues, etc.

R2-2403529	Introduction of NR UE Rx-Tx time difference measurement in NR UL E-CID [ECID-UERxTx]	Ericsson, Polaris Wireless, China Telecom, NTT Docomo, AT&T, FirstNet, Intel, Comtech, Nokia, Nokia Shanghai Bell, Verizon Wireless, Huawei, ZTE	CR	Rel-18	38.305	18.1.0	0164	-	F	NR_pos-Core