3GPP TSG-RAN2 Meeting #125bis	R2-2403960

# Changsha, China, April 15-19, 2024

Title:	Reply LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1

Response to:	R1-2312668 / R2-2400032

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW-Core

Source:	RAN WG2

To:	RAN WG1, RAN WG4

Cc:	-

Contact person:	Umesh Phuyal

uphuyal@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	R2-2403961 (CR for TS 38.331), R2-2403962 (CR for TS 36.331)

# 1	Overall description

RAN2 thanks RAN1 for the LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1 (R1-2312668).

RAN2 had previously replied to Question 1 in R2-2401885.

RAN2 further discussed question 2 in the LS and would like to provide following response:

Question 2: If the answer to Question 1 is Yes, is it possible for RAN2 to define a scheme to avoid the backward compatibility issue?

Answer 2: RAN2 will add a Rel-18 parallel list in SIB4 indicating the dl-CarrierFreq-r18 and frequencyBandList-r18 of the <5MHz neighbor cells. If the new fields are included, legacy fields will need to be gracefully ignored by the legacy UEs. However, legacy fields are mandatory to signal in current specification.

From RAN2 point of view, it is feasible to use ARFCN-ValueNR = 250 (corresponding to GSCN = 2) and freqBandIndicatorNR = 1024 as reserved values.

When the new fields are included, legacy fields will be set to the reserved values by the network and ignored by the new UE.

RAN2 requests RAN4 to define these ‘reserved’ values of GSCN =2 / ARFCN-ValueNR = 250 and NR operating band = 1024, and capture in RAN4 specifications, e.g. in Table 5.4.3.1-1 and Table 5.2-1 in TS 38.101-1.

Furthermore, RAN2 will introduce changes in NR SIB11 >> measIdleConfigSIB-r16, and LTE SIB24 to enable signalling of <5MHz cells applicable only for UEs supporting <5MHz CBW. Please refer to the attached endorsed CRs.

# 2	Actions

ACTIONS:

To RAN WG4: RAN2 would like to request RAN4 to take the above information into account, define the ‘reserved’ GSCN / ARFCN-ValueNR and NR operating band and capture in RAN4 specifications, and provide feedback, if any.

To RAN WG1: RAN2 would like to request RAN1 to take the above information into account and provide feedback, if any.

# 3	Dates of next TSG RAN WG2 meetings

TSG-RAN2 Meeting #126	May 20 - 24, 2024	Fukuoka, Japan

TSG-RAN2 Meeting #127	Aug 19 - 23, 2024	Maastricht, NL