3GPP TSG RAN WG2#125bis	R2-2402126

Changsha, China, 15th - 19th April 2024

# 3GPP TSG-RAN4 Meeting #110	R4-2403086

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

Title:	Reply LS on RRC network assistant signalling for advanced receiver on MU-MIMO scenario

Release:	Rel-18

Response to:	R2-2313706

Work Item:	NR_demod_enh3-Core

Source:	RAN WG4

To:	RAN WG2 RAN WG1

Cc:

Contact Person:

#### Name:	Jingzhou Wu

E-mail Address:	wujingzhou@chinatelecom.cn

#### Name:	Qiuge Guo

E-mail Address:	guoqiuge@catt.cn

Attachments:

# 1. Overall Description:

RAN4 thank RAN2 for the reply LS (R2-2313706) on RRC network assistant signalling for advanced receiver on MU-MIMO scenario.

Regarding the questions for the new network assistant signalling, RAN4 would like to give the following answers:

- Question 1: Whether the 3 assumptions listed in R2-2313706 (from 1 to 3) are feasible.

- RAN4 answer: RAN4 confirms the feasibility of the 3 RAN2 assumptions listed in R2-2313706.

- Question 2: Based on the below RAN1 conclusion, whether the DMRS power boosting information for advanced receiver is still needed.

Continuation of discussions triggered by R1-2307902 (rejected) from RAN1#114 R1-2310120 Clarify number of CDM groups without data for DMRS Qualcomm Incorporated Conclusion The following specification in TS 38.214 is interpreted as the UE may assume that “CDM groups without data” are not used for data transmission for any co-scheduled user in the same serving cell. When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively..

- RAN4 answer:

During the RAN4 discussion, majority of the companies think that, based on the above RAN1 conclusion, the previous required RRC indication on ‘Whether the DM-RS power boosting configurations (i.e., Number of DM-RS CDM groups without data, TS38.214 table 4.1-1) of all the co-scheduled UE(s), which has the same DM-RS sequence as the target UE, is the same as the target UE.’ in R4-2316980, is no longer needed since RAN1 already agreed that UE may assume that “Number of DM-RS CDM groups without data, TS38.214 table 4.1-1) of all the co-scheduled UE(s), which has the same DM-RS sequence as the target UE, is the same as the target UE”. Additionally, they think that if RAN2 introduces the signalling and UE implements solely based on RAN1 conclusion, misalignment between network and UE may occur.

At the same time, some companies think that, the scheduling of co-scheduled UEs with non-aligned DMRS power boosting between co-scheduled UEs is not prohibited based on the above RAN1 conclusion, because RAN1 only concluded that UE “may assume” rather than “shall/should assume” such scheduling, so they think that it is optional for BS to perform such scheduling, and UE can also “may not assume” this scheduling, then they think that it is still necessary to introduce the RAN2 RRC signalling, otherwise there could be interoperability issue between BS and UE, causing UE performance degradation.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully request RAN2 to take the above answers into account while designing the network assistant signalling for advanced receivers for MU-MIMO.

To RAN1

ACTION: 	RAN4 respectfully request RAN1 to provide clarification on the understanding of the above RAN1 conclusion if any.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #110-bis 	15 April - 19 April 2024

TSG-RAN WG4 Meeting #111 	20 May - 24 May 2024