3GPP TSG RAN WG2#125bis	R2-2402129

Changsha, China, 15th - 19th April 2024

# 3GPP TSG-RAN WG4 Meeting #110	R4-2403493

# Athens, GR, 26 Feb – 01 Mar, 2024

Title:	Reply LS on RAN2 agreements for satellite switch with resync

Response to:	R2-2314016

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Jie Cui

E-mail Address:	Jie_cui@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1. Overall Description:

RAN4 would like to thank RAN2 for the LS on RAN2 agreements for satellite switch with resync. RAN4 #110 had discussion on the feasibility of UE to perform the downlink synchronization with the target satellite and keep the communication with the source satellite of the same serving cell simultaneously in soft satellite switch. Following conclusion have been achieved:

- It is feasible that a soft satellite switch capable UE can perform downlink synchronization with the target NGSO satellite and keep the connection (DL and UL) with the source NGSO satellite simultaneously under the following conditions.

  - Only if SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

- If the UE starts synchronization to a different satellite from T-ServiceStart, scheduling restriction is expected, if:

  - the UE is not capable of parallelMeasurementWithoutRestriction-r17, or

  - the UE is not capable of simultaneousRxDataSSB-DiffNumerology but the SSB has a different SCS than PDCCH/PDSCH

  - Note: RAN4 is investigating how to minimize the duration of the scheduling restriction.

# 2. Actions:

To RAN WG2 group.

ACTION: RAN4 kindly requests RAN2 to take the above agreement into consideration for future work.

# 3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #110bis	15 – 19 April 2024	Changsha, China

TSG RAN WG4 Meeting #111	20 – 24 May 2024	Fukuoka, Japan