3GPP TSG RAN WG2#125bis	R2-2402137

Changsha, China, 15th - 19th April 2024

3GPP TSG-RAN WG4 Meeting #110	R4-2403830

Athens, Greece, February 19 – March 01, 2024

Title:	LS Reply on Aerial Pmax values

Response to:	R2-2313949

Release:	Release 18

Work Item:	NR_UAV-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:	Rafhael Amorim

E-mail Address:	rafhael.medeiros_de_amorim@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN2 for their further LS reply in R2-2313949, which confirms the introduction of the new aerial capability.

Regarding the clarification requested by RAN2 to RAN4 in their LS (“whether it is needed to have additionalPmax-r18 in NR-NS-PmaxValueAerial-r18”), the position of RAN4 is the following:

- Although the objectives of this WI were concluded in RAN4 and no applicability for a new  additionalPmax-r18 was discussed, this IE will be useful in the future, as it could be by the network to control maximum output power of UAV device within carrier in case of any specific deployment restrictions or serve as other purpose if necessary. Therefore, additionalPmax-r18 should be included in this Release of the specification.

RAN4 also takes this opportunity to clarify with RAN2 the intended behavior of the aerial UE. The aerial UE (UAV UE) shall first check whether there is a configured NR-NS-PmaxValueAerial-r18 in the considered band.

- If such IE is configured, the UE shall apply the parameters configured in this IE.

  - If the NS value indicated in NR-NS-PmaxValueAerial-r18 is not supported by the UE, the UE shall not connect to the cell.

- If such IE is not configured, the UE shall then check whether a NS value is configured in the (legacy) NR-NS-PmaxValue IE for that same band.

RAN4 politely asks RAN2 to confirm whether this intended procedure is reflected on current RAN2 specifications.

2. Actions:

To RAN WG4.

ACTION: 	RAN4 respectfully asks RAN2 to:

- take the RAN4 view on the inclusion of the new IE additionalPmax-r18  into consideration when deciding RAN2 signalling.

- confirm the intended behavior of the aerial UE regarding NR-NS-PmaxValueAerial-r18 is reflected on the RAN2 signaling design.

3. Date of Next TSG-RAN WG4 Meeting:

RAN4#110-bis	from 15-04-2024	to 19-04-2024	Changsha, Hunan Province, CN

RAN4#111	from 20-05-2024	to 24-05-2024	Fukuoka City, Fukuoka, JP