3GPP TSG-RAN WG2 Meeting #125bis		R2-2403765

Changsha, China, 15th – 19th April, 2024

Title:	Reply LS on improved GNSS operations in Rel-18 IoT NTN

Response to:	LS on improved GNSS operations in Rel-18 IoT NTN (R1-2401754)

Release:	Release 18

Work Item:	IoT_NTN_enh-Core

Source:	RAN2

To:	RAN1

Cc:	-

Contact person:

Name:	Bin Xu

E-mail Address:	xubin10@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN2 thanks RAN1 for the LS on improved GNSS operations in Rel-18 IoT NTN. In the LS, RAN1 mentions three alternatives to be considered by RAN2 as follows:

From RAN1 perspective, when timeAlignmentTimer is not infinity, the following alternatives were considered, and it is up to RAN2 to specify: - Alt-1: the end of X should be at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is reset with length equal to remaining timeAlignmentTimer every time when a MAC CE (to be defined by RAN2) is received - Alt-1a: the end of X should be at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is set to remaining timeAlignmentTimer at the start point of X and ULTransmissionExtentionTimer is reset with length equal to configured timeAlignmentTimer value every time when a MAC CE (to be defined by RAN2) is received - Alt-2: the end of X should be at the point where timeAlignmentTimer expires and timeAlignmentTimer is reset every time when a legacy MAC TAC is received Note 1: It is up to RAN2 to decide whether the MAC CE is the legacy TAC or a new TAC or a new MAC CE. Note 2: It is up to RAN2 to implement the above behaviour based on new timer, existing timer, or by extending GNSS validity. Note 3: For Alt-1a, from RAN1 perspective, eNB should be able to update TA without extending X

RAN2 would like to inform RAN1 that during RAN2#125bis meeting, RAN2 has made the conclusion to adopt Alt-1a:

  - Alt-1a: the end of X should be at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is set to remaining timeAlignmentTimer at the start point of X and ULTransmissionExtentionTimer is reset with length equal to configured timeAlignmentTimer value every time when a MAC CE (to be defined by RAN2) is received.

2	Actions

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account.

3	Dates of next TSG-RAN WG2 meetings

3GPP TSG RAN WG2#126	20 May - 24 May 2024		Fukuoka, JP

3GPP TSG RAN WG2#127	19 Aug - 23 Aug 2024		Maastricht, NL