3GPP TSG-RAN2 Meeting #126												R2-2404168

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Agenda Item:	7.15.2

Source:	OPPO

Title:	Left issues on RRC

Document for:	Discussion, Decision

- Introduction

This is to discuss RRC left issues.

- Discussion

In [POST125bis][107][V2XSL], one left issue was identified, i.e., what is the relationship between

- Legacy-carrier usage of SRB before the initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition

- Network configured carrier set split for the RRC_CONNECTED UE

SL-SCCH-CarrierSetConfig-r18 ::=     SEQUENCE {

sl-DestinationList-r18               SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16,

sl-SRB-Identity-r18                  SEQUENCE (SIZE (1..3)) OF SRB-Identity,

sl-AllowedCarrierFreqSet1-r18        SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16),

sl-AllowedCarrierFreqSet2-r18        SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

}

There is question wondering whether we should rely on 2) (instead of 1) for the initial SRB message delivery, for RRC_CONNECTED UE with duplication being configured.

Yet given the following agreement, we understand the intention of using legacy-carrier was for Rx-UE to only turn-on the Rx chain of legacy-carrier to prepare for the in-coming PC5-S message, for power saving and backwards compatibility reason, which should not be affected by the usage of duplication, which would be used after link establishment being completed.

Agreements on PC5-RRC

- Confirm the working assumption “SL CA/PDCP duplication is applied to PC5-RRC after SL link is established”.

- R2 confirm the configured carrier-set configuration for two RLC legs in case of PDCP duplication would not affect the usage of legacy-carrier before initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition.

- Conclusion

Proposal 1	R2 confirm the configured carrier-set configuration for two RLC legs in case of PDCP duplication would not affect the usage of legacy-carrier before initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition.