3GPP TSG RAN WG2 Meeting #126             	                	                          R2-2404188

Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item:	8.1.4

Source:	Intel Corporation

Title:	Discussion on UE-sided training data collection

Document for:	Discussion and Decision

# Introduction

During Rel-18, solutions to support data collection from UE device to UE-side OTT server for UE-sided model training were discussed. However, except for data collection from UE to UE-side OTT server, to support model training at UE side, some information transferring from other entities to UE/UE-side OTT server for model training should also be considered.

In this contribution, we focus on the discussion of supporting dataset transfer for positioning UE-sided model training. Following use cases are considered:

  - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

# Discussion

Following label information is captured in RAN1 reply LS on data collection [1] for positioning use cases:

| LCM purpose | Case | Data content | Typical data size (per data sample) | Typical latency requirement |
| --- | --- | --- | --- | --- |
| Training | All Cases | Measurements (corresponding to model input): timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | Relaxed |
|  | Direct AI/ML positioning | Label: Location coordinates as model output | 56 to 144 bits See Note 3 | Relaxed |
|  | AI/ML assisted positioning | Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | Relaxed |

It is further replied in R1-2308730 [2]:

- For positioning enhancement use case: - For model training, training data can be generated by UE/PRU/gNB/LMF and terminated at LMF/OTT server.

In Table 1, we summarize the likely types of data and their signalling types as relevant to positioning use-cases.

Table 1. Summary of data types and their signalling

| Content | Signalling type |
| --- | --- |
| Ground-truth label | Measurement report |
| Measurements corresponding to model input | Measurement report |
| Quality indicator (for ground-truth labels and/or measurement data) | Measurement report |
| RS configurations (for deriving measurement) | Assistance signalling |
| Assistance info for label calculation/generation, for label validity/quality condition(s), e.g., between LMF and UE/PRU | Assistance signalling |
| Time stamp (time stamp in existing positioning measurements or new time stamp report) | Measurement report |

For ground-truth label with any associated label quality generation, the entities responsible for data collection are summarized in Table 2. Here, ground-truth labels may include target location coordinates for direct AI/ML-based positioning (i.e. Case 1).

Table 2. Summary of entities responsible for ground-truth labels for Case 1

| Entity | Case 1 |
| --- | --- |
| UE with estimated/known location | Yes |
| Positioning Reference Unit (PRU) | Yes |
| Network entity (other than LMF) |  |
| LMF with known PRU location | Yes |

Furthermore, a working assumption was made in RAN1 #116bis meeting:

Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

Instead of relying on UE/PRU to determine estimates of location coordinates, e.g., with or without use of assistance data from LMF, the LMF may provide ground-truth labels that are determined based on estimates using measurement data from PRUs/UEs. Such an option can reduce the reliance on PRUs for location coordinate estimates as LMF can provide such estimates based on measurement data reported by UE. Thus, this approach can be interpreted to include dataset transfer/delivery from LMF to where model training is located, i.e. UE or UE-side OTT server.

Observation 1: For positioning Case 1, labelled dataset transfer from LMF to UE/UE-side OTT server should be supported.

There are mainly three approaches:

Option 1: LMF directly transfer labelled dataset to UE via LPP signaling

Option 2: LMF transfer labelled dataset to UE-side OTT server via CN (e.g. NWDAF)

Option 3: LMF transfer labelled dataset to UE-side OTT server via OAM

For Option 2/3, further coordination with SA2 and SA5 is needed.

Proposal 1: Following options are considered to support labelled dataset transfer from LMF to UE/UE-side OTT server:

Option 1: LMF directly transfer labelled dataset to UE via LPP signaling

Option 2: LMF transfer labelled dataset to UE-side OTT server via CN (e.g. NWDAF). Coordination with SA2 is needed.

Option 3: LMF transfer labelled dataset to UE-side OTT server via OAM. Coordination with SA5 is needed.

# Conclusion

In this contribution, we discussed the need of supporting labelled data transfer from LMF to UE/UE-side OTT server and potential solutions:

Observation 1: For positioning Case 1, labelled dataset transfer from LMF to UE/UE-side OTT server should be supported.

Proposal 1: Following options are considered to support labelled dataset transfer from LMF to UE/UE-side OTT server:

Option 1: LMF directly transfer labelled dataset to UE via LPP signaling

Option 2: LMF transfer labelled dataset to UE-side OTT server via CN (e.g. NWDAF). Coordination with SA2 is needed.

Option 3: LMF transfer labelled dataset to UE-side OTT server via OAM. Coordination with SA5 is needed.

# Reference

[1] R1-2310681, Reply LS on Data Collection Requirements and Assumptions (Part B), RAN1

[2] R1-2308730, Reply LS on Data Collection Requirements and Assumptions (Part A), RAN1