3GPP TSG-RAN WG2 #126	Tdoc R2-2404365

Fukuoka, Japan, May 20 – 24, 2024

Agenda Item:	7.3.3

Source:	Ericsson

Title:	Clarification on cellDTRX-RNTI

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the need to clarify cellDTRX-RNTI in stage-2 and how to address it.

# 2	Discussion

In 38.300, the UE identities are clarified in clause 8.1, i.e. a list with short description of RNTIs used. However, this list does currently does not include cellDTRX-RNTI.

cellDTRX-RNTI is not clarified in 38.300.

Therefore, this should be included in 38.300 for completeness.

Add cellDTRX-RNTI description to 38.300.

A TP to 38.300 capturing the proposal above is provided in the Annex in section 4. If the change above is agreed for 38.300, it can be considered to also add cellDTRX-RNTI in the abbreviation section for 38.331.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	cellDTRX-RNTI is not clarified in 38.300.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add cellDTRX-RNTI description to 38.300.

# 4 Annex: TP to 38.300

START OF FIRST CHANGE

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

AR	Augmented Reality

ARP	Allocation and Retention Priority

ATG	Air to Ground

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

BRID	Broadcast Remote Identification

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

cellDTRX-RNTI Cell Discontinuous Transmission and Reception RNTI

CD-SSB	Cell Defining SSB

CFR	Common Frequency Resource

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAA	Detect And Avoid

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DCR	Direct Communication Request

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

DSR	Delay Status Report

DTX	Discontinuous Transmission

E-CID	Enhanced Cell-ID (positioning method)

EC	Energy Cost

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LBT	Listen Before Talk

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

ML	Machine Learning

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MO-SDT	Mobile Originated SDT

MP	Multi-Path

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MT-SDT	Mobile Terminated SDT

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

N3C	Non-3GPP Connection

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NES	Network Energy Savings

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

END OF FIRST CHANGE

START OF SECOND CHANGE

# 8	NG Identities

## 8.1	UE Identities

In this clause, the identities used by NR connected to 5GC are listed. For scheduling at cell level, the following identities are used:

-	C-RNTI: unique UE identification used as an identifier of the RRC Connection and for scheduling;

-	CG-SDT-CS-RNTI: unique UE identification used for Configured Grant-based SDT in the uplink;

-	CI-RNTI: identification of cancellation in the uplink;

-	CS-RNTI: unique UE identification used for Semi-Persistent Scheduling in the downlink or configured grant in the uplink;

-	INT-RNTI: identification of pre-emption in the downlink;

-	MCS-C-RNTI: unique UE identification used for indicating an alternative MCS table for PDSCH and PUSCH;

-	P-RNTI: identification of Paging and System Information change notification in the downlink;

-	SI-RNTI: identification of Broadcast and System Information in the downlink;

-	SP-CSI-RNTI: unique UE identification used for semi-persistent CSI reporting on PUSCH.

For power and slot format control, the following identities are used:

-	SFI-RNTI: identification of slot format;

-	TPC-PUCCH-RNTI: unique UE identification to control the power of PUCCH;

-	TPC-PUSCH-RNTI: unique UE identification to control the power of PUSCH;

-	TPC-SRS-RNTI: unique UE identification to control the power of SRS.

During the random access procedure, the following identities are also used:

-	RA-RNTI: identification of the Random Access Response in the downlink;

-	MSGB-RNTI: identification of the Random Access Response for 2-step RA type in the downlink;

-	Temporary C-RNTI: UE identification temporarily used for scheduling during the random access procedure;

-	Random value for contention resolution: UE identification temporarily used for contention resolution purposes during the random access procedure.

For NR connected to 5GC, the following UE identity is used at NG-RAN level:

-	I-RNTI: used to identify the UE context in RRC_INACTIVE.

For UE power saving purpose, the following identities are used:

-	PS-RNTI: used to determine if the UE needs to monitor PDCCH on the next occurrence of the connected mode DRX on-duration;

-	PEI-RNTI: used to determine if the UE needs to monitor the associated PO.

For IAB the following identity is used:

-	AI-RNTI: identification of the DCI carrying availability indication for soft symbols of an IAB-DU.

For Network-Controlled Repeater the following identity is used:

-	NCR-RNTI: identification of the DCI carrying side control information.

For MBS, the following identities are used:

-	G-RNTI: Identifies dynamically scheduled PTM transmissions of MTCH(s);

-	G-CS-RNTI: Identifies configured scheduled PTM transmissions of MTCH(s) scheduled with configured grant;

-	MCCH-RNTI: Identifies transmissions of MCCH and MCCH change notification.

For sidelink, the following identities are used:

-	SL-RNTI: unique UE identification used for NR sidelink communication scheduling;

-	SL-CS-RNTI: unique UE identification used for configured sidelink grant for NR sidelink communication;

-	SL Semi-Persistent Scheduling V-RNTI: unique UE identification used for semi-persistent scheduling for V2X sidelink communication;

-	SL-PRS-RNTI: unique UE identification used for SL-PRS transmission scheduling on dedicated SL-PRS resource pool;

-	SL-PRS-CS-RNTI: unique UE identification used for configured sidelink grant for SL-PRS transmission on dedicated SL-PRS resource pool.

For NES, the following UE identity is used:

-	cellDTRX-RNTI: identification of the DCI used for activating and/or deactivating cell DTX/DRX and/or NES mode for CHO indication.

END OF SECOND CHANGE