3GPP RAN WG2 Meeting #126	R2-2404558

# Fukuoka, Japan, May 20th - 24th, 2024

Agenda Item:	8.3.2.2

Source:	HONOR

Title:	Discussion on AI aided RRM measurement prediction

Document for:	Discussion and Decision

1. Introduction

In RAN#102, Rel-19 AI mobility study was agreed in RP-234055 [1]. The study will focus on L3-based NR PCell change enhancement in RRC_CONNECTED mode over air interface. The objective of the SI includes:

- AI/ML based RRM measurement and event prediction - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2]

In RAN2#125bis meeting, AI/ML based RRM measurement has been discussed and the following agreements were made [2]:

- For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results. Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal prediction. - The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction： Spatial-domain prediction: RSRP difference to the actual measurement Temporal prediction: RSRP difference to the actual measurement measurement reduction rate as one KPI - As a first step we will focus on measurement prediction accuracy. FFS whether and what system level performance evaluation is needed

In this contribution, we will discuss the detailed use cases, model inference and model monitoring for AI aided RRM measurement in the study.

2. Discussion

2.1. Motivation

As concluded in the justification of the SID, in case of high UE mobility speed, high-density area or for future critical services, it might be difficult to set proper thresholds for current L1L3 Mobility scheme. So that unintended handover failure may happen which may interrupt data transmission and impact user experience. If we consider AI/ML algorithm to provide some predicted value based on proactive scheme, it will improve handover performance and reduce UE measurement at the meantime.

2.2 Considerations on use cases of cell-level measurement prediction

For conventional intra/inter frequency NR cell level measurement, SSB- based or CSI-RS based measurement can be configured. The cell level measurement prediction is considered as the extension of R18 RAN1 SI on AI/ML aided beam prediction, so that both SSB-based and CSI-RS based L3 RSRP measurement prediction should be supported.

Proposal 1: Both SSB-based and CSI-RS-based cell level L3 RSRP measurement prediction should be supported.

For temporal domain prediction, UE measures SSB and/or CSI-RS RSRP for serving cell and certain configured neighbor cells for now and UE or NW predicts next Tms RSRP for the measured cells. For spatial domain prediction, UE measures SSB and/or CSI-RS RSRP for serving cell and probably a set of configured neighbor cells (set B), then UE or NW predicts other intra-frequency sets of neighbor cells (set A).

Except for temporal domain and spatial domain prediction, in order to reduce the impact from measurement gap, frequency domain prediction should also be considered. In this case, UE measures SSB and/or CSI-RS RSCP for serving cell and intra-frequency neighbor cells, then UE or NW could predict corresponding inter-frequency neighbor cells.

Proposal 2：For spatial domain prediction, the UE or NW can predict other intra-frequency sets of neighbor cells (set A) based on the measurement result of SSB and/or CSI-RS RSRP for serving cell and probably a set of configured neighbor cells (set B).

Proposal 3: Except for temporal domain and spatial domain prediction, frequency domain prediction should also be considered.

2.3 Considerations on model inference of cell-level measurement

For RRM measurement prediction, model input could be measured RSRP for SSB/CSI-RS and pattern information for different use cases of prediction. For temporal prediction, the pattern information could be predicted time instances and for spatial domain and frequency domain prediction, setA/setB cells should be provided. It is FFS whether L1 RSRP or L3 RSRP after filtering should be used as the input of the model. And model inference output should be the predicted L3 RSRP of F time instances or for Top-K neighbor cells for different use cases of prediction.

For UE side model inference, L3 signaling can be used for above configuration and reporting. It is FFS set A for prediction can be dynamically configured or based on certain fixed pattern similar like setA/setB configuration discussion in R18 beam prediction. For the prediction results to report, both event triggered reporting and periodical reporting can be supported. Current RRC measurement report message and format could be reused. In the report, whether the results are derived from actual measurement or prediction should be indicated to network. For temporal-domain measurement prediction, new types of reporting involving an array with different measurement results predicted for different time interval might be included.

Besides, for each predicted result, the confidence/probability information can also be reported to network to help its mobility decision. Another alternative is that NW could configure certain confidence/probability threshold in order to control the corresponding reporting.

For network side model inference, only inputs i.e., measured results are needed to be configured, current L3 measurement report configuration and report message should be considered as the baseline. In some cases, other UE assistant information could be helpful e.g., timestamp for temporal-domain measurement prediction. This could be further studied considering UE complexity and overhead for air interface.

Proposal 4: Discuss following model input/output for RRM measurement prediction:

Model input:

- Measured RSRP for SSB/CSI-RS (FFS L1 or L3 measurement)

- Pattern information:

- Time instances

- SetA/SetB pattern, i.e., Measured serving/neighbor cells vs predicted intra/inter-frequency neighbor cells

Model output:

- Predicted L3 RSRP

  - For F time instances

  - For intra/inter-frequency Top-K neighbor cells

- Confidence/probability information (FFS)

Proposal 5: For UE side model inference, current RRC signaling should be used to configure model input and output, measurement report message can be extended to support the prediction reporting. For NW side model inference, current measurement configuration and reporting should be used as the baseline and FFS other essential assistant information.

2.4 Considerations on model monitoring of cell-level measurement prediction

For UE side model inference, UE side monitoring is considered as the baseline solution. Network should configure the evaluation metrics and reporting criteria. For example, network could configure UE to measure in a certain time interval and the measured results are considered as the ground truth. If the predicted value is within a certain range compared with the ground truth, the prediction is considered as successful with good performance. Otherwise, it might trigger a configured event to report the monitoring results.

For network side inference model, NW side monitoring should be considered as the baseline solution, and current L3 measurement reporting from UE can be reused as assistant information to help network to evaluate the performance metrics. Other additional information like timestamp could be studied further.

Proposal 6: For UE side model monitoring, further study the following aspects:

- Performance metric and reporting criteria should be configured by network.

- In order to reduce the reporting overhead, event-based model monitoring reporting should be adopted.

Proposal 7: For network side monitoring, current L3 measurement reporting from UE can be reused as assistant information to help network to evaluate the performance. Other additional information like timestamp could be studied further.

3. Conclusions

In this contribution, we discussed the AI aided RRM measurement prediction, and made the following proposals:

Proposal 1: Both SSB-based and CSI-RS-based cell level L3 RSRP measurement prediction should be supported.

Proposal 2：For spatial domain prediction, the UE or NW can predict other intra-frequency sets of neighbor cells (set A) based on the measurement result of SSB and/or CSI-RS RSRP for serving cell and probably a set of configured neighbor cells (set B).

Proposal 3: Except for temporal domain and spatial domain prediction, frequency domain prediction should also be considered.

Proposal 4: Discuss following model input/output for RRM measurement prediction:

Model input:

- Measured RSRP for SSB/CSI-RS (FFS L1 or L3 measurement)

- Pattern information:

- Time instances

- SetA/setB pattern, i.e., Measured serving/neighbor cells vs predicted intra/inter-frequency neighbor cells

Model output:

- Predicted L3 RSRP

  - For F time instances

  - For intra/inter-frequency Top-K neighbor cells

- Confidence/probability information (FFS)

Proposal 5: For UE side model inference, current RRC signaling should be used to configure model input and output, measurement report message can be extended to support the prediction reporting. For NW side model inference, current measurement configuration and reporting should be used as the baseline and FFS other essential assistant information.

Proposal 6: For UE side model monitoring, further study the following aspects:

- Performance metric and reporting criteria should be configured by network.

- In order to reduce the reporting overhead, event-based model monitoring reporting should be adopted.

Proposal 7: For network side monitoring, current L3 measurement reporting from UE can be reused as assistant information to help network to evaluate the performance. Other additional information like timestamp could be studied further.

4. References

[1]	RP-234055 New WID: Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR

[2]   	R2-2404002 Report of 3GPP TSG RAN WG2 meeting #125bis