3GPP TSG-RAN WG2 Meeting #126                                      R2-2404613

Fukuoka, Japan, May 20th – 24th, 2024

Source:         ZTE Corporation

Title:           Discussion on remaining corrections in LPP

Agenda item:    7.2.4

Document for:	Discussion and Decision

- Introduction

This discussion paper discusses some remaining issues in Rel-18 LPP specification.

- Discussion

RAN1 has sent reply LS R1-2403636[1] including the following answers to RAN2’s questions:

Question 2: Is there only one dl-PRS-ID or are there multiple dl-PRS-IDs associated with the aggregated main and additional measurement, respectively? RAN1’s Response: In RAN1’s understanding, if nr-DL-PRS-ResourceSetID uniquely identifies the DL-PRS Resource Sets across all the multiple dl-PRS-IDs of the same TRP, then only one dl-PRS-ID is needed to be associated with the aggregated main measurement and additional measurements. Otherwise, multiple dl-PRS-IDs are associated with the aggregated main and additional measurement in the measurement report. Question 3: If there are multiple dl-PRS-IDs associated with main and additional measurements, respectively, should the list of the dl-PRS-IDs in additional measurements be included in the list of dl-PRS-IDs in the main measurement? RAN1’s Response: There is no need to provide dl-PRS-ID(s) in additional measurement reporting since both main measurement and additional measurement are derived from the same TRP.

In 37.355, the PRS resource set ID has the following field description:

- – NR-DL-PRS-ResourceSetID The IE NR-DL-PRS-ResourceSetID defines the identity of a DL-PRS Resource Set of a TRP. -- ASN1START NR-DL-PRS-ResourceSetID-r16 ::= INTEGER (0..nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16) -- ASN1STOP nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16 INTEGER ::= 7 nr-DL-PRS-ResourceSetID This field specifies the DL-PRS Resource Set ID, which is used to identify the DL-PRS Resource Set of the TRP across all the frequency layers..

Based on above it can be seen that, the value range of set ID is from 0 to 7 (up to 4 PFLs per TRP, up to 2 PRS resource sets per TRP per PFL), and the above field description means the nr-DL-PRS-ResourceSetID is uniquely recognized inside one TRP, even if the TRP has more than one dl-PRS-IDs.

Observation 1: The nr-DL-PRS-ResourceSetID is uniquely recognized inside one TRP, even if the TRP has more than one dl-PRS-IDs.

According to RAN1’s answer, if nr-DL-PRS-ResourceSetID uniquely identifies the DL-PRS Resource Sets across all the multiple dl-PRS-IDs of the same TRP, there is no need to report multiplr dl-PRS-IDs, the mandatory field dl-PRS-ID-r16 can be used directly. So, there is no need to include additional dl-PRS-ID in the NR-AggregatedDL-PRS-ResourceSetID-Element.

Furthermore in Q3, additional measurement does not need to report dl-PRS-ID because the additional measurement should have same dl-PRS-ID as the main measurement, i.e., the mandatory field dl-PRS-ID-r16 in the MeasElement.

- Proposal 1: According to RAN1’s LS and the observation 1, RAN2 should delete dl-PRS-ID in the NR-AggregatedDL-PRS-ResourceSetID-Element.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: The nr-DL-PRS-ResourceSetID is uniquely recognized inside one TRP, even if the TRP has more than one dl-PRS-IDs.

- Proposal 1: According to RAN1’s LS and the observation 1, RAN2 should delete dl-PRS-ID in the NR-AggregatedDL-PRS-ResourceSetID-Element.

- References

- R1-2403636, Reply LS on questions on RAN1 parameter list, RAN1, 2024-04