3GPP TSG-RAN WG2 Meeting #126	R2-2404701

Fukuoka, Japan, May 20 - 24 , 2024

Source: 			Qualcomm Incorporated, Ericsson, ZTE Corporation, Sanechips

Title: 	Discussion on UE capabilities for two PUCCH groups

Document for: 	Decision

Agenda Item: 	5.1.3.2

# Introduction

RAN1 originally indicated in their feature list that twoPUCCH-Group (FG6-7) is prerequisite of diffNumerologyAcrossPUCCH-Group (FG6-8). This is captured in TR38.822 as follows.

|  | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] |
| --- | --- | --- | --- | --- | --- |
| 6-7 | Two NR PUCCH group with same numerology | 1) For NR CA UE, same numerology across NR carriers for data/control channel at a given time 2) For EN-DC UE, same numerology across NR carriers for data/control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2 | 6-5, 6-6 | twoPUCCH-Group | FeatureSetUplink |
| 6-8 | Different numerology across NR PUCCH groups | For both NR CA UE and EN-DC UE, different numerology between two NR PUCCH groups for data/control channel at a given time | 6-5, 6-7 | diffNumerologyAcrossPUCCH-Group | CA-ParametersNR |

However, TS38.306 currently does not capture such requirement (see below). In this document we discuss how to deal with this gap.

| Definitions for parameters | Per |
| --- | --- |
| twoPUCCH-Group Indicates whether two PUCCH group in CA with a same numerology across CCs for data and control channel [at a given time] is supported by the UE. For NR CA, two PUCCH group is supported with the same numerology across NR carriers for data and control channel at a given time. For (NG)EN-DC/NE-DC, two PUCCH group is supported with the same numerology across NR carriers for data and control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2. The UE supports two PUCCH groups with PUCCH on a band X and a band Y if it sets this capability parameter for both band X and band Y. | FS |
| diffNumerologyAcrossPUCCH-Group Indicates whether different numerology across two NR PUCCH groups for data and control channel at a given time in NR CA and (NG)EN-DC/NE-DC is supported by the UE. | BC |

# Discussion

It should be noted that for the feature of two PUCCH groups with different numerologies (diffNumerologyAcrossPUCCH-Group) to work, the support of two PUCCH groups with the same numerology (twoPUCCH-Group) is not required.

RAN1’s original intent might have been that twoPUCCH-Group is “easier” to support and therefore the UE supporting diffNumerologyAcrossPUCCH-Group should also support twoPUCCH-Group.

However, we observe such decision lacks considerations on practical use cases and corresponding deployment of the features.

  - Band combination with different numerologies among bands

It is possible that the UE supports a band combination where only different numerologies are supported among the bands. The UE may support two PUCCH groups with different numerologies (diffNumerologyAcrossPUCCH-Group) in the band combination. It is quite counterintuitive to report the capability of two PUCCH groups with the same numerology (twoPUCCH-Group) for such band combination. Even if reported, the reported UE capability does not carry any meaning.

  - IOT availability

Two PUCCH group with different numerologies (diffNumerologyAcrossPUCCH-Group) has clear benefit that HARQ timeline is maintained per the slot timing of each subcarrier spacing. This for example can be seen in FR1 + FR2 CA, where only different numerologies make sense essentially. Two PUCCH groups with the same numerology (twoPUCCH-Group) does not share the same benefit/goal.

It is our understanding from the field that the features of two PUCCH groups with different numerologies (diffNumerologyAcrossPUCCH-Group) and two PUCCH groups with the same numerology (twoPUCCH-Group) will go through independent phases of implementation, IOT and deployment, because those features have somewhat different benefits.

As another example is FR1-FDD +FR2-TDD CA band combination. The UE may support the same and different numerologies between FDD and TDD bands. Now one wants to deploy two PUCCH groups with different numerologies (diffNumerologyAcrossPUCCH-Group) for this band combination, the lack of IOT opportunity for two PUCCH groups with the same numerology (twoPUCCH-Group) should not hinder such legitimate deployment.

  - Backward compatibility

As shown in the previous section, it is not very straightforward to mandate the support of twoPUCCH-Group for the UE supporting diffNumerologyAcrossPUCCH-Group. As the current TS38.306 stands, it is not sensible to assume all existing UE implementations are according to the information captured in TR38.822. RAN2 should at least allow the UE implementation according to the current Technical Specification.

Observation:	Mandating the support of twoPUCCH-Group for the UE supporting diffNumerologyAcrossPUCCH-Group in Technical Specification invites a lot of issues, without a compelling benefit.

Proposal:	RAN2 to make the explicit decision not to specify that twoPUCCH-Group (FG6-7) is prerequisite of diffNumerologyAcrossPUCCH-Group (FG6-8).

# Proposal

Based on the discussion in this document, we made the following observation and proposals.

Observation:	Mandating the support of twoPUCCH-Group for the UE supporting diffNumerologyAcrossPUCCH-Group in Technical Specification invites a lot of issues, without a compelling benefit.

Proposal:	RAN2 to make the explicit decision not to specify that twoPUCCH-Group (FG6-7) is prerequisite of diffNumerologyAcrossPUCCH-Group (FG6-8).