3GPP TSG-RAN WG2 Meeting #126	R2-2404765

# Fukuoka, Japan, 20th - 24th May., 2024

Title:		Remaining issues for R18 POS stage2 TS 38.300

Source:	Huawei, HiSilicon, vivo

Agenda item:	7.2.6

Document for:	Discussion and Decision

- Background

This paper discusses the remaining RAN2 issues in the stage2 spec TS 38.300 for R18 positioning

- Discussions

In the current RAN stage2 spec, the following aspects are missing for the description of SL positioning in R18:

- Resource allocation scehems for SL positionig, included scheme1 and scheme2

- Identities for scheduling in dedicated SL-PRS resource pool

Hence, we propose to add the following descriptions:

Proposal1: Add the following descriptions for RAN stage2 spec for SL positioning, Adopt the TP in Annex A

- Resource allocation scehems for SL positionig, included scheme1 and scheme2

- Identities for scheduling in dedicated SL-PRS resource pool

- Conclusions:

We propose the following for the description in TS 38.300

Proposal1: Add the following descriptions for RAN stage2 spec for SL positioning, Adopt the TP in Annex A

- Resource allocation scehems for SL positionig, included scheme1 and scheme2

- Identities for scheduling in dedicated SL-PRS resource pool

- Annex: Text Proposal

3GPP TSG-RAN WG2 Meeting #126	R2-240

# Fukuoka, Japan, 20th - 24th May., 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38300 for SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1/ added resource allocation scehems for SL positionig, included scheme1 and scheme2 2/ added identities for scheduling in dedicated SL-PRS resource pool |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are made per above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage2 description for SL positioning will not be accurate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2, 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===================================CHANGE BEGIN===================================

5	Physical Layer

5.7	Sidelink

5.7.1	General

Sidelink supports UE-to-UE direct communication using the sidelink resource allocation modes, physical-layer signals/channels, and physical layer procedures below.

5.7.2	Sidelink resource allocation modes/schemes

Two sidelink resource allocation modes are supported for sidelink communications: mode 1 and mode 2. Two sidelink resource allocation schemes are supported for SL-PRS transmission: scheme 1 and scheme 2. In mode/scheme 1, the sidelink resource allocation is provided by the network. In mode/scheme 2, UE decides the SL transmission resources in the resource pool(s).

5.7.3	Physical sidelink channels and signals

Physical Sidelink Control Channel (PSCCH) indicates resource and other transmission parameters used by a UE for PSSCH. PSCCH transmission is associated with a DM-RS.

Physical Sidelink Shared Channel (PSSCH) transmits the TBs of data themselves, and control information for HARQ procedures and CSI feedback triggers, etc. At least 6 OFDM symbols within a slot are used for PSSCH transmission. PSSCH transmission is associated with a DM-RS and may be associated with a PT-RS.

Physical Sidelink Feedback Channel (PSFCH) carries HARQ feedback over the sidelink from a UE which is an intended recipient of a PSSCH transmission to the UE which performed the transmission. PSFCH sequence is transmitted in one PRB repeated over two OFDM symbols near the end of the sidelink resource in a slot.

The Sidelink synchronization signal consists of sidelink primary and sidelink secondary synchronization signals (S-PSS, S-SSS), each occupying 2 symbols and 127 subcarriers. Physical Sidelink Broadcast Channel (PSBCH) occupies 9 and 7 symbols for normal and extended CP cases respectively, including the associated DM-RS.

5.7.5	Physical layer measurement definition

For measurement on the sidelink, the following UE measurement quantities are supported:

-	PSBCH reference signal received power (PSBCH RSRP);

-	PSSCH reference signal received power (PSSCH-RSRP);

-	PSСCH reference signal received power (PSCCH-RSRP);

-	Sidelink received signal strength indicator (SL RSSI);

-	Sidelink channel occupancy ratio (SL CR);

-	Sidelink channel busy ratio (SL CBR).

5.7.6	Sidelink Reference Signal and Measurements for Positioning

The Sidelink Positioning Reference Signal (SL-PRS) is defined to facilitate Ranging/Sidelink positioning methods, such as SL-TOA, SL-TDOA, SL-RTT, SL-AoA, through the following set of measurements SL-PRS RSRP, RSRPP, RTOA, RSTD, Rx-Tx timing difference and AoA as described in TS 38.305 [42].

=================================NEXT CHANGE=======================================

8	NG Identities

8.4	NR sidelink communication, V2X sidelink communication and Ranging/Sidelink positioning related identities

The following identities are used for NR sidelink communication and SL-PRS transmission on shared SL-PRS resource pool:

-	Source Layer-2 ID: Identifies the sender of the data in NR sidelink communication and/or SL-PRS transmission. The Source Layer-2 ID is 24 bits long and is split in the MAC layer into two bit strings:

-	One bit string is the LSB part (8 bits) of Source Layer-2 ID and forwarded to physical layer of the sender. This identifies the source of the intended data and/or SL-PRS in sidelink control information and is used for filtering of packets at the physical layer of the receiver;

-	Second bit string is the MSB part (16 bits) of the Source Layer-2 ID and is carried within the MAC header. This is used for filtering of packets at the MAC layer of the receiver.

-	Destination Layer-2 ID: Identifies the target of the data in NR sidelink communication and/or SL-PRS transmission. For NR sidelink communication and SL-PRS transmission on SL-PRS shared resource pool, the Destination Layer-2 ID is 24 bits long and is split in the MAC layer into two bit strings:

-	One bit string is the LSB part (16 bits) of Destination Layer-2 ID and forwarded to physical layer of the sender. This identifies the target of the intended data and/or SL-PRS in sidelink control information and is used for filtering of packets at the physical layer of the receiver;

-	Second bit string is the MSB part (8 bits) of the Destination Layer-2 ID and is carried within the MAC header. This is used for filtering of packets at the MAC layer of the receiver.

-	PC5 Link Identifier: Uniquely identifies the PC5 unicast link in a UE for the lifetime of the PC5 unicast link as specified in TS 23.287 [40]. The PC5 Link Identifier is used to indicate to upper layers the PC5 unicast link in which sidelink RLF was declared and corresponding PC5-RRC connection was released.

V2X sidelink communication related identities are specified in clause 8.3 of TS 36.300 [2].

The following identities are used for SL-PRS transmission on dedicated SL-PRS resource pool:

-	Source ID: Identifies the sender of the SL-PRS transmission. The length of the identifier is either 12 or 24 bits determined by RRC layer configuration. The source ID identifies the source of the intended SL-PRS in sidelink control information and is used for filtering of packets at the MAC layer of the receiver;

-	Destination ID: Identifies the target of the SL-PRS transmission. The length of the identifier is 24 bits. The Destination ID identifies the target of the intended SL-PRS in sidelink control information and is used for filtering of packets at the MAC layer of the receiver.

=======================================CHANGE ENDS==============================