3GPP TSG-RAN WG2 Meeting #126	R2-2404801

Fukuoka, Japan May 20th – 24th, 2024

Agenda item:		8.9.2

Source:		Lenovo

Title:		Store and Forward support in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2#125bis meeting made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

- S&F implies that at least the full eNB will be onboard. - An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this). - The S&F satellite operation is common for NB-IoT and eMTC. - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture).

In this contribution we discuss the requirements of S&F and analyze the potential impacts on Uu to support S&F operations in IoT NTN.

- Discussion

S&F operation is an operation mode of a 5G system with satellite-access, where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g., to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment. Although the principles and some architectural details of S&F operation are still under SA2 discussion, RAN2 have also made some initial progress in the latest RAN2#125bis meeting and is expected to discuss further on possible Uu impacts.

## Indication of S&F operation

To fulfill some of the fundamental requirements consolidated by SA2, RAN2 have agreed that “an IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied”, and whether it is via NAS or AS has not been decided. In our understanding the indication via NAS or AS is not mutually exclusive to each other. Even if a UE can be indicated via NAS that S&F is applied in the current IoT NTN, some cell level information is still necessary for the UE, considering that the gNBs on-board satellites that can support S&F operations have some characteristics including store capability and accessibility to CN entity in time. As a result, the source or target UE of an S&F operation needs to be aware that the current cell supports S&F delivery or configuration, either for determination of initiating S&F data transmission or for configuration of transmitting/receiving S&F data/response with large delay.

Observation 1: The indication of S&F operation (e.g., capability or applicability) at cell level to a UE is necessary from RAN perspective.

Therefore, we think it can be a starting point to discuss this in RAN2.

Proposal 1: RAN2 to discuss a per-cell indication of S&F operation so that UE can determine whether it can transmit or receive for S&F services in an NTN cell.

## Cell barring for S&F operation

Another discussion from RAN2 perspective is that whether a UE of previous releases (e.g., Rel-17 & Rel-18) or a UE incapable of S&F operation shall be barred to access an NTN cell that supports S&F operation. In our understanding, firstly the general access control strategy for S&F operation has not been decided, neither the UE capability design for S&F operation. It is too early to decide introducing cell barring mechanism for S&F operation. Besides, according to the TR23.700-29 last updated, it seems that SA2 may not preclude Rel-17 & Rel-18 UEs in S&F operations:

3GPP TR23.700-29 v0.5.0 7.2 Key Issue #2 Principles for evaluation It is proposed to evaluate the support of Store & Forward Satellite operation (key issue#2) following the principles below: 1. Support of single and multi-satellites. 2. Support of Roaming (the definition of roaming in S&F Satellite operation shall be clarified). 3. Support of MO/MT SMS service, CIoT CP/UP. 4. UE location verification 5. Security considerations (in coordination with SA3) 6. UE power consumption 7. Delay for a UE to complete the attach procedure and for a registered UE to access service 8. Support of Legacy UEs (Rel-17 & Rel-18 UEs) 9. Minimal impact on the existing 3GPP procedures 10. Whether Proprietary NFs are needed to support S&F 11. Compute and storage requirements on satellite considering limitation of satellite payload, which can be estimated based on NFs deployed on the satellite. 12. Management of the transition between phases where feeder link is available or not.

Observation 2: The access control strategy and UE capability for S&F operation are under discussion and SA2 is considering to support Rel-17 & Rel-18 UEs as well.

Therefore we think for S&F cell barring it is too early to make further decisions in RAN2.

Proposal 2: Whether to support cell barring for S&F is pending on SA2 progress.

## Uu information exchange for S&F operation

Referring to SA2 works as in TR22.865, at least the following scenarios or use cases can be considered for S&F operations: MO, MT, data transfer for IoT devices in remote areas, and emergency report. The main different between S&F operation and normal operation in NTN is whether there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network i.e., whether it requires the service and feeder links to be active simultaneously. Such differences have brought in some new requirements as consolidated by SA2:

- support S&F operation for authorized UEs;

- support to inform a UE whether S&F Satellite operation is applied;

- support to allow the operator or a trusted 3rd party to apply an S&F data retention period;

- support to allow the operator or a trusted 3rd party to apply an S&F data storage quota;

- support a mechanism to configure and provision specific required QoS and policies for UE’s data;

- support to provide related information (e.g. estimated delivery time to the authorised 3rd party) to an authorized UE.

- support to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g. estimated delivery time to the authorised UE).

- support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

- support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available.

- support mechanisms for a UE to register with the network when the network is in S&F Satellite operation.

- support mechanisms to authorize subscribers for receiving services when the network is in S&F Satellite operation.

Observation 3: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

According to the listed cases and requirements, the services that can leverage S&F operations have some specific characteristics including delay-tolerant (could be hours) and remote (source or target could be reached only be limited satellite coverage), and the UE may need some availability information about the S&F operation supported by the network, e.g., for which area or for how long the data can be delivered, or some functional information about the S&F operation supported by the network, e.g., support of providing delivery status, support of cast types for delivery (unicast, multicast and broadcast), support of eNB/gNB-only S&F (no CN entity is involved during the S&F operation).

Observation 4: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

On the other hand, network may also need to be aware of UE requirements for S&F operation, e.g., to configure UE for S&F data transmission/reception or to understand for which area or for how long the data shall be delivered, or whether providing delivery status is needed, or which cast types for delivery is expected.

Observation 5: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

When both the network and UE supports S&F operation, the UE needs to determine when an S&F operation service can be initiated e.g., whether a service can leverage S&F operation. From protocol design perspective, the AS-NAS functional spilt in UE for S&F shall be specified, e.g., which stratum is responsible for the initiation and the other stratum can provide assistances. Moreover, AS-NAS interaction could be needed to ensure appropriate S&F initiation and data delivery, e.g., start/suspend NAS->AS S&F data delivery when satellite coverage is valid/invalid to avoid congestion.

Observation 6: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

Based on the above analysis, the following aspects can also be considered for RAN2’s study on supporting S&F operations:

Proposal 3: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).

- Conclusion

In this contribution we discuss the requirements of S&F and analyze the potential impacts on Uu to support S&F operations in IoT NTN. The following observations are given:

Observation 1: The indication of S&F operation (e.g., capability or applicability) at cell level to a UE is necessary from RAN perspective.

Observation 2: The access control strategy and UE capability for S&F operation are under discussion and SA2 is considering to support Rel-17 & Rel-18 UEs as well.

Observation 3: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

Observation 4: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

Observation 5: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

Observation 6: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

And it is proposed that:

Proposal 1: RAN2 to discuss a per-cell indication of S&F operation so that UE can determine whether it can transmit or receive for S&F services in an NTN cell.

Proposal 2: Whether to support cell barring for S&F is pending on SA2 progress.

Proposal 3: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).