3GPP TSG-RAN WG2 Meeting #126	                     		      R2-2404871

Fukuoka, Japan May 22nd – 26th, 2024

Source:	OPPO

Title:	Discussion on UE capability on bandwidth aggregation positioning

Agenda Item:	7.2.4

Document for:	Discussion and Decision

1. Discussions

It is well known that the UE in the RRC_Inactive state and RRC_Idle state cannot be configured with measurement gap(MG).

But in the current spec NR-DL-PRS-ProcessingCapability field description, following description about MG for RRC_INACTIVE AND RRC_IDLE state has been captured:

prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive

Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_INACTIVE and RRC_IDLE state.

The UE can include this field only if the UE supports dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive and maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters:

-	maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR1, which is supported and reported by UE.

-	maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE.

-	maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL.

-	maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL.

-	dl-PRS-BufferTypeOfBWA: Indicates the DL PRS buffering capability.

-	prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE.

-	prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms.

-	prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms.

-	maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1.

-	maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2.

NOTE 20:	dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType.

NOTE 21:	The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms.

NOTE 22:	Each two linked PRS resources are counted as 1 resource

NOTE 23:	maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive.

and

prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive

Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_INACTIVE and RRC_IDLE state. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters:

-	maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz of for FR1, which is supported and reported by UE.

-	maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE.

-	maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL

-	maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL

-	dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability.

-	prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE.

-	prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms.

-	prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms.

-	maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1.

-	maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2.

NOTE 24:	dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType.

NOTE 25:	The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms.

NOTE 26:	Each two linked PRS resources are counted as 1 resource

NOTE 27:	maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive.

Therefore, we propose to delete such words from the current spec.

Observation 1: the UE in the RRC_Inactive state and RRC_Idle state cannot be configured with measurement gap(MG).

Proposal 1: RAN2 to agree to delete the words “within a MG” from both of the IE names and the field descriptions for the IEs prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive and prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive.

3 Conclusions

In this contribution, following observation and proposal are provided:

Observation 1: the UE in the RRC_Inactive state and RRC_Idle state cannot be configured with measurement gap(MG).

Proposal 1: RAN2 to agree to delete the words “within a MG” from both of the IE names and the field descriptions for the IEs prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive and prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive.