3GPP TSG-RAN WG2 Meeting #126	R2-2405048

Fukuoka, Japan, May 22nd – 26th, 2024

Agenda Item:	7.5.3.2

Source:	OPPO

Title:	Discussion on spec impact on SDAP PDU set remapping

Document for:	Discussion, Decision

Introduction

This paper will discuss the spec impact due to the SDAP PDU set remapping.

Discussion

RAN2#125 discusses the case where some PDUs within a PDU set of a QoS flow are mapped to one DRB while the remaining PDUs within the same PDU set are mapped to another DRB following the new QoS flow to DRB mapping rule. The conclusion drawn by RAN2 is that this remapping only occurs at the boundaries of the PDU set.

QoS flow remmaping R2-2400891 Clarification on PDU Set discard handling when QoS flow remapping ASUSTeK Proposal 1 When a timer discardTimerForLowImportance for a PDU Set expires on a DRB, remaining PDUs of the same PDU set delivered to another DRB shall be also discarded. - Xiaomi thinks that we should consider this in Rel-19 - Qualcomm thinks that the principle => We can only remap the boundaries of the PDU set => Noted Agreements: 3 We can only remap the boundaries of the PDU set and no specification change is needed

RAN2#125bis re-discusses whether any specification impact is necessary for the above case, as some companies recognised that the current spec may not explicitly support the RAN2#125 agreement. Consequently, RAN2 now tends to capture a NOTE in the spec. If the NOTE is implemented, we understand this note would be reflected in Stage-2 spec since the RAN2 aims to establish a general principle to guide the UE behaviour, which is typically suitable for inclusion in TS 38.300.

SDAP PDU set remapping R2-2402583 Correction on support of remapping PDU Set boundary in SDAP ASUSTeK discussion Rel-18 37.324 NR_XR_enh-Core Proposal 1 When a timer discardTimerForLowImportance for a PDU Set expires on a DRB, remaining PDUs of the same PDU set delivered to another DRB shall be also discarded. Proposal RAN2 captures the text proposal (“NOTE 1: All SDAP SDUs belonging to a PDU Set are mapped to the same DRB.”) in TS 37.324 or the text proposal (“In case of QoS flow remapping, UE maps all PDUs belonging to a PDU Set of a QoS flow to the same DRB”) in TS 38.300. - Nokia thinks it makes sense as a note. - Qualcomm thinks that the agreement in the chair note is sufficient. Lenovo asks if this implies that now the SDAP needs to be aware of the PDU SET. => RAN2 aim to capture the text proposal (“NOTE 1: All SDAP SDUs belonging to a PDU Set are mapped to the same DRB.”). FFS which spec. => Noted

The TS 38.300 is updated with a NOTE that all SDAP SDUs belonging to a PDU Set are mapped to the same DRB.

Conclusion

We have the following proposals:

Proposal 1	The TS 38.300 is updated with a NOTE that all SDAP SDUs belonging to a PDU Set are mapped to the same DRB.

Text Proposal

16.15.4.2.2	Discard

When the PSIHI is set for a QoS flow, as soon as one PDU of a PDU set is known to be lost, the remaining PDUs of that PDU Set can be considered as no longer needed by the application and may be subject to discard operation at the transmitter to free up radio resources.

NOTE 1:	It cannot always be assumed that the remaining PDUs are not useful and can safely be discarded. Also, in case of Forward Error Correction (FEC), active discarding of PDUs when assuming that a large enough number of packets have already been transmitted for FEC to recover without the remaining PDUs is not recommended as it might trigger an increase of FEC packets.

In uplink, the UE may be configured with PDU Set based discard operation for a specific DRB. When configured, the UE discards all packets in a PDU set when one PDU belonging to this PDU set is discarded due to discard timer expiry.

The gNB may perform downlink PDU Set discarding based on implementation by taking at least PSDB, PSI, PSIHI parameters into account.

In case of congestion, the gNB may use the PSI for PDU set discarding. For uplink, dedicated downlink signalling is used to request the UE to apply a shorter discard timer to low importance SDUs in PDCP.

NOTE 2:	How SDUs are identified as low importance is left up to UE implementation. When a PSI is available, it can be used to classify the PDCP SDUs of a PDU Set according to the guidelines specified in TS 26.522 [58].

NOTE 3:	All SDAP SDUs belonging to a PDU Set are mapped to the same DRB.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)