3GPP TSG-RAN WG2 Meeting #126                                    R2-2405641

Fukuoka, Japan May 20th – 26th, 2024

Agenda item:		7.17.2

Source:				Samsung

Title:					Discussion on PSCell release for MUSIM operation

Document for:		Discussion & Decision

# 1	Introduction

This contribution discusses how to indicate UE's preference on release of PSCell for MUSIM operation.

# 2	Discussion

In RAN2#125 meeting it was agreed that if UE has a preference for PSCell to be released for temporary capability restriction, its preference can be indicated by the musim-CellToRelease in the UEAssistanceInformation message. Hence, the following changes are currently implemented to reflect previous RAN2 agrement in TS 38.331 (see only relevant parts).

1> if transmission of the UEAssistanceInformation message is initiated to provide musim-CapRestriction according to 5.7.4.2 or 5.3.5.3: 2> if UE has a preference for temporary capability restriction: 3> if UE has a preference for serving cell(s), except Pcell, and/or SCG to be released: 4> include the musim-Cell-SCG-ToRelease; 5> set musim-CellToRelease to include the serving cell(s) the UE prefers to be released;

Observation#1: RAN2 agreed to support that UE can indicate its preference on the temporary capability restriction on the PSCell to release for MUSIM operation.

After review, there was discussion whether an PSCell can be indicated by the IE SCellIndex in Rel-16 and it was concluded that it can not be used to identify the PSCell [1].

Observation#2: In the description of the IE SCellIndex, PSCell was deleted to clarify that it cannot identify an PSCell in Rel-16.

According to the following ASN.1 of the IE MUSIM-CellToRelease, the IE SCellIndex is used to indicate any serving cell(s) except PCell, which should be changed to the IE ServCellIndex.

MUSIM-CellToRelease-r18 ::=             SEQUENCE (SIZE (1..maxNrofServingCells)) OF SCellIndex

Proposal #1: In the ASN.1 of the IE MUSIM-CellToRelease, change "SCellIndex" into "ServCellIndex".

# 3	Conclusion

In section 2, the following observations are made:

Observation#1: RAN2 agreed to support that UE can indicate its preference on the temporary capability restriction on the PSCell to release for MUSIM operation.

Observation#2: In the description of the IE SCellIndex, PSCell was deleted to clarify that it cannot identify an PSCell in Rel-16.

Based on above, RAN2 is requested to discuss and agree on the following proposal:

Proposal #1: In the ASN.1 of the IE MUSIM-CellToRelease, change "SCellIndex" into "ServCellIndex".

# 4	Reference

[1] R2-2106728	Clarification on SCellIndex and servCellIndex	NTT DOCOMO