3GPP TSG RAN WG 2 Meeting #126	R2-2405948

Fukuoka, Japan, 20th - 24th May, 2024

Source: 	Fujitsu

Title: 	Summary of [AT126][501][R18MobE] Power Control Parameters after LTM cell switch (Fujitsu)

Agenda Item:	7.4.1

Document for:	Discussion and decision

1	Introduction

This document is a summary of:

[AT126][501][R18MobE] Power Control Parameters after LTM cell switch (Fujitsu)

Scope: Sort out the usage of outer config vs candidate config, establish R2 view.

Intended outcome: Report

Deadline: CB, see schedule

2	Background

In the LS from RAN1 [1], it was pointed out that the power control parameters were missing in current ASN.1. To solve this issue, RAN1 proposes two approaches.

Approach 1: Capture the RAN1 agreement on the linkage between TCI states for candidate cell(s) and those for target cell(s) in RAN2 specification(s).

Approach 2: Introduce the new RRC parameters under LTM-TCI-Info.

RAN2 has discussed which way we would adopt in the [Post125bis][519][R18 Mob] discussion, but no consensus has been made.

3	Discussion

The rapporteur proposes the following way forward:

RAN2 should discuss to select either Approach 1 or Approach 2.

However, if RAN2 cannot select, RAN2 should find compromised solution, e.g., both Approaches are adopted.

[CATT, Xiaomi] Approach 1 is simple and preferred. [MTK] doesn’t think Approach 2 is necessary.

[LGE, ZTE] Approach 2 is simple and preferred. [Ericsson,]Approach 2 can cover both approaches.

[RAN1] Without linkage, the system will work.

=> RAN2 adopts Approach 2.

4	Conclusion

Proposal: RAN2 to adopt Approach 2, i.e., introduce new RRC parameters under LTM-TCI-Info.

5	References

- R2-2404115/R1-2403683, " LS on the identification of the power control parameters after LTM cell switch"