3GPP TSG-RAN WG2 Meeting #127 	   R2-2406857

# Maastricht, Netherlands, Aug 19th – 23th, 2024

Source:	TCL

Title:	discussion on RLC AM enhancement

Agenda item:	8.7.5

Document for:	Discussion and decision

- Introduction

In RAN2 #126 meeting, we have discussed RLC AM retransmission enhancement for the PDUs with small delay budget extensively and reached the following agreements:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this contribution we will discuss the details of the approaches to avoid the unnecessary retransmission and to timely retransmit the PDUs with small delay budget.

- Discussion

  - Unnecessary retransmissions

When the transmission delay of a PDU exceeds the corresponding PDB or PSDB, even if the PDU or segments has (have) been sent to the lower layer, the PDU or segments should be discarded and the retransmissions should be stopped to avoid unnecessary retransmissions. According to the agreement, the TX initiated approach and the RX initiated approach are further discussed and compared in detail in the following table.

| Additional impact on standard compared to the legacy approach | Tx initiated approach | Rx initiated approach |
| --- | --- | --- |
| Additional Tx operation | - Identify and discard the obsolete PDUs or segments - notifies the Rx side about the SN gap | No |
| Additional Tx operation | updates state variables according to the SN gap from Tx side | - Identify the obsolete PDUs or segments and mark them - Send a status report and updates state variables as legacy approach - Discard the received PDUs or segments or delivery them to higher layer |
| How to identify the obsolete PDUs or segments | Identify obsolete PDUs or segments based on a configured threshold and the remaining time | FFS. One potential solution is to identify the obsolete PDUs or segments based on an additional timer and a configured threshold |
| Is there additional delay | - Yes. the transmission of the control PDU for SN gap could introduce additional delay | No |

According to the comparison, it is obvious that the Rx initiated approach is simpler and less impacts on the current specification. However, it is not clear how to determine the obsolete PDUs or segments in the receiving side. Based on the periodic and real-time characteristic of XR service, a potential solution is a status report scheme based on the periodicity and the PSDB or PDB for the PDU Set or PDU at the RLC AM receiving side as illustrated in Figure 1. A periodic deadline to receive PDU Set or PDU could be configured based on the periodicity, burst arrival time and PSDB or PDB, and one or more occasions for status reports can be configured before each deadlines.

Figure 1

Another solution is that RLC AM receiving side can determine the obsolete PDUs or segments based on the SN information of the discarded PDCP PDU information in the PDCP status report from the transmitting side [2]. However this solution is only applicable for the case where PDCP SN is mapped to RLC SN one by one and RLC AM receiving side should be enhanced to establish and maintain the relationship between PDCP SN.

Observation 1: Rx initiated approach is simpler and less impacts on the current specification than Tx initiated approach.

Proposal 1: It is suggested RAN2 to choose the Rx initiated approach as the baseline to further discuss the solution for Unnecessary retransmissions.

  - Timely RLC transmission and retransmission

    - Prioritized the initial transmission of the PDUs with small delay budget

Except timely retransmission of the PDUs with small delay budget, it’s obviously that the transmitting side of an AM RLC entity shall also transmit timely the initial transmission of the those PDUs.

One of the potential solutions to ensure the timely initial transmission for the SDU with small delay budget is to prioritize the initial transmission of the PDUs for those SDUs. With the prioritized initial transmission mechanism , SDUs with a small delay budget could have more time and more opportunities for retransmission, which is beneficial not only for the timely RLC retransmission(s) but also for the transmission reliability.

However in current TS 38.322, the AM RLC transmitting entity only supports to prioritize the transmission of control PDUs and retransmitted PDUs. So we propose to prioritize the initial transmission of the PDUs with small delay budget.

Proposal 2: It is suggested RAN2 to discuss the prioritization of initial transmission of the PDUs with small delay budget.

    - autonomous retransmission

As is well known, because RLC AM transmitting side cannot determine the failed transmission parts based on status information, blind retransmission leads to unnecessary repeated transmissions, resulting in a significant decline in transmission efficiency. Moreover, the better the wireless channel conditions, the greater the efficiency drop.

Additionally, according to the existing RLC AM standard, the transmitting side can autonomously decide to perform blind retransmission of SDUs or segments without affecting the operation of the receiving side. Therefore, blind retransmission is an implementation issue and does not require any enhancements to the standard. The decision of when or how to perform blind retransmission is implemented internally by the transmitting side.

Proposal 3: It is suggested RAN2 not to discuss the autonomous retransmission of the PDUs with small delay budget.

    - Timely retransmission

According to the previous agreement, there are two approaches for timely retransmission, one is based on timely status report in RLC AM receiving side, another is based on timely polling in RLC AM transmitting side.

For the transmitting side scheme of timely polling in RLC AM transmitting side, a straightforward solution is to reuse the legacy t-PollRetransmit timer based mechanism with the enhancement to dynamically employ the expiration value according to the SDU's delay budget.

For the receiving side scheme of the timely status report in RLC AM receiving side, a potential solution is reuse the solution for unnecessary retransmission illustrated in Figure 1. More often status report could be needed to ensure that each missed PDU in a PDU Set has a timely chance for retransmission.

Similar as the comparison in 2.1, the receiving side scheme is simpler and less impact on the current specification than the transmitting side scheme.

Proposal 4: It is suggested RAN2 to further discuss solutions for timely retransmission based on the Rx initiated approach for Unnecessary retransmissions.

- Conclusion

In this contribution, we discussed the solutions for RLC AM retransmission enhancement and provided our proposals as follows:

Observation 1: Rx initiated approach is simpler and less impacts on the current specification than Tx initiated approach.

Proposal 1: It is suggested RAN2 to choose the Rx initiated approach as the baseline to further discuss the solution for Unnecessary retransmissions.

Proposal 2: It is suggested RAN2 to discuss the prioritization of initial transmission of the PDUs with small delay budget.

Proposal 3: It is suggested RAN2 not to discuss the autonomous retransmission of the PDUs with small delay budget.

Proposal 4: It is suggested RAN2 to further discuss solutions for timely retransmission based on the Rx initiated approach for Unnecessary retransmissions.

- References

- RP-234057: "New WID: XR (eXtended Reality) for NR Phase 3".

- R2-2404813	AM RLC Enhancement, Lenovo